STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the IBM LAN server

driver.

The DOS LAN requester driver, 8008NDIS.DOS, and the network-

information file, SMCDOS.NIF, are located in the \NDIS subdirectory

on the SuperDisk.

Use these instructions with the instructions in the chapter

entitled "LAN Adapter Protocol Support Configuration," in the IBM

OS/2 LAN Server Network Administrator Reference Volume 1: Planning

and Installation manual.

Note: This procedure applies to OS/2 LAN Server and OS/2 LAN

Requester installations only.

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

4. Select the Install button.

5. Specify the target drive (usually C:\).

6. Select the Install button.

7. Type: <drive>:\NDIS

In the above line, <drive> is the floppy drive

containing the diskette with the \NDIS subdirectory.

8. Press Enter.

9. For each driver that is found, press Enter at the prompt.

10. Select the Configure button.

11. Select "Configure LAN Transports" followed by Continue.

The Configure Workstation screen appears.

12. From the Network Adapter window, select the EtherNote

ElitePort adapter followed by Add. The adapter name

appears in the Current Configuration window.

13. From the Protocols window, select "IBM IEEE 802.3" unless

it already appears below the adapter name (you may also

need to select "IBM NetWare Requester", depending on your

network).

14. Select Add so the protocol appears in the Current

Configuration window below the driver.

15. Select OK followed by Exit.

16. Proceed with the rest of the instructions according to the

LAN Server manual.

Refer to the general installation document (INSTALL.DOC) for

information on PROTOCOL.INI parameters.

EtherCard Elite, EtherNote, ElitePort, and SuperDisk are

trademarks, and SMC and Standard Microsystems are registered

trademarks, of Standard Microsystems Corporation. Other

trademarks mentioned herein belong to their respective

companies.