STANDARD MICROSYSTEMS CORPORATION

Banyan Vines

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  To copy the LAN Manager workstation driver files from the

SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

1. Copy all the files in the SuperDisk directory \NDIS to a

Destination Directory on your PC's hard disk (the recommended

Destination Directory is C:\BANYAN).

2. Refer to the Banyan Vines documentation and to the NDISBAN.DOC

file on the Vines distribution diskette.

3. Copy the file 8008NDIS.DOS from the SuperDisk to the Destination

Directory on your PC's hard disk. (The Destination Directory

should already contain BAN.EXE.)

4. Copy the file PROTOCOL.INI from the SuperDisk to a \LANMAN

subdirectory off the Destination Directory.

PREPARING YOUR OPERATING SYSTEM

1. Using any text editor, add the following lines to your CONFIG.SYS

file:

DEVICE=\BANYAN\PROTMAN.DOS /I:C:\BANYAN

DEVICE=\BANYAN\8008NDIS.DOS

2. Run PCCONFIG (a VINES utility) to configure the PC for the

network:

o  Select "Network Card Settings". Then press the Enter key.

o  For hardware configuration options, select "NDIS Ethernet".

Press Enter.

o  Enter the interrupt that the adapter is using. Press Enter.

o  On the "PROTOCOL.INI bindings =" line, type: SMC8008_NIF

o  Press Enter.

o  Press F10 to save your configuration.

o  Press Esc to exit from the current screen.

4. Reboot your computer.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.