STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v2.0 OS/2 Requester

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the IBM LAN

server version 2.0 OS/2 requester.

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.0 diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the INSTALL button.

5. Specify the target drive (usually C:\) and press Enter or

select OK.

6. Select INSTALL again to load the SMC drivers from the

SuperDisk.

7. At "Install Additional Network Drivers" panel add "NDIS" to

the path name (a:\NDIS).  Press ENTER for all default files

and drivers.

8. Select the Configure button.

9. Select "Configure LAN transports" followed by Continue.

The Configure Workstation screen appears.

10. From the Network Adapters window, select the "SMC Ethernet

Adapter for OS/2" and select the Add button.

This adapter name appears in the Current Configuration window.

11. From the Protocols window, select "IBM IEEE 802.2" followed by

the Add button. Then select "IBM OS/2 NETBIOS" followed by Add.

(You may also need to select the "IBM NetWare Requester" option

and Add, depending on your network.)

Each time you select the Add button, the selected option appears

in the Current Configuration window, below the chosen driver.

12. Select OK.

13. Select Exit.

The CONFIG.SYS Update screen appears.

14. Specify the pathname where the CONFIG.SYS file resides.

15. Select the CONTINUE button.

16. Select OK.

17. Select Exit.

18. Edit the CONFIG.SYS file to include the following statement:

DEVICE=\IBMCOM\MACS\8008NDIS.OS2

19. Change to the C:\IBMCOM directory.

20. Edit the PROTOCOL.INI file as follows:

Driver Name = NDS8008$, e.g. change SMC8000$ to NDS8008$

21. Delete everything under [SMCOS2AT_NIF] except the Driver Name

entry.  Save the file.

22. Copy the 8008NDIS.OS2 file from the floppy disk to

C:\IBMCOM\MACS.  8008NDIS.OS is in the a:\NDIS subdirectory.

23. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

24. Reboot your computer, enter OS/2, and install the OS/2

Requester as per OS/2 manuals.

This completes the installation procedure.

EtherCard PLUS, EtherNote, ElitePort, and SuperDisk are trademarks,

and SMC and Standard Microsystems are registered trademarks, of

Standard Microsystems Corporation. Other trademarks mentioned

herein belong to their respective companies.