STANDARD MICROSYSTEMS CORPORATION

Setup and Diagnostic Program

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use the SETUP.EXE program to configure and test the SMC

Ethernet parallel port LAN adapter.

o  How to print to a network printer from Windows, should you

encounter an error message.

o  How to use the Auto H/W Sensing option

RUNNING THE SETUP PROGRAM

To run the SETUP.EXE program:

1. Insert the SuperDisk in drive A.

2. Type: A:

3. Press the Enter key.

4. Type: SETUP

5. Press Enter.

Note: If you see a series of zeros for the Node ID,

type SETUP/F in step 4 and press Enter.

MENU OPTIONS

When you run SETUP.EXE, you can configure the following hardware

settings for your adapter:

<1> Node ID

<2> Printer Port IRQ

<3> H/W Model

<4> Media Type

<5> Printer Mode

<6> RAM Size

<7> Write Delay Factor

<8> Read Delay Factor

<9> Interrupt Trigger

<10> Auto Detecting Setting

PRINTING TO A NETWORK PRINTER FROM WINDOWS

If you receive an error message when you try to print to a network

printer from Windows:

1. Exit Windows.

2. Run Windows Setup.

3. Look at the Network option. If this option is set to "Novell

NetWare (shell versions 3.26 and above)," select "Change System

Settings" from the Options menu and select "Novell NetWare

(shell versions 3.21 and above)" for the Network option.

4. Exit Windows Setup.

5. Start Windows and try to print again.

USING AUTO H/W SENSING

The Auto H/W Sensing option for Set Configuration allows the driver

to automatically detect your adapter's configuration. The View

Configuration option looks at the EEPROM settings, which are not

affected by Auto H/W Sensing. Therefore, View Configuration may

show settings that differ from the actual adapter configuration.

When the driver is loaded, however, will show you the current

adapter configuration.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.