STANDARD MICROSYSTEMS CORPORATION

Novell LAN WorkPlace v3.0 for OS/2 Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to copy the Novell LAN WorkPlace version 3.0 for OS/2

driver from the SuperDisk.

o  How to load the LAN driver and establish network connection.

COPYING THE DRIVER FILE

The files 8008ODI.SYS and NET.CFG are located in the \OS2_ODI

subdirectory on the SuperDisk. Copy these files to the \NETWARE

subdirectory on your PC.

LOADING THE DRIVER

Use the following instructions with those in the NetWare Requester

for OS/2 manual. Note that when you are prompted for a Destination

Directory, select \NETWARE.

1. Load the NetWare INSTALL program according to the instructions

in the NetWare manual.

2. Select "Requester on workstation" from the menu.

3. Specify the source and target directories.

4. Select "Edit CONFIG.SYS and Copy Files".

5. Select "8008ODI.SYS" from the list. If the SMC driver is not in the

list, type: 8008ODI.SYS

6. If prompted as to whether this is a third-party NIC driver, select

"Yes".

7. Select or deselect any remaining options, then select "Save".

In the "Save File As:" field, type: CONFIG.SYS

8.  In the Copy Requester Files window, select "Copy".

9.  Proceed with the instructions in the NetWare Workstation for

OS/2 manual until you complete the Requester installation.

10. Make sure the NetWare Requester software is loaded and running.

11. Obtain IP-related information about your network, such as

IP-address, router address, subnet mask, and Ethernet frame

type. This information may be obtained from your System

Administrator. You will need to provide this information for

the upcoming installation.

12. Insert disk 1 of the LAN WorkPlace for OS/2 version 3.0 in a

disk drive.

13. From a full-screen OS/2 window, type <drive>:INSTALL and press

the Enter key.

OR

If you have a monochrome monitor, type <drive>:INSTALL -M and

press the Enter key.

In the above lines, <drive> is the floppy drive where you

inserted disk 1 of the LAN WorkPlace for OS/2 version 3.0.

14. Select the hard drive you want to install to. This should be the

same drive where you installed the requester.

15. When prompted to update your CONFIG.SYS, NET.CFG, and RESOLV.CFG

system files, select "Yes".

16. When prompted for a frame type, select the "Standard Ethernet

TCP/IP framing".

17. Use the information you obtained from your System Administrator

in step 11 as you continue with the installation.

18. This completes the installation. Reboot your computer.

If you experience problems attaching to your network server or

TCP/IP host, verify the frame types in your NET.CFG file. Most

NetWare servers require an Ethernet 802.3 frame type. This usually

appears in the NET.CFG file as:

Link Driver 8008ODI

Frame Ethernet_802.3

The majority of TCP/IP hosts require an Ethernet II frame type.

This usually appears in the NET.CFG file as:

Link Driver 8008ODI

Frame Ethernet_II

After the LPW installation, you should have multiple frame types

loaded. The following sample NET.CFG file shows an example of

how these multiple frame types can be specified:

Link Driver 8008ODI

Frame Ethernet_802.3

Frame Ethernet_II

or

Link Driver 8008ODI

Frame Ethernet_802.3

Frame Ethernet_802.2

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.