STANDARD MICROSYSTEMS CORPORATION

DEC PATHWORKS v4.1 for DOS

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the DEC PATHWORKS version 4.1 for DOS NDIS driver

files from the SuperDisk to your PC.

o  Prepare your operating system so you can attach to the network.

COPYING THE DRIVER FILES

1. Copy all the files from the \NDIS subdirectory on the SuperDisk

to a Destination Directory on your PC (the recommended Destination

Directory is C:\DECNET).

2. Refer to DEC Pathworks installation manual.

3. Copy the file 8008NDIS.DOS from the SuperDisk to the Destination

Directory on your computer's hard disk.

PREPARING YOUR OPERATING SYSTEM

1. Insert the DEC Pathworks Client Setup diskette #1 into your

computer's disk drive.

2. Type: NETSETUP

3. Press the Enter key.

4. When prompted about whether you have an LK250 keyboard, select

"N(o)" if you do not have this type of keyboard.

5. Select the DECNET option and press Enter.

6. Insert the DNET TRNSPRT diskette into your computer's disk drive.

7. Specify the destination drive letter.

8. When the prompt "Are you creating an initial workstation

diskette" appears, select "N(o)".

9. Type the node name and address.

10. Press Enter.

11. Type the server node name and number.

12. Select "Ethernet" and press Enter.

13. Select "Other NDIS".

14. When prompted to "Enter Full Drive:\Path\Filename for

NDIS Driver," enter the full path where the NDIS driver

resides on the SuperDisk. For example:

A:\NDIS\8008NDIS.DOS

15. Press Enter.

16. When prompted to "Enter Full Drive:\Path\Filename for

PROTOCOL.INI STUB File," enter the full path where

this file resides on the SuperDisk. For example:

A:\NDIS\DEC_PATH

17. Press Enter.

18. Insert the DEC PATHWORKS v4.1 DNET TRANSPORT disk in

your computer's floppy drive and follow the on-screen

instructions.

19. Insert the SuperDisk in your computer's floppy drive

and follow the on-screen instructions.

20. Use any text editor to edit the PROTOCOL.INI file in

the \DECNET subdirectory as follows:

o  Change all BINDINGS statements to SMC. For example,

BINDINGS=xxx should be changed to BINDINGS=SMC.

o  Change the header [8008NDIS.DOS] to [SMC].

o  Delete the semicolon (;) from in front of NI_IRQ

and make sure the interrupt value specified here

matches the interrupt used by your adapter.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.