STANDARD MICROSYSTEMS CORPORATION

PC-NFS Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the PC-NFS driver

for the SMC EtherNote ElitePort parallel port LAN adapter.

OVERVIEW

The SMC PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the SMC Ethernet parallel port LAN adapter.

You can use two methods to install this driver.

Method 1

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual. When prompted, select "3C501" (the 3Com EtherLink card)

as your network interface card.

2. When prompted, select "3" for the interrupt level (IRQ) and "300h" for

the base I/O address.

3. Copy the following files to the \NFS subdirectory of your computer's

hard disk (the paths shown in parentheses indicate where these files

reside on the SuperDisk):

PDNFS.SYS  (this file is in the \PKTDRVIR\PCNFS subdirectory)

8008PD.SYS (this file is in the \PKTDRIVR subdirectory)

8008PD.COM (this file is in the \PKTDRIVR subdirectory)

4. Use any text editor to modify your CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE =C:\NFS\8008PD.SYS <packet_int_no>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

Note: In the above example, <packet_int_no> is a number from 96

to 128, and indicates the software-interrupt vector number

used by the packet driver program.

5.  Use any text editor to modify your AUTOEXEC.BAT file as follows:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

Note: The name "john" is used here as an example. Insert the user

name appropriate for your installation.

6. Reboot your PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate with other

computers using the TCP/IP protocol.

Method 2

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual. When prompted, select "3C501" (the 3Com EtherLink card)

as your network interface card.

2. When prompted, select "3" for the interrupt level (IRQ) and "300h" for

the base I/O address.

3. Copy the following files to the \NFS directory of your hard

disk:

PDNFS.SYS

8008PD.SYS

8008PD.COM

4. Use any text editor to modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

5. Use any text editor to modify your AUTOEXEC.BAT file as follows:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

SET NFSDRIVE = C

8008PD <packet_int_no>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

Note: In the above example, <packet_int_no> is a number from 96

to 128, and indicates the software-interrupt vector number

used by the packet driver program. Also, the name "john"

is used here as an example. Insert the user name appropriate

for your installation.

6. Reboot your PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate with other

computers using the TCP/IP protocol.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.