STANDARD MICROSYSTEMS CORPORATION

Novell NetWare OS/2 Workstation Requester v2.0 Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the Novell NetWare OS/2 workstation requester version 2.0

driver from the SuperDisk.

o  Install the driver using the Netware OS/2 requester INSTALL

program.

COPYING THE DRIVER FILES

The OS/2 Requester driver, 8008ODI.SYS, and the NET.CFG file are

located in the \OS2_ODI subdirectory on the SuperDisk.

1. Run the Novell OS/2 Requester Install program.

2. Copy the 8008ODI.SYS and NET.CFG files to the Destination

Directory specified in the Install program (the recommended

Destination Directory is C:\NETWARE).

LOADING THE DRIVER

Use the following instructions with those in the NetWare Requester

for OS/2 manual:

1. Load the NetWare INSTALL program according to the instructions

in the NetWare manual.

2. Select "Requester on workstation" from the menu.

3. Specify the source and target directories.

4. Select "Edit CONFIG.SYS and Copy Files".

5. Select "8008ODI.SYS" from the list. If the SMC driver is not in

the list, type: 8008ODI.SYS

6. If prompted as to whether this is a third-party NIC driver,

select "Yes".

7. Select or deselect any remaining options, then select "Save".

In the "Save File As:" field, type: CONFIG.SYS

8. In the Copy Requester Files window, select "Copy".

9. Proceed with the instructions in the NetWare Workstation for

OS/2 manual until you complete the installation.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.