XIRCOM POCKET ETHERNET ADAPTER III

README.DOC

Version 3.09

October 18, 1994

===================================================================

Thank you for purchasing the Xircom Pocket Ethernet Adapter III

and welcome to a new level of connectivity convenience.  The Xircom

Pocket Ethernet Adapter III is the easiest-to-use parallel port

Ethernet adapter available.

Xircom is committed to making all of its networking products compatible

with the widest range of notebook and laptop PCs possible.  As a company,

we are equally committed to customer satisfaction and timely response to

the needs and concerns of our customers.

Remember to register your Xircom Pocket Ethernet Adapter III

to receive information about future software upgrades and

new products.

CONTENTS:

---------

1)  Features

2)  ODI Notes

3)  Novell NetWare Server Driver Notes

4)  NDIS 3.0 Notes

5)  OS/2 Notes

6)  PPX Notes

7)  EPP Notes

8)  EPP Bios

9)  Machine Specific Notes

10)  Xircom Technical Support

1) FEATURES OF RELEASE

===================

Features of release 3.08 of the Xircom Pocket Ethernet Adapter III

Network drivers:

- NDIS 3.0 drivers for Microsoft's Windows for Workgroups and

Windows NT 3.1

- Novell NetWare Server Driver for NetWare 3.11., 3.12, 4.01,

and 4.02

- Support of EPP BIOS

2) ODI NOTES

=========

If you are using a NET.CFG configuration file with your ODI driver,

using "=" is not a valid syntax.

For example, use INT 7 instead of INT=7.

If you are using a frame type other than 802.2, please be sure to

copy the file NET.CFG from the ODI subdirectory on your Xircom

Network Drivers diskette to the same subdirectory where your ODI

drivers reside on your boot disk or hard disk.

3) NOVELL NETWARE SERVER NOTES

============================

At the time of this release, a new version of MONITOR.NLM for

LAN drivers has been released by Novell for NetWare operating systems.

If you are running a NetWare 3.11 file server and want to use the

Pocket Ethernet Adapter III server driver, you will need to upgrade

the MONITOR.NLM.  (There is no need to upgrade for Netware 3.12 or 4.01)

4) NDIS 3.0 NOTES

================

This version of the Xircom NT Driver PE3NDIS.386 supports

Windows NT version 3.1.  Microsoft has developed a driver for

the Pocket Ethernet Adapter III for Windows NT 3.5.  This driver is

available on the Microsoft Windows NT version 3.5 CD.

Enhanced Parallel Port (EPP) mode is not supported with NDIS 3.0

Windows for Workgroups and Windows NT.

5) OS/2 NOTES

==========

The OS/2 drivers available on the disk are intended to support OS/2

versions 2.0 and above.  These drivers do not support Microsoft OS/2

version 1.3.

6) PPX NOTES

=========

Software support for the Xircom Parallel Port Multiplexor (PPX) is

programmed into the DOS drivers provided on the current version of the

Xircom Network Drivers diskette.  The network device driver will detect

whether or not PPX hardware is present.  DO NOT load a separate PPX

driver (i.e., PPX.COM or PPX.SYS).

When you use the Xircom adapter with a Xircom Parallel Port Multiplexor

(PPX) and a printer, be sure that your printer is turned on before

you load your network drivers.

The Xircom Parallel Port Multiplexor is not supported with the NDIS 3.0

drivers for Windows for Workgroups and Windows NT.  Support will be

includedin a future release.

The PE3NDIS.OS2 and PE3ODI.OS2 drivers do not support the Xircom Parallel

Port Multiplexor (PPX).  Local printing is supported under DOS only.

Some machines with Enhanced Parallel Port (EPP) capability may not

support EPP mode when used in conjunction with a PPX.

7) EPP NOTES

=========

The drivers on this disk support personal computers (PCs) that have

Enhanced Parallel Port (EPP) capability.  To enable EPP mode on an

EPP-capable computer, see your Xircom User's Guide.

This release contains newer versions of the EPP utility files.

Please note that the network drivers on this disk MUST be used

with the EPP utility files on this disk.  The EPP utility files

on this disk will support previous versions of the network drivers.

As of this release, the following EPP-capable PCs have been or

are currently being tested:

AST Power Exec 3/25SL

COMPAQ Contura

COMPAQ LTE Lite/20 and Lite/25**

DECpc 325p

Dell 325 NC

Dell NL25

Everex Carrier SL/25

GRiD 1660

IBM N45 SL

NEC Ultralite SL/25c

NCR 3170 Safari

Panasonic CS-480

Zenith MastersPort SLe

Zenith Z-Lite

Zenith Z-Note 320L and 325L

**  To enable the EPP utility on the COMPAQ LTE Lite, first use

Compaq's SETUP program to change the default LPT port to LPT2.

8) EPP BIOS

========

The drivers listed below support an EPP BIOS standard for

DOS.  Since the IEEE P1284.3 standard for EPP BIOS had not reached

final approval at the time of this release, Xircom and numerous laptop

PC manufacturers have chosen to support one of the earlier draft

revisions of the standard.  Specifically, this release supports draft

revision 3, dated March 1993, of the EPP BIOS specification.

EPP BIOS support removes the need to load a separate utility to make

use of a PC's EPP capability.  If EPP BIOS is present, the driver will

detect it and the Pocket Ethernet Adapter III will operate in EPP mode.

EPP BIOS is a standard for DOS with a PC's CPU operating in real mode.

Therefore,  EPP BIOS is only supported with the following DOS drivers.

Novell ODI Workstation driver, PE3ODI.COM

NDIS driver, PE3NDIS.EXE

Packet driver, PE3PD.COM

- KEYWORD "NOEPP"

The ODI driver configuration file, NET.CFG, contains a new keyword

specifically for EPP BIOS.  This keyword is "NOEPP". When this

keyword is present in the NET.CFG file, or the PROTOCOL.INI file EPP

mode, will NOT be invoked by way of EPP BIOS.

The following machines have been identified as supporting EPP BIOS.

AST Ascentia 900N

COMPAQ Elite

COMPAQ Contura Aero

Dell Latitude XP

Dell Latitude M Series

Gateway 2000 Colorbook

IBM EduQuest Fifty-five

NEC VERSA

NEC VERSA/V50

Zenith Data Systems Z-NOTEFLEX

9) MACHINE-SPECIFIC ISSUES

=======================

COMPAQ LTE Lite/20 or Lite/25

- When running these models in EPP mode, run Compaq's setup program and

change the printer port from LPT1 to LPT2 before loading the Xircom

EPP utility.  When the network driver is loaded, a message will

indicate that LPT2 is being used.

IBM PS/2 Model 70, IBM ThinkPad 720, and ThinkPad 350

- These machines utilize a different implementation of the parallel port

which requires the "RDS" keyword to be placed in the appropriate

network configuration file.  If you are using ODI, place the keyword

in the NET.CFG file.  If you are using NDIS, add it to the PROTOCOL.INI

file.  Other machines may also have this different implementation and

may require this keyword to be added.

IBM ThinkPad 500

- When using NDIS 3.0 with Windows for Workgroups 3.11 make sure that

the parallel port setting is set to OFF.  To do this, go to CMOS setup

by pressing Func+F3.  Set the parallel port setting OFF (not Auto).

The port should remain in this mode for it to function properly.  The

driver should then load normally.

10) XIRCOM TECHNICAL SUPPORT

========================

Corporate Offices         Technical Support and Information

------------------------  ------------------------------------------------

Xircom, Inc.              United States................ TEL: 800-874-4428

Corporate Headquarters    (THE FOLLOWING 805 NUMBERS VALID JANUARY, 1994)

2300 Corporate Center Dr. 24-Hr Fax-on-demand.(USA).... TEL: 800-775-0400

Thousand Oaks, CA  91320  24-Hr Fax-on-demand (Intl)... TEL: 805-376-9020

TEL: (805) 376-9300      24-Hr Fax.................... FAX: 805-376-9100

FAX: (805) 376-9311      24-Hr BBS Data Lines......... BBS: 805-376-9130

24-Hr Internet E-mail........ INT: cs@xircom.com

24-Hr CompuServe............. CIS: GO XIRCOM

HOURS:  5:00 a.m. to 6:00 p.m., Monday through Friday (Pacific Time)

Xircom Europe NV          Europe, Mid-East, Africa TEL: +32/(0)3.360.38.60

Bisschoppenhoflaan 82-84  24-Hr Fax-on-demand......FAX: +32/(0)3.360.38.00

2100 Antwerp, Belgium     24-Hr Fax................BBS: +32/(0)3.326.31.50

TEL: +32/(0)3.360.38.11  24-Hr BBS Data Lines.....TEL: +32/(0)3.326.23.68

FAX: +32/(0)3.326.31.50

HOURS:  9:00 a.m. to 5:00 p.m., Monday through Thursday

0:00 a.m. to 4:00 p.m., Friday

Xircom Asia Ltd.          Asia Pacific................ TEL: +852-2525-2078

1604 Century Square

1-13 D'Aguilar Street     24-Hr Fax................... BBS: +852-2537-6048

Hong Kong                 24-Hr Data Lines............ BBS: +852-2537-6048

TEL: +852-2525-2078

FAX: +852-2525-8955

HOURS: 9:00 a.m. to 5:00 p.m., Monday through Friday

--------------------------------------------------------------------------

Thank you for making Xircom a part of your network.

--------------------------------------------------------------------------

Xircom acknowledges all tradenames and trademarks used in this document

as the property of their respective owners.

================================[  END  ]=================================