Parallel Port Multiplexor III

User Information

Version 1.72a

September 24, 1994

=========================================================================

Thank you for purchasing Xircom's Parallel Port Multiplexor III

("PPX-03"). This file contains information about when to use the

PPX device drivers and how to install them, along with other

information not contained in the Hardware Installation Guide.

The software provided with the PPX-03 can also be used with

Xircom's previous Parallel Port Multiplexor (model PPX-02).

The information in this README file applies to both models.

README FILE CONTENTS

====================

This README file contains the following topics:

- FCC Warning

- Changes in this Release

- When to use the Device Driver software supplied with the

PPX

- PPX Device Driver diskette contents

- Parallel Port Multiplexor III Specifications

- Compatibility

- Installing the PPX Device Driver

- Additional Parameters

- Using SETPPX

- Microsoft Windows 3.1 or 3.0 Configuration

- Troubleshooting

- Limited Warranty, Disclaimer and Limitation of Liability

- Xircom Technical Support

CHANGES IN THIS RELEASE

=======================

This release is to correct a syntax error in the SETPPX utility.

SETPPX previously was case sensitive and only lower case "a" or "b"

could be used.  This version allows the use of SETPPX with either

upper case or lower case "a" and "b".  See the "Using SETPPX" section

for complete information on using SETPPX.

FCC WARNING

===========

This equipment has been tested and found to comply with the

limits for Class A digital devices pursuant to Part 15 of the

FCC Rules. These limits are designed to provide reasonable

protection against harmful interference when the equipment is

operated in a commercial environment. This equipment generates,

uses, and can radiate radio frequency energy and, if not installed

and used in accordance with the instruction manual, may cause

interference to radio communications. Operation of this equipment

in a residential area is likely to cause interference in which

case the user will be required to correct the interference at

his own expense.

The user is cautioned that changes and modifications made to the

equipment without the approval of the manufacturer could void

the user's authority to operate this equipment.

It is suggested that only shielded and grounded cables be used

with the equipment to ensure compliance with FCC rules.

WHEN TO USE THE DEVICE DRIVER SOFTWARE SUPPLIED WITH THE PPX

===============================================================

Current Xircom Pocket LAN Adapters have PPX support built into

their DOS drivers, and do not require a separate PPX driver.

Consult the following chart to determine whether you need to

load the PPX.COM or PPX.SYS device driver supplied on your PPX

diskette.  Installation instructions for PPX drivers are provided

in this README file.

Your Pocket LAN Adapter software version is printed on the Pocket

Adapter software diskette label. Call Xircom Technical Support

at 1-800-874-4428 if you need to upgrade to a version that has

built-in PPX support, or download the current driver software

from Xircom's 24-hour Bulletin Board System (BBS).  BBS phone

numbers are provided at the end of this README file.

NOTE: The PPX is NOT supported in OS/2 environments.

==========================================================

Xircom Adapter Model   Software Version   Load PPX Driver?

==========================================================

Pocket Ethernet III        ALL                 NO

Pocket Ethernet II     2.0 and later           NO

Pocket Ethernet            ALL                 YES

Pocket Token Ring II   3.2 and later           NO

Pocket Token Ring      3.2 and later           NO

Pocket Arcnet II       2.0 and later           NO

Pocket Arcnet              ALL                 YES

Pocket 3270                ALL                 YES

=========================================================

PPX DEVICE DRIVERS DISKETTE CONTENTS

====================================

This diskette contains the following files:

README.DOC    This document.

PPX.SYS       Device driver for PPX-02 and PPX-03 for use in

CONFIG.SYS

PPX.COM       Device driver for PPX-02 and PPX-03 for use from

the command line or in a batch (.BAT) file

SETPPX.COM    Utility program to set default pass-through port.

PPX-03 SPECIFICATIONS

=====================

Model:                   PPX-03

Size:                    9.25 in. x 3.04 in. x 0.7 in.

(235 mm x 77 mm x 18 mm) including

connectors

Weight:                  4.8 oz (134 g)

Input connector:         1 DB-25

Output connectors:       2 DB-25

Agency certification:    FCC Part 15, Class A

COMPATIBILITY

=============

The PPX is designed to allow the use of a local printer with a

Xircom Pocket Adapter. It can also be used to access two local

printers from a single parallel port. Xircom makes no representa-

tions with respect to other applications that may be able to take

advantage of PPX technology, and does not support any other uses

of the PPX. The PPX is supported in DOS environments only.

The PPX requires that the parallel port be addressed through

the PC's system BIOS. Products that require manual switching or

reconfiguration of the port, such as some external floppy or hard

drives, will not work. Products that access the port directly,

such as print spoolers or file transfer programs, will not

recognize the presence of the PPX and will not work.

INSTALLING THE PPX DEVICE DRIVER

================================

If you are using the PPX with a Pocket Adapter whose software

does not support the PPX, or with two local printers, you will

need to load a PPX device driver. You may load the PPX device

driver each time you turn on your PC by putting the PPX.SYS

driver in the CONFIG.SYS file.  You may also load the driver

by executing the file PPX.COM at the DOS prompt or in a batch

(.BAT) file, such as AUTOEXEC.BAT.

To Load PPX.SYS in CONFIG.SYS

-----------------------------

1.   Copy PPX.SYS from this diskette to your hard drive.

For example:

COPY A:PPX.SYS C:\

If you wish to copy the file to a subdirectory, be

sure to include the correct pathname.

2.   Using a text editor, add the following line to your

CONFIG.SYS file before any other device drivers:

DEVICE=[path]PPX.SYS

where [path] is the directory into which you copied

the PPX.SYS file.

To Load PPX.COM in AUTOEXEC.BAT (or other batch file)

-----------------------------------------------------

1.   Copy PPX.COM from this diskette to your hard drive.

For example:

COPY A:PPX.COM C:\

If you wish to copy the file to a subdirectory, be

sure to include the correct pathname.

2.   Using a text editor, add the following line to your

AUTOEXEC.BAT (or other batch) file before any other

drivers:

[path]PPX

where [path] is the directory into which you copied

the file PPX.COM.

ADDITIONAL PARAMETERS

=====================

The following parameters are available with the PPX device

drivers.  The default value used when the driver is loaded with

no switches is shown in brackets. In most cases, the default

values will not need to be changed.

LPT=[1]   The physical parallel port on the PC to which the

PPX is attached. Options are 1 (default), 2, or 3.

A=[2]     LPT value defined for PPX port A. Options are 1,

2 (default), or 3, where A=1 means that PPX port

A is defined as LPT1, A=2 means port A is LPT2, and

A=3 means port A is LPT3.

B=[1]     LPT value defined for PPX port B. Options are 1

(default), 2, or 3, where B=1 means that PPX port

B is defined as LPT1, B=2 means port B is LPT2, and

B=3 means port B is LPT3.

Note that the A= and B= switches MUST have different

values.

INT=[A]   Designates which PPX port (A or B) will have

interrupt capability. Options are A or B.

For an on-screen summary of these options, type PPX ? at your

DOS prompt.

USING SETPPX

============

The SETPPX.COM utility on the Device Drivers diskette can be used

to set either port of the PPX as the default pass-through port.

The intended purpose of this utility is to allow the use of a

printer without loading either Xircom Pocket LAN Adapter drivers

or PPX device drivers.

The command line format is as follows:

SETPPX A    sets the pass-through to port A

SETPPX B    sets the pass-through to port B

SETPPX ?    displays the command line options

MICROSOFT WINDOWS 3.1 OR 3.0 CONFIGURATION

==========================================

To set up Windows version 3.1 for printing through the PPX on

LPT1 (default):

1.   Choose Control Panel and select Printers.

2.   Click on CONNECT.

3.   Choose LPT1.DOS or LPT1.OS2 as the printer port.

4.   Disable "Fast Printing Direct to Port" option.

5.   Click on OK.

6.   Click on CLOSE.

To set up Windows version 3.0 for printing through the PPX on

LPT1 (default):

1.   Choose Control Panel and select Printers.

2.   Click on CONFIGURE.

3.   Choose LPT1.OS2 as the printer port.

4.   Click on OK.

5.   Click on OK.

TROUBLESHOOTING

===============

If you have followed the installation and configuration

instructions in the Hardware Installation Guide and this

README file and are not able to get your printer to work

with the PPX, try the following test before calling Xircom

Technical Support:

1.   Check all hardware connections.

2.   If you have configured the PPX utility to load as a device

driver (PPX.SYS) or as part of a *.BAT file (PPX.COM),

remove (or "remark out") the PPX reference from the

appropriate file.

3.   Turn the computer off.

4.   Connect the printer to the "A" port of the PPX.

5.   Turn the computer back on.

6.   From the Device Drivers diskette, run the SETPPX utility

to set the default pass-through to port A. For example, type

A:SETPPX A

7.   Without loading any PPX device driver, attempt to access the

printer attached to the PPX. For example, try printing a

document.

a.   If the printer works through port A, verify that you are

using the correct LPT and A and B values for your

configuration.  Contact Xircom Technical Support for

assistance.

b.   If the device does not work through port A, remove the

PPX from the computer's parallel port and attach the

device directly to the computer. If the device works

without the PPX, contact Xircom Technical Support for

assistance.

COPYRIGHT AND LICENSE INFORMATION

=================================

Xircom reserves the right to make improvements and/or changes in

the products and programs described in this README file at any

time without notice.

The software described in this README file is furnished under

license and may be used or copied only in accordance with such

license.

(c)1993  Xircom, Inc. All rights reserved. Xircom is a registered

trademark of Xircom, Inc.

LIMITED WARRANTY, DISCLAIMER, AND LIMITATION OF LIABILITY

=========================================================

(a) Xircom warrants to the original purchaser that:

1.   When purchased, the Parallel Port Multiplexor III (the

"PPX-03") is free from defects in material and workmanship;

and

2.   For two years from the purchase date, the software

accompanying the PPX-03 (the "Software") and the PPX-03

itself will perform as stated in the README file.

(b) Xircom will honor claims under this Warranty only if the User

completes all the following steps before making the claim and

makes the claim according to the procedure below:

1.   Read the Hardware Installation Guide and this README file

and follow their installation, setup, software setup, and

operation instructions;

2.   Use the PPX-03 and Software only in the environments

described in the Hardware Installation Guide and this

README file.

(c) Xircom will not honor this Warranty (and this Warranty will be

automatically void) if there has been any (1) tampering with the

PPX-03's external label containing the PPX-03's serial number;

(2) attempt to remove the label or to open the PPX-03's case; or

(3) repair or attempt to repair made by anyone other than a

Xircom-authorized technician.

(d) This Warranty does not cover, and Xircom will not be liable

for, any damage or failure caused by misuse, abuse, acts of God,

accidents (such as dropping the PPX-03 or the diskette containing

the Software), electrical irregularity, or other causes beyond

Xircom's control, or claim by other than the original purchaser.

(e) If, after inspection, Xircom determines there is a defect or

failure covered by this Warranty, Xircom will elect to repair or

replace the PPX-03 or Software or refund the purchase price (if

replacement is not possible or repair is not practical). The

replacement may be a new or refurbished product.

(f) To report a problem or to request warranty service in the

U.S.A., the purchaser must first contact Xircom's Technical

Support department at (800) 874-4428 and obtain a Return Material

Authorization (RMA) number. Outside the U.S.A., contact your

regional Xircom office or local reseller.

Products returned for any reason must be complete and must include

the PPX-03, the software diskette, Hardware Installation Guide,

and all other components from the original package, as well as a

copy of the receipt of purchase, and must be shipped prepaid, with

the RMA number clearly visible on the outside of the shipping

container, to the following address:

Xircom Service Department

5717 Corsa Avenue

Westlake Village, CA 91362

(g) The statements in this Warranty are the only warranty made

by Xircom for the PPX-03 and Software.

(h) XIRCOM DISCLAIMS AND WILL NOT BE LIABLE FOR ANY INCIDENTAL,

CONSEQUENTIAL, OR OTHER DAMAGES (SUCH AS PERSONAL INJURY OR

DAMAGE TO ANY COMPUTER OR DATA).

(i) (The following paragraph applies to the countries of the

European Community only.)  IN ANY CASE OF DAMAGE TO WHICH THE

EUROPEAN COMMUNITY DIRECTIVE OF 25TH JULY 1985 IS APPLICABLE AND

WHICH DOES NOT RESULT FROM NEGLECTING THE INSTRUCTIONS SET OUT

UNDER PARAGRAPH (b) OF THIS WARRANTY OR FROM THE OCCURRENCE OF

ANY CIRCUMSTANCES AS MENTIONED UNDER PARAGRAPHS (c) OR (d),

THE LIABILITY OF XIRCOM WILL BE EXTENDED ACCORDING TO THE

PROVISIONS OF THIS DIRECTIVE WITHOUT PREJUDICE TO THE RIGHT OF

XIRCOM TO REFER TO THE LIMITATIONS OF LIABILITY AS PROVIDED BY

THE DIRECTIVE OR THE APPLICABLE NATIONAL LAW.

(j) EXCEPT FOR THE WARRANTY EXPLICITLY STATED HEREIN, XIRCOM MAKES

NO WARRANTIES FOR THE PPX-03, EXPRESS OR IMPLIED, EITHER IN

FACT OR BY OPERATION OF LAW.  XIRCOM SPECIFICALLY DISCLAIMS ANY

IMPLIED WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR

PURPOSE OR NONINFRINGEMENT.

(k) Some states or countries do not allow exclusion or limitation

of incidental or consequential damages, so the above limitations

or exclusions may not apply.

This Warranty gives you specific legal rights and you may also

have other rights which vary from state to state or country to

country.

Inquiries under this Warranty should be directed to Xircom at:

U.S.A.

Xircom

Technical Support

26025 Mureau Road

Calabasas, California  91302

(800) 874-4428 or

(818) 878-7600

Europe, Middle East, and Africa

Xircom Europe

Bisschoppenhoflaan 82-84

2100 Antwerp, Belgium

+32/(0) 3 326 34 94

Canada

Keating Technologies Inc.

25 Royal Crest Court, Suite 200

Markham, Ontario

Canada L3R 9X4

(416) 479-0230

Asia Pacific

Xircom Asia LTD.

1604 Century Square

1-13 D'Aguilar Sreet

Hong Kong

+852-525-2078

Latin America

Contact your local reseller or distributor.

XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call

or send a facsimile to the appropriate number listed below. In

the U.S.A., telephone support is available Monday through Friday

from 6:00 a.m. to 4:30 p.m. Pacific time. For Xircom Europe NV,

telephone support is available Monday through Thursday from 8:30

a.m. to 5:30 p.m., and Friday from 8:30 a.m. to 4:30 p.m. The

Xircom FAX numbers and the bulletin board systems may be called

24-hours a day, seven days a week. Access the Xircom INFOSYS BBS

at up to 14400 bps, No parity, 8 data bits, 1 stop bit (N-8-1).

Corporate Offices          Technical Support and Information

-----------------------   ------------------------------------------

Xircom, Inc.               Technical Support - - TEL: (800) 874-4428

Technical Support          Technical Support - - FAX: (818) 878-7175

26025 Mureau Road          Drivers & Tech Info - BBS: (818) 878-7618

Calabasas, CA  91302

TEL: (818) 878-7600

FAX: (818) 878-7630

Xircom Europe NV           Technical Support - - TEL: 32 0 3 326 34 94

Bischoppenhoflaan 82-84    Technical Support - - FAX: 32 0 3 326 31 50

2100 Antwerp, Belgium      Drivers & Tech Info - BBS: 32 0 3 326 23 68

TEL: 32 0 3 326 34 94

FAX: 32 0 3 326 31 50

Enjoy your Xircom Parallel Port Multiplexor III!

-------------------------------------------------------------------------

Some of the product names herein have been used for identification

purposes only and may be trademarks of their respective companies.

===============================[  END ]==================================