------------------------------------------------------------------------

FAX Software Release Notes

System Release 4.01

for OS/2

Copyright 1994 Dialogic Corporation

All Rights Reserved

OS/2 is a trademark of International Business Machines Corporation.

------------------------------------------------------------------------

This Release Note contains release information not currently documented

in the user's guides.

Releases documented: System Release 4.01

NOTICE

The Dialogic VFX/40 integrated 4-channel voice/fax solution is a

hardware assembly (set) consisting of the following components:

-   D/41E baseboard

-   FAX/40 fax resource daughterboard

The Dialogic hardware and software documentation will refer to the

VFX/40 solution by its individual components (D/41E, FAX/40). All

references made to the D/41E and FAX/40 boards apply to the VFX/40

solution.

To install the VFX/40 solution hardware (D/41E voice board with attached

FAX/40 daughterboard) in the computer chassis, follow the instructions

for installing the D/41E voice board found in the Voice Hardware

Reference.

If you have separate VFX/40 solution components (D/41E voice board;

FAX/40 daughterboard), assemble them before installing the VFX/40

hardware in the computer chassis. Follow the instructions to install the

FAX/40 daughterboard onto the D/41E voice board found in the FAX/40

Hardware Reference.

------------------------------------------------------------------------

CHANGE SUMMARY

------------------------------------------------------------------------

System Release 4.01

The following new FAX features are supported by the FAX/40 fax resource

board:

-  Conversion and transmission of ASCII data

-  fx_sendascii( ) convenience function

-  Sub-page addressing using ASCII data

-  DF_ASCIIDATA data structure for setting ASCII data graphical

attributes (margins, font, line spacing, etc.)

-  Embedded escape sequences to override DF_ASCIIDATA structure

settings

-  Set preferred maximum receive page length

------------------------------------------------------------------------

CHANGE DETAILS

------------------------------------------------------------------------

System Release 4.01

NEW FEATURES (FAX/40 ONLY)

-  Conversion and transmission of ASCII data: This feature allows the

application to transmit an ASCII file specifying a selected set of

graphical attributes (font, margins, etc.). The conversion of the

ASCII text to Modified Huffman fax data takes place on the Dialogic

FAX/40 daughterboard.

-  DF_IOTT/DF_ASCIIDATA structures: The DF_IOTT structure is used to

specify an ASCII source for fax data. The ASCII data may be present

in memory or resident in a file. When ASCII data is specified in

conjunction with the DF_IOTT structure, the optional DF_ASCIIDATA

structure can specify the following graphical attributes for the

ASCII data (refer to the DF_ASCIIDATA structure description in the

FAX Software Reference for details):

-  font

-  margins (left, right, top, bottom)

-  line spacing

-  page length

-  page padding

-  Embedded escape sequences: The ASCII data may contain embedded

escape sequences from the Dialogic ASCII to FAX command set. These

escape sequences can be used to change some of the graphical

attributes from within the ASCII file, overriding the values

specified at the application level in the DF_IOTT/DF_ASCIIDATA

structures (refer to the FAX Software Reference for details).

-  fx_sendascii( ) convenience function: The fx_sendascii convenience

function transmits a single ASCII source file at the default width,

length and margins for ASCII data (refer to the FAX Software

Reference for details).

-  Set preferred maximum receive page length: The preferred maximum

length of the page that can be received by a fax channel from a

remote transmitter can be specified in the rcvflag field of the

fx_rcvfax( ) and the fx_rcvfax2( ) functions. The preferred maximum

page length can be specified as A4 size (approximately 11 inches),

B4 size (approximately 14 inches) or unlimited length (refer to the

FAX Software Reference for details).

-  Sub-page addressing using ASCII data: ASCII data may be merged on

the same page with an image. For example, a fax page can be

transmitted consisting of a short image file (such as a company

logo) followed by an ASCII text file. NOTE: The DF_IOTT structure

in conjunction with the DF_ASCIIDATA structure would describe the

ASCII sub-page (refer to the FAX Software Reference for details).

COMPATIBILITY ISSUES

-  For applications developed using versions of the FAX library

earlier than System Release 4.01: The sndflag argument of the

fx_sendfax( ) function, and rcvflag argument of the fx_rcvfax( )

and fx_rcvfax2( ) functions have been changed from an int type to

an unsigned long type. Compiler warnings may result if the sndflag

or rcvflag argument is being passed to the send or receive function

as a variable when existing applications are compiled and linked

with the System Release 4.01 FAX library. When this is the case,

before recompiling your application with the System Release 4.01

library, change any applicable sndflag and rcvflag variable

declarations from int type to unsigned long type.

RESTRICTIONS AND LIMITATIONS

-  FAX/40 - ASCII data formatted with the Dialogic ASCII to FAX

Command Set (embedded Escape sequences in the ASCII text):

.  To ensure compatibility with future software versions, on EACH

line of the ASCII text, for groups of ASCII characters

requiring underline and/or bold attributes, set the ON Escape

sequence before the first ASCII character in the group and the

OFF Escape sequence after the last character in the group (see

the ASCII to FAX Tables in the Appendix of the FAX Software

Reference for Escape sequence details).

.  At the beginning of each page of ASCII text, the font and line

wrap Escape sequences must be set if they are to be different

than the settings specified in the DF_ASCIIDATA structure.

-  FAX/40 - FAX Page Header Attribute Parameter: The fax page header

attribute parameter FC_HDRATTRIB will only support the default

value (DF_HDRINSERT) and the disable header value (DF_HDRDISABLE).

The other parameter values, if set, will not take effect.

-  Using the DFC_MPG io_phdcont value to merge pages for FAX

transmission (sub-page addressing): Following the last DFC_MPG

entry in the DF_IOTT list for a merge page, the first non-DFC_MPG

entry must specify only one page.

-  Function Calls from Different Threads: Be careful when calling

voice and/or fax functions from different threads on the same

channel device at the same time. Calling a function (such as

dl_stprm( ) or fx_setparm( )) while a channel device is sending or

receiving a fax, may reset the channel device status to "idle."

-  FAX/120 Hardware Diagnostic (FAXCHK): To use the FAX/120 Hardware

Diagnostic (FAXCHK) program to check the operation of FAX/120 board

components, the associated D/121A or D/121B voice board's IRQ level

must be set at 9 or lower.

------------------------------------------------------------------------