==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

INTRODUCTION

Tango-PCB PLUS  Version 2.22  contains the  enhancements and bug fixes,

described below.

Please note  the following  compatibility issues:  While Tango-PCB PLUS

Version 2.22 PCB files are backward compatible with Version 2.11A, they

are not  backwards compatible  with versions  prior  to  Version  2.00.

Files saved  with Binary Save option in Version 2.22 are not compatible

with the  current version  of Tango  Route PRO.   When you are ready to

route your  file you must use the ASCII save option.  Files saved using

the Binary Save option cannot be read by any PCB Version prior to 2.11.

INSTALLATION INSTRUCTIONS

ACCEL has  standardized on  an automatic installation program, INSTALL,

which uses  compressed files on the distribution diskettes.  The Tango-

PCB PLUS Reference Manual contains complete instructions for using this

utility.   The minimum free disk space required for a full installation

of this release is 3.2 MB.

NEW FEATURES AND ENHANCEMENTS

The following  features have  been added  or improved  in this release.

Please consult  the Release  Notes for  a detailed description of these

enhancements.

DXF FILE  LOAD -  DXF-format files  may now  be loaded  into  PCB  PLUS

through the  File Load  command.  Dimension blocks and most simple

entities are supported; other block, shapes, and 3D items are not.

AUTOMATIC DRILL  TABLE GENERATION  -  The  new  Utilities  Drill  Table

command will create a table containing the drill symbols used, the

number of  holes using those symbols, and the size of those holes.

Additional user-defined fields are also supported.

AUTOMATIC COMPONENT  RENUMBERING -  The new  Utilities Renumber command

allows you  to specify  some or  all components for renumbering in

any of eight different ways.

DESIGN-BASED .INI  AND .DFN FILES - Now when you save your design file,

a corresponding  .INI and  .DFN files can also saved.  This allows

you to  customize your projects and environment.  Check the bottom

box in  Setup Options  to enable  the feature,  which  is  off  by

default.

MULTIPLE POWER  AND GROUND NETS - The new Nets Plane Assign command and

other enhancements  add support  for the  connection  of  multiple

power and ground nets to aid in creating split-plane designs.

NETS ROUTE  SUSPENSION -  Now you  can suspend  a  route  and  leave  a

connection instead  of placing  a line  to the  destination point.

Page 1

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

Just use either slash key instead of pressing <Esc>, and come back

to it later.

OPTIONAL NET  OPTIMIZATION -  Now you  can elect  NOT to have your nets

optimized after Nets Load or Place Component processing.

EUROPEAN CHARACTER  SET -  Support for  the Multilingual  850 (Latin I)

character  set   has  been   added  through   the  external   file

UPPER128.FON.   European characters  can now be placed directly in

your design and output normally.

DATE, TIME,  AND FILENAME  KEYWORDS -  Now when  you place the ".DATE",

".TIME", and .FILENAME" keywords in your design, they are replaced

with their  values.   The new  keywords are  not supported for use

with components.

ONE-KEY ZOOM IN AND ZOOM OUT - You can now execute a Zoom In command by

pressing the "+" key, and Zoom Out with the "-" key.

DRIVERS -  Several drivers have been fixed, and two new printer drivers

have been added:

TIGA.DRV       The  TIGA  graphics  driver  now  works  well  with

Hercules- and  related-brand TIGA  cards.  Gemini-brand cards

use interrupts  that can  cause the  program to  appear to be

"hung": this  can happen  during rapid  mouse movement  while

closing down  a dialog  box; the  program will  revive if the

mouse is  moved to the same location as when it "hung".  This

is not  a problem with the TIGA.DRV driver, and should not be

a problem  if you  are careful  with your mouse during screen

repaints.

HPPJ180C.DRV   The Hewlett-Packard  PaintJet 180-DPI color printer

driver was broken and was not included with the last release;

it returns with 2.22.

RDXY800.DRV    The Roland  DXY-800 plotter  driver now outputs the

correct coordinates,  instead of  them being  four times  too

large.

HPDJ150C.DRV

HPDJ300C.DRV   We  have  added  support  for  the  Hewlett-Packard

DeskJet color printers in 150- and 300-DPI modes.  Applicable

printers include the HP DeskJet 500C.

HPLJ600.DRV    We  have  added  support  for  the  Hewlett-Packard

LaserJet printers  in  600-DPI  mode.    Applicable  printers

include the HP LaserJet 4.

OKI321.DRV     We have  added support  for the  Okidata  Microline

321.

Page 2

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

COMSPEC ENVIRONMENT VARIABLE - Now you can choose the command shell run

by the File DOS command.  Include the line "set COMSPEC=shell.com"

in your  AUTOEXEC.BAT file--substituting  your command  shell  for

"shell.com"--and your  favorite alternative command shell will run

whenever you File DOS.

PROBLEM FIXES

The following bugs were fixed in this release:

1.   250-MIL HOLES  - These  are now  output correctly  for all  output

types.

2.   CHARACTER SET  - The tilde character ("~") may now be output.  The

question mark  in the  basic character  set now  has a  dot at the

bottom.

3.   CONNECTIONS  -   Deleting  highlighted   lines   produces   normal

connections, not highlighted ones.

4.   CURRENT PAD - Illegal values are now trapped with error messages.

5.   DELETE BLOCK  - Connections  are no longer consolidated unless one

was actually created during the deletion process.

6.   DRC -  The DRC  report now  correctly flags  clearance  violations

involving surface pads with the same center coordinates.

7.   DXF POLYGONS  -  All  non-self-intersecting  polygons  should  now

output correctly.

8.   EDIT COMPONENT  - Releasing  a component caused a host of problems

in conjunction with other complex operations.  These problems have

been fixed.

9.   EDIT PAD  - Editing  an asymmetrical  pad no  longer  changes  its

rotated counterpart.

10.  LEADING ZEROES - Leading zeroes are no longer deleted from strings

containing them.

11.  METRIC DISPLAY - The status-line display of metric coordinates now

rounds instead  of truncates.   This makes the display as accurate

as possible.

12.  MISSING STRINGS  -  Component  strings  would  sometimes  fail  to

display; this has been fixed.

13.  MOVE BLOCK  - Surface  pads  can  now  only  move  when  they  are

selected.

Page 3

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

14.  NETS - Errors 701 and 1619 (no through-hole pads in plane net) are

no longer incorrectly displayed when optimizing nets.

15.  NETS LOAD  - The value and type component fields are now correctly

updated when loaded from the netlist file.

16.  NETS ROUTE  - If  you  route  a  connection  starting  on  a  line

endpoint, changing layers now places a via for you.

17.  OUTPUT APERTURES  - Clearing the aperture and tool definitions and

assignments and pressing the OK button now clears ALL assignments,

not just the one associated with items on the current design.

18.  OUTPUT REPORTS  - Manual  form-feeds are  no longer  required when

outputting only  one report.   Dialog data no longer get corrupted

when you  press OK  and output no reports; the data also get saved

to the .INI file when no reports are output.

19.  PLACE BLOCK  COPY -  Component reference designators are no longer

lost during this operation.

20.  PLACE COMPONENT  - Editing component patterns will no longer cause

items to disappear.

21.  POLYGON OUTPUT  - Polygon  vector output  has been  enhanced: many

gaps have been eliminated by using greater overlap of fill lines.

22.  REFDES SPACES  - The  Edit Component command now strips all spaces

from reference  designators.   The Nets  Load command  also strips

them upon  input, so  that files  created with  refdes spaces will

still load.

23.  TIGA DRIVER  - If the TIGA graphics driver has insufficient memory

for initialization, PCB PLUS now issues an error message and exits

gracefully.

24.  WAS-IS FILE  - Changes  made through the Place Designator and Edit

Component commands  will now  correctly be reflected in the was-is

file; was-is file corruption has been eliminated.

25.  OUTPUT TYPE  - If  the output  driver type is missing, it will now

default to DXF, instead of garbage.

26.  OUTPUT CAM - CAM File Queue Entries no longer get corrupted.

KNOWN PROBLEMS

1.   DXF OVAL PADS - Oval pads show up as lines in DXF output, when the

lines/arcs/circles option is selected.

Page 4

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

2.   NETS OPTIMIZE  - This command currently does not support partially

routed nets,  and may  drop  connections  from  them  rather  than

optimizing.   If this  happens, use  Nets Reconnect to replace the

connections.

3.   OUTPUT DRIVERS  - Even  though the  driver input  control  in  the

second Output  Plot/Print dialog  will accept  manual entry,  this

entry is  ignored; drivers  can only  be selected from the drivers

list box.

4.   MOVE ENDPOINT  - It is sometime possible to fool Move Endpoint and

leave a  dangling blueline.   If  a line  connected to  another is

deleted and replaced with a blueline, then the line is replaced by

Place Line  (not Nets  Route), Move  Endpoint at that intersection

may move the lines but leave the blueline.  This can be fixed with

Nets Reconnect,  but it  is always  best to route connections with

Nets Route.

5.   SELECT LINE  - Some  diagonal lines  cannot be  selected except at

their endpoints;  others can  be selected  even if  the cursor  is

close to  but not  over the line.  These problems never occur with

lines at  90- or  45-degree angles,  but only  with non-orthogonal

lines.

6.   SQUARE APERTURES  - If you use a Square Aperture draw aperture you

may see  some 'chunks'  missing from corners of your polygons when

you load your photoplot file.

7.   BLUE LINES  - Deleting  Polygons or  Arcs, or  Lines that  are not

routed to  Pad Centers,  will not  give 'blue  lines' if  they are

deleted.

8.   MACROS -  Under extreme low-memory conditions, you may receive two

out-of-memory error  messages while  recording  a  macro.    After

acknowledging the  first, the program may appear to be hung but it

is actually  waiting for  another acknowledgment:  hit <Enter>  to

return to normal processing.

9.   POLYGONS -  Complicated polygons  are those with many simultaneous

vertical edges;  e.g., a polygon that looks like a horizontal saw,

with all  of its  points at  the same  y-coordinate.   Complicated

polygons outline  correctly, but  do not  fill  correctly  on  the

display.    Also,  there  may  not  be  enough  memory  to  output

complicated polygons,  depending on  the  size,  scaling  factors,

available memory,  and driver used (the Postscript drivers are not

affected).  If you run out of memory while outputting, you can try

purging the  Undo information  (File DOS  will do this), break the

polygon into  two or more smaller polygons, or rotate the image to

change the  vertical edges  into horizontal ones.  Also, extremely

narrow polygons may not be displayed at some zoom levels.

Page 5

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

ADDITIONAL CONSIDERATIONS

1.   NETS  RECONNECT  WITH  MULTIPLE  PLANE  NETS  -  When  using  Nets

Reconnect with  a split-plane design, you must make sure that free

pads and  vias are  disconnected from the power and ground planes.

If they  are connected,  Nets Reconnect  (and Nets Verify and DRC)

will detect  a short, and stop without filling primitives with net

information.   If you wish to reload the netlist for  a board with

free pad  or via  plane-connections, you  must follow these steps:

1) disconnect  all free  pads and  vias from  the power and ground

planes, keeping  track of  them for  future reference; 2) load the

netlist; 3) Nets Reconnect; 4) use Nets Plane Assign for the plane

nets (this  should replace  all of your plane connections); 5) run

Nets Reconnect  again if  superfluous  bluelines  remain;  and  6)

referring to  your notes from step 1), reconnect any leftover pads

and vias  to the power or ground planes.  Once the primitives have

net information,  you should  never have to disconnect the pad and

via plane connections in order to load your netlist.

2.   NETS ROUTE  SUSPENSION -  You can  use "set  TANGOSLASH=string" in

your AUTOEXEC.BAT  file to  assign alternate  keys for suspension;

there are  several things  to keep  in mind.   You  must use  only

printable characters in your string.  These characters can have no

other meanings:  hotspot and  macro keys will not work.  Using the

TANGOSLASH variable  at all  replaces the  slash  keys  with  your

string; the slash keys will no longer work unless included in your

string.

3.   POSTSCRIPT CARTRIDGES  - Some  PostScript  cartridges  cause  port

timeout errors; this usually happens because they are not finished

processing the current page when sent more data, and they have not

set the  port busy-flag.  Some cartridges  redefine the  page size

regardless of instructions received to the contrary; these devices

print out  an A4-size output on two A4-size sheets instead of one.

Neither of  these problems occur on any true PostScript printer we

have tested or on other cartridges.

4.   KEYWORDS -  The .DATE,  .TIME, or .FILENAME keywords are refreshed

whenever  output--whether  the  output  is  to  the  screen  or  a

peripheral device.  Only the keyword strings are stored, not their

resultant text.   This  means that selecting them for movement may

create "text placed on text" for the keyword.  This does not occur

for .FILENAME.   It  can occur for .DATE if placed today and moved

tomorrow, and  occurs often  for .TIME.   Zoom Redraw corrects the

display.   When printing, if the .TIME keyword crosses two or more

printing passes, its output may change between them.

Page 6

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

5.   ROUTING WITH MULTIPLE PLANE NETS - Tango-Route PLUS fully supports

designs with one power and one ground net, but has limited support

for the  Multiple Plane  Nets feature  in this  version:    unless

surface-mount pads are fanned out to vias in Tango-PCB PLUS, Tango

Route PLUS  will not  route them  or otherwise connect them to the

plane; if  fanned out  to vias,  the vias will be connected to the

plane.  Tango RoutePRO supports a maximum of one power net and one

ground net.

6.   DESIGN-BASED .INI  FILES -  If  the  program  is  started  without

specifying a  design file,  the PCB.INI  file is always loaded and

processed first,  before the  design-based .INI file referenced in

PCB.INI.     The  colors  and  settings  (including  speed-palette

display) stored in PCB.INI will appear momentarily, to be replaced

immediately by  the design-based  .INI colors  and settings.  This

will occur  when you start the program without specifying a design

file.

7.   DESIGN-BASED INI  FILES - Remember that design-based INI files are

only saved  if the  design file  is explicitly  saved.  This means

that Setup  DRC and  Output Reports  information and  changes, for

instance, will  be lost unless you save the design file.  The rule

of thumb  is now  more valid  than ever:   when  you want  to keep

something, save!  PCB.INI always contains the current setting when

you exit the program.

8.   DESIGN-BASED DFN  FILES -  When Tango-PCB  PLUS finds  and loads a

design-based DFN  file, it  first clears  all definitions.  If you

have line, pad, via, text, custom speed-palette or CAM definitions

or assignments  that you  want to  keep, make  sure they are saved

into a  design-based DFN  file (you  will be prompted to do this),

because they  may be  overwritten when  you load  your next  file.

Clearing the  DFNs keeps your environment free from the clutter of

unused DFNs.   For instance, before this release it was quite easy

to accumulate  hundreds of unused pad definitions, which you would

have to prune by hand (using the Current Pad command) just to keep

them  manageable.     If  you  wish  to  load  additional  non-CAM                                                           additional

definitions, use the File Load command to load the desired file as                                                                            as

a block,  then delete  the block:  new DFNs are always created for            a block

new items  placed in  the workspace.   PCB.DFN always contains the

current DFNs when you exit the program.

9.   OUTPUT TYPE  SETTINGS -  Remember that  each  output  type  (Plot,

Print, PostScript,  and DXF)  has its  own complete  set of output

parameters including  scaling values  and the destination port (or

file).   When you  change types  (using the  driver  List  button,

driver names can be typed into the input control but they will not

register unless  selected using  List), the  program remembers the

parameters for the old type, and replaces them with the remembered

parameters for the new type--the changes have not been lost.

Page 7

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

10.  DXF-IN DIMENSIONS  - When  working with  dimensions in AutoCAD, be

sure to use the PURGE command before output to DXF.  Each time you

move or  otherwise change  a dimension,  AutoCAD generates another

image for  the dimension;  these leftover images are output to DXF

unless you the PURGE command beforehand.

11.  DXF-IN VALUES - Remember that DXF items that exceed Tango-PCB PLUS

specifications cannot  be loaded  into the  workspace.  Items that

fall outside  the workspace  boundary, are  too big,  or too thick

will be discarded after an appropriate error message.  If you need

such items  to be  loaded, you must alter them in your CAD program

before loading  the DXF file.  Remember too that many CAD programs

use external utilities for DXF Export and Import:  changes made in

the program itself may not be reflected in the DXF file unless you

run the utility.

12.  GERBER FILE  LOAD -  Remember that  in order to load and display a

photoplot file  correctly, your current aperture descriptions must

match those  used to  create the  file.   If  you  have  loaded  a

different design  file, the current aperture data will be replaced

if there  is a DFN file corresponding to that design file.  If you

are unsure  about your aperture descriptions, the safest procedure

is to  reload the  design file  to get its DFN file, then load the

photoplot file;  do a File Clear before loading the photoplot file

if you  don't want  the clutter--DFNs  are cleared only by loading

another DFN file.

13.  AUTOSAVE -  If an AutoSave hits during a Zoom command, the save is

canceled.

14.  BLUE  LINES/DELETE   VIA  -  Deleting  a  Via  that  contains  Net

information will  not generate  a connection  line (or consolidate

connection lines).   If  you are  deleting 2 line segments and the

Via that  connects them, be sure to delete the Via before deleting

both line  segments.   If you  delete the  Via after deleting both

line segments,  the connection lines will not be consolidated, and

Nets Route  will force you to route to the location of the deleted

Via.   If you  use Delete  Highlight or Delete Block to remove the

Lines and Via you will not see this problem.

15.  .DRV FILES  - Do  not place  any non-supplied  .DRV files  in  the

directory with your Tango .DRV files.  Tango software looks at the

.DRV files  to present the list of output drivers to the user.  If

non-supplied files exist in the directory, unexpected behavior may

result.

16.  LISTING FILES  - Due  to the changes we made in filename filtering

in the  File Load  list box,  you can no longer use X*.PCB to list

all of  your .PCB  files that  start with  X.   You  can  use  the

technique to filter files in the second list box.

Page 8

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

17.  BINARY FILES  COMPATIBILITY -  The current versions of Tango Route

PRO will  not support  binary files.   You  must save off the file

using ASCII format before routing.

18.  METRIC GRIDS - Because the resolution of our database is 1 mil, we

can only   display  or save  items in 1 mil increments (not .1mm).

The metric grid coordinates that display on the Status Line should

be thought of as an on-screen calculator that converts an imperial

location to  a metric location.  The visible grid only displays in

imperial units.

19.  MACROS -  Because some  commands have  been changed,  and some new

commands have  been added, you may find that some Macros that were

recorded in  versions prior  to 2.22 may no longer work correctly.

You will  have to  re-record any  macros that no longer work using

version 2.22.

20.  MACROS -  If  a  macro  contains  two  or  more  separate  editing

commands, UNDO  will only  affect the last one; it will not affect

the entire macro.

21.  VERSION 1.16  TO 2.22  CONSIDERATIONS - 2.22 requires more netlist

information than  is contained  in version  1.XX files; be sure to

reload your netlist file to insure proper processing.

22.  PIN DESIGNATOR  MISMATCH -  If the  pin designators of a component

you are  auto-placing are  different from  those of  the  netlist,

those connections  cannot be  made.   Also, if  you  have  already

placed a  component whose  pin designators disagree with a netlist

that you  load, connections  cannot be  made, and  no  warning  is

given.   Run  the  Unplaced  Nodes  DRC  Report  to  detect  these

potential problems.

23.  OBSOLETE OVERLAY  FILES -  Overlay files  (.OV1 and  .OV2) are  no

longer used. Delete PCB.OV1 and PCB.OV2 (left over from 1.16) from

your PCB directory.

Page 9

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

GRAPHICS CARDS/TANGO DRIVER LIST

RESOLUTION COLORS   GRAPHICS CARD         Tango DRIVER

640 X 480      IBM MCGA (Monochrome)         MCGA

720 X 348      Hercules Graphics (Mono)      HERCULES

640 X 350  16  ATI EGA WONDER                EGA

640 X 350  16  ATI VGA WONDER                EGA

640 X 350  16  EIZO MD-B07 VGA               EGA

640 X 350  16  EIZO MD-B10 VGA               EGA

640 X 350  16  GENOA SuperVGA                EGA

640 X 350  16  IBM EGA or Compatible         EGA

640 X 350  16  QUAD GTI SuperVGA             EGA

640 X 350  16  Sigma VGA/H                   EGA

640 X 350  16  SOTA  VGA/16                  EGA

640 X 350  16  Tecmar VGA/AD                 EGA

640 X 350  16  Video7 VEGA Deluxe            EGA

640 X 480  16  ATI VGA Wonder                VGA

640 X 480  16  Colorgraphics EGA             EGA

640 X 480  16  EIZO MD-B07 VGA               VGA

640 X 480  16  EIZO MD-B10 VGA               VGA

640 X 480  16  EVEREX Micro Enhancer EGA     EGA

640 X 480  16  GENOA SuperEGA                EGA

640 X 480  16  GENOA SuperVGA                VGA

640 X 480  16  IBM VGA                       VGA

640 X 480  16  IBM PS/2 VGA                  VGA

640 X 480  16  NEC GB1                       VGA

640 X 480  16  Paradise EGA                  EGA

640 X 480  16  PCG VGA                       VGA

640 X 480  16  QUAD EGA ProSync              EGA

640 X 480  16  QUAD GTI SuperVGA             VGA

640 X 480  16  Sigam VGA/H                   VGA

640 X 480  16  SOTA VGA/16                   VGA

640 X 480  16  Tecmar VGA/AD                 VGA

640 X 480  16  Tseng Labs EGA 480            EGA

640 X 480  16  Video7 VEGA Deluxe            VGA;VSEVDLX

640 X 528  16  GENOA SuperEGA                CUSTOM/SVGA

720 X 540  16  Video7 VGA                    VSEV720

800 X 600  16  Ahead VGA Wizard+/Wiz Deluxe  CUSTOM/SVGA

800 X 600  16  AST VGA Plus                  CUSTOM/SVGA

800 X 600  16  ATI ULTRA                     ATI800

800 X 600  16  ATI VGA Wonder                ATI800

800 X 600  16  BOCA Super VGA                CUSTOM/SVGA

800 X 600  16  Cardinal VGA600               CUSTOM/SVGA

800 X 600  16  Chips & Tech. Super VGA       CUSTOM/SVGA

800 X 600  16  Diamond Speedstar             TSENG800

800 X 600  16  Diamond Speedstar 24X         PARA800

800 X 600  16  Diamond Stealth               ORCS3800

800 X 600  16  EIZO MD-B10 VGA               CUSTOM/SVGA

Page 10

==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.22==

800 X 600  16  EVEREX Micro Enhancer EGA     CUSTOM/SVGA

800 X 600  16  EVEREX ViewPoint 256 / 512    CUSTOM/SVGA

800 X 600  16  GENOA SuperVGA 5400/6400      GEN800

800 X 600  16  GENOA SuperVGA 7400 Series    TSENG800

800 X 600  16  GENOA SuperVGA 8000 Series    ORCS3800

800 X 600  16  Hewlett-Packard VGA           CUSTOM/SVGA

800 X 600  16  OAK OTI-067 VGA               OAK800

800 X 600  16  Orchid Fahrenheit 1280        ORCS3800

800 X 600  16  Orchid ProDesigner Plus       ORC800

800 X 600  16  Orchid ProDesignerII/IIs      ORC800

800 X 600  16  Paradise EGA or VGA           PARA800

800 X 600  16  PCG PHOTON MEGA PLUS VGA      CUSTOM/SVGA

800 X 600  16  QUAD GTI SuperVGA             CUSTOM/SVGA

800 X 600  16  Renaissance RVGA I/II         CUSTOM/SVGA

800 X 600  16  Sigma VGA/H                   CUSTOM/SVGA

800 X 600  16  SOTA VGA/16                   CUSTOM/SVGA

800 X 600  16  STB VGA                       CUSTOM/SVGA

800 X 600  16  STB Powergraph VGA            TSENG800

800 X 600  16  Tatung TVGA-16                CUSTOM/SVGA

800 X 600  16  Tatung VGA                    CUSTOM/SVGA

800 X 600  16  Tecmar VGA/AD                 CUSTOM/SVGA

800 X 600  16  TIGA Hercules Graph Station   TIGA

800 X 600  16  Trident TVGA VGA / 8900       TRI800

800 X 600  16  TSENG 4000                    TSENG800

800 X 600  16  VESA Super VGA                CUSTOM/SVGA

800 X 600  16  Video7 VGA                    CUSTOM/SVGA

800 X 600  16  Video7 Fastwrite              VSEV800

800 X 600  16  Wizard Deluxe EGA             CUSTOM/SVGA

800 X 600  16  ZyMOS POACH 51 VGA            CUSTOM/SVGA

1024 X 768 16  ATI VGA WONDER XL             ATI1K

1024 X 768 16  ATI ULTRA                     ATI1K

1024 X 768 16  Diamond Speedstar             TSENG1K

1024 X 768 16  Diamond Stealth               ORCS31K

1024 X 768 16  EIZO B10                      GEN1024

1024 X 768 16  GENOA SuperVGA 5400/6400      GEN1024

1024 X 768 16  GENOA SuperVGA 7400 Series    TSENG1K

1024 X 768 16  GENOA SuperVGA 8000 Series    ORCS31K

1024 X 768 16  OAK OTI-067 VGA               OAK1K

1024 X 768 16  Orchid ProDesigner VGA Plus   ORC1024

1024 X 768 16  Orchid ProDesigner II/IIs     TSENG1K

1024 X 768 16  Orchid Fahrenheit 1280        ORCS31K

1024 X 768 16  Paradise VGA1024              GEN1024

1024 X 768 16  Sigma Designs VGA/X16         GEN1024

1024 X 768 16  SOTA VGA/16                   GEN1024

1024 X 768 16  STB Powergraph                TSENG1K

1024 X 768 16  TIGA Hercules Graph Station   TIGA

1024 X 768 16  Trident 8900 Series           TRI1024

1024 X 768 16  TSENG 4000 Series             TSENG1K

1024 X 768 16  Video7 VRAM                   VSEV1024

1024 X 768 16  Video7 VRAM II                VSEV1024

1024 X 768 16  Western Digital 90C00         ORC1024

Page 11