Motion02.dll Interface

This document describes how to interface to the motion02.dll WINDOWS device driver that handles nuLogic's motion control boards for PCs. This device driver was written in C and exists in the form of a DLL. A header file by the name of nudll02.h contains all the function prototypes for the DLL and should be included in any application that uses the DLL interface. This header file can be found on the same software distribution disk as the DLL.

Configuration File

The device driver DLL, motion02.dll, requires the existence of a configuration file located in the same directory as the DLL and having the name 'nulogic.cfg'. The configuration file is created by running nuLogic's board configuration utility, nuconfig.exe, which should also reside in the same directory as the DLL. However, this information is provided in case someone wants to generate their own configuration file. The configuration file is a binary file containing sixteen words, each word being sixteen bits. The board configuration file establishes a board type and address for a total of eight possible board ID's, numbered 0 through 7. Each board ID corresponds to two sequential words in the configuration file, the first word is the board type and the second word is the board address. The board type has the following code: 0 for pcControl, 1 for pcStep - OL, and 2 for pcStep - CL. The board address is a hexadecimal I/O address. The board ID is implicit in the offset of the data in the configuration file. For instance the first word of the configuration file is the board type for board ID 0, the second word is the address for board ID 0.

Function Return Statuses

Every call to the driver has a function return value. This return status indicates whether or not the command completed without error. The following is a list of all the possible function return statuses and their descriptions:

0: successful (no errors)

1: communications error: ready to receive timeout

2: command error: command error on current packet

3: no data available in the return data buffer

4: communications error: return packet is not complete

5: board failure

6: application error: bad axis number passed to driver

7: application error: return data buffer not empty

8: command error: command error occurred prior to current packet

9: command error: unable to clear command error bit

10: application error: bad command ID passed to driver

11: communications error: returned packet ID is corrupted

12: applications error: bad board ID or board ID not configured

13: applications error: bad word count passed to 'send_command'

14: applications error: closed loop command only

15: communications error: unable to flush the data buffer

16: applications error: servo command only

17: applications error: stepper command only

18: applications error: closed loop stepper command only

19: can not find board configuration file

20: steps/rev or lines/rev not configured for given board/axis

21: system reset did not occur within allotted time

The Return Data Buffer

Most of the 'READ' commands, or any command that returns data other than the function return status, have a corresponding function that generates the data but does not immediately return it. In these cases the data is left in a buffer on the board to be read some time later. This buffer is called the 'Readback Buffer' or the 'Return Data Buffer' and can be read at any time using the 'read_rdb' function. The 'READ' functions that leave data in the Return Data Buffer use a naming convention that ends in '_rdb'. It should also be noted that any 'READ' function that does immediately return data requires that the Return Data Buffer be empty before execution.

Board Level Commands

Most of the functions contained in this device driver correspond directly to the functions supported by nuLogic's motion control boards. These functions are described in the user manuals for the boards. The manuals should be used as a primary reference for programming these functions. The hexadecimal command ID is useful for referring to the user manual. Note that some of the commands pertain to only one type of board. Again, the appropriate user manual will list which functions pertain to a given type of board.

The following gives a list of function prototypes, the corresponding hex command ID and the command name in English as it appears in the appropriate user manual. Pertaining to the function prototypes, BYTE is an unsigned character, eight bits, and WORD is an unsigned int, sixteen bits. It should also be noted that all functions use the recommended FAR PASCAL calling convention and that all pointers are passed as FAR.

Command ID 0x42: Read Limit Switch Status

int FAR PASCAL read_lim_stat(BYTE boardID, WORD FAR *switchstatptr);

int FAR PASCAL read_lim_stat_rdb(BYTE boardID);

int FAR PASCAL read_lim_stat4A(BYTE boardID, WORD FAR *switchstatptr);

int FAR PASCAL read_lim_stat4A_rdb(BYTE boardID);

Command ID 0x43: Set Step Output Mode & Polarity

int FAR PASCAL set_step_mode_pol(BYTE boardID, WORD mpdata);

Command ID 0x43: System Reset

int FAR PASCAL system_reset(BYTE boardID, WORD password);

Command ID 0x44: Set I/O Port Polarity & Direction

int FAR PASCAL set_io_pol(BYTE boardID, WORD pddata);

Command ID 0x45: Set I/O Port Output

int FAR PASCAL set_io_output(BYTE boardID, WORD podata);

Command ID 0x46: Enable I/O Port Trigger Inputs

int FAR PASCAL enable_io_trig(BYTE boardID, WORD trigdata);

Command ID 0x47: Read I/O Port

int FAR PASCAL read_io_port(BYTE boardID, WORD FAR *iodataptr);

int FAR PASCAL read_io_port_rdb(BYTE boardID);

Command ID 0x48: Trigger I/O Port #

int FAR PASCAL trigger_io(BYTE boardID, WORD portnum);

Command ID 0x49: Read Per Axis H/W Status

int FAR PASCAL read_axis_stat(BYTE boardID, BYTE axis, WORD FAR *statptr);

int FAR PASCAL read_axis_stat_rdb(BYTE boardID, BYTE axis);

Command ID 0x4A: Set Limit Switch Input Polarity

int FAR PASCAL set_lim_pol(BYTE boardID, WORD polarity);

Command ID 0x4B: Enable Limit Switch Inputs

int FAR PASCAL enable_limits(BYTE boardID, WORD enabledata);

Command ID 0x4C: Stop Motion

int FAR PASCAL stop_motion(BYTE boardID, BYTE axis);

Command ID 0x4D: Kill Motion

int FAR PASCAL kill_motion(BYTE boardID, BYTE axis);

Command ID 0x4E: Load Target Position

int FAR PASCAL load_target_pos(BYTE boardID, BYTE axis, DWORD position);

Command ID 0x4F: Load Velocity

int FAR PASCAL load_vel(BYTE boardID, BYTE axis, DWORD velocity);

Command ID 0x50: Reset Position Counter to Zero

int FAR PASCAL reset_pos(BYTE boardID, BYTE axis);

Command ID 0x51: Find Home

int FAR PASCAL find_home(BYTE boardID, BYTE axis, WORD direction);

Command ID 0x52: Find Index

int FAR PASCAL find_index(BYTE boardID, BYTE axis, WORD direction, WORD offset, WORD FAR *indexptr);

int FAR PASCAL find_index_rdb(BYTE boardID, BYTE axis, WORD direction, WORD offset);

Find Index is the only board level command whose interface has been slightly modified in the device driver. The difference pertains only to pcControl (servo) boards. The difference is that the 'find_index' driver call does not require the encoder line count to be passed down with each call. Instead, the encoder line count needs to be loaded only once per axis with the 'store_elc' function call. After issuing 'store_elc', the driver will use the stored encoder line count to perform a Find Index.

Command ID 0x53: Read Present Position

int FAR PASCAL read_pos(BYTE boardID, BYTE axis, DWORD FAR *positionptr);

int FAR PASCAL read_pos_rdb(BYTE boardID, BYTE axis);

Command ID 0x54: Read Steps/Sec (Velocity)

int FAR PASCAL read_steps_vel(BYTE boardID, BYTE axis, DWORD FAR *veloptr);

int FAR PASCAL read_vel_rdb(BYTE boardID, BYTE axis);

Command ID 0x54: Read Present Velocity

int FAR PASCAL read_vel(BYTE boardID, BYTE axis, WORD FAR *velocityptr);

int FAR PASCAL read_vel_rdb(BYTE boardID, BYTE axis);

Command ID 0x55: Set Stop Mode

int FAR PASCAL set_stop_mode(BYTE boardID, BYTE axis, WORD mode);

Command ID 0x56: Load Following Error Count

int FAR PASCAL load_fol_err(BYTE boardID, BYTE axis, DWORD errorcount);

Command ID 0x57: Load Steps and Lines Per Rev

int FAR PASCAL load_steps_lines(BYTE boardID, BYTE axis, DWORD stepslines);

Command ID 0x57: Load Filter Derivative Sample Period

int FAR PASCAL load_deriv_per(BYTE boardID, BYTE axis, WORD period);

Command ID 0x58: Load Filter Proportional Gain

int FAR PASCAL load_prop_gain(BYTE boardID, BYTE axis, WORD gain);

Command ID 0x59: Read Encoder

int FAR PASCAL read_encoder(BYTE boardID, BYTE axis, DWORD FAR *encoderptr);

int FAR PASCAL read_encoder_rdb(BYTE boardID, BYTE axis);

Command ID 0x59: Load Filter Integral Gain

int FAR PASCAL load_intg_gain(BYTE boardID, BYTE axis, WORD gain);

Command ID 0x5A: Set Loop Mode

int FAR PASCAL set_loop_mode(BYTE boardID, BYTE axis, WORD mode);

Command ID 0x5A: Load Filter Derivative Gain

int FAR PASCAL load_deriv_gain(BYTE boardID, BYTE axis, WORD gain);

Command ID 0x5B: Set Base Velocity

int FAR PASCAL set_base_vel(BYTE boardID, BYTE axis, WORD basevelo);

Command ID 0x5B: Load Filter Integration Limit

int FAR PASCAL load_intg_lim(BYTE boardID, BYTE axis, WORD limit);

Command ID 0x5C: Multi-Axis Start

int FAR PASCAL multi_start(BYTE boardID, WORD startdata);

Command ID 0x5D: Trigger Buffer Delimiter

int FAR PASCAL trig_buff_delim(BYTE boardID);

Command ID 0x5E: Acquire Samples

int FAR PASCAL acquire_samples(BYTE boardID, WORD numsamples);

Command ID 0x5F: Begin Trigger # Prestore

int FAR PASCAL begin_prestore(BYTE boardID, WORD trigselect);

Command ID 0x60: End Trigger Prestore

int FAR PASCAL end_prestore(BYTE boardID);

Command ID 0x61: Start Motion

int FAR PASCAL start_motion(BYTE boardID, BYTE axis);

Command ID 0x62: Load Acceleration

int FAR PASCAL load_accel(BYTE boardID, BYTE axis, DWORD acceleration);

Command ID 0x63: Load Acceleration Factor

int FAR PASCAL load_accel_fact(BYTE boardID, BYTE axis, WORD accfactor);

Command ID 0x63: Set Direction

int FAR PASCAL set_direction(BYTE boardID, BYTE axis, WORD direction);

Command ID 0x64: Set Position Mode or Set Operation Mode

int FAR PASCAL set_pos_mode(BYTE boardID, BYTE axis, WORD posmode);

Command ID 0x65: Load Position Breakpoint

int FAR PASCAL load_pos_brk(BYTE boardID, BYTE axis, DWORD breakpos);

Command ID 0x66: Enable Breakpoint Function

int FAR PASCAL enable_brk(BYTE boardID, BYTE axis, WORD func);

Command ID 0x67: Load Anticipation Time Breakpoint

int FAR PASCAL load_time_brk(BYTE boardID, BYTE axis, DWORD time);

Command ID 0x68: Load Position Reference Offset

int FAR PASCAL load_pos_ref(BYTE boardID, BYTE axis, DWORD offset);

Command ID 0x69: Load Position Scale Factor

int FAR PASCAL load_pos_scale(BYTE boardID, BYTE axis, DWORD scalefactor);

Command ID 0x6A: Set Scale Factor Sequence

int FAR PASCAL set_scale_seq(BYTE boardID, BYTE axis, WORD sequence);

Command ID 0x6B: Set Run/Stop Status Pulse Width

int FAR PASCAL set_rs_pulse(BYTE boardID, BYTE axis, WORD pulse);

Command ID 0x6C: Load Rotary Counts

int FAR PASCAL load_rot_counts(BYTE boardID, BYTE axis, DWORD countdata);

Other Driver Functions

The following driver functions are not specific to any board level command.

Send Command

int FAR PASCAL send_command(BYTE boardID, BYTE axis, BYTE cmd, BYTE wcount, DWORD cmddata);

'Send Command' can be used to send any command in the board's command set. The command requires board ID, axis number, the hex command ID, the command packet word count (2, 3 or 4), and 32 bits of command data.

Read the Return Data Buffer

int FAR PASCAL read_rdb(BYTE boardID, WORD FAR *numptr, WORD FAR *axisptr, WORD FAR *cmdptr, DWORD FAR *rdbptr);

This function is used to read the Return Data Buffer. This function requires pointers to three arrays that will be used to return axis number, command ID and corresponding data for the number of packets specified. The number of packets to be read from the RDB is specified in *numptr. The number of packets actually read will be returned in this location.

Read The Communications Status Register

int FAR PASCAL read_csr(BYTE boardID, BYTE FAR *csrptr);

This function returns the contents of the Commmunications Status Register in the location pointed to by *csrptr.

Flush the Return Data Buffer

int FAR PASCAL flush_rdb(BYTE boardID);

This function will flush the RDB of the board specified in boardID.

In Position

int FAR PASCAL in_pos(BYTE boardID, BYTE axis, DWORD target, DWORD deadband, DWORD *resultptr);

This function compares the current position to the position specified in 'target'. If the current position is within a given deadband from the target, specified in 'deadband', then zero is returned in *resultptr; otherwise, the difference between the current position and the target is returned in *resultptr.

Load RPM

int FAR PASCAL load_rpm(BYTE boardID, BYTE axis, double rpm);

This function will take an RPM and load a velocity value to the board that corresponds to that RPM. This function requires that the information containing the number of encoder lines per revolution for pcControl (servo) boards, or the number of steps per revolution for stepper boards has been previously loaded to the driver. These values can be loaded with the 'store_elc' and 'store_steps_rev' functions respectively. For stepper boards, the number of steps per revolution will also get saved when the function 'Load Steps & Lines Per Rev' is issued.

Read RPM

int FAR PASCAL read_rpm(BYTE boardID, BYTE axis, double FAR *rpm);

Returns the present velocity in RPM in the location pointed to by *rpm. This requires the number of steps per rev or encoder lines per rev have been previously loaded (see 'Load RPM' above).

Load Revs Per Second Per Second

int FAR PASCAL load_rpsps(BYTE boardID, BYTE axis, double rpsps);

Takes an acceleration value in RPS/Sec and loads the appropriate acceleration value to the board. This requires the number of steps per rev or encoder lines per rev have been previously loaded (see 'Load RPM' above).

Store Steps Per Rev

int FAR PASCAL store_steps_rev(BYTE boardID, BYTE axis, WORD steps);

Stores the number of steps per rev for a given axis and board. This information is required by the driver in order to perform the RPM or RPSPS functions for any pcStep board. This function is not required if the 'Load Steps and Lines Per Rev' function has already been issued.

Store the Encoder Line Count

int FAR PASCAL store_elc(BYTE boardID, BYTE axis, WORD elc);

Stores the number of encoder lines per revolution for a given board and axis. This information is required by the driver in order to perform the RPM, RPSPS  or 'Find Index' functions for any pcControl board.

Set Initialization Status

int FAR PASCAL set_init_stat(BYTE boardID, BYTE axis);

Indicates to the driver that an axis of a given board has been initialized. This function is the complement of 'get_init_stat' which returns the initialization status of a given axis. These two functions are intended to be used in the case that two or more separate applications are communicating with the board. However, their use is not required.

Get Initialization Status

int FAR PASCAL get_init_stat(BYTE boardID, BYTE axis, BYTE FAR *status);

Indicates whether or not the initialization status for a given board and axis has been set. This function returns a zero in the location pointed to by *status if that board/axis has been initialized, if not initialized a one is returned in the same location. This function is the complement of 'set_init_stat'. Please refer to it for more information.

Verify Address

int FAR PASCAL verifyaddr(WORD boardaddr);

Attempts to communicate with a board at a specified hex I/O address. Returns a zero if successful. Returns a one if unsuccessful.