+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

--------------------------------

SIS - Silicon Integrated Systems

--------------------------------

------------------------------------------------------------------------------

Filename   Port      Description  SIS 85C471

--------   -------   ---------------------------------------------------------

V089B410   SIS_410   4 Mem Banks, REG_6A=FF

V089B411   SIS_411   6 Mem Banks, REG_6A=FF

V089B412   SIS_412   8 Mem Banks, REG_6A=FF

V089B413   SIS_413   4 Mem Banks, REG_6A=00

V089B414   SIS_414   6 Mem Banks, REG_6A=00

V089B415   SIS_415   8 Mem Banks, REG_6A=00

NOTE: The versions above are but a few samples of the

many ingenious multiplex schemes possible with

this chipset.  (Hint: try V089B410 first).

------------------------------------------------------------------------------

Filename   Port      Description  SIS 85C460,461V  386/486 single chip

--------   -------   ---------------------------------------------------------

V072B400   SIS_400   460,461V  386,486,Cx

V072B404   SIS_404   (TL-)  with turbo-LED active low

V072B405   SIS_405   (TL+)  with turbo-LED active high

V072B40G   SIS_40G   Green-PC, Appian ADI2 VL-IDE, (GENOA and FREE)

V072B40Q   SIS_40Q   Green-PC, Promise 20230C, UMC Superio  (JBOND)

------------------------------------------------------------------------------

Filename   Port      Description  SIS "RABBIT"

--------   -------   ---------------------------------------------------------

V009B303   SIS_303   386, 85C3X0  Revs A,B,C  Turbo via PIO-0 (85C320 Pin 83)

V009B306   SIS_306   (TL-)  PIO-0 and 8042 pins

V009B307   SIS_307   (TL+)  PIO-0 and 8042 pins

V009B308   SIS_308   (T-)   PIO-0 not used, only 8042 pins

V009B309   SIS_309   (T+)   PIO-0 not used, only 8042 pins

------------------------------------------------------------------------------

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High