+----------------------------------------------+

|   MR BIOS (r)  Production BIOS Version 1.65  |

+----------------------------------------------+

|   Copyright (c) 1994,  Microid Research Inc  |

|    2336-D Walsh Ave, Santa Clara, CA 95051   |

|   Tel (408) 727-6991  /  Fax (408) 727-6996  |

+----------------------------------------------+

-------------------------------------

Texas Instruments  "TIGER"  TACT83000

-------------------------------------

Filename   Port      Description

--------   -------   ---------------------------------------------------------

V010B300   TACT300   TACT83000 386SX/DX, non-cache

V010B30A   TACT30A   x010B300  (TL-)

V010B30B   TACT30B   x010B300  (TL+)

V010B302   TACT302   TACT83000 386SX/DX, Intel 385 cache

V010B30E   TACT30E   x010B302  (TL-)

V010B30F   TACT30F   x010B302  (TL+)

V010B303   TACT303   TACT83000 386SX/DX, Austek cache

V010B30C   TACT30C   x010B303  (TL-)

V010B30D   TACT30D   x010B303  (TL+)

V010B400   TACT400   TACT83000 486DX, no external cache

V010B40A   TACT40A   x010B400  (CL-)

V010B40B   TACT40B   x010B400  (CL+)

-----------------------------------------------------------------

8042 Turbo/Turbo-LED control:  (T-) (T+) (TL-) (TL+) (CL-) (CL+)

-----------------------------------------------------------------

Hot-Key TURBO and/or LED control via keystrokes  <CTRL> <ALT> <-/+>  will

simultaneously activate all commonly used 8042 pins: 23,24, 27,28,29,30.

The system board layout must connect one of these pins to an existing

Turbo/LED circuit.  Consideration should be given to the remaining pins

which are also driven.  MR BIOS (r) 8042 or AMI KF keyboard controller

may be used for this purpose.  Award, Phoenix, and Quadtel 8042's cannot

be used with any MR BIOS (r) version designated as having this special

8042 functionality.

Active

Legend  Function  State   Description

------  --------  -----   --------------------------------------------

(T-)    Turbo      Low    8042 turbo/LED circuit ONLY (no chipset register

(T+)    Turbo      High   turbo).  Also, Setup Utility SPEED screen appears.

(TL-)   Turbo-LED  Low    Both chipset turbo and 8042 pins are activated.

(TL+)   Turbo-LED  High

(CL-)   Cache-LED  Low    Used mainly with 486, Cache state indicates "turbo".

(CL+)   Cache-LED  High