OPERATION

MANUAL

FOR

LOCKIT I with Administrator Option (L1+)

LOCKIT I Extended Edition (L1E)

LOCKIT I Hardware (L1H)

Copyright (c) 1989 - 1993 by

Security Microsystems, Inc.

and

Johnson Computer Systems, Inc.

215 Cromwell Avenue

Staten Island, New York 10305-1309

Orders (800) 345-7390  Information (718) 667-4714

Tech Support (718) 667-4720   BBS  (718) 667-0947

Main Offices (718) 667-1019   FAX  (718) 667-0131

Page i

Copyright (c) 1993 by Security Microsystems, Inc.

All Rights Reserved

LOCKIT (r) is a registered trademark

of Security Microsystems, Inc.

Other brand and product names are trademarks or regis-

tered trademarks of of their respective holders. WINDOWS,

as used in this manual, refers to Microsoft's implementa-

tion of a windows system.

Printed in the U.S.A.

April 1994 edition

Revision 0 1 2 3

Version 5.3

Page ii

Table of Contents

INTRODUCTION...................................................1

CHAPTER 1 - THE BASICS.........................................3

About this manual...........................................3

What the LOCKIT  I products do..............................4

Access control............................................4

A: drive boot protection..................................4

Ctrl-Break control........................................5

Preventing access to DOS..................................6

Lunch-Break lock description..............................6

About the products..........................................7

LOCKIT  I with Administrator Option ......................7

LOCKIT  I Extended Edition................................8

LOCKIT  I Hardware........................................8

Summary of LOCKIT  I features...............................9

System requirements........................................12

Restrictions and other concerns............................12

Disclaimer of warranty.....................................15

Your LOCKIT  I license.....................................15

Using LOCKIT  I menus......................................17

LOCKIT  I password and user names..........................17

CHAPTER 2 - INSTALLATION......................................19

Before installing LOCKIT  I................................19

Copy-protected versions of LOCKIT  I.......................19

Pre-installation setup ....................................20

The pre-installation setup menu............................23

Recording your choices for later use.....................24

Define original passwords and names......................24

Select options...........................................24

Set limits...............................................25

Define hot-key...........................................25

Lock files during installation...........................26

Choose who will install LOCKIT  I........................26

Installing LOCKIT  I ....................................29

CHAPTER  3 - AFTER INSTALLATION...............................35

Using the LOCKIT  I main program...........................35

Page iii

Table of Contents (Con't)

Using the administrator's main menu........................36

Installing Windows Lunch-Break support ..................36

Change passwords.........................................37

Set password lifetimes, after which they expire........38

Choose who can change expired passwords................40

Set number of different passwords required.............40

Choose characters displayed defining passwords.........41

Change name/password screen............................41

Select options...........................................43

Set limits...............................................43

Locking LOCKIT  I related files..........................43

Unlocking LOCKIT  I  related files ......................44

Accessing fixed disk after diskette boot.................44

Removing LOCKIT  I from the computer.....................45

Define new hot key combination...........................45

Set keyboard idle time...................................46

Control directory access by user (L1E only)..............48

Control I/O Port access by user (L1E only)...............51

Select file accesses to be logged (L1E only).............52

Selecting LOCKIT  I options ...............................57

Maximum floppy boot protection...........................58

Display password entry asterisks ........................58

Relaxed MBR protection...................................58

Sidekick compatibility mode..............................59

Ctrl-Break prohibited during boot........................59

Time/date change prohibited..............................59

Blank screen during Lunch-Break .........................60

Freeze computer during Lunch-Break ......................60

All users may exit Lunch-Break...........................60

Special display blanking ................................61

User names are required .................................61

User may change his/her name ............................61

Setting LOCKIT  I  limits .................................62

Maximum keyboard idle time...............................62

Minimum number of password characters....................63

Maximum invalid logons before alarm......................63

Maximum invalid logons before lockout....................63

Seconds to wait before auto logon........................64

Page iv

Table of Contents (Con't)

Alternate keyboard/clock handling........................64

The user's main menu ......................................65

Using the LOCKIT  I program after it is installed..........67

CHAPTER 4 - THE LOCKIT  I FILES...............................69

ACCESS.SYS (L1E only).....................................69

BRK-CNTL.COM .............................................70

CLEANDSK.DRV..............................................70

JCSSEC.386................................................70

EXEC.COM..................................................71

FLUSHLOG.COM (L1E only)...................................72

FPERM.INF (L1E only)......................................72

LOCKIT.EXE................................................72

LOCK.EXE and UNLOCK.EXE ..................................72

LOG.EXE (L1E only)........................................73

LOGOFF.COM................................................74

MEM-STAT.SYS..............................................74

SET-TIME.COM..............................................75

VIOLS.COM (L1E only)......................................76

VMOUSE.COM ...............................................77

WHO.COM ..................................................78

Optional LOCKIT  I files..................................80

DesMaster (TM) file encryption..........................80

HELPUSER.COM............................................80

LOGO.EXE................................................81

Custom features and network support.....................81

APPENDIX - PROBLEMS AND SOLUTIONS.............................82

Installing new software with LOCKIT  I installed..........82

Using Stacker or other compression programs...............82

If CLEANDSK.DRV does not run..............................83

If CLEANDSK.DRV gets erased or corrupted..................84

If LOCKIT  I does not install properly or hangs...........85

How LOCKIT  I protects against viruses....................87

What to do if you forget your password....................87

LOCKIT  I Hardware installation problems..................88

Where to get help with LOCKIT  I .........................90

Page v

Introduction

Thank you for investing in this LOCKIT  I product.

We believe you will find it to be both an effective

and convenient security system for your IBM PC, XT,

AT, PS/2, laptop, notebook, workstation, clone or com-

patible. Please note that you are not licensed to use

this product until you have read and agree to the

disclaimer of warranty on page 15.

This latest version of LOCKIT  I is the result of

many man-years of effort that it has taken to write the

almost one million lines of source code that make up

the LOCKIT  I system. These products have grown up

with the personal computer over the last nine years to

become the most reliable and secure products of their

type in existence today. We hope that you enjoy using

these products and that you tell your friends if you

like them and tell us if you don't. We look forward to

providing you with the best in service and support and

will do our utmost to keep the LOCKIT  I products

current despite the rapid changes that are occurring

in the computer and operating system markets today.

To enable us to be there when you need us, we ask

you to do us one favor. Please respect our license

agreement. When you make copies of our software and

give them away for free you are indeed hurting your-

self. If we cannot afford to support our products then

you, the user, will not receive the support and service

that you deserve. Every unpaid copy of LOCKIT  I

means less money that we have to spend for technical

support people, phone costs, new product develop-

ment, and product quality assurance.

If you have any suggestions for improvements,

please tell us about them. While we cannot make every

change in either the manuals or the programs which

have been suggested by our users, we do give careful

consideration to each suggestion and have implemented

many of them.

Page 1

(This page intentionally left blank)

Page 2

CHAPTER 1 - THE BASICS

ABOUT THIS MANUAL

While many people have told us that they are able

to use LOCKIT  I without reading the manual, we

know that others prefer a complete written description

of the programs they use. We have attempted to make

LOCKIT  I as easy to use as possible without having

to read the manual. If you have any difficulties using

LOCKIT  I, please consult the Table of Contents and

then refer to the appropriate section. All the features of

the LOCKIT  I products are accessed from a few simple

menus. This manual describes each menu and pro-

vides a detailed description of each feature accessible

from that menu.

A manual can never hope to answer all of the ques-

tions that arise from the use of a sophisticated prod-

uct like LOCKIT  I on the many thousands of not so

"compatible" personal computers.  We have included a

section in the back of this manual that addresses the

most common questions our technical support people

receive. We have also made available a bulletin board

system for registered LOCKIT  I users where the ans-

wers to new questions and problems are posted as

well as providing an ongoing forum based on messages

from our users.

We have tried to arrange this manual in a logical

and concise way, starting with a brief overview of the

product and continuing with the installation and pro-

gram use details. It is our belief that you must under-

stand what the basic function of the LOCKIT  I prod-

uct is before you can tackle the details of specific

features.

Page 3

WHAT THE LOCKIT  I PRODUCTS DO

LOCKIT  I products provide many different func-

tions to protect data on a hard disk from theft, corrup-

tion and destruction and to prevent the unauthorized

use of a personal computer.

Access Control -

All LOCKIT  I products prevent the unauthorized

use of a personal computer or LAN workstation. After

installing LOCKIT  I, users can be required to enter a

password or optionally, a user name and password,

each time the computer is booted from its hard disk.

Users who are allowed to use the computer can just

type in their password (or name and password) and

press Enter. (There is also a special no-password-re-

quired "Auto-boot" feature which will be described later.)

The boot process will then continue normally.

A: Drive Boot Protection -

The main function of the LOCKIT  I products is to

plug the security hole that has been designed into

every personal computer since the first IBM XT, namely

the ability to boot from a diskette in the A: drive and to

be able to access the hard disk. Since there is no

feasible way to prevent booting from a diskette in the

A: drive by means of software alone, the LOCKIT  I

software products do the next best thing, they prevent

access to the hard disk's contents if the computer is

booted from a diskette. The ability to boot from a dis-

kette was intentionally designed into the personal com-

puter for other reasons than to help hackers get at

important data on your hard disk. There are many

instances where, for one reason or another, the com-

puter has refused to boot-up from the hard disk. When

this occurred, you could not use your computer un-

less there was an alternative way of booting, specifi-

Page 4

cally, from a diskette in the A: drive. LOCKIT  I must,

therefore, allow booting from the A: drive and provide a

mechanism for allowing only an authorized person to

access the data on the computer's hard disk. LOCKIT

I accomplishes this through the use of the password.

When a user boots from a diskette the computer will

boot normally, but it will not be possible to access the

hard disk unless it is first unlocked by means of the

"Access fixed disk after diskette boot" feature which

requires the administrator's password. LOCKIT  I Hard-

ware has the ability to completely prevent boot-up from

a diskette for all users. The administrator can always

override this protection and boot the computer from a

diskette, should the need arise. It should be noted that

some computer manufacturers are just now starting to

put LOCKIT  I type protection into their BIOS ROMs

and including the ability to prevent booting from the

A: drive, however they still lack many of LOCKIT  I's

other security features, and most of them can be easily

defeated by removing the computer's battery for sev-

eral seconds.

Ctrl-Break Control -

Another serious security flaw in most personal com-

puters is caused by the Ctrl-Break key combination

that allows a user to "break out" of the AUTOEXEC.BAT

file as it is executing. While this is not as serious a

flaw as the A: drive boot-up problem, it still prevents

an administrator from ensuring that the computer is

started up in the way that he/she has designated. It

also allows DOS shell-type programs (like LOCKIT

III) to be escaped out of, thus lessening their ability to

limit access to DOS functions like FDISK, FORMAT,

DEL and ERASE. Once again, LOCKIT  I addresses

this problem with total control of the Ctrl-Break key

combination. With any LOCKIT  I product installed,

the administrator can turn Ctrl-Break on or off at will,

during boot-up or anytime after. This also applies to

Ctrl-C and the Alt-3 (number pad) sequences.

Page 5

Prevent access to DOS -

There are instances where a computer is being used

for a dedicated task. For example, some PC's are used in

the alarm monitoring industry, running a single dedi-

cated program, 24 hours a day. Certainly, in this ex-

ample, it would be necessary to prevent the operator

from escaping from the alarm monitoring program and

getting access to DOS so that a game or other program

could be run. LOCKIT  I provides a utility that can

constrain the PC to run a dedicated task and prevent

escape to DOS from within that task. Coupled with the

Ctrl-Break control feature of LOCKIT  I, the EXEC.COM

program, provided with all LOCKIT  I products, allows

an administrator to control the operation of a PC as no

other software product on the market today can.

Lunch-Break Lock Description -

The Lunch-Break feature is a temporary locking

mechanism for your computer that allows you to pre-

vent anyone else from using your computer while you

step away for any reason (typically a lunch break.)  When

the Lunch-Break feature is invoked, the keyboard and

mouse is locked to all input except password entry, the

screen can be completely blanked, and processing can

be allowed to continue or be suspended. This means

that a long spreadsheet computation, database opera-

tion, printout or other process may be allowed to con-

tinue normally while the computer is in the Lunch-

Break locked condition. An observer will not be able

either to see or exercise control over the computer op-

eration. Lunch-Break may be activated by pressing the

user-selectable hot key combination or, if you so choose,

the Lunch-Break feature can be automatically activated

after a selectable period of keyboard and mouse inactiv-

ity. When the correct password is entered, the screen

and keyboard will return to normal operation. This fea-

ture not only provides protection for the data on the PC's

hard disk but also protects any mainframe or network

(LAN) to which the PC is connected.

Page 6

ABOUT THE PRODUCTS

This manual covers three members of the LOC-

KIT  I product family, LOCKIT  I with Administrator

Option (L1+), LOCKIT  I Extended Edition (L1E) and

LOCKIT  I Hardware (L1H). Since all three products

contain the same basic features, these features will be

explained first and later sections will deal with the

differences in each product. You may want to take a

moment to check the diskette and packaging to make

sure which LOCKIT  I product you have purchased.

The order codes L1+, L1E and L1H are marked on the

packaging and the diskette label bears the full product

names:

LOCKIT  I with Administrator Option  (L1+) -

LOCKIT  I with Administrator Option provides mul-

tiple passwords on a computer so that each user can

have his/her own password to use the computer. It

also provides one password, called the administrator

password, that has a higher authority level than the

user passwords. Only the administrator's password will

allow access to the hard disk after booting from a

floppy disk, and only the administrator's password will

allow the removal of LOCKIT  I from the computer.

Certain files can be locked away from all users except

the administrator. LOCKIT  I with Administrator Op-

tion prevents users from seeing or modifying any file

that has the system, read-only and not the hidden

attributes set. Users will not be able to modify these

"locked" files or their attributes. As administrator, all

the features, options and limits of LOCKIT  I  will be

under your control but some of these can be made

available to the users, if you so choose. All three prod-

ucts L1+, L1E and L1H all provide the Administrator

Option feature.

Page 7

LOCKIT  I Extended Edition  (L1E) -

The Extended Edition of LOCKIT  I contains all of

the features of LOCKIT  I with Administrator Option. In

addition, LOCKIT  I Extended Edition provides subdi-

rectory protection,  port control and an audit log facility.

The subdirectory protection feature lets the administra-

tor designate each subdirectory on the hard disk as

Read-Only, Write-Only or Execute-Only, or any combi-

nation of R-W-X for each user individually. Thus the

administrator can set up private subdirectories for each

user or allow access to certain files and programs to

users individually. Up to four COM ports and the paral-

lel printer ports can be allowed access to or locked for

each individual user by the administrator.  The audit log

is a chronological list of user activity, including file open-

ings and closings, on the computer. It also records at-

tempted access permission violations and invalid logon

attempts. The audit log facility allows the administrator

to keep track of who is using the computer and what is

being done on it. It also serves to warn the administrator

of possible break-in attempts.

LOCKIT  I Hardware  (L1H) -

The LOCKIT  I Hardware product is a half-slot circuit

board that provides the all the same features and function-

ality as the software-only product LOCKIT  I with Adminis-

trator Option, without having to consume any of the system

RAM that LOCKIT  I with Administrator Option needs for

its device driver. In addition, LOCKIT  I Hardware can

prevent boot-up from a system-formatted diskette com-

pletely, not just lock out the hard disk if the computer is

booted from a floppy as is done by the software LOCKIT  I

products. LOCKIT  I Hardware, besides providing the high-

est level of boot protection, can also provide data encryption

and decryption via an optional DES chip. This type of data

encryption is the fastest and most secure available for personal

computers today. The hardware board also is more com-

patible with the many different types of hard disks and

software products that are available.

Page 8

SUMMARY OF LOCKIT  I FEATURES

All LOCKIT  I products provide the following features:

� Requires a password be entered to boot the computer,

� Allow user to change his/her own password,

� Passwords length up to 16 characters maximum,

� Can require a minimum password length,

� Can require passwords to be changed on a regular

basis for each user individually,

� Prevent each user from reusing passwords for up

to ten previously used passwords.

� Sounds an alarm, after a selectable number of in-

valid logon attempts,

� Freezes the computer for five minutes after a select-

able number of invalid logon attempts,

� Screen blanker and keyboard lock invoked by hot

keys, called manual Lunch-Break feature,

� Lunch-Break can also be entered from the DOS

command line or from a batch file,

� Hot keys to invoke Lunch-Break are user selectable,

� Can automatically invoke Lunch-Break after a

period of keyboard and mouse inactivity,

� User selectable time for automatic Lunch-Break,

� Automatic Lunch-Break time can be set from DOS

command line or batch file,

� Lunch-Break available in DOS and WINDOWS,

� Can lock the AUTOEXEC.BAT, CONFIG.SYS and

LOCKIT  I device driver, CLEANDSK.DRV, to

prevent inadvertent changes and deletions.

� Allows boot-up without having to use a password

for special situations.

� Special options to allow LOCKIT  I to work

properly with ill-behaved programs.

� Prevent users from using Ctrl-Break to exit the

AUTOEXEC.BAT file and other batch files,

� Prevent users from obtaining a DOS prompt,

� Access the hard disk after booting from a diskette

under password control,

� Automatically recover from most virus attacks that

destroy the hard disk partition table.

Page 9

� Only authorized person can remove LOCKIT  I.

� Up to 12 different users with individual passwords

plus a separate, higher level user called the "admin-

istrator" with his/her own password,

� Can require all users to enter both a user name

and password,

� Ability to force each user into a specific application,

� Ability to steer AUTOEXEC.BAT and other batch

files based on who is logged on,

� An environment variable can be automatically set

to the user name of the user who is logged on,

� Administrator can change any user's name and/or

password plus his/her own,

� Can pre-install selected features and passwords

prior to installation on the target computer,

� Administrator can set upper time limit users can

specify for automatic Lunch-Break,

� Administrator can lock and unlock files that users

can not unlock,

� Installation can require a password and be set up

to be done by administrator or user.

� Special high security maximum floppy boot protec-

tion option to prevent hacker tools from working.

(L1+ and L1E).

� Special relaxed MBR option to allow proper opera-

tion with other virus protection programs.

LOCKIT  I Extended Edition (L1E) provides the follow-

ing additional features:

� Administrator can assign read-only, write-only and

execute-only permissions for each subdirectory

individually by user,

� Absolute sector oriented hard disk reading and writ-

ing can be prevented,

� Diskette drives can be designated as any combina-

tion of read-only, write-only, execute-only or no

access,

� Administrator can lock out all four COM ports and

parallel ports from individual users,

Page 10

� .COM and .EXE files can be protected from reading

and writing on a per user, per subdirectory basis,

� Initial permissions for newly created subdirectories

can be specified,

� Log of invalid logon attempts kept which can be

displayed by the administrator or any user,

� Log of denied file accesses, files executed or all file

opening and closings can be kept,

� Events to be logged can be designated by the ad-

ministrator for each user individually,

LOCKIT  I products are definitely more secure than

a dead bolt lock on your front door. A knowledgeable

and determined individual may be able to circumvent

the software products, as indeed all software security

systems can be circumvented given sufficient time and

motivation. The level of protection provided is, how-

ever, sufficient for most purposes and exceeds that of

any similar program known to us.

The LOCKIT  I Hardware product, when used with

a case-locking device provides one of the highest levels

of security available on a personal computer.

Data encryption is the most secure method for

protecting your data against theft. Our optional

DesMaster(tm) products provides full Data Encryption

Standard (DES) encryption with very short encryption/

decryption times.  For more information on DesMaster(tm),

please contact us or our sales representatives.

A super fast hardware DES chip is also available

for  LOCKIT  I Hardware or our NSA/NIST listed LOC-

KIT  Professional system.

Please contact our sales department for more in-

formation on the full line of LOCKIT  and DesMaster(tm)

products.

Page 11

SYSTEM REQUIREMENTS

All LOCKIT  I products require an IBM compatible

computer with at least one floppy diskette drive and at

least one hard disk drive. The products will run on

8086, 8088, 80286, 80386, 80486 and other compat-

ible processors. All standard MDA, CGA, MCGA, Her-

cules, EGA, and VGA video adapters are supported as

well as the new SVGA adapters and laptop plasma and

LCD displays. LOCKIT  I software products rely on a

device driver that requires a certain amount of system

RAM. (6K for L1+ and 13K for L1E). If you are using an

extended memory manager with a 386 or higher type

computer and DOS 5.0 or higher, you can locate the

LOCKIT  I device driver in extended memory. LOC-

KIT  I software products also require a hard disk with

DOS as the only operating system partition. The hard

disk must use IBM-standard 512 bytes/sector for at

least the bootable partition. The products will work

with most hard disk cards. The only known exception

is when the system uses two physical hard drives and

two separate controller cards and a software driver

is required to access the second drive.

LOCKIT  I Hardware is compatible with all com-

puters that contain an available ISA or EISA slot to

accommodate the hardware board. It is compatible with

all hard disks and partitioning schemes.

If you have questions about a specific application,

please call our technical support department at (718)

667-4720 or leave a message for the SYSOP on our

BBS system (718) 667-0947.

RESTRICTIONS AND OTHER CONCERNS

LOCKIT  I may work with Windows versions 1.x or

2.x but we no longer explicitly support these versions.

We do support Windows 3.0, 3.1 and beyond.

DOS releases prior to the 3.3 release do not sup-

port hard disks larger than 32 megabytes. Version 3.3

Page 12

allows you to divide a large disk into volumes, each of

which must be 32 megabytes or less. DOS versions

3.31 and beyond allow large hard drives to be used as

a single volume.  Many products are available which

correct the deficiencies of the earlier versions of DOS.

LOCKIT  I software products work correctly with vir-

tually all of these products. A few, however, force your

disk to use a nonstandard sector size. LOCKIT  I soft-

ware will detect this condition and tell you that it will

not install because of the presence of a nonstandard

sector size. This will not affect the installation of LOC-

KIT  I Hardware.

Norton Cache version 6.0 contains a very serious

bug which may destroy all of the data on your hard

disk if you have both Norton Cache Intelliwrites and

LOCKIT  I Maximum Floppy Boot Protection enabled

at the same time. (It may also cause similar problems

when used with other disk related software products.)

This bug was fixed in Norton Cache version 6.01 which

is distributed free to all registered version 6.0 users.

We strongly recommend that you upgrade to version

6.01 on all your machines whether or not you plan to

use LOCKIT  I on them.

Your hard drive(s) must not contain partitions be-

longing to operating systems other than DOS. You

should not attempt to use FDISK or any other parti-

tioning software while a LOCKIT  I software product

is installed.

If you are using DOS 3.31 or greater and use the

shell statement in your CONFIG.SYS file it should look

like:

SHELL=C:\DOS\COMMAND.COM  C:\DOS  /P

or

SHELL=C:\DOS\COMMAND.COM C:\DOS  /P /E:XXX

Page 13

if you want to keep COMMAND.COM in the \DOS direc-

tory. Most users forget to include the C:\DOS after the

COMMAND.COM. Omitting the C:\DOS may cause the

computer to hang with LOCKIT  I installed.

Use the latter form to change the environment size.

XXX represents the number of memory bytes that you

want to reserve for the storage of environment vari-

ables.

The rest of this manual will refer to the three differ-

ent LOCKIT  I products by their order codes for the

sake of brevity. It is important to note which product

you are installing and using by the three character

order code.

Important Note

Certain features described in this manual

may not apply to the LOCKIT  I product that

you  have purchased. It is very important that

you know which product you own so that you

don't expect it to do something it was not de-

signed to do. We have purposely split the LOC-

KIT  I family into five different products to bet-

ter meet the different sets of requirements of

our customers. This not only saves you money

but also allows the LOCKIT  I product to bet-

ter fit the needs of the situation in which it will

be used.

Two other LOCKIT  I products, LOC-

KIT  I Single Password and LOCKIT  I for Lap-

tops are not included in this manual. For more

information on these two products, please call

our sales department. Features will have the

LOCKIT  I order codes marked in parenthesis

to indicate to which products they apply.

Page 14

DISCLAIMER OF    WARRANTY

LOCKIT  I  products, and associated software and this

documentation  are  sold "AS-IS" in regards to perfor-

mance or merchantability. The seller's salespersons and/

or this or other documentation provided by Security

Microsystems, Inc. may have made statements about this

product. Any such statements do not constitute warran-

ties and shall not be relied on  by the buyer in deciding

whether to purchase and/or use this product. LOCKIT  I

products and associated software and this documentation

are sold without any expressed or implied warranties

whatsoever. Because of the diversity of conditions and

hardware under which this program may be used, no

warranty of fitness for a particular purpose is offered. The

user is advised to backup all data on the hard disks before

trying it, and to thoroughly test it before relying on it. The

user must assume the entire risk of using the program.

Any liability of seller or manufacturer will be limited

exclusively to product replacement or refund of the pur-

chase price.

YOUR LOCKIT  I  LICENSE

After you have read and agree to the disclaimer of

warranty, you are licensed to install and use LOCKIT  I on

the ONE computer for which you have paid the license fee.

You are permitted to remove LOCKIT  I from one com-

puter and install it on another so long as LOCKIT  I does

not reside on two or more computers at the same time.

Backup copying of LOCKIT  I for archival reasons is per-

mitted. LOCKIT  I software products are fully copyrighted

software products and Johnson Computer Systems, Inc.

through their agent Security Microsystems, Inc. reserves all

rights not specifically granted in this license.

Security Microsystems, Inc. will seek prosecution and

damages, up to the maximum extent allowed by law, of any

person or company intentionally attempting to deprive Se-

curity Microsystems, Inc. of it's license fees by willfully

making and distributing illegal copies of it's products.

Page 15

(This page intentionally left blank)

Page 16

USING LOCKIT  I  MENUS

Whenever the LOCKIT.EXE program is run, and af-

ter entering your password, when required, a menu

will appear on the screen. Each menu contains the list

of functions which you may perform when that menu

is displayed. You may select any item from a menu

simply by pressing the letter displayed in front of that

item, or by using the up and/or down cursor control

keys to position the light-bar (inverse video) over the

item and pressing Enter. Additional information about

a menu item may be displayed by moving th light bar

to the item and pressing the ? key.  Letters and the ?

may be typed in either upper or lower case. Either the

Esc or the E keys may be used to exit any menu. The

menus shown in this manual may differ slightly from

those displayed on the screen due to page/screen size

limitations.

LOCKIT  I PASSWORD AND USER NAME

The LOCKIT  I password consists of zero to six-

teen characters (keystrokes). LOCKIT  I does not rec-

ognize a difference between upper and lower case let-

ters. The minimum password length may be set to any

value from zero to sixteen characters. All LOCKIT  I

products require at least a password to be entered at

boot-up (except for auto-boot discussed later). They

can also be setup to require both a user name and

password at boot-up time, to run LOCKIT.EXE, and to

exit Lunch-Break. A user name can be up to 7 charac-

ters long. After LOCKIT  I is installed, each time the

computer is booted you will have to begin by entering

your password and pressing the Enter key. If the entry

is correct, the computer will begin to boot, otherwise

you will hear one long tone and the system will wait

for you to start the process over. The Backspace key

may be used to correct errors in the normal manner.

The Esc key may be used to terminate the present

attempt and to start all over. The Enter key signifies

Page 17

the end of both the user name and the password. After

entering a password, you may hear a sequence of tones

alternating between two different pitches. This is called

an alarm and it occurs when a number of consecutive

incorrect password entries have occurred. The number

of consecutive incorrect entries required to trigger the

alarm is determined by a menu item in the "Set lim-

its" menu, available from the administrator's main

menu. If the number of consecutive errors exceeds

another limit, also chosen by the administrator, the

computer will sound the alarm and then lock for five

minutes following each count of incorrect entries. Turn-

ing the computer off will not influence the count of

incorrect entries. If the computer is turned off during a

five minute lockup, the five minute period will be re-

peated from the beginning when the computer is next

rebooted. For more information on the user names and

passwords, see the section on "Changing passwords".

For more information on the alarm function and the

lookup feature, see the "Setting LOCKIT  I limits"

section.

Page 18

Chapter 2 -  Installation

BEFORE INSTALLING LOCKIT  I

We strongly recommend that you make a complete

backup of all the data on the hard disk(s) so that, in

the unlikely event that something goes wrong during

installation, you will be able to restore the contents of

the hard disk(s). If no third party backup utilities such

as Fastback or PC Tools Backup are available then you

should use BACKUP.COM, that is supplied with DOS,

to copy all the data from your hard disk to floppies. If a

tape drive is available then this is obviously the pre-

ferred alternative.

The LOCKIT.EXE file on the original diskette should

not, and in some cases (L1E) cannot, be used to install

LOCKIT  I.

We suggest that a working copy of the original LOC-

KIT  I diskette be made prior to running the installation

procedure. While the DOS DISKCOPY and DISKCOMP

commands can be used to make the working copy, it

has been our experience that formatting a blank dis-

kette and using the DOS command:

COPY  A:*.*  B: /V

to copy all the files from the original LOCKIT  I diskette

to the working copy is the preferred method.

COPY-PROTECTED VERSIONS OF LOCKIT  I

LOCKIT  I software products (L1+ and L1E) are sup-

plied in both copy-protected and non-copy-protected ver-

sions. If your original disk states that it is "Copy-Pro-

tected" then you must follow the instructions included

in the separate pages that accompany this manual titled

"Installing Copy-Protected Versions of LOCKIT  I". If

your original diskette is not marked "Copy-Protected"

then please proceed with the instructions in the follow-

ing section.

Page 19

PRE-INSTALLATION SETUP

The pre-installation setup allows the administrator

to modify a copy of the LOCKIT  I main program,

LOCKIT.EXE, so that when LOCKIT  I is installed, the

features and options the administrator has selected

will automatically be in effect. This is most useful

when the administrator must install LOCKIT  I on

several computers and it is required that the product features

and administrator password be the same on each com-

puter. Of course the LOCKIT  I license requires that

you have purchased a copy of LOCKIT  I for each com-

puter that you are installing LOCKIT  I on. The

pre-installation setup is only required for LOCKIT  I

Extended Edition (L1E) however if the pre-installation

setup is not done on LOCKIT  I with Administrator

Option and LOCKIT  I Hardware they will revert to a

single password product with features similar to LOC-

KIT  I Single Password. The pre-installation setup al-

lows the administrator to define the administrator's

password. Defining the administrator's password al-

lows the additional user passwords to be defined ei-

ther at this time or after the product is installed.

IMPORTANT

The Pre-Installation setup process described in this

section makes no changes to the computer used to

perform it. It modifies only the LOCKIT.EXE file on

the diskette or hard disk on which it resides.

To do a pre-install setup on LOCKIT  I, place a

diskette containing a copy of the LOCKIT.EXE file (not

your original please) in drive A: and type:

A:LOCKIT     /P

and press the Enter key. If you wish to do a pre-

installation on a copy of LOCKIT.EXE located on your

hard disk or another diskette drive you can type:

Page 20

[d:][path]LOCKIT   /P[=[d:][path]]

where:

d: is the optional drive name

path is the optional path name

For example, if you wanted to do a pre-installation

setup on a copy of LOCKIT.EXE that resided on your

hard drive "C:" in a subdirectory called \SM you would type:

C:\SM\LOCKIT   /P=C:\SM

If a pre-install has not already been done on this

copy of LOCKIT.EXE, the screen shown in Fig. 1 will

be displayed. Read the screen and then press the Y

or N  key.

��������������������������������������������������������������ͻ

�    LOCKIT I Hard Disk Protection System  -  Version 5.2      �

�      (C)Copyright 1992 by Security Microsystems, Inc.        �

�                                                              �

�             LOCKIT I Pre-Installation Setup                  �

�                                                              �

�  Preinstallation allows you to modify the LOCKIT.EXE file so �

�  that it contains the passwords and other options you choose.�

�  Whenever the modified copy is used to install LOCKIT I on a �

�  computer, your choices will be copied to the computer's hard�

�  disk and they will automatically be in effect.              �

�                                                              �

�  The copy of LOCKIT I that will be modified is:              �

�                                                              �

�                   C:\SM\LOCKIT.EXE                           �

�                                                              �

�  If you wish to modify a different copy, end this program    �

�  and restart it with the command: LOCKIT /P=Path where       �

�  Path specifies the drive/directory containing the desired   �

�  copy of LOCKIT.EXE                                          �

�                                                              �

�  Do you wish to modify the above file (Please press Y or N)  �

�                                                              �

�      Please do NOT modify your original LOCKIT I diskette.   �

��������������������������������������������������������������ͼ

Fig. 1 - Pre-installation Notice

If a pre-install has already been done on this copy of

LOCKIT.EXE, the screen shown in  Fig. 2  will be dis-

played.

Page 21

�������������������������������������������������������������ͻ

�                                                             �

�                                                             �

�                                                             �

�                                                             �

� An administrator password has already been assigned to this �

� file. You must provide that password to make additional     �

� changes.                                                    �

�                                                             �

�                                                             �

�                                                             �

�                                                             �

�        Do you wish to continue?  (Please enter Y or N)      �

�                                                             �

�                                                             �

�                                                             �

�������������������������������������������������������������ͼ

Fig. 2 - Pre-installation Password Request

In this case you must enter your password in order to

get to the screen shown in Fig. 3.

��������������������������������������������������������������ͻ

�            LOCKIT I Pre-Installation Setup Menu              �

�                                                              �

�   Please press the LETTER in front of the option you wish.   �

�                                                              �

�           E.  END this program.                              �

�                                                              �

�           H.  HOW to use this menu.                          �

�                                                              �

�           P.  Define original PASSWORDS and names.           �

�                                                              �

�           O.  Select OPTIONS.                                �

�                                                              �

�           S.  SET limits.                                    �

�                                                              �

�           K. Define HOT Key                                  �

�                                                              �

�           L.  LOCK files during installation.                �

�                                                              �

�           W.  Choose WHO will install LOCKIT I.              �

�                                                              �

��������������������������������������������������������������ͼ

Fig. 3 - Pre-Installation Setup Menu

Page 22

THE PRE-INSTALLATION SETUP MENU

The Pre-Installation setup menu, shown in figure

3, contains items that allow you to setup various LOC-

KIT  I options and features prior to installing LOC-

KIT  I on your computer. The single most important

thing that must be done during pre-installation is the

selection of an administrator's password. An adminis-

trator password must be defined prior to installation

or you will not be able to install LOCKIT  I Extended

Edition. It is also important, but not required, for LOC-

KIT  I with Administrator Option and LOCKIT  I Hard-

ware, however, if you do not define an administrator's

password for these products during the pre-installa-

tion, they will revert to a single password product with

no administrator features. Once the administrator's

password is selected and recorded in the LOCKIT.EXE

file, this password will be required to remove LOCKIT

I and to change any of the options and features that

cannot be changed by a user. Below is a description of

each of the items in the pre-installation setup menu.

E.  END this Program -

Pressing E or ESC will exit the pre-installation menu

and return you to DOS. If any changes were made, they

will be automatically recorded when you exit the

pre-installation menu.

H.  HOW to use this menu -

The H key provides a screen full of basic informa-

tion about how to use the menu you are currently in.

You may also get specific help for other menu items by

Page 23

selecting the menu item you are interested in and

pressing the question mark key.

P.  Define original PASSWORDS and names  -

To define an original administrator name/password

(or names/passwords for other users), select the P

option from the menu. The exact procedure and the

screens  you will see during name/password definition

are discussed in the section "Change passwords" on

page 37. Please consult that section now for further

information.

O.  Select OPTIONS  -

The O menu item brings up another menu that

allows you to determine the way LOCKIT  I will oper-

ate once it is installed. There are twelve LOCKIT  I

options that are available for selection or not. These

options that you select at this time may also be se-

lected and/or deselected by you, as administrator, af-

ter installation. However, any changes made in the

pre-installation will be reinstated upon subsequent in-

stallations.

Page 24

For example, if you choose to display asterisks

while entering your password, this option will be in

effect when you first install LOCKIT  I on your com-

puter. If you so desire, you can deselect this option

once LOCKIT  I is installed. If you deselect it, from

then on, no asterisks will be displayed when you enter

your password. If you remove LOCKIT  I from your

computer and later reinstall it, the asterisks will again

be displayed because this option was stored as the

default and written into the LOCKIT.EXE file. For a

detailed discussion of all the LOCKIT  I options please

refer to the section "Selecting LOCKIT  I Options" on

page 57.

S.  SET Limits  -

The S menu item brings up another menu that

allows you to set various program parameters and

bounds on certain user selections such as minimum

password length, number of invalid logons before alarm,

number of invalid logons before lockup and maximum

keyboard/mouse idle time before Lunch-Break is au-

tomatically invoked, etc.  For detailed information on

LOCKIT  I limits see "Setting LOCKIT  I Limits" on

page 62.

K.  Define  Hot KEY   -

The K menu item allows you to define the combi-

nation of keys that, when pressed will cause the com-

puter to enter the Lunch-Break locked condition. Any

combination of the Alt, Ctrl, Left Shift and Right

Shift keys may be selected, however, if you intend to

use the Lunch-Break feature from within WINDOWS, you

should not include the Alt key in your selection.

The default hot key combination is the Left Shift and

Right Shift keys pressed and released simultaneously.

For more information on  the LOCKIT  I  Hot Key see

"Defining a new hot KEY combination" on page 45.

Page 25

L.  LOCK files during installation -

The L menu item will cause the CONFIG.SYS,

AUTOEXEC.BAT, and CLEANDSK.DRV (the LOCKIT  I

device driver) files to be locked during installation.

Locked files cannot be altered by anyone other than

the administrator. A user cannot delete them or change

their names, contents, or attributes. For additional in-

formation on locked files see "Locking and Unlocking

LOCKIT  I Related Files" on pages 43 and 44.

W.  Choose WHO will install LOCKIT  I -

The W menu item allows you to choose whether

the administrator's or user's menu will be displayed

after LOCKIT  I is installed. If the administrator's menu

is displayed, the person who installs LOCKIT  I will

be able to change all the user names, passwords, op-

tions, and limits. Thus he/she will be considered to be

the new administrator and have all the administrator's

rights and privileges. If the user's menu is chosen

to be displayed, the installer may change only the User 1

password and perform other functions available to all

users. When you press W the question in figure 4 will

appear.

Page 26

�������������������������������������������������������������ͻ

�                                                             �               �                                                             �

�                                                             �               �                                                             �

�                                                             �               �                                                             �

�                                                             �

�                                                             �

� Do you wish to assume that the Administrator, rather than   �

� User 1, will install LOCKIT I ? (Please press Y or N)       �

�                                                             �

�                                                             �

�                                                             �

�                                                             �

� The administrator is currently assumed to be the installer. �

�                                                             �

�                                                             �

�                                                             �

�������������������������������������������������������������ͼ

Fig. 4 - Installer select screen

Answer the question displayed with a Y to select the

Administrator or N to select User 1. If you choose to have

the installer be the administrator you can also choose to

require additional information as shown in figure 5 below.

�������������������������������������������������������������ͻ

�                                                             �                                                                         �

�                                                             �                                                                         �

�                                                             �                                                                         �

� It will be assumed that the administrator is the installer. �

�                                                             �

�                                                             �

�                                                             �

�  Please press:                                              �

�                                                             �

� 1. To require that the admin password be entered to install �

� 2. To require that the installer define a new admin password�

� 3. To allow installer to choose between options 1 and 2 or  �

� 4. If you want installation to occur without any password   �

�    being entered.                                           �

�                                                             �

�  Option 1 -3 will ensure that LOCKIT I cannot be installed  �

�  unless someone knows the administrator password.           �

�                                                             �

�������������������������������������������������������������ͼ

Fig. 5 - Administrator as Installer selections

Page 27

We recommend that you select the administrator

as installer and also select that a password be re-

quired for installation. This will ensure that you

can never install LOCKIT  I on a computer without know-

ing the administrator's password, which is required to

remove it. If you have a specific situation that requires

you to select the user as installer then you may select

this option, however, be aware that unless the

administrator's password for this copy of LOCKIT.EXE

is known, it may be possible to install LOCKIT  I on a

computer and, since the administrator's password is

not known, it will be impossible to remove it.

Once you have completed the pre-installation set

up and exited to record your selections, you are ready

to actually install LOCKIT  I on your computer.

Page 28

INSTALLING   LOCKIT  I

IMPORTANT

While we have a very high degree of confi-

dence in LOCKIT  I, it is impossible to guaran-

tee that any software program will work on all

of the millions of differently configured com-

puters on which it may be used. For this rea-

son we ask that you ensure that you have a

current backup of your hard disk before in-

stalling LOCKIT  I. We do not anticipate that

you will experience any problems in installing

and using LOCKIT  I, but we do want you to

be able to recover in the unlikely event that a

problem does occur.

If you have an earlier version of LOCKIT  I in-

stalled on your computer, please remove it, at this

time, by using the destall or remove procedure for that

particular version of LOCKIT I.

To install LOCKIT  I, you will need to have the

file LOCKIT.EXE on a diskette in drive A: or on your

hard disk.

To install simply type:

LOCKIT

making sure to conclude by pressing the Enter key.

You may need to type the drive letter if the drive

containing LOCKIT.EXE is not the default drive, for

example:

A:\LOCKIT

or

C:\SM\LOCKIT

Page 29

FOR LOCKIT  I  Hardware (L1H) ONLY

There are two parts to installing LOCKIT

I Hardware, installation of the circuit

board into the computer's expansion slot

and installation of the security software

into the circuit board's ROM. Before inst-

alling the circuit card a starting address

for the board's ROM must be determined.

Running the LOCKIT.EXE program, without the

board installed in the computer, will dis-

play a screen indicating the proper setting

of the dipswitch on the L1H board. If a

message appears indicating that the ex-

pansion ROM space is full then please

turn to the problem and answer section in

the back of this manual for further in-

structions. After setting the dipswitch, you

must remove the paper tab from between the

battery and the battery clip. Once this tab

has been removed, you should not place the

board on a metal surface as this may cause

the battery to be shorted out. Install the

board by first turning off power to the

computer, removing the case cover of the

computer system unit and locating an empty

expansion slot. Use a screwdriver to remove

the blank slot cover and replace it with the

LOCKIT  I board. Make sure the LOCKIT  I board

is seated squarely into the expansion bus

connector. You may use the board in either

an 8 bit or a 16 bit expansion slot. It will

also work in an EISA slot. It is not intended

to be used, and will not work, in a microchannel

bus computer such as the IBM PS/2 50, 60, 70, 80,

etc. After installing the board, power up the

computer to make sure it is still functioning,

and run the LOCKIT.EXE program again. This time

you should see the menu in figure 6.

Proceed with installation as with the LOCKIT

I Software products. This is described in the

following section.

Page 30

��������������������������������������������������������������ͻ

�     LOCKIT I Hard Disk Protection System  -  Version 5.2     �

�      (C)Copyright 1993 by Security Microsystems, Inc.        �

�                                                              �

�                                                              �

�                                                              �

�          LOCKIT I is not installed on this computer.         �

�                                                              �

�                                                              �

�   Please press the LETTER in front of the option you wish.   �

�                                                              �

�                  E.  END this program.                       �

�                                                              �

�                  H.  HOW to use this menu.                   �

�                                                              �

�                  I.  INSTALL LOCKIT I.                       �

�                                                              �

�                                                              �

��������������������������������������������������������������ͼ

Fig. 6 - LOCKIT I Installation Menu

If LOCKIT  I is not installed, (but the board is

installed for L1H) the menu shown in figure 6 will

be displayed. Simply select the INSTALL option by

pressing I or using the cursor keys to move the

lighted bar down to the I option and pressing the

Enter key.

A screen may appear reminding you that an

administrator's password has already been defined

for this copy of LOCKIT  I. If you don't remember

or otherwise don't know the password you defined,

please do not proceed with the installation. If you

do proceed, you may not be able to access your

hard disk. If you have recently performed the pre-

install and are sure you know the administrator's

password and name then press "Y" to continue. The

administrator's password may be required to install

LOCKIT  I. If it has been setup this way during the

pre-install then it will not be possible to install

LOCKIT I unless you know the administrator's pass-

word.                                                                     Page 31

Page 31

Please Note

If LOCKIT  I is already installed on your computer,

you will just be requested to enter your password

(or name and password if so configured) when you

type LOCKIT.

After LOCKIT  I installation has been completed

a screen giving important information will be displayed.

Please read it carefully in its entirety. After reading

the screen, press any key and a main menu will be dis-

played.

If LOCKIT  I has been setup (during the pre-

install) assuming that the administrator is installing it,

the administrator's main menu shown in figure 7A for

L1+, figure 7B for L1E, or figure 7C for L1H, will appear

next. Thus the person installing will have the ability to

change any of the user names and passwords as well as his/

her own. All of the LOCKIT  I features, options and limits

will also be available for change at this time.

If LOCKIT  I has been setup assuming that user 1 is

installing, then the user's main menu shown in figure 8

will appear and only the password (and possibly the name)

for user 1 will be available for change. Certain user

selectable LOCKIT  I features and limits will also be

available for modification.

IMPORTANT

If any error messages appear during the instal-

lation process, please write them down exactly

so that you can convey them to the tech. support

person at Security Microsystems.

Page 32

�����������������������������������������������������������ͻ

�             LOCKIT I Administrator's Main Menu            �

�  Please press the LETTER in front of the option you wish. �

�                                                           �

�         E.  END this program.                             �

�         H.  HOW to use this menu.                         �

�                                                           �

�                                                           �

�         P.  Change PASSWORD.                              �

�         O.  Select OPTIONS.                               �

�         S.  SET Limits.                                   �

�                                                           �

�         L.  LOCK   LOCKIT I related files.                �

�         U.  UNLOCK LOCKIT I related files.                �

�         A.  ACCESS fixed disk after diskette boot.        �

�                                                           �

�         R.  REMOVE LOCKIT I from this computer.           �

�         K.  Define new hot KEY combination.               �

�         T.  Set keyboard IDLE time.                       �

�����������������������������������������������������������ͼ

Fig. 7A - Administrator's Main Menu (L1+)

�����������������������������������������������������������ͻ

�             LOCKIT I Administrator's Main Menu            �

�  Please press the LETTER in front of the option you wish. �

�                                                           �

�         E.  END this program.                             �

�         H.  HOW to use this menu.                         �

�                                                           �

�                                                           �

�         P.  Change PASSWORD.                              �

�         O.  Select OPTIONS.                               �

�         S.  SET Limits.                                   �

�                                                           �

�         L.  LOCK   LOCKIT I related files.                �

�         U.  UNLOCK LOCKIT I related files.                �

�         A.  ACCESS fixed disk after diskette boot.        �

�                                                           �

�         R.  REMOVE LOCKIT I from this computer.           �

�         K.  Define new hot KEY combination.               �

�         T.  Set keyboard IDLE time.                       �

�                                                           �

�         D.  Control DIRECTORY access by user.             �

�         I.  Control I/O PORT by user.                     �

�         F.  Select FILE accesses to be logged.            �

�����������������������������������������������������������ͼ

Fig. 7B - Administrator's Main Menu (L1E)

The installation process is completed by selecting any

desired items from the menu displayed.When all desired

selections, if any, have been made, select the "E.

Page 33

End this program" option to return to DOS. Protection is

now in effect. The Lunch-Break feature will not be avail-

able until the computer is rebooted.

�����������������������������������������������������������ͻ

�             LOCKIT I Administrator's Main Menu            �

�  Please press the LETTER in front of the option you wish. �

�                                                           �

�         E.  END this program.                             �

�         H.  HOW to use this menu.                         �

�                                                           �

�                                                           �

�         P.  Change PASSWORD.                              �

�         O.  Select OPTIONS.                               �

�         S.  SET Limits.                                   �

�                                                           �

�         L.  LOCK   LOCKIT I related files.                �

�         U.  UNLOCK LOCKIT I related files.                �

�         A.  ALLOW Boot-up from A: Drive.                  �

�                                                           �

�         R.  REMOVE LOCKIT I from this computer.           �

�         K.  Define new hot KEY combination.               �

�         T.  Set keyboard IDLE time.                       �

�                                                           �

�����������������������������������������������������������ͼ

Fig. 7C - Administrator's Main Menu (L1H)

�����������������������������������������������������������ͻ

�                 LOCKIT I User's Main Menu                 �

�                                                           �

�  Please press the LETTER in front of the option you wish. �

�                                                           �

�         E.  END this program.                             �

�                                                           �

�         H.  HOW to use this menu.                         �

�                                                           �

�                                                           �

�                                                           �

�         P.  Change PASSWORD.                              �

�                                                           �

�         K.  Define new hot KEY combination.               �

�                                                           �

�         I.  Set keyboard IDLE time.                       �

�                                                           �

�����������������������������������������������������������ͼ

Fig. 8 - User's Main Menu

Page 34

Chapter 3 - After  Installation

USING THE LOCKIT  I  MAIN  PROGRAM

To access the LOCKIT  I main menu simply type:

LOCKIT

making sure to conclude by pressing the Enter key. You

may need to type the drive letter if the drive containing

LOCKIT.EXE is not the default drive, for example:

A:\LOCKIT

Please note

Installing LOCKIT  I does not copy the main

LOCKIT  I program file called "LOCKIT.EXE" onto

your hard disk. If you intend to use the LOCKIT

I program to change passwords regularly or for

convenience reasons you should copy LOCKIT.EXE

from your working copy diskette into the root

or any subdirectory  for which a path exists.

To copy the LOCKIT.EXE to your hard disk, place

your LOCKIT  I working copy diskette in the A:

drive and type:

COPY  A:\LOCKIT.EXE  C:\

and press the Enter key. This will copy the

LOCKIT.EXE file onto your hard disk in the

root directory. Alternatively, you may decide

to copy LOCKIT.EXE into a subdirectory that

exists in your "path" statement. For example

the command:

COPY A:\LOCKIT.EXE  C:\DOS\LOCKIT.EXE

copies LOCKIT.EXE into the \DOS subdirectory.

Page 35

When you run the LOCKIT.EXE program on a com-

puter that LOCKIT  I has been installed on, you

will be prompted for a password. If the adminis-

trator's password is entered, the administrator's

main menu, shown in figures 7A, 7B or 7C will be

displayed. If a user's password is entered, the

user's menu shown in figure 8 will be displayed.

USING THE ADMINISTRATOR'S MAIN MENU

For general information on using LOCKIT I menus,

see the section "Using LOCKIT  I menus" on page 17.

The administrator's main menu contains 12 to 15 items

depending on which LOCKIT I program you have pur-

chased. Below is a listing of the items in the

administrator's main menu and a description of what

they do.

E.  END this program -

You may return to DOS from the administrator's

main menu by selecting the E option or by pressing

the Esc key.

H.  HOW to use this Menu  -

Pressing H will bring up a "How to use this menu"

screen for assistance.

Page 36

P.  Change PASSWORDS

Passwords and optionally, user names, may by changed

by selecting the P. Change PASSWORD option from the

administrator's main menu. This menu item also allows

the administrator to select password aging, password

reuse and the user name and password for each user in-

dividually. The person who is to be allowed to change

an expired password and the characters that are to be

displayed while users are defining their passwords can

also be selected from this menu item. When this menu

item is selected, the screen in figure 9 will be dis-

played. The administrator has the ability to change

all 12 user names and passwords as well as his/her own

name and password. To change a name or password, the

administrator must first select which name and password

he/she wishes to change by entering a number from 0 to

12. To change password aging or reuse for an individual

or all users the letters "E" or "R" can be selected.

To change the characters displayed during pass-

Page 37

word definition, "D is selected and pressing "W" allows

the selection of who can change expired passwords.

Pressing the Enter key returns the administrator back

to the main menu.

������������������������������������������������������������ͻ

�           LOCKIT Name and Password Definition              �

�                                                            �

�  Please press: 0 - 12 to change a password and/or name.    �

�        E to set password lifetimes after which they expire �

�        W to choose WHO can change expired passwords.       �

�        R to change the number of different passwords Req'd �

�        D to choose the char DISPLAYED when defining a PW   �

�    ENTER to return to the main menu.                       �

�                                                            �

�    Allowed to change expired PWs:  Administrator, User     �

�    Displayed during PW definition: Actual characters       �

�                                                            �

� User   Name   Days  Recycle   User   Name   Days  Recycle  �

�  0.    Admin   0       0       7.   User 7   0       0     �

�  1.    User 1  0       0       8.   User 8   0       0     �

�  2.    User 2  0       0       9.   User 9   0       0     �

�  3.    User 3  0       0      10.   User 10  0       0     �

�  4.    User 4  0       0      11.   User 11  0       0     �

�  5.    User 5  0       0      12.   User 12  0       0     �

�  6.    User 6  0       0                                   �

�             Please enter your selection:                   �

������������������������������������������������������������ͼ

Fig. 9 - Administrator's Name/Password Selection Screen

Selecting the letters E, W, R or D allows the ad-

ministrator to select other password related features

described below.

E.  Set password lifetimes, after which they EXPIRE

System security can be enhanced by requiring

users to change their passwords periodically. This important

security feature, also known as "password aging", can

allow the administrator to specify the number of days a

newly defined password will remain valid, for each user

individually. To set this limit, press the E key. A

screen will appear asking you to select a user number

or to press "A" for all users. If you want to set the

password aging for a single user, press the number of

that user (1-12). If you want to set the password aging

to be the same for all users, press "A". The next

Page 38

screen will ask you to choose the desired password

lifetime in days. Enter a number from 0 to 185. A

value of zero means that the password will never ex-

pire. If a number greater than zero is selected, the

user's password will become invalid after this number

of days. The administrator can also determine whether

or not the user will be able to define a new password

once it expires. LOCKIT  I can be setup so that only

the administrator can define new passwords.

Important Note

If your computer is an XT class machine, (i.e.

one that requires a command in the AUTOEXEC.BAT

to set the time and date from the real-time

clock) and you choose to use password aging, you

must place the command:

LOCKIT   /A

at or near the beginning of your AUTOEXEC.BAT file,

but after any command that calls up the time and

date from the battery-operated real-time clock.

Also, the LOCKIT.EXE file must be copied into the

root directory of your hard disk or into a subdir-

ectory which is included in the PATH statement.

If you have an AT class computer (generally

an 80286, 80386, and above CPU), you will not

have to place the above line in your AUTOEXEC.BAT

file.

A user may try to prevent password expiration by

setting the computer's clock/calendar back. For this

reason, all passwords are marked as expired whenever

the clock regresses by four or more hours.

When password aging is selected, passwords de-

fined during pre-installation expire the first time

they are used.

Page 39

W.  Choose WHO can change expired passwords

Any time a user enters an expired password, he/

she will be required to change it and may or may not

be given an opportunity to do so depending on the

setting made in  this "W" selection. When "W" is

pressed,  the program will ask you to select whether

you want both the administrator and the user, or just

the administrator to be able to change an expired pass-

word. If you choose to allow only the administrator to

change expired passwords, then users will be locked out

of the computer when their passwords expire. Users will

still be able to change their own passwords before the

passwords expire, if the administrator gives them access

to the LOCKIT.EXE program, however, changing their pass-

words will not postpone the expiration date set by the

administrator.

R.  Set Number of Passwords REQUIRED before reuse

Password aging is ineffective if a user is allowed to

change a password to the same password he or she is cur-

rently using. As the administrator, you can require each

user to select several different passwords before being

allowed to reuse an earlier one. You can specify any

number, up to ten, for the number of different passwords

that must be defined before the first one can be reused.

To set this limit, press the "R" key. A screen will appear

asking you to select a user number or to press. If you

want to set the number of passwords required before reuse

for a single user, press the number of that user (1-12).

If you want to set the number of passwords required before

reuse to be the same for all users, press "A". The program

will then ask you to select the number of passwords that

must be defined prior to reusing a password. Enter a number

from 0 to 10. Entering zero means that the user can define

the current password as the new password.

Page 40

D.  Choose the character DISPLAYED defining passwords

When a user changes his/her password, the characters

that are echoed to the screen, while the password is

being entered, can be selected by the administrator.

The three choices are: asterisks, actual characters

or no characters. The choice of which character to

display is a trade-off between security and ease-of-use.

When users are allowed to see the characters they

are entering, there is little chance of making a

mistake, however, displaying the actual characters

on-screen allows someone looking over the user's

shoulder to obtain the user's new password. Using

asterisks is a compromise. The user can still see how

many characters he/she is entering, however, the

actual characters remain hidden. When you press the

"D" to select characters displayed, a menu of three

choices will be presented. Select "C" to have the

actual characters entered appear on the screen,

"A" to have asterisks appear on the screen, or

"N" to have nothing appear on the screen as passwords

are being defined. Normally LOCKIT  I will require a

user to enter a password twice to ensure that the

password entered is the same as the one that the user

believes he/she has entered. If the option of displaying

no characters on the screen during password definition

has been chosen, LOCKIT  I will require that the pass-

word be entered three times to make sure that the

password entered is correct.

Change Name/Password Screen -

The administrator can change both the user name and

password for each user and his/her own name and pass-

word by pressing a number from 0 to 12. This will cause

the screen in figure 10 to appear. An important point to

remember is that even though the administrator may have

chosen not to use both user names and passwords, the

user's name is the first item to be changed, when a number

from 0 to 12 is pressed. After selecting a new name,

entering it twice (or thrice if the administrator has

chosen no password characters to be displayed on screen)

or just pressing Enter to avoid changing the user name,

the user's password can be defined in a similar manner.

Page 41

��������������������������������������������������������������ͻ

�                                                              �

�                   LOCKIT I Name Definition                   �

�                                                              �

�                                                              �

� The current name for this user is:  Admin                    �

�                                                              �

� Press return to retain this name, or enter a new name:  Tiny �

�                                                              �

� Please enter the new name again to be sure its correct: Tiny �

� You may be required to enter this name to gain access.       �

�                                                              �

��������������������������������������������������������������ͼ

Fig. 10 - Name Definition Screen

After changing or not changing the name, the password

definition screen shown in figure 11 will appear. If you

do not wish to change the password, press the Esc key.

��������������������������������������������������������������ͻ

�                                                              �

�                 Password Definition Screen                   �

�                                                              �

�                                                              �

�  Passwords may be one to sixteen key strokes, and include    �

�  letters, numbers, and the keys:  space - =  [ ] ; , .       �

�                                                              �

�       Case is not significant for any password keys.         �

�                  Three special keys are:                     �

�     Backspace - Used to correct an error in the normal way.  �

�     Enter     - Means, "Password entry is complete."         �

�     Escape    - Means, "I don't want to enter a password."   �

�                                                              �

�                                                              �

�  Please enter new password and press return:  SECRET-STUFF   �

�                                                              �

�  Your new password is defined.  Whenever LOCKIT I asks for   �

�  your password, type it in and then press enter.  You MUST   �

�  be able to enter it correctly.  We suggest you use your     �

�  print screen key to make a hard copy of this screen and     �

�  keep it in a safe place.                                    �

�                                                              �

�             Please press any key to continue.                �

�                                                              �

��������������������������������������������������������������ͼ

Fig. 11 - Password Definition Screen

Page 42

The example in figure 11 shows that the user has

selected "SECRET-STUFF" as the new password. After

entering the new password you will be asked to enter

it a second time just to be certain it has been entered

correctly. The lower portion of the screen shown in

figure 11 is then displayed and the new password is

stored. The password is stored in one-way encrypted

form. Whenever you enter a password to gain access, it

is encrypted and then compared to the stored value.

There is no way to decrypt passwords. It is, therefore,

extremely important for the administrator to remember

his/her password. If the password is forgotten and your

organization has not purchased an Extended Service

Contract, it will be necessary to perform a low level

format of your hard disk to remove LOCKIT  I.  If there

were another way to get in, the security provided by

LOCKIT  I would be seriously compromised.

O.   Select  OPTIONS  -

Choosing the O menu item displays a menu of

other LOCKIT  I option items to select. These items

are described in the section called "Selecting LOCKIT

I Options" on page 57.

S.  SET Limits -

Choosing the S menu item displays a menu of other

LOCKIT  I limit items to set. These items are de-

scribed in the section called "Setting LOCKIT  I

Limits" on page 62.

L.  LOCK  LOCKIT  I  related files -

This menu item locks (sets the system and read-

only file attributes) for three files: CONFIG.SYS,

AUTOEXEC.BAT and CLEANDSK.DRV, the LOCKIT  I device

driver. Locked files cannot be deleted, modified or

unlocked by any user. Only the administrator can

unlock locked files.

Page 43

U . UNLOCK LOCKIT  I related files-

This menu item unlocks (clears the System and Read-

Only file attributes) the CONFIG.SYS, AUTOEXEC.BAT,

and the LOCKIT  I device driver, CLEANDSK.DRV, files.

A.  ACCESS fixed disk after diskette boot-

It may become impossible to boot from your hard

disk due to causes unrelated to LOCKIT  I. For example,

if COMMAND.COM is accidentally deleted or a defective

device driver is installed, you cannot boot from the

hard disk whether LOCKIT  I is installed or not. You

will have to boot from a diskette and repair the problem.

With LOCKIT  I installed, however, when you boot from a

floppy diskette, you will not be able to access the hard

disk(s). This menu item allows you to access your hard

disk so you can repair it. Simply boot from a diskette,

run LOCKIT.EXE, enter the administrator's password and

select "A. ACCESS fixed disk after diskette boot". You

will be told that LOCKIT  I protection has been tempor-

arily suspended and that the next time you boot from a

floppy diskette you will have access to your hard disk.

If you reboot from your hard disk, full protection will

be automatically restored.

IMPORTANT

The diskette used to boot the computer must

be a system formatted diskette with the same

version of DOS on it as is on the computer's

hard disk or this feature will not work. To

determine the version of DOS that is on your

hard disk, boot from the hard disk, type:

VER

and press the Enter key.

Page 44

R.  REMOVE LOCKIT  I from this computer -

Selecting the "R. REMOVE LOCKIT I from this

computer" option will destall LOCKIT I including the

WINDOWS support, which means deleting the file

JCSSEC.386 from the root directory.  The LOC-

KIT I device driver, CLEANDSK.DRV will be deleted,

and the corresponding "DEVICE=CLEANDSK.DRV" state-

ment will be removed from the CONFIG.SYS file, LOC-

KIT I related files will be unlocked and other changes

made to your hard disk will be restored. This is the

ONLY way that LOCKIT I should be removed from

your computer.

*************************  WARNING *************************

If you try to remove LOCKIT I by deleting the device

driver line "DEVICE=CLEANDSK.DRV" from your CONFIG.SYS file,

this will NOT remove LOCKIT I from your computer and in fact,

will cause it to lock up upon the next reboot.

**************************************************************

K.  Define new hot KEY combination -

The LOCKIT  I hot key is used to place your computer

in Lunch-Break. (For more information on Lunch-Break,

see "Lunch-Break Description" on page 6) The hot key

is actually a combination of two or more keys held

down simultaneously and released. The default hot

key combination is the Left Shift and Right Shift

keys pressed and released simultaneously. You may

change it to any combination of two or more of the

following keys: Left Shift, Right Shift, Alt, and Ctrl,

however, if you intend to use the Lunch-Break feature from

within WINDOWS, you should not include the Alt key in

your selection. To change your hot key, select the K.

Define new hot KEY option from the administrator's main

menu. The hot key selection screen shown in figure 12

Page 44

will then be displayed. Simply follow the directions

on the screen and your new hot key will be in effect.

��������������������������������������������������������������ͻ

�                                                              �

� You may now select the keys which will cause your computer's �

� screen to blank (if selected) and your keyboard to lock      �

� until you enter your password.                               �

�                                                              �

� Please press any two or more of the following keys:          �

�                                                              �

�        Left Shift    Right Shift    Alt    Ctrl              �

�                                                              �

� Hold them down until you hear a two tone beep and you are    �

� asked to release them.  You will have to hold the keys down  �

� approximately four seconds.                                  �

�                                                              �

� If you are using WINDOWS and wish to invoke Lunch-Break via  �

� the hot-key please DO NOT select the Alt key as one of your  �

� hot-keys.                                                    �

� If you are using only DOS then you may select the Alt key    �

� as one of your hot-keys.                                     �

��������������������������������������������������������������ͼ

Fig. 12 - Hot Key Selection Screen

I.  Set keyboard IDLE time -

You may choose to have your computer automatically

enter the Lunch-Break state when your keyboard has

been idle for a specified period from 3 to 60 min-

utes. (Remember that your computer can be allowed

to continue the task it was performing if the freeze

option is not selected. This is true for both hot-

key and automatic activation of Lunch-Break.) If

you select a time of 61 minutes, automatic acti-

vation of Lunch-Break is disabled and your computer

will go into Lunch-Break only when the hot key is

pressed. If you find that LOCKIT I prevents you

from entering a value below 61, then the maximum

idle time has been set to a lower value from within

the "Administrator's Limits Menu". To select, de-

select or change the automatic Lunch-Break time,

choose the I. Set keyboard IDLE time item from

the administrator's main menu. The screen shown in

Page 46

figure 13 will then be displayed. Simply enter

the desired time and press the Enter key.

�������������������������������������������������������������ͻ

�                                                             �

� You may request that your machine automatically go into the �

� Lunch-Break state if the keyboard is idle for a specified   �

� time period.  You may select a time period from 3 to 61     �

� minutes.                                                    �

�                                                             �

� A time of 61 minutes means that automatic Lunch-Break will  �

� never occur.                                                �

�                                                             �

� The current keyboard idle time is 5  minutes.               �

�                                                             �

� Please enter new keyboard idle time in minutes:             �

�                                                             �

�������������������������������������������������������������ͼ

Fig. 13 - Idle Time Selection Screen

The administrator can set the keyboard idle time

for his/her own use by selecting this menu item. There

is also a menu item in the "Administrator Limits Menu"

that appears to have the same function. This is not the

case. The item in the "Administrator Limits Menu" controls

the upper limit of the keyboard idle time for ALL users.

Thus if the administrator chooses an upper limit of say

10 minutes, each user will only be able to set the key-

board idle time from 3 to 10 minutes. They will not be

able to disable the automatic Lunch-Break feature because

this requires a keyboard idle time of 61 minutes. For

more information please read the section on "Maximum

keyboard idle time" in the "Administrator Limits Menu"

section. The keyboard idle time can also be set from the

DOS prompt by using the SET-TIME.COM program. For more

information see the description of the SET-TIME.COM pro-

gram on page 75.

Page 47

D.  Control DIRECTORY access by user (L1E) -

This section applies only to LOCKIT I Extended

Edition (L1E). This menu item will not appear on the

administrator's main menu for LOCKIT I with Admin-

istrator Option (L1+) or LOCKIT I Hardware (L1H).

As administrator you may control each user's access

to the subdirectories on your hard disk(s), to sector

oriented hard disk I/O, and to diskettes. These func-

tions are accomplished by selecting the D. Control

DIRECTORY access by user item from the administrator's

main menu. When this item is selected, a table similar

to the one shown in figure 14 will be displayed.

����������������������������������������������ͻ To Select User

�  Directory/Area  �   TheBoss   �   User 2    �  Cursor keys

�                  �EXE/COM�Other�EXE/COM�Other�

����������������������������������������������͹ To Select Dir.

� Diskette Access  � R-W-X � R-W � ----- � R-W �  Cursor, PgUp/Dn

����������������������������������������������Ķ  Home, End

�  HardDisk AbsI/O � ----- � R-W � ----- � --- �

����������������������������������������������Ķ To Control Access

� C:\              � R-W-X � R-W � R---X � R-- �  R=Read W=Write

����������������������������������������������Ķ  X=Execute

� C:\DATABASE      � R-W-X � R-W � --X-- � R-- �  A=All N=None

����������������������������������������������Ķ

� C:\BJONES        � R-W-X � R-W � R-W-X � R-W � To chg all users'

����������������������������������������������Ķ  access to this

� C:\UTILS         � R-W-X � R-W � --X-- � --- �  dir, use Alt

����������������������������������������������Ķ  with R,W,X,A,N

� C:\GAMES         � R-W-X � R-W � --X-- � --- �

����������������������������������������������Ķ To chg this usr's

� C:\PAYROLL       � R-W-X � R-W � ----- � --- �  access to all

����������������������������������������������Ķ  dirs, use Cntrl

� D:\              � R-W-X � R-W � R-W-X � R-W �  with R,W,X,A,N

����������������������������������������������Ķ

� D:\BACKUP        � R-W-X � R-W � R---X � R-- � To Save Choices

����������������������������������������������ͼ  Esc or E

Note:  R, W and X toggle the corresponding permission.

Fig. 14 - Directory Access Control Table

Page 48

Access rights assigned to a root directory apply

only to that directory, while those assigned to a first

level subdirectory also apply to all of its subdirect-

ories.

In all cases except HardDisk Abs I/O (described

below), you may separately grant access rights to .EXE

and .COM files (programs) and to all other files.  The

access rights that may be granted are "Read" and "Write",

and in the case of programs, "Execute".

READ access means that applications can read data

from files. WRITE access means that the files can be

created, written to, overwritten, renamed, deleted or

have their attributes changed.  EXECUTE access means

that files containing programs can be executed.  For

example, if the WordPerfect word processor program is

a file named WP.EXE, it may be executed only by users'

having execute access to it. Execute access does not

imply read access. Thus, if a user has only execute

access to WP.EXE, the command,

COPY    C:WP.EXE     A:WP.EXE

will fail because the copy command is not allowed to

read the file.

Some programs such as WordPerfect sometimes

modify themselves. If you are using DOS 3.1 or above,

LOCKIT  I will always allow an executing program to

read and write itself even if the access is not explicitly

granted. In versions of DOS prior to 3.1, LOCKIT I cannot

determine exactly which file is executing and so the

access is denied if it is not explicitly granted.

Thus, WordPerfect running under DOS 3.1 or above will

be allowed to modify itself even if write permission

has not been granted.

Some programs are designed to read and/or write

files that they require to be in the same directory as

the executing program. If you wish to prevent such

programs from being copied, grant only execute access

to .EXE/.COM files but full access to other files. Alter-

nately, you could deny write access to diskettes.

All users are always granted read access to the file

Page 49

named AUTOEXEC.BAT in the root directory of the hard

drive from which the system was booted. This is done

to allow all users to execute AUTOEXEC.BAT when the

system is booting.

When a user has write permission to a root direc-

tory, he may create first level subdirectories. The

administrator can define which access rights the user

will have in the newly created subdirectory by specify-

ing the access rights in the "New Level 1 Dirs" row of

the directory access control table. A very few programs

ask DOS to read/write specific physical locations on the

disk rather than performing operations on files. If such

a program can find the physical location of a file, it

may be able to read from the file even if the user does

not have read access to its directory.  The "HardDisk Abs

I/O" item in figure 14 allows the administrator to allow/

prevent this type of access. Preventing the access may

prevent some programs from running, but will result in an

even more secure system.  We suggest that you do not grant

this access unless you find that you must run a program that

requires it. The column labeled .EXE/.COM is not applicable

to absolute I/O.

Please note that in rare instances an application

that runs well when LOCKIT  I Extended Edition is

not installed will fail to run correctly when LOCKIT

I Extended Edition is installed.  This does not nec-

essarily indicate an error in LOCKIT  I Extended

Edition. For instance, an application may try to change

the attribute of a file from read-only to read-write.

If the user has not been granted appropriate access to

the file's directory, DOS will return an "access denied"

error. It is possible that the application may not handle

the error correctly. This bug in the application may

never have been noticed because the application never

encountered that error before.

A sample directory access control table is shown in

figure 14. The first three lines allow control of diskette

drives, sector oriented I/O and newly created level 1

directories. The remaining lines control access to the

root and first level subdirectories of your hard drive(s).

Page 50

Access granted to a root directory applies only to that

directory.  Access granted to a first level subdirectory

applies to that directory and all of its subdirectories.

Each column shows the access currently granted to the user

whose name appears at the top of the column. User names

are assigned using the PASSWORD option of the main menu.

In the example shown, user 1 has been assigned to TheBoss

and no new name has been assigned to user 2.

The cursor control, Pg Up, Pg Dn, Home, and End

keys may be used to move the highlight bar from one

position to another. Pressing the R, W, and X keys will

toggle (turn on and off) read, write, and execute per-

missions respectively. To grant/deny all permissions

in the highlighted square, press A or N respectively.

Pressing Ctrl with R, W, X, A, or N, will grant/deny

the corresponding access to all directories (the entire

column). Similarly, using the Alt key will affect all

users' access to that directory (the entire row). Thus,

if a user is to be granted access to almost everything,

begin by moving the bar to the user's column and

press Ctrl-A. Then remove the undesired accesses. At-

tempting to move the bar off the screen will cause

more users or directory names to be displayed.

When you have the access permissions set as you

desire, press the Esc or the E key to return to the

main menu. If no new directories have been created

since you last booted the computer, your selections

will be in effect immediately.  If this is not the case,

you will be notified that your selections will be effec-

tive when you reboot the computer.

I. Control I/O PORT access by user-

This section applies only to LOCKIT I Extended

Edition (L1E). This menu item will not appear on the

administrator's main menu for LOCKIT  I with Admin-

istrator Option (L1+) or LOCKIT  I  Hardware (L1H).

The administrator can control access to the parallel

Page 51

(LPT1, LPT2) ports and up to four serial ports (COM1,

COM2, COM3, COM4) for each user individually. When the

"I" menu item is selected, the table shown in figure 15

�������������������������������������������������������������ͻ

�    �������������������������������������������������ͻ      �

�    �                Port Control                     �      �

�    �������������������������������������������������͹      �

�    �  User1  �  User2  � User 3  � User 4  � User 5  �      �

�    �������������������������������������������������Ķ      �

�    �P-1-2-3-4�P-1-2-3-4�P-1-2-3-4�P-1-2-3-4�P-1-2-3-4�      �

�    �������������������������������������������������ͼ      �

�                                                             �

�    Press:  Right/Left cursor keys to select a user.         �

�            P - to toggle access to parallel ports.          �

�            1 - to access to serial port COM1                �

�            2 - to access to serial port COM2                �

�            3 - to access to serial port COM3                �

�            4 - to access to serial port COM4                �

�            A - to select all of the above (P,S).            �

�            N - to select none of the above (P,S).           �

�  Alt with one of the above to make it apply to all users.   �

�            Esc to save your choices, go to main menu.       �

�                                                             �

� If there is a serial mouse, you may wish to grant access to �

� its COM port. Remember that a user may be able to install   �

� another device there.                                       �

�������������������������������������������������������������ͼ

Fig. 15 - Port Control Table

will be displayed and the administrator will be able to select

the various accesses shown for each user.

In the table the existence of the letter "P" under a

user's name indicates permission to use the parallel ports.

A dash (-) indicates that the user is restricted from using

the parallel ports. The numbers "1 - 4" represent access

permission to the four COM or serial ports, respectively.

F. Select FILE accesses to be logged (L1E) -

This section applies only to LOCKIT I Extended

Edition (L1E). This menu item will not appear on the

administrator's main menu for LOCKIT I with Admin-

istrator Option (L1+) or LOCKIT I Hardware (L1H).

Choosing the F. Select FILE accesses to be logged

item from the main menu causes the table shown in

Page 52

figure 16 to be displayed. You may then select which

type(s) of file access you wish to log. Access types

which may be selected are Denials, Program Executions,

and All Other accesses.

�������������������������������������������������������������ͻ

�                                                             �

�    �������������������������������������������������ͻ      �

�    �                   Log Control                   �      �

�    �������������������������������������������������͹      �

�    �  Admin  � TheBoss � User 2  � User 3  � User 4  �      �

�    �������������������������������������������������Ķ      �

�    �  -----  �  D-X--  �  D-X-O  �  D----  �  D----  �      �

�    �������������������������������������������������ͼ      �

�                                                             �

�    Press:  Right/Left cursor keys to select a user.         �

�            D - to toggle logging of denied accesses.        �

�            X - to toggle logging of programs executed.      �

�            O - to toggle logging of all other accesses.     �

�            A - to select all of the above (D, X, F).        �

�            N - to select none of the above (D, X, F).       �

�            Esc to save your choices, go to main menu.       �

�                                                             �

�������������������������������������������������������������ͼ

Fig. 16 - Logging Control Table (L1E only)

Denied accesses occur when LOCKIT  I Extended Edi-

tion refuses to grant a requested access. For example,

an attempt by a user to delete, write to, or change the

name or attributes of a file in a directory to which the

user has read only access will result in a denial. It is

not possible to select logging of denied accesses for the

administrator because all administrator access requests

are granted.

The following lines, shown in figure 17, extracted

from an actual log, indicate the type of information

that is available to the administrator.

A small portion of each line was deleted so that it

would fit on one line in this document. This portion

indicates if files were opened with write access, etc.

The sample indicates that User 2 booted the machine,

DOS opened ANSI.SYS and AUTOEXEC.BAT, and then

SAV-DTAB was executed.  Following this the user at-

Page 53

tempted to change to a directory, CBH, for which he

had no access. The user apparently called the admin-

istrator who placed the machine in Lunch-Break, en-

tered the administrator passwordto exit Lunch-Break

so that the administrator's permissions would be in

effect, did the directory change for the user, renamed a

file, and reentered Lunch-Break. User 2 then entered

his password and continued as shown. Later User 1

booted the machine, etc.

�������������������������������������������������������������ͻ

� Log file starting date is   4-04-93                         �

� 17:18:43  User 2 - Allowed: Open.        C:\COMMAND.COM     �

� 17:18:40  User 2 - ������� RE-BOOT on   4-04-93             �

� 17:18:41  User 2 - Allowed: Open.        C:\DOS3.31\ANSI.SYS�

� 17:18:44  User 2 - Allowed: Open.        C:\AUTOEXEC.BAT    �

� 17:18:44  User 2 - Execute: ExecPrgm.    C:\SAV-DTAB.COM    �

� 17:18:55  User 2 - NotAlwd: Change Dir.  C:\CBH\            �

� 17:19:02  User 0 - Allowed: Change Dir.  C:\CBH\            �

� 17:19:39  User 0 - Allowed: FCB Rename.  C:\CBH\SPC\EV.CFG  �

� 17:20:15  User 2 - Allowed: Open.        C:\AUTOEXEC.BAT    �

� 17:20:15  User 2 - Allowed: Open.        C:\AUTOEXEC.BAT    �

� 17:20:27  User 2 - NotAlwd: Create.      A:\AUTOEXEC.BAT    �

� 17:20:53  User 2 - Allowed: Change Dir.  C:\DOC\            �

� 17:21:08  User 2 - Allowed: FCB Delete.  C:\JNK             �

� 17:21:41  User 1 - ������� RE-BOOT on   4-04-93             �

� 17:21:41  User 1 - Allowed: Open.        C:\AUTOEXEC.BAT    �

� 17:21:41  User 1 - Execute: ExecPrgm.    C:\SAV-DTAB.COM    �

� End of log file.                                            �

�������������������������������������������������������������ͼ

Fig. 17 - Typical Log Screen

A small area of memory is reserved for recording

log entries. These entries are written to the log file on

the disk whenever the area is nearly full, when the

FLUSHLOG utility is run, and whenever a denial is

logged.  It is, therefore, possible that a few entries

(other than denials) may be lost when the machine is

rebooted unless FLUSHLOG is run just prior to booting.

The log file is a locked file named ACCESS.SYS and

is located in the root directory of the hard drive from

which the machine is booted. Normally, one would run

Page 54

the FLUSHLOG utility to write any entries remaining in

memory to the ACCESS.SYS file and then change the

name of ACCESS.SYS to another name. (LOCKIT  I

will create a new ACCESS.SYS whenever it needs to

write log entries and the file does not already exist.)

The LOG utility may then be run as described on page 74

to produce a file similar to the sample above.

ACCESS.SYS is created as a locked file. The ad-

ministrator can unlock this file by using the command

FLUSHLOG /U

or by using the UNLOCK utility provided with LOCKIT  I.

Page 55

(This page intentionally left blank)

Page 56

SELECTING LOCKIT I OPTIONS

Selecting the O. Change OPTIONS item from the

administrator's main menu causes a new menu to be

displayed. This menu is shown in figure 18 below.

���������������������������������������������������������������ͻ

�                                                               �

�          Administrator Options Selections Menu                �

�                                                               �

�  Press the LETTER in front of the option you wish to change.  �

�                                                               �

�  E.  END option selection and return to main menu.            �

�  H.  HOW to use this menu, how to get additional help.        �

�                                                               �

�  M.  MAXIMUM floppy boot protection     - Not Selected.       �

�  D.  DISPLAY password entry asterisks.  - Selected.           �

�  R.  RELAXED MBR protection             - Not Selected        �

�                                                               �

�  K.  SIDEKICK compatibility mode.       - Not Selected.       �

�  C.  CTRL-BREAK prohibited during boot. - Not Selected.       �

�  T.  TIME/Date change prohibited.       - Not Selected.       �

�                                                               �

�  B.  BLANK screen during LunchBreak.    - Selected.           �

�  F.  FREEZE computer during LunchBreak. - Not Selected.       �

�  A.  ALL users may exit LunchBreak.     - Not Selected.       �

�  S.  SPECIAL Display blanking           - Not Selected.       �

�                                                               �

�  N.  User NAMES are required.           - Not Selected.       �

�  U.  USER may change his/her user name. - Not Selected.       �

�                                                               �

���������������������������������������������������������������ͼ

Fig. 18 - Administrator's Options Menu

Pressing the letter in front of the option will change its

selection/deselection state. Each of the options is described

in the following paragraphs.

E.   END option selection -

You may return to the administrator's main menu

by selecting the E option or by pressing the Esc key.

H.  HOW to use this menu  -

Pressing H will bring up a "How to use this menu"

screen for assistance.

Page 57

M.  MAXIMUM floppy boot protection  -

This option makes it more difficult for an unauthorized

person ("hacker") to break into your computer. It erects

another significant barrier to protect your data if the

computer is booted from a diskette. If this option is

selected, you will see no difference in the operation of

your computer. Maximum floppy boot protection is an option

because it is not supported by some smaller hard disks

partitioned with older versions of FDISK. If your computer

will not support this option, attempting to select it will

result in an informative message and the option will not

select. In either case, your computer is still well pro-

tected, and the hard disk cannot be accessed by booting

from a floppy-disk.

D.  DISPLAY  password  entry  asterisks  -

This menu item causes asterisks to be displayed

for each password character entered during log in only.

If this option is not selected, nothing will be displayed.

Note, that the character that is displayed while defining

a new password is not controlled by this option but is

selected in the Change Password menu item in the Adminis-

trator's main menu. Also, please note that nothing is

displayed while entering a password to exit from Lunch-

Break since the screen is usually blanked.

R.  RELAXED MBR protection -

The Master Boot Record (MBR) and DOS boot record

are areas of your hard disk that must be correct before

you can boot your computer from its hard disk. LOCKIT

I provides very strong protection intended to keep even

the most sophisticated viruses from writing to these

records. Unfortunately, this level of protection prevents

some anti-virus programs from running. Selecting this

option weakens the protection afforded by LOCKIT I in

order to allow these anti-virus programs to run. Select

this option only if you are having a problem getting an

anti-virus program to work with LOCKIT  I.

Page 58

K.  SIDEKICK compatibility mode -

Prevents the computer from responding to Borland's

Sidekick program's hot key during Lunch-Break. Se-

lect this option only if you are using Sidekick and you

find that the computer responds to Sidekick's hot key

during Lunch-Break.

This paragraph contains a detailed technical de-

scription of this option so feel free to skip to the next

paragraph if you wish. LOCKIT  I intercepts both the

clock (IRQ 0) and keyboard (IRQ 1) interrupts at boot

time and again on entry into Lunch-Break. Each time

the clock interrupt is issued, Sidekick determines if

any program has intercepted the keyboard interrupt

since it has. If so, it re-intercepts the keyboard

interrupt. This is why they say it must be loaded

last, and why it can see its hot key even during Lunch-

Break. If LOCKIT  I's Sidekick Compatibility option

is selected, LOCKIT  I passes clock interrupts inter-

cepted to the IRQ 0 interrupt address that was in effect

when its device driver was loaded at boot time. This

effectively passes clock interrupts around Sidekick

(and perhaps other TSRs) so that it never re-intercepts

the keyboard interrupt. This also assures that the DOS/

BIOS system clock continues to run.

C.  CTRL-BREAK prohibited during boot -

Prevents anyone other than the administrator from

breaking out of the AUTOEXEC.BAT file during boot by

pressing Ctrl-Break or Ctrl-C. This option is used in

conjunction with the BRK-CNTL.COM file described later

on.

T.  TIME/Date change prohibited  -

Selecting this option will prevent users, but not

the administrator, from changing the system date and/

or time. Even other utilities for changing the time and

Page 59

date not supplied with DOS will be blocked by this

feature.

B.  BLANK screen during Lunch-Break -

Selecting this option causes the screen to become

completely blank during Lunch-Break. If this option is

not selected, the keyboard and mouse will lock but the

screen will remain active. This allows you to use the

system to monitor some process while prohibiting ob-

servers from interfering with the process.

F. FREEZE computer during Lunch-Break -

Prevents the computer from continuing to process

during that time. There are certain rare instances when

processing cannot continue during Lunch-Break and

utilizing this option is necessary. Normally it should

not be needed.

WINDOWS USERS NOTE

If you want Lunch-Break to work properly

while in WINDOWS, you must not select the

"FREEZE computer during Lunch-Break" option.

WINDOWS must see activity or it will not

work properly.

A. ALL users may exit Lunch-Break -

When this option is selected, any user may unlock

the computer when it is in Lunch-Break by entering

their own password (and optionally a user name) to

unlock the computer. We don't recommend selecting

this option if security between authorized users is a

concern. When deselected, only the user that was logged

on when Lunch-Break was invoked or the administrator

can unlock the computer.

Please remember, if user names are required, the

user must enter BOTH the user name (and press the

Page 60

Enter key) followed by the password (and press the En-

ter key) before the Lunch-Break lock will come off.

S.  SPECIAL display blanking  -

This option should be selected when the BLANK

screen during Lunch-Break option is selected, but

your screen does not blank as it should. A few non-

standard display adapters require selection of this

option to blank properly.

N.  User NAMES are required -

When this option is selected, the administrator and

all users will have to enter both their user name and

password to gain access to the computer, access menus,

and to exit Lunch-Break. If this option is deselected,

then only the password will be required at boot-up,

when running LOCKIT.EXE and to exit Lunch-Break.

U. USER may change his/her name  -

The administrator can always define a user's name

as well as his/her own name. If this option is selected,

then users will be able to change both their user name

and password, instead of just their password.

Page 61

SETTING  LOCKIT  I  LIMITS

Selecting the S menu item from the administrator's

main menu allows you to set certain limiting values.

Each of the limits is described in the following para-

graphs. The Set Limits menu is shown in figure 19.

��������������������������������������������������������������ͻ

�              Administrator Limits Selection Menu             �

�                                                              �

�  Press the LETTER in front of the option you wish to change. �

�                                                              �

�  E.  END limit selection and return to main menu.            �

�                                                              �

�  H.  HOW to use this menu, how to get additional help.       �

�                                                              �

�  I.  Maximum keyboard IDLE time (minutes).  -  Currently 61  �

�                                                              �

�  P.  Minimum number of PASSWORD characters. -  Currently 0   �

�                                                              �

�  A.  Maximum invalid logons before ALARM.   -  Currently 5   �

�                                                              �

�  L.  Maximum invalid logons before LOCKOUT. -  Currently 0   �

�                                                              �

�  S.  SECONDS to wait before auto logon.     -  Currently 0   �

�                                                              �

�  K.  Alternate KEYBOARD/clock handling      -  Currently 0   �

�                                                              �

��������������������������������������������������������������ͼ

Fig. 19 - Administrator's Limits Selection Screen

E.   END limits selection -

You may return to the administrator's main menu

by selecting the E option or by pressing the Esc key.

H.  HOW to use this Menu  -

Pressing H will bring up a "How to use this menu"

screen for assistance.

I. Maximum keyboard IDLE time -

Keyboard idle time is the time in minutes between

the most recent keystroke or mouse movement and

Page 62

the time when the computer automatically goes into

Lunch-Break. This option allows you to determine the

maximum keyboard idle time a user can specify. If the

user specifies a time of 61 minutes, automatic Lunch-

Break will never occur. If you select this option and

enter 10, a user may set the actual idle time to any

value between 3 and 10 minutes.

P. Minimum number of PASSWORD characters -

Allows you to determine the minimum number of

characters in a password. When you select this option

you will be asked to enter a number from 0 to 16.

Newly defined passwords must contain at least the

number of characters you specify.

A. Maximum invalid logons before ALARM -

After an excessive number of consecutive unsuc-

cessful attempts to boot the computer, exit Lunch-

Break, and/or use the LOCKIT  I program, an alarm

can be programmed to sound. The alarm consists of

several repetitions of a two tone signal. Turning the

computer off between attempts will not keep the alarm

from working. This option allows you to select the

number of failed attempts to allow prior to the alarm

being sounded. If you select the value zero, the

alarm will never sound.

L. Maximum invalid logons before LOCKOUT -

After an excessive number of consecutive attempts

to boot the computer, exit Lunch-Break, and/or use

the LOCKIT  I program, the computer can be made to

lock for a period of five minutes. Turning the computer

off during a lockout period will cause the five minute

lockout to be restarted from the beginning on the next

power up. This option allows you to select the number

of failed attempts prior to the lockout occurring. If you

select the value zero, the lockout will never occur.

Page 63

S. SECONDS to Wait Before Auto Logon -

Normally, LOCKIT  I requires that a correct pass-

word be entered each time the computer is booted. If

this selection limit is set to a value other than zero, it

specifies the number of seconds that LOCKIT  I will

wait for a correct entry.  If no password entry attempt is

made during the specified interval, the computer will

automatically boot as though the password for User 6

had been correctly entered. This feature is very useful in

applications where the computer must self-boot for un-

attended or remote limited operation but allow full op-

eration by authorized users only. It is also very handy

when using LOCKIT III, where no LOCKIT I password is

necessary, but "A" drive boot-up protection is required.

K. Alternate KEYBOARD/Clock Handling -

There are a very few combinations of computers, add-

on boards and application programs that are not compat-

ible with LOCKIT I. Changing the interrupt vectors that

LOCKIT  I uses will sometimes eliminate the incompatibil-

ity. We suggest using this feature only in the event of

incompatibilities between LOCKIT I and other hardware

or software products. When you select this feature, you will

be asked to choose one of several software interrupt groups

for LOCKIT  I to use. You do not have to know what an

interrupt is. When you choose this option, LOCKIT  I will

list the values from which you may choose, and even give a

recommended choice. If LOCKIT I still has any incompat-

ibilities with a specific combination of computer and soft-

ware that you are using, please call our tech support

department for assistance.

WINDOWS USERS NOTE

To have Lunch-Break work properly while in

WINDOWS, you must not set the Alternate Key-

board/Clock handling to any value other than

zero.

Page 64

U. Highest USER no. to access Admin. Menu -

This menu item allows the administrator to designate

which users are to have access to the Administrator main

menu. To have only the Administrator, User 0, be able to

access the administrator's main menu, set this number to

0 (the default value). If you want Users number 0, 1 and 2

to have access to the Administrator's main menu then set

this number to 2.

________________________________________________________

THE USER'S MAIN MENU

For general information on using LOCKIT  I menus,

see "USING LOCKIT  I MENUS" on page 17. The user's

main menu, shown in figure 8, on page 34, will appear

whenever the LOCKIT.EXE program is executed on a

computer that has LOCKIT  I installed and one of the

user's passwords is entered. The following section will

describe the menu items in the user's main menu.

E.   END this program -

You may return to DOS from the user's main menu

by selecting the E option or by pressing the Esc key.

H.  HOW to use this Menu  -

Pressing H will bring up a "How to use this menu"

screen for assistance.

Page 65

P.  Change PASSWORDS -

Passwords and optionally, user names, may by

changed by selecting the P. Change PASSWORD option

from the user's main menu.

If user names are being used and the user is al-

lowed to change his/her own name, the screen in fig-

ure 10, on page 42, will appear. After selecting the

user name, entering it twice or pressing Enter to

avoid changing the user name, the password is defined

in a similar manner.

The change password screen shown in figure 11, also on

page 42, will appear next. If you do not wish to change

the password, press the Esc key. The example in the figure 11

shows that the user has selected "SECRET-STUFF" as the new

password. After entering the new password you may be asked

to enter it a second or third time just to be certain it has

been entered correctly. The lower portion of the screen

shown in figure 11 is then displayed and the new password

is stored.

K.  Define new hot KEY combination -

The LOCKIT I hot key is used to place your computer

in Lunch-Break. (For more information on Lunch-Break,

see "Lunch-Break Description" on page 6) The hot key is

actually a combination of two or more keys held down

simultaneously and released. The default hot key consists

of the left and right shift keys. You may change it to any

combination of two or more of the following keys: Left

Shift, Right Shift, Alt, and Ctrl. To change your hot key,

select the K. Define new hot KEY option from the user's

main menu. The hot key selection screen shown in figure

12, on page 46, will then be displayed. Simply follow

the directions on the screen and your new hot key will

be in effect.

Page 66

I.  Set keyboard IDLE time -

You may choose to have your computer automati-

cally enter the Lunch-Break state when your keyboard

has been idle for a specified period from 3 to 60 minutes.

(Remember that your computer can be allowed to con-

tinue the task it was performing if the freeze option is

not selected. This is true for both hot key and automatic

activation of Lunch-Break.) If you select a time of 61

minutes, automatic activation of Lunch-Break is disabled

and your computer will go into Lunch-Break only when the

hot key is pressed. If you find that LOCKIT  I prevents

you from entering a value below 61, then the maximum idle

time has been set to a lower value from within the

"Administrator's Limits Menu". To select, deselect or

change the automatic Lunch-Break time, choose the I. Set

keyboard IDLE time item from the user's main menu.

The screen shown in figure 13, on page 47, will then

be displayed. Simply enter the desired time and press

the Enter key. Each user can set the keyboard idle time

for his/her own use by selecting this menu item.

USING LOCKIT  I PROGRAM AFTER IT IS INSTALLED

Once LOCKIT  I is installed, the only way to change

any of the LOCKIT  I features or remove LOCKIT  I

is to run the LOCKIT.EXE program. To run the LOC-

KIT.EXE program you must place your LOCKIT  I disk

in the A: drive and type:

A:\LOCKIT

or

LOCKIT

if you have copied the LOCKIT.EXE file to your hard

disk and have the path set so that DOS can locate it.

After pressing the Enter key you will be asked to

enter your password (or optionally a name and password).

If you enter a correct password, the appropriate user or

Page 67

administrator main menu will be displayed. Each of

these menu items is described in detail in the pre-

ceding sections. Your selections made from this menu

will be effective immediately.

Important Note

Running LOCKIT.EXE by typing:

LOCKIT /P

only allows you to change options, limits, names

and passwords defined within the LOCKIT.EXE

file, prior to installation. Once LOCKIT  I is

installed on a computer, performing a  LOCKIT /P

will not allow you to change anything on the

computer. LOCKIT /P only modifies the LOCKIT.EXE

file for future installations.

Running LOCKIT.EXE by typing:

LOCKIT /I

allows you to install LOCKIT  I immediately.

If LOCKIT  I is already installed running, typing:

LOCKIT

allows you to change options, limits, names and

passwords for the computer that LOCKIT  I is

installed on.

Typing:

LOCKIT /M

forces the LOCKIT program into the monochrome (Black

and white mode. Likewise typing:

LOCKIT /C

forces the LOCKIT program into the color (Blue

and white mode. By default LOCKIT will sense which

mode (color or B/W) your computer supports.

Page 68

Chapter 4 -  LOCKIT  I Files

This chapter describes each of the files on your

LOCKIT  I distribution diskette, as well as those files

created by LOCKIT  I during or after installation.

ACCESS.SYS  (L1E only) -

This LOCKIT  I Extended file is not on your diskette.

It is created in the root directory of your first (or

only) hard drive at any time it does not already exist

and there are log entries to be written. This file is

used by the LOG utility to generate the user readable log.

The LOG utility is described later on.

BRK-CNTL.COM  -

This program is used to enable/disable Ctrl-Break

and Ctrl-C at any time after your computer is booted.

The owner can prevent users from breaking out of the

AUTOEXEC.BAT file during system boot. This program can

be placed in the AUTOEXEC.BAT file to re-enable breaks.

Use BRK-CNTL ON to enable breaks and BRK-CNTL OFF to

disable them. Resident programs, such as some of the

DOS keyboard utilities for various languages which com-

pletely take over the keyboard interrupt, will cause

your computer to recognize breaks even when you have

them disabled. They will also prevent LOCKIT  I from

"knowing" when you are typing on your keyboard. Thus,

if you have selected the automatic Lunch-Break feature,

LOCKIT  I may go into Lunch-Break right while you are

typing. To prevent both of these anomalies you may also

use the optional RES parameter. This will direct BRK-

CNTL to remain resident. For example,

BRK-CNTL   ON   RES

will enable breaks and cause BRK-CNTL to remain resi-

Page 69

dent. The RES parameter should be used after the

resident program which takes over the keyboard and

should be used only once per system boot.

CLEANDSK.DRV -

This file is the LOCKIT  I device driver. It is not

located on your LOCKIT  I diskette, but is created in

the root directory of your hard disk when you install

LOCKIT  I. It is automatically deleted when you re-

move  LOCKIT  I.

IMPORTANT NOTE

You must never delete the CLEANDSK.DRV file

manually. It will be deleted automatically only

when LOCKIT  I is removed from the computer.

Deleting this file with LOCKIT  I still installed

will cause your computer to lockup.

JCSSEC.386 (replaces DRVR-DLL.DLL, DRVR-APP.EXE)  -

The DRVR files are no longer used to support WINDOWS except

if you are running WINDOWS 3.0 in the standard mode.

A Virtual device driver called JCSSEC.386 is created when

you install LOCKIT and Windows is detected. The VXD allows

the Lunch-Break feature to work within all modes of WINDOWS

3.1. In Windows LOCKIT now places a Blue screen when Lunch-

Break is invoked. Pressing the enter key calls up the

familiar "Please enter your Password" message. Lunch-Break

now works in DOS boxes, within Windows, in full screen DOS

sessions under Windows, or in regular Windows applications.

EXEC.COM  -

Allows the system administrator to execute a pro-

gram for a user and prevent the user from escaping to

the DOS prompt or executing any other program. Typi-

cally, EXEC would be placed in the AUTOEXEC.BAT file

to call a program such as 123, Dbase, or WordPerfect

into execution. The EXEC command line syntax is:

EXEC  [/R] [d:][\path\]filename [other parameters]

where:

/R  is used to restart the application automatically if

the user tries to exit from the  application.

Page 70

d   is the letter of the drive where the file is located

\path\ is the full path designation for the file

filename is the complete name of the file, including

the .EXE or .COM extension.

other parameters are any input parameters required

by the specific application.

For example, to cause a computer to run only

WordPerfect 5.1 upon boot-up, and to prevent exiting

from the WordPerfect program to DOS, the following

line could be placed as the last line in the

AUTOEXEC.BAT file:

EXEC    /R       C:\WP51\WP.EXE

Two parameters have been added to the EXEC program.  Their

use is somewhat technical, and most users can skip the

following description.  The EXEC command line now has the form:

EXEC [/R] [/Ln] [/Tn] drive:\full_path_name\prog.ext param-list

The new parameters, /L and /T allow the program started by

EXEC to execute other programs.  /Tn allows a total of n pro-

grams to be executed, additional execution attempts will fail.

The n may be any number from 0 (the default) through 9.  This

parameter is necessary to start Windows 3.1, for example, be-

cause it asks DOS to execute 2 or 3 programs (Standard or 386

modes) during its startup.  Once started windows could execute

any number of Windows programs, but no DOS programs since that

would exceed the selected value of a properly chosen n.  The

/Ln parameter allows the program started by EXEC to execute

another program, which may execute another program, etc. to a

depth of n levels. Thus if EXEC /L1 started a menu program

(level0), the menu program could execute any number of other

programs (all at level 1).  The level 1 programs started by

the menu program could not execute any other programs because

a depth of level 2 would not be allowed.  The n may be any

number from 0 (the default) through 9. You may not use both

the /L and /T parameters.

Page 71

FLUSHLOG.COM  (L1E only) -

This LOCKIT  I Extended utility causes any log

entries remaining in memory to be written out to the

ACCESS.SYS file as described in the section on log

control on page 54. ACCESS.SYS is created as a locked

file. The administrator may use the command:

FLUSHLOG    /U

to unlock ACCESS.SYS and write any remaining log

entries to it. The ACCESS.SYS file can also be un-

locked with the UNLOCK.EXE utility program described

later.

FPERMF.INF (L1E only) -

This file is recreated whenever LOCKIT  I Extended

directory access permissions are changed and is not

deleted when LOCKIT  I Extended is removed. It will

be used to recover the directory permissions the next

time you install.

LOCKIT.EXE  -

This is the main LOCKIT  I program. It is de-

scribed in the preceding sections of this manual. In-

stalling LOCKIT  I does not copy this or any other

files, except CLEANDSK.DRV to your hard disk. If you

want to, you may copy LOCKIT.EXE or any of the other

LOCKIT  I files to your hard disk or you can use them

from the floppy. Copying the LOCKIT.EXE file onto the

hard disk may reduce some of the security.

LOCK.EXE and UNLOCK.EXE  -

LOCK and UNLOCK are two utilities provided with

LOCKIT  I that allow the setting and clearing of a

file's system and read-only attributes. LOCKIT  I

recognizes a file as "locked" if the system and read-

only attributes are set. If LOCKIT  I is installed, locked

files may not be unlocked by any user. Only the administra-

tor can unlock files. Locked files cannot be changed, over-

written or deleted by a user. The LOCKIT  I administrator's

main menu also provides the ability to LOCK and UNLOCK

Page 72

three special files, AUTOEXEC.BAT, CONFIG.SYS and CLEANDSK.DRV.

LOCK and UNLOCK provide this functionality from the DOS

command line or from within batch files. The syntax for using

LOCK and UNLOCK is as follows:

LOCK [d:][\path\]filename

where:

d      is the letter of the drive where the file is located.

\path\    is the full path designation for the file.

filename  is the complete name of the file.

UNLOCK [d:][\path\]filename

where:

d      is the letter of the drive where the file is located.

\path\    is the full path designation for the file.

filename  is the complete name of the file.

Example:

LOCK  C:\DOS\FORMAT.COM

will lock the FORMAT.COM program in the \DOS subdirectory

on the C: drive.

UNLOCK  C:\DOS\FORMAT.COM

would unlock the FORMAT.COM program.

LOG.EXE (L1E only) -

This LOCKIT  I Extended utility is used to read

the log file produced by LOCKIT  I Extended and gen-

erate a user readable log or journal of the users'

activity.  A sample of the output from this utility

is shown in figure 17, on page 54. To use this utility enter:

LOG   in-filename   out-filename

at the DOS prompt. For example one might use the commands:

Page 73

FLUSHLOG

REN C:\ACCESS.SYS  OLDLOG

LOG  OLDLOG  PRN

to flush any log entries remaining in memory to the

disk, ensure (by renaming) that no new log entries will

be added to the file, and write a user readable log of

system activity to the printer.

LOGOFF.COM  -

The LOGOFF utility provides a means for one user

to log off and another to log on without having to

reboot the computer. Typing:

LOGOFF

and pressing Enter at the DOS prompt will clear the

screen, and display a box requesting the user to press

any key to begin the logon process.  When a key is

pressed, the boot time logon screen will be displayed.

Any valid user password will be accepted. LOGOFF is a

quick way of letting another user take over operation

of the computer without first having to do a Ctrl-Alt-

Del or power-off and on sequence.

MEM-STAT.SYS -

(And/or MEM_STAT.SYS) These small files contain

information about the state of your hard disk(s) before

the first installation of LOCKIT  I. They can be used

for disaster recovery, and have allowed us to help us-

ers to recover from a number of problems which had

nothing to do with LOCKIT  I. If they do not already

exist, these files are created when LOCKIT  I installs

but are not deleted when LOCKIT  I is removed. We

suggest that you allow them to remain on your disk if

you may reinstall LOCKIT  I at a later time. Please do

NOT copy them from one hard disk to another.

Page 74

SET-TIME.COM  -

This program provides an alternate method for set-

ting the keyboard idle time before automatic Lunch-

Break to any value between 3 and 61 minutes up to

the maximum allowed by the "Maximum keyboard idle

time" menu item found in the administrator's Limits

Menu. For more information on automatic Lunch-Break

see "Set keyboard IDLE time". If you use a nonresident

program that completely takes over the keyboard, and

you have automatic Lunch-Break selected, you may find

that LOCKIT  I goes into Lunch-Break even while you

are typing. This can be prevented by using a simple

batch file to temporarily disable the automatic Lunch-

Break. If the name of your program is DOWORLD and you

normally use a maximum keyboard idle time of five min-

utes, the batch file:

SET-TIME   61

DOWORLD

SET-TIME   5

will prevent automatic Lunch-Break while program

DOWORLD is running. If the program causing the prob-

lem is resident, read about the BRK-CNTL.COM program

described previously. The SET-TIME utility also accepts

0 to invoke Lunch-Break immediately from the DOS prompt

or from a batch file.

SET-TIME     0

is the command that will force the computer into Lunch-

Break lock. The currently logged on user's password

or the administrator's password can be used to unlock

the Lunch-Break lock (or any password if so setup).

The SET-TIME utility will also accept a command line

parameter of "D" which will set the keyboard idle time

back to the previously Defined value. Thus the follow-

Page 75

ing three lines would duplicate the previous example

except that whatever keyboard idle time was originally

defined would be reinstated by the last line.

SET-TIME   61

DOWORLD

SET-TIME   D

If the keyboard idle time was set to 17 minutes and the

above three lines were executed, after the first line the

automatic Lunch-Break feature would be disabled. Next,

the DOWORLD program would be executed. Finally the

keyboard idle time would be set back to 17 minutes.

VIOLS.COM  (L1E only) -

LOCKIT  I Extended records each unsuccessful

attempt to enter a password or a user name/password

combination. Such attempts are called "violations".

When a correct password is entered, LOCKIT  I erases

the record of any violations which occurred during the

immediately preceding two or three minutes. This pre-

vents recording "typos" made by a valid user.

The record of each violation contains the user num-

ber of the name entered (if any), and the date and time

of the violation.  When booting the computer, the DOS

clock has not yet been set, so we must use the hard-

ware clock. Since XT class machines do not have a

standard hardware clock, we cannot record the times

on these machines. We do, however, keep a record of

each violation.

This program has three separate functions related

to password entry violations. One, two or all three of

the functions may be used on a single execution of

VIOLS.COM. The command:

VIOLS /L=filename  /C  /R

will perform all three functions.  The "/L" will generate

a report of any recorded violations and give the date

Page 76

and time that you last logged on. If a date and time

when you know you weren't on the computer are displayed,

it is an indication that someone else may know your

password. If "=filename" is present, the report will

be written to the file specified by "filename". If it

is not present the report will be written to the screen.

The "/C" will remove (clear) all entries from the vio-

lation record. If both /L and /C are present, the record

will be cleared after the report is generated.

The "/R" will cause VIOLS.COM to remain resident.

This should not be done more than once per system boot.

Violations during Lunch-Break will not be recorded unless

VIOLS is resident. In addition, the alarm and lockout

features will not work during Lunch-Break unless VIOLS

is resident.

VIOLS terminates with a DOS errorlevel of 8 if an

error occurs, an errorlevel of 4 if violations are re-

ported, and 0 otherwise. The following statements in

your AUTOEXEC.BAT file would automatically install

VIOLS as resident and alert the user to any previous

violations:

VIOLS  /L  /R

IF NOT  ERRORLEVEL 4  GOTO   CONTINUE

ECHO  WARNING _ Violations are listed above

PAUSE

:CONTINUE

VMOUSE.COM  -

This utility causes the automatic Lunch-Break fea-

ture to treat mouse activity as keyboard activity.  It

also prevents anyone from using the mouse while your

computer is in Lunch-Break. VMOUSE should be

loaded after your mouse driver. If your mouse

driver is a device driver (commonly MOUSE.SYS or

MSCMOUSE.SYS) place a VMOUSE statement near

the beginning of your AUTOEXEC.BAT file. If your

mouse driver is loaded from your AUTOEXEC.BAT

file, place the VMOUSE statement immediately af-

Page 77

ter the statement that loads your mouse driver.

WINDOWS USERS NOTE:

LOCKIT  I WINDOWS support provides these

capabilities while WINDOWS is running. Thus

if you use your mouse only while you are in

WINDOWS, you do not need to use VMOUSE.

WHO.COM  -

This utility allows the AUTOEXEC.BAT or other

batch files to do different things for different

users. WHO will always return the user number as a

DOS errorlevel, or will return an errorlevel of 255

if LOCKIT  I is not installed. In addition, WHO /L

will display the name of the user, and  WHO /E

will set a DOS environment variable called PCVU

to the name of the current user. (Any spaces in

the name will be deleted.) For more information

on DOS errorlevels and environment variables and

IF statements, please see your DOS manual. The

error level facility of the WHO program may be

used by structuring your AUTOEXEC.BAT file as

shown in the following example. Upper case char-

acters indicate actual lines of the AUTOEXEC.BAT

file.

Page 78

EXAMPLE -

commands common to all users

.

.

WHO

IF ERRORLEVEL 3 GOTO ERROR

IF ERRORLEVEL 2 GOTO USER2

IF ERRORLEVEL 1 GOTO USER1

.

commands to be executed when the

administrator's password was used.

.

GOTO COMMON

:USER1

.

commands for user 1

.

GOTO COMMON

:USER2

.

.

commands for user 2

.

GOTO COMMON

:ERROR

.

commands to be used when LOCKIT  I

is not installed or User 3 or above logged on.

.

:COMMON

.

commands common to all users

.

The environment variable facility may be used as follows:

WHO /E

IF %PCVU%==TheBoss  GOTO  LABEL_A

IF %PCVU%==User2  GOTO  LABEL_B

where "TheBoss" and "User2" are two user names

Page 79

OPTIONAL LOCKIT I FILES

These programs, available separately, are designed

to work with LOCKIT  I. The DesMaster(tm) program

can be used by itself.

DesMaster(tm) (Product Order code ENC or DES) -

DesMaster(tm) provides Data Encryption Standard

encryption of files. It provides all four DES modes,

ECB, CBC, CFB, and OFB. DES is the national data

encryption standard defined by the National Institute

of Standards and Technology. Files protected by

DesMaster(tm) are useless without knowledge of the

password that was used to encrypt them. Encryption i

s the most secure method of file protection available

and DesMaster(tm) provides the most secure method of

encryption currently available. DesMaster(tm) is avail-

able in both software-only and combination hardware/

software models for the highest speed and government

certification. For more information about DesMaster(tm)

please contact Security Microsystems or one of our

dealers.

HELPUSER.COM -

This optional utility allows a corporate security

officer to grant access to a LOCKIT  I protected com-

puter on a one time basis. The CSO does not need to

know any passwords, does not need to be physically

present, and cannot grant access to another

organization's computers.

The HELPUSER program is available to Right-to-

Copy license holders only. A custom version of LOC-

KIT  I is supplied, so that the HELPUSER program

provided will work only on their copies of LOCKIT I.

HELPUSER support is also available to user's pur-

chasing our ESC (Extended Service Contract) for LOC-

Page 80

KIT  I. For details about our ESC, see the brochure

included in the software package. For more informa-

tion  please contact Security Microsystems, Inc. or

their sales representatives.

HELPUSER has saved many of our LOCKIT  I cus-

tomers from having to low level format their hard disks

when they have forgotten their administrator password.

LOGO.EXE -

This optional utility may be used with all LOCKIT

I programs. It is only available with corporate Right-to-

Copy licenses. It allows the administrator to design the

appearance of the initial LOCKIT  I screen which ap-

pears when the system is booted. Our logo may be

completely replaced with one of your own design. Use

of color is supported. Custom initial screens can be

purchased for all LOCKIT  I products for users not

purchasing Right-To-Copy licenses. For more informa-

tion please contact Security Microsystems, Inc. or their

sales representatives.

Custom Features and Add-Ons -

Many of our customers have requested special fea-

tures and utilities to be used with the LOCKIT  I

programs. If you have a special requirement or need a

custom feature, please call Security Microsystems and

ask to speak with the engineering department. It is

even possible that the feature you need already exists

but is not included with the off-the-shelf product.

LOCKIT I now contains many built-in features that are

available to network administrators to allow them to

supervise, install and maintain LOCKIT I remotely,

either over the network or via special batch files. For

more information about these features, and how to

access them, please contact Security Microsystems.

Page 81

Appendix

Problems and Solutions

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Installing new software with LOCKIT  I installed

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

A large number of software products, including LOC-

KIT  I, change the computer's CONFIG.SYS file when

they are installed. After you've installed LOCKIT  I,

you should prevent changes to the CONFIG.SYS file since

these changes could result in the LOCKIT  I device

driver not being one of the first lines in your CONFIG.SYS.

LOCKIT  I must have the line "DEVICE=CLEANDSK.DRV"

as the one of the first lines in the CONFIG.SYS file or

your computer will lockup when you try to boot it from

the hard disk. Certain extended memory managers like

"386Max", "QEMM" and products like the Intel "Inboard"

and certain SCSI drivers like ASPI4DOS must have their

"DEVICE =" statements first in CONFIG.SYS. To prevent

problems with other software installing themselves before

the LOCKIT  I device driver, make sure to remove LOCKIT

I from the computer before installing any new software.

After the new software is installed, make sure your computer

is working properly and then reinstall LOCKIT  I. If

removing LOCKIT  I first is not practical, at least check

the CONFIG.SYS file to make sure that the LOCKIT I device

driver statement, "DEVICE= CLEANDSK.DRV" has not been

preceded before rebooting.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If you are using Stacker or other similar hard disk comp-

ression programs.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Prior to DOS 6.0, Stacker, and other hard disk

compression programs, had to create a special com-

pressed drive, which is really just a big file on the

actual hard disk. These programs then swap the let-

ters of the actual hard disk and the special compressed

disk so that the compressed disk appears as the ac-

tual hard disk. If you install LOCKIT  I on a computer

Page 82

that is using one of these programs you should follow

the installation instructions contained in this manual

except that before you reboot your computer, you should

copy two files, CONFIG.SYS and CLEANDSK.DRV from your

compressed hard drive to the actual hard drive. For

example, if your compressed hard drive is labelled C:

and your actual hard drive is labelled D:, you should

enter the following commands after installing LOCKIT I,

but before rebooting:

COPY  C:\CONFIG.SYS  D:\

COPY  C:\CLEANDSK.DRV   D:\

This will ensure that when your computer boots up, the

LOCKIT I device driver will be properly installed. If

you have already rebooted your computer before doing

the above file copying, you will probably be locked out

of the computer because LOCKIT I will think that the file

CLEANDSK.DRV is missing. To fix this problem follow the

instructions below to remove LOCKIT I from the computer

after booting from a diskette. Next, reboot from the hard

disk, reinstall LOCKIT  I and make sure to copy the two

files as indicated above before rebooting from the hard

drive. These instructions are not necessary when using

DOS 6.0 with Stacker version 3.1 or Doublespace.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

What to do if you get the message "CLEANDSK has not run !".

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Once LOCKIT I is installed a file called "CLEANDSK.

DRV" is created in the root directory and a line is

added to the CONFIG.SYS file that runs CLEANDSK.DRV,

as a device driver.

If CLEANDSK.DRV is not executed when the computer

is booted from the hard disk, the computer will lock-

up and the message "CLEANDSK.DRV has not run !" will

appear on the screen. There are several reasons why

this may happen.

1) The file CLEANDSK.DRV may have been deleted

2) The line "DEVICE=CLEANDSK.DRV" may have

Page 83

been removed from the CONFIG.SYS file.

3) Something may have been changed in the

CONFIG.SYS file, preventing the line "DEVICE=

CLEANDSK.DRV" from being executed.

If the computer appears to be locked-up, don't panic.

Just reboot your computer from a DOS system-formatted

floppy diskette in the A: drive. (Note: The version of

DOS on the floppy MUST be the same or greater than the

version of DOS on your hard disk.) Next, place your

installation copy of the LOCKIT I diskette in the A

drive and type:

A:\LOCKIT

and press the Enter key. After entering your admin-

istrator's password you will see the administrator's

main menu. One of the items on this menu will be:

A.  Access fixed disk after diskette boot

Select this item and press Enter. You should see the

message "LOCKIT I protection has been suspended." Next,

place the DOS diskette back in the A: drive, close the

disk drive door and reboot the computer. (Ctrl-Alt-Del

is fine) When the computer returns to the A> DOS prompt

you should be able to type C: and press Enter to get to

your hard disk.

Once you can access your hard disk you can check

to see that the CLEANDSK.DRV file is still there and

that the "DEVICE=CLEANDSK.DRV" line is still located

in the CONFIG.SYS file.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If CLEANDSK.DRV gets erased or corrupted.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If something has deleted or corrupted the CLEAN-

DSK.DRV file, the only recourse is to remove LOCKIT

I from the computer and reinstall. The reason for

this is that the LOCKIT.EXE program creates the

CLEANDSK.DRV file when it is run in the installation

mode. The CLEANDSK.DRV file itself does not exist on

Page 84

the LOCKIT  I diskette. To remove and reinstall, place

your LOCKIT  I installation diskette in drive A: and

type:

LOCKIT

and press the Enter key. Next, select the "R. Re-

move LOCKIT  I from this computer" option. LOCKIT

I will note that it cannot delete the CLEANDSK.DRV

file but this is OK. When LOCKIT I is finished you

should be able to reboot from your hard disk and

reinstall LOCKIT  I properly.

If the problem is due to an error in the CONFIG.SYS

file, preventing the line "DEVICE=CLEANDSK.DRV" from

executing, then use a text editor such as EDLN or the

DOS 5.0 or 6.0 EDIT program to replace the "DEVICE=

CLEANDSK.DRV" line in  the CONFIG.SYS file. Reboot

again, this time from the hard disk, and LOCKIT I

should function normally.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If LOCKIT  I does not install properly or  hangs during

installation or reboot.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Some users report having problems installing LOCKIT

I on computers that have several TSR (terminate-

stay-resident) programs running. We recommend that

before trying to install LOCKIT I on a computer with

a lot of these type programs, that you rename the

AUTOEXEC.BAT file to AUTOEXEC.SMC and reboot your

computer. After successfully installing LOCKIT  I,

rename the file AUTOEXEC.SMC back to AUTOEXEC.BAT,

reboot, and all should be fine. Other problems can

sometimes be traced to an unusual device driver or

other statement in the CONFIG.SYS file. For example,

if there is a:

SHELL = C:\DOS\COMMAND.COM

statement in the CONFIG.SYS file, it must be amended

to read:

Page 85

SHELL=C:\DOS\COMMAND.COM  C:\DOS /P

or

SHELL=C:\DOS\COMMAND.COM  C:\DOS /P /E:XXX

(Use the second line if you are changing the environ-

ment size. "XXX" represents the new environment size

in bytes.)

To find a problem in the CONFIG.SYS, remark out

all lines (do this by placing REM in front of each line)

except the DEVICE=CLEANDSK.DRV line and reinsert

lines one at the time (by removing the REM) until the

offending statement is located. Make sure to reboot

the computer each time a statement is added.

The position in the CONFIG.SYS file of the LOCKIT

I  "DEVICE=CLEANDSK.DRV" statement has become impor-

tant due to the use of memory managers and SCSI drives

that use device drivers. If there is a line in the

CONFIG.SYS file that reads "DEVICE= ASPI4DOS.SYS",

the LOCKIT  I device driver statement must be placed

AFTER it or the computer will hang when it is rebooted.

The same applies to memory managers such as QEMM386

and 386MAX, etc. The line "DEVICE=CLEANDSK.DRV" should

come AFTER the line that installs the memory manager,

especially if you want to load the LOCKIT I device

driver into high memory. LOCKIT I will automatically

place the line "DEVICE=CLEANDSK.DRV" as the first

line in the CONFIG.SYS  unless there is a line in

the CONFIG.SYS file that reads:

Rem CLEANDSK.DRV Here

or

Rem CLEANDSK.DRV Here (High)

or

Rem CLEANDSK.DRV (Special, text)

If LOCKIT  I sees any of these lines, it will place

the line "DEVICE=CLEANDSK.DRV" as the next line in

the CONFIG.SYS file.

If you are using a DOS 6.0 or above multi-section

CONFIG.SYS file, you may place one of the above lines

Page 86

in each section. If (High) is included, LOCKIT  I will

use a standard DOS DEVICEHIGH statement. If (Special,text)

is included, LOCKIT I will create the following

CONFIG.SYS line:

DEVICE=text CLEANDSK.DRV

This allows automatic creation of device statements

for QEMM386 or other memory managers. For example, using:

Rem CLEANDSK.DRV Here (special,c:\qemm\loadhi.sys /r:2)

will create the line:

DEVICE=c:\qemm\loadhi.sys /r:2 CLEANDSK.DRV

Later versions of DOS ignore any CONFIG.SYS state-

ment beginning with "REM".  Earlier versions will

display a message stating that they cannot recognize

the statement but will otherwise ignore it.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

LOCKIT   I can help after a VIRUS or other calamity has

destroyed your hard  disk partition  table.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

During installation, LOCKIT  I takes a snapshot

of your hard disk partition table after it modifies it.

This snapshot is stored in an unused sector of the

hard disk. If something happens to your boot sector/

partition table and the hard disk becomes unbootable

LOCKIT  I will recognize this and try to restore your

boot sector. LOCKIT  I will do this automatically.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

What do I do if I forget my password ?

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Probably the call our tech support department

gets most frequently concerns someone forgetting their pass-

word. The purpose of LOCKIT I is to protect a computer

from someone trying to get at the data on the hard disk.

Page 87

If you are the owner of LOCKIT  I and you are

trying to access the data on your own computer's

hard disk then there should be no problem with us

helping you to get into your computer, but there is a

problem. We don't know whether you are indeed the

rightful owner of the computer or that you are trying

to break into someone else's computer that is also

protected by LOCKIT  I. We do have a responsibility

to all of our customers. However, we do understand

that there are circumstances when we should be able

to help you to get into your computer. Here's what

to do:

If you've forgotten a user's password then you

should be able to go to the person who installed

LOCKIT I on your computer and ask him/her to use

the administrator's password to issue you a new

password. If you have forgotten the administrator

password then you have two options:

1) Low level format your hard disk (if possible),

run FDISK to delete all partitions then create one

DOS partition,  run FDISK /MBR to regenerate the

boot record then run FORMAT /S and reload all your

data from the backup diskettes that you always keep

up to date.

2) Call Security Microsystems' Tech Support line :

(718) 667- 4720

and request a "Lost Password Information Sheet". If

you have a FAX, it can be FAXed to you, otherwise,

it will come by mail. Listed on the sheet are the

instructions and requirements for allowing us to help

you obtain entry into the computer.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

What to do when LOCKIT  I Hardware reports that the expansion

ROM space is completely full.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If, during the installation of LOCKIT  I Hardware, you

see the message that:

Page 88

IT APPEARS THAT THE COMPUTER'S EXPANSION ROM SPACE IS FULL

It means that the installation program was unable to

determine a location in the expansion ROM area that

could be used by the LOCKIT  I Hardware board's

internal ROM. If the computer is using a memory man-

ager such as EMM386, QEMM386 or 386MAX, it is possible

that they are using the entire expansion ROM

area to expand or extend system RAM. LOCKIT  I

Hardware requires a 16K block of memory space starting

at an address segment from C800 to EC00. The possible

dipswitch settings are listed in figure 20. The most

popular settings to try are D000, D400, D800 and DC00.

If your computer has a network card installed, the

CC00 or C800 (not for XT type computers) settings should

be tried first.

��������������������������������������������������������������ͻ

�        1             2              3               4        �

� ������������Ŀ ������������Ŀ ������������Ŀ ������������Ŀ  �

� ��ܿ�ܿ�Ŀ�ܿ� ��ܿ�ܿ�Ŀ�Ŀ� ��ܿ�Ŀ�ܿ�ܿ� ��ܿ�Ŀ�ܿ�Ŀ�  �

� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� ��  �

� ������������ٳ ������������ٳ ������������ٳ ������������ٳ  �

� �������������� �������������� �������������� ��������������  �

�  Segment=C800   Segment=CC00   Segment=D000   Segment=D400   �

�        5              6              7             8         �

� ������������Ŀ ������������Ŀ ������������Ŀ ������������Ŀ  �

� ��ܿ�Ŀ�Ŀ�ܿ� ��ܿ�Ŀ�Ŀ�Ŀ� ��Ŀ�ܿ�ܿ�ܿ� ��Ŀ�ܿ�ܿ�Ŀ�  �

� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� ��  �

� ������������ٳ ������������ٳ ������������ٳ ������������ٳ  �

� �������������� �������������� �������������� ��������������  �

�  Segment=D800   Segment=DC00   Segment=E000   Segment=E400   �

�       9             10                                       �

� ������������Ŀ ������������Ŀ                                �

� ��Ŀ�ܿ�Ŀ�ܿ� ��Ŀ�ܿ�Ŀ�Ŀ�                                �

� �� �� �� �� �� �� �� �� �� ��                                �

� ������������ٳ ������������ٳ                                �

� �������������� ��������������                                �

�  Segment=E800   Segment=EC00                                 �

��������������������������������������������������������������ͼ

Fig. 20 - Alternate Dipswitch Settings

(L1H Only)

When you select a setting to try, you must be sure to

"exclude" that 16K block from being used by the

Page 89

memory management software. Please consult the docu-

mentation that came with your memory management

software for instructions on how to exclude areas of

memory from being used by the memory manager.

Remember to run the LOCKIT.EXE program after

changing a dipswitch setting and reinstalling the board.

If you are still experiencing problems installing the

board, try rebooting the computer from a diskette in

drive A: (with no CONFIG.SYS or AUTOEXEC.BAT files)

and then try installing the board.

If all else fails, please call Security Microsystems

Tech. Support for assistance.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

Where to get  help with LOCKIT  I.

""""""""""""""""""""""""""""""""""""""""""""""""""""""""

If you are a registered user, you are entitled to 90

days of free support with LOCKIT  I. If you have not

sent in your registration card, you must have your

serial number and version number or support will not

be given. To obtain technical support call:

(718) 667-4720 or (718) 667-1019

Help can also be obtained via FAX. The number is :

(718)   667-0131

A Bulletin Board System (BBS) is available to regis-

tered users only. To access the BBS call:

(718)   667-0947

using a modem set for 1200 - 9600 baud, 8 bits, no

parity, 1 stop bit.

Technical support is also available 24 hours a day

to our Extended Support Contract customers. A sep-

arate 24 hour telephone hot line number is given to

ESC customers when they sign up.

Page 90