Updates to the REMOTE LINK User's Guide

USING DIGIBOARD INT 14 SUPPORT

To use REMOTE LINK with a DigiBoard multiport async adapter and the

DigiBoard INT 14H device driver, do the following:

1. Add the appropriate DigiBoard device driver statement to your

CONFIG.SYS file; for example, XIDOS5.SYS.

2. Run the DigiBoard XIDOSCFG program to configure the INT 14H support;

e.g., what COM port address to use and what driver support to use.

Set the channel parameters such that the receive flow control is RTS

and the transmit flow control is CTS. Be aware that the COM port

address is one more than the starting channel number; for example,

if the starting channel is #4, the first COM port address is COM5.

3. Configure the Application Server entry to use INT 14 support, and

set the COM port to the one you defined with the XIDOSCFG utility.

RESTRICTIONS ON .SRV FILE

The application server list file (RLINK.SRV) will be updated the first

time the latest version of REMOTE LINK is used. After this update occurs,

the file is incompatible with previous versions of REMOTE LINK. The file

must be writable so the update can occur or you will get an error message.

If you are using REMOTE LINK with a release of DOS prior to 4.0, you must

have a unique, unshared, copy of your RLINK.SRV file or workstation printing

will not operate properly.

WORKSTATION PRINTING RESTRICTIONS

If you press the Main Menu Hotkey (default: SHIFT+F1) while a workstation

printer is printing, the connection between the application server and

the workstation printer is broken. The job that was printing will be

requeued; in order to restore the printer connection and resume printing,

you must terminate the current connection and reconnect to the application

server.

If you update a previous version of the application server without

performing a reformat, you must create the LPT2Q and LPT3Q print

queues on the application server if they do not exist. Failure to

do this may cause incorrect operation of Simplified Workstation

Printing with printers mapped to the application server LPT2 and

LPT3 ports.

USING MODEMS WITH REMOTE LINK

For asynchronous remote connectivity, Citrix recommends the use of

a multiport board on the host side that can support data rates up

to 115,200 bits per second (bps). This capability will allow

14,400 bps modems to run at a maximum data rate (with compression)

of 57,600 bps, and 28,800 bps modems to run at a data rate of

115,200 bps.  Adapters with this capability include the DigiBoard

X/em series, Computone IntelliPort II and IIex series,  Equinox

Super Serial Technology (SST), and Stargate AvanStar series

adapters.

MULTIUSER includes support for a wide variety of modems, supporting

base data rates (without compression) up to 28,800 bps. The REMOTE

LINK client program also supports these modems as well as many

PCMCIA modems not supported by the application server. The most

current modem compatibility information can be found on the Citrix

BBS, (305) 346-9004, or on the Citrix forum on CompuServe

(GO CITRIX).

The modem setup files in MULTIUSER and REMOTE LINK have a default

configuration baud rate setting of no greater than 19,200 bps. In

order to fully utilize today's high-speed modems, use the proper

client and host hardware and configure both the host workstation

and the client for a baud rate equal to four times the modem data

rate. For example, if you are using a 14,400 bps modem, set the

baud rate to 57,600. If you are using a 28,800 bps modem the baud

rate can be set to a maximum of 115,200, provided you have serial

ports capable of handling these speeds.  This allows the modem to

take advantage of its data compression features to increase the

effective data rate over the actual (physical) data rate.

If line conditions are poor, you may experience random line

disconnections. If this happens, try reducing the link (telephone

line) baud rate until you have better connection stability. It is

also a good idea, when possible, to use modems from the same

family or manufacturer for both the host and client sides.  Although

many manufacturers claim general compatibility for their modems,

modems from the same manufacturer are often more tolerant of one

another.

If you experience random disconnections or a high error rate,

follow these guidelines for resolving the problem:

1. From the Remote Link main menu, select "AppServer List"

and make sure that:

A. The remote client has the proper modem configured.

B. The baud rate is set for 19,200.

C. Error correction is enabled.

Press F4 to save the configuration. Try to establish a reliable

connection.

2. Try turning off Remote Link compression. Press F4 to save the

configuration. Try to establish a reliable connection.

After you can reliably establish a connection, try to increase the

baud rate as described above. Assure that both the host and the

client profiles are configured for the same baud rate.

SELECTING PC CLIENT SERIAL PORT SUPPORT

In order for a remote PC client to support high baud rates, the

client will need to have a serial port capable of handling the high

baud rate. Serial port performance has a crucial effect on overall

performance that far outweighs the attention it usually receives.

No amount of tuning can make a PC with an inadequate serial port

interface perform well in a remote access situation.

The burden placed on a PC's processor by an adapter is often

inversely proportional to the maximum data rate of the adapter.

Most high-speed interfaces, such as LAN cards, use hardware that

move large blocks of data with virtually no CPU intervention.

Because the interfaces are so fast, high-speed hardware on the

LAN card is a requirement for the PC to handle the data rate

without errors.

Serial interfaces, on the other hand, operate at relatively slow

speeds, which often can be serviced directly by the PC's CPU,

thereby eliminating the need for high-speed hardware on the adapter.

Until recently, most PCs were built with simple 8250 or 16450

serial controller chips (or UARTs). With these chips, CPU

interrupts are generated each time a character of data is sent

or received, reducing the amount of time the CPU can devote to

other tasks. The overhead incurred is acceptable at low baud

rates (9600 baud or less); however, higher baud rates impose an

unacceptable burden on the system.

When the CPU cannot keep up with the serial port, a PC experiences

data overruns, where a new character arrives before the CPU has

processed the previous one.  REMOTE LINK transmits data in blocks

known as frames. If a character is lost from a frame due to a data

overrun, the entire packet is resent, usually not until a lengthy

timeout interval has expired.

If the processor is much too slow, overruns occur in almost every

packet and the remote access connection fails.  If the processor

speed falls just short, complex interactions between the serial

port, the CPU, memory managers, and other system software may cause

intermittent overruns.  In this case, the remote access connection

may seem to work but have intermittent problems such as sudden

drops in speed, spurious loss of connections, and so on.  Such

inconsistent behavior is one of the most annoying problems faced

by both end users and support personnel.

In addition to the differences in performance between serial

controller chips, serial performance may also vary by operating

system.  For example, Microsoft Windows adds an extra layer of

software for handling CPU interrupts, forcing the processor to

execute extra instructions each time it is interrupted to receive

or transmit characters.  On the other hand, DOS, as the simplest

operating system, requires the least extra processing by the CPU

each time it is  interrupted.  Therefore, maximum serial port

throughput for a given hardware configuration usually occurs when

running DOS.

However, the simplicity of DOS may also lead to less consistency

because it provides no context for handling CPU interrupts.

Some low-level DOS programs, particularly memory managers, may

disable interrupts for extended periods of time. During the

intervals when interrupts are disabled, characters may be lost

because the CPU is not aware that it must service them.  The

need to support higher and higher serial port speeds has led to

the use of more sophisticated serial controller chips.  Three

such solutions are listed below, in order of their data rate

capabilities.

The first solution is to install a serial communications board with

a 16550A Universal Asynchronous Receiver/Transmitter (UART) chip.

A 16550A UART can receive or transmit up to 16 characters without

processor intervention; therefore, the processor is interrupted

less frequently and moves more data with less effort.  Adapters

utilizing a 16550A UART are available for about $50 and represent

a good solution for PC clients utilizing a 14,400 bps modem. Most

PCMCIA modem cards contain 16550A UARTs.

The next solution is to install a Hayes ESP Communications

Accelerator that allows you to send and receive data at speeds up

to 115,200 bps.  The ESP Communications Accelerator is a non-

intelligent adapter that provides automatic flow control and two

1024-byte buffers versus the 16550A's 16-byte buffer.  In addition,

it is fully backward compatible with conventional 16450 and 16550A

serial ports.  The Hayes ESP Communications Accelerator is available

for around $100 and is a good choice for connecting modems running

above 14,400 bps, such as the Hayes 28,800 Optima modem, that have

a maximum data rate with compression of up to 115,200 bps.

The last, and most expensive, solution is to install an

intelligent serial port card such as the DigiBoard 2-port ISA

adapter (PC/2e). This card has its own on-board processor (an

8MHz 80186) and 64KB of dual-ported RAM, which relieves the

processor of the burden of managing the serial ports.  The

DigiBoard intelligent adapter, or an equivalent intelligent

adapter, will provide the highest remote access performance. The

adapter must support either a standard COM port interface or an

INT14 interface to be compatible with REMOTE LINK.

RESTRICTIONS ON SCRIPT FILE SUPPORT

REMOTE LINK script files should be used only to automate the

connection to the application server. They may not work reliably

after the connection to the application server has been established.

RESTRICTIONS ON OS COMMAND PROMPT

No TSRs (Terminate and Stay Resident routines) can be loaded while

shelled out to DOS in REMOTE LINK using the "OS Command Prompt" option.

Any TSRs must be loaded before REMOTE LINK is started.

The "OS Command Prompt" option saves the current memory state before

starting the DOS shell, allowing REMOTE LINK to provide a much larger

memory space for the DOS session. If a TSR program is loaded, the TSR

may allocate memory, preventing the non-resident portion of REMOTE LINK

from being reloaded when the DOS session is exited. This will cause

a "Fatal Error ESH0001" error and REMOTE LINK will terminate.