WHATS IN A NAM                by The Mad Phone-man

---------------------------------------------------------

Nam stands for "Number Assignment Module" or to the Teckies a PROM

(Programable Read-Only Memory) A blank Nam usualy costs between $1. to

$2.75. Sometimes its more expensive depending on the operating temperature

and packaging specifications.

Two flavors of NAM's are used for cellular. NEC uses the open colector

(Signetics p/n 82S32 or equivalent). All others use the tri-state (Signetics

82S123 or equivalent). Blank Nams are manufactured by Signetics,National

Semiconductor, Monolithic Memorys, Fujitsu, Texas Instruments, and Advanced

Microdevices. Blank Nams can be purchased at your electronic distribuitor's

and many radios come with a blank included.

The NAM contains the subscriber number and lock code, the home system

identification and other system required information. You may wonder how this

info is arranged.

The NAM is organized into 32 rows and 8 columns. It is 32 words of 8 bits

each. (256 bits total) Starting from the top of the NAM (address 00)

you will find the abreviation SIDH, This means "system identification number

home" , a number starting at 0001 assigned by the FCC.

Each market allows two systems. Even for the wire-line and odd for

the non-wireline.

At address 03 we find LU (Local use) on the left and MIN on the right these

areusualy set to 1. Locations with zeros are reserved. Going down the map,

there's MIN1 and MIN2 the subscriber number and the area code respectivly

Dont try to read them from a raw printout of the NAM data, they are scrambled

beond recognition. The reason? The way they are arranged is the way they must

be transmitted to the cellular systems receivers. The programmer does this to

make the radio's job easier.

Next is the station class mark, which identifys the class and power capability

of the phone. The system will treat a handheld (low power) differently than

a standard 3 watt mobile.

IPCH is the inital paging channel. The radio listens for a page on this

channel. Wirelines use 334 and non-wirelines use 333.

ACCOLC (ACCess Overload Class)  is designed in throwing off customers in

the event of an overload. Thru neglect this standard has been largely unused.

(A class 15 station is supposed to be police, fire, or military)

Usualy its set to 0 plus the last digit of the phone number to provide random

loading.

PS- Prefered system. This is always 1 in non-wireline and 0 in wireline.

The lock code is about the only thing you can read directly by studying

the NAM data. The "spare" bit must be a 0 if the radio contains a 3 digit code.

Because the number of clicks when you dial 0 on a (dial) phone equals 10

zeros in the lock code are represented by an "A" the hexadecimal equiv of 10.

EE,REP,HA, and HF correspond to end-to-end signaling (DTMF tones possible

you talk) REPeratory dialing (provision for 10 or more numbers in memory)

Horn Alert and hands free. Like all options, they are 1, if turned on and

0 if turned off.

Addresses 13 thru 1D (all these numbers are in hex) are supposed to be used

by radio mfgrs to store option switches. Usualy 13 is used, 14 sometimes and

the rest less often.

Last you will find checksum adjustment and checksum. These numbers are

calculated automaticly after the data has been edited for the NAM. The sum

of all words in the nam plus these last two must equal a number with 0's

in the last two digits. The radio checks this sum and if it isnt correct

the radio assumes the NAM is bad or tampered with. In the case the radio

refuses to operate until a legal NAM is installed.

MARK            most        BIT SIGNIFICANCE       least        Hex

DEFINITION                                                      address

----------------------------------------------------------------------------

|    0         SIDH (14-8)                   |       00

----------------------------------------------------------------------------

|              SIDH (7-0)                    |       01

----------------------------------------------------------------------------

LU=Local use |   LU  |    0  0  0  0  0  0          | MIN |       02

----------------------------------------------------------------------------

| 0   0          MIN2 (33-28)                |       03

----------------------------------------------------------------------------

|   MIN2 (27-24)        |     0  0  0  0     |       04

----------------------------------------------------------------------------

|  0  0  0  0      |     MIN1 (23-20)        |       05

----------------------------------------------------------------------------

|                MIN1  (19-12)               |       06

----------------------------------------------------------------------------

|                MIN1  (11-4)                |       07

----------------------------------------------------------------------------

|        MIN1 (3-0)  |   0   0   0   0       |       08

----------------------------------------------------------------------------

|     0   0   0   0    |     SCM (3-0)       |       09

----------------------------------------------------------------------------

|     0   0   0   0   0   |   IPCH  (10-8)   |       0A

----------------------------------------------------------------------------

|          ICPH   (7-0)                      |       0B

----------------------------------------------------------------------------

|     0   0   0  0    |   ACCOLC  (3-0)      |       0C

----------------------------------------------------------------------------

PS=Perf Syst |     0   0   0   0   0   0   0   |   PS     |       0D

----------------------------------------------------------------------------

|    0   0   0   0   |     GIM (3-0)         |       0E

----------------------------------------------------------------------------

|   LOCK DIGIT 1       |   LOCK DIGIT 2      |       0F

----------------------------------------------------------------------------

|   LOCK DIGIT 3       |   LOCK SPARE BITS   |       10

----------------------------------------------------------------------------

EE=End/End   |   EE  |   0    0    0    0    0    0 | REP |       11

----------------------------------------------------------------------------

REP=Reprity  |   HA  |   0    0    0    0    0    0 | HF  |       12

----------------------------------------------------------------------------

HF=Handsfree |                                            |

HA=Horn Alt  |         Spare Locations (13-1D)            |

|         contain all 0's                    |       13

|                                            |       to

|                                            |       1D

----------------------------------------------------------------------------

|         NAM CHECKSUM ADJUSTMENT            |       1E

----------------------------------------------------------------------------

|            NAM CHECKSUM                    |       1F

----------------------------------------------------------------------------

##############################################################################################################################

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X