Cellular Phone File - #1

written, created and tested

by Count Zero

{CHiNA}

This simple (?) mod has been tested on the:

UNIDEN CS-1000/1200 Series Cellular

MPPS Red 12/13 (Pretty much same as above model)

and has proven effective for over four months running.  However, (yes, here

comes the big disclaimer...)

----------------------------------------------------------------------------

D I S C L A I M E R

CHiNA and its members claim no responsibility for irresponsible

use of the information and designs contained herein.  This file is being

presented on a "for knowledge's sake" basis to the members of the modemming

community at large.  Any use of this file except for educational and

operational efficiency purposes is hereby forbidden.

So there!

The Conflict * Maxwell Smart * Count Zero * Monalisa Overdrive * The Viper

& Rubiks the Cube

----------------------------------------------------------------------------

What this mod does is prevent a correct unit identification code (called UIC

from here on) from being transmitted.  The messages sent to and from the

local transmittal stations should be surpisingly familiar to any one of our

readers.

But here's the mod and a bit of theory that I used to discover it.

(1) Your individual UID is "burned into" a simple 8x8 EPROM that may

be erased and "re-written" to accomodate a new code.  This may be

difficult, and in fact IS difficult because you will have a lot of

trouble finding where it begins and ends.

(2) The contact sequence when you first power up the unit (which usually

goes on while the handset's "NO SERVC" or "SVC UNAVAIL" is lit) goes

like this:

YOU     A0 A0 A0 A0 A0 A0 A0 A0

IT      ACK or NAK (up to a max of 4 times)

YOU     12 3A + UID

IT      12 3A + UID

YOU     ACK or NAK

IT      00 00 00 or FF FF FF

(Available / Not Available)

The best route to handle this is to FORCE your system to ACK when asked

if a false code is its code.

The following should outline the procedure:

You will need:

* A Temperature-Controlled Soldering Iron

* Rosin-Core Solder

* Solder wick (for you slobs)

* Pair of Diag-Cutters (or wire-cutters)

* About 15 minutes of time.

Step 1 - Unplug the unit and allow to sit for at least a half hour to allow

all capacitors to become completely discharged.  Also, as a

precaution, "discharge" yourself on a common ground (no woolly

socks, ok?)  Remove cover from "handset" portion (yes, the one with

the keypad)

Step 2 - Locate the indicated EPROM should have a serial number that begins

with an "IA" prefix and will be noted on the circuit board as

"IC4" or "IC5".  Given this knowledge and the following picture:

+5v -!-------!- GND

-! IA... !- RST

-!       !-

+1.5v -!       !-

IC4       D1  -!       !-  D5

D2  -!       !-  D6

D3  -!       !-  D7

D4  -!-------!-  D8

...you should be able to find it.

Step 3 - Cut the D1 pin and pull completely back from the motherboard at

a 90 deg angle.  This will not interfere with your system messages

but will disable any "odd number" from being sent!  Thus your code

alone will come out false.

Step 4 - Locate the following components:

R14 - Resistor #14      1.5 ohm

Cut and jumper with solder and small gauge wire

R15 - Resistor #15      3.5 ohm

Cut and replace with 1.5 ohm from previous step

C22 - Capacitor #22

Cut and leave out!

Now make sure you have no "cold" joints and all soldered points are secure!

If you are going to screw up at any point in the procedure, this will be it.

Make sure to double-check your work!  I don't want anyone weeping to me

because their handset if now fused to their right ear!

Step 5 - (explanation of Step 4)

This step "forces" the system to send an ACK (by routing the NAK

trigger through ACK output) and thus verifying the bogus code.

Step 6 - Reassemble handset.

Just a hint, do NOT go overboard on your calls as these calls are not free,

they are just being billed to another person's code (if it is a legit code)

Again, re-read the disclaimer.

Step 7 - Operate the unit normally.

TROUBLESHOOTING:

Problem                 Solution

* NO POWER                      Be sure all power leads were reconnected

correctly when you put the handset back

together.

* STILL GETTING CHARGED FOR     Cut the correct pin from the IC!

CALLS                         If still getting charged, cut D2 as

well though this may be risky.

* CALLS "CAN'T BE COMPLETED"    Recheck mods made in Step #4.

AS DIALED or SYSTEM

UNAVAILABLE

Well, this should get you started.  A few notes before I go:

Thanks to The Conflict (for the inspiration), Maxwell Smart (for that "Smart"

report on Operation Wolf), Monalisa Overdrive (for letting me call him

repeatedly while testing this mod out!), Lord Blix (for the cracking help when

I needed it), The Viper (because he wants to be thanked)

Call on of our CHiNA nodes today for the latest in "knowledgable" text files

unlike other groups...

OVER AND OUT ---------> COUNT ZER0 !

+- Shamelessly Leeched from The Mudd Club -+

Press a key...

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Salted Slug Systems                      Strange              408-454-9368

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Tomorrow's 0rder of Magnitude            Finger_Man           408-961-9315

My Dog Bit Jesus                         Suzanne D'Fault      510-658-8078

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X