====================================================================

Wireless Network Management Sample Programs

for AIX NetView/6000 Version 1

(c) Copyright International Business Machines Corporation - 1994 -

All rights reserved

====================================================================

Contents

--------

A) Product Type and Conditions of Use

B) Prerequisites

C) Installation Procedure

D) IBM Wireless Management Sample Program Description

D.1) Basic Configuration

D.2) Sample Configuration Files (MIB Application Builder)

D.3) Event Configuration (Trap Customization)

E) Example of Utilization

F) Appendix: File List

A) Product Type and Conditions of Use

----------------------------------

The IBM Wireless LAN Utilities diskette contains the Wireless MIB, the

code point description file, the trap definition file, and a number of

ready-to-use Wireless Applications built through the AIX NetView/6000

MIB Application Builder (see File List in Appendix).

The product is provided as a list of SAMPLE files shipped on a

3.5 inch diskette (DOS format).

Since this software is provided as SAMPLE configuration files for

managing IBM Wireless cells, there will be no associated service.

These files are distributed without any IBM warranty. However, they

were reviewed and tested as any regular IBM software product.

B) Prerequisites

-------------

The Wireless Network Management Sample Program requires no prerequisites

in addition to those required by AIX NetView/6000. Wireless devices,

however, must already have been installed on your network.

Installation can be completed only on a RISC System/6000 where

AIX NetView/6000 Version 1 is installed.

To perform the installation procedure you must have the root privilege,

and the read-write map authorization.

C) Installation Procedure

----------------------

This procedure is divided into four major steps:

1. Install the AIX installation script:

a) Insert the IBM Wireless LAN Utilities diskette in the drive

b) Open an AIX window

c) Type the command: mkdir <dirname>

where <dirname> is the name of the temporary directory

where you want to copy the installation script

d) Type the command: cd <dirname>

to move to that directory

e) Type the command:

dosread -a /wlsample/nv6000v1/instnv1.aix instnv1.aix

to copy the installation script

2. Execute the installation script file by typing the command:

sh instnv1.aix

This script copies the sample files to the appropriate AIX

NetView/6000 directories, and updates the configuration

information.  This may take up to 5 minutes.

3. a) Erase the installation script by typing the command:

rm instnv1.aix

b) Remove the temporary directory by typing the commands:

cd ..

rmdir <dirname>

c) Remove the IBM Wireless LAN Utilities diskette from the drive.

d) Stop and restart AIX NetView/6000.

4. Load the Wireless MIB into the NetView/6000 database:

a) Select the Load/Unload MIBs... option from the Configure

pull-down menu. The Load/Unload MIBs dialog box shows all

currently loaded MIBs.

b) Check that the mib-2 MIB is loaded before loading the

Wireless MIB. If it is not loaded, do the following:

- Click on the Load... pushbutton

- Select /usr/etc/nm/mibs/mib-2 from the list

- Click on the OK pushbutton

c) Click on the Load... pushbutton

d) Select /usr/etc/nm/mibs/ibm-wlan.mib from the list

e) Click on the OK pushbutton

f) Exit the Load/Unload MIBs dialog box by clicking on the Close

pushbutton.

IBM Wireless Network Management Sample Program installation is now complete.

You can now traverse the MIB tree and select objects from the Wireless MIB

when you use the MIB Browser operation, configure AIX NetView/6000 to

collect MIB data, build new applications through the MIB Application

Builder, or run the applications built by the MIB Application Builder

that are part of the IBM Wireless package.

New menus will appear under the Monitor...MIB values menu.

Try one of these new menus.

D) IBM Wireless Network Management Sample Program Description

----------------------------------------------------------

The IBM Wireless Network Management Sample Program is mainly a menu driven

(pull-down menus) application offering specific management of the Wireless

Network workstations (configuration files).

Configuration for both wireless standard traps and specific traps is also

provided.

The purpose of these sample files is to offer basic Wireless Network

management using the standard NetView/6000 tools provided.

This complements the way Wireless MIB variables are retrieved using the

NetView/6000 MIB Browser tool.

You may decide to build your own files, using these sample files as

a reference.

D.1) Basic Configuration

-------------------

* MIB

Note: It was decided to use the following MIB subtree as a repository for

the IBM Wireless objects:

1 . 3 . 6 .   1    .   4   .     1     . 2 .  6    . 10

(iso.org.dod.internet.private.enterprises.ibm.ibmProd.IBM Wireless LAN)

* Community Name

The community names must be correctly set between the SNMP agent of the

base stations you want to manage and the NetView/6000 platform.

For detailed information about this configuration, refer to:

- "IBM Wireless LAN Installing and Operating Your Network" (SA33-0232)

- 'Configuring AIX NetView/6000 and SNMP Agents' in "AIX NetView/6000

Administration Reference" (SC31-6196).

* IP Address of Trap Receiver

The SNMP agent must be configured to send its traps to Netview/6000.

For detailed information about this configuration, refer to

"IBM Wireless LAN Installing and Operating Your Network" (SA33-0232).

D.2) Sample Configuration Files (MIB Application Builder)

----------------------------------------------------

These applications were built using the NetView/6000 integrated MIB

Application Builder tool (see "AIX NetView/6000 Administration

Reference" for more information).

When the Wireless Network Management Sample Program is installed, the

IBM Wireless LAN menu appears under the AIX NetView/6000 Monitor...MIB values

menu.  From this menu, user-friendly displays of IBM Wireless MIB

information are provided. The information is presented either as a table,

a form, or as a graph. Graphs are used for the display of network statistic

information.  Refer to the "AIX NetView/6000 Administration Reference"

(SC31-6196) for detailed information about the display of graphs.

Select a Wireless Base Workstation on the NetView/6000 Network or Segment

view.

From there, the different Wireless Network Management Sample Program

applications can be invoked by clicking on the IBM Wireles LAN menu item.

IBM Wireless information retrieved through the Wireless Network Management

Sample Program includes:

- Configuration of the selected wireless base station and its

associated wireless workstations

- Vital Product Data for the selected wireless base station and its

associated wireless workstations

- Various statistics.

The following information outlines the structure of the sample

applications provided from the NetView/6000 main menu bar, under the

Monitor...MIB Values...IBM Wireless LAN menu:

Monitor

--> MIB Values (Standard NetView/6000 menu)

--> IBM Wireless LAN (Wireless Network Management specific applications)

--> Base Configuration...

This form displays the configuration of the selected

wireless base station.

--> Base Adapters

--> Configuration...

This table displays the configuration and the status

of each wireless adapter in the selected wireless

base station. Each adapter is identified by a

number. This number is the interface number

locally assigned to each adapter by the operating

system of the base station.

--> Vital Product Data...

This table displays the Vital Product Data of each

wireless adapter (controller card and transceiver   )

in the selected wireless base station.

--> Statistics...

These applications display statistics for the

wireless adapters in the form of graphs. In each

graph, you can select the wireless adapter you're

interested in.  The graph shows four counters,

which allow you to monitor the efficiency and

the error rate of the radio channel.

Transmitted packets: This counter monitors the

number of radio packets sent by the adapter to

the wireless workstation it controls.

Received packets: This counter monitors the

number of radio packets received by the adapter

from the wireless workstations.

Retransmitted packets: This counter monitors the

number of radio packets that had to be re-sent

by the adapter because they were not acknowledged

by the wireless workstations.

Line errors: This counter monitors the number of

radio packets received by the adapter that were

discarded because they were corrupted during

transmission in the air.

--> Workstations

--> Configuration...

This table lists the workstations registered to

the base station you've selected. Each workstation

is identified by two numbers: One to identify

the wireless adapter of the base station that

controls that workstation, the other to identify

the workstation itself. For example, workstation

1.4 is controlled by adapter number 1 and has been

assigned number 4.

--> Vital Product Data...

The table provides the Vital Product Data of each

wireless workstation adapter (controller card and

transceiver).

--> Statistics...

This graph displays statistics about each wireless

workstation. To select a workstation, you must

provide both the number of the adapter that

controls it and the workstation number.

The graphed counters are similar to those in the

Base Adapter Statistics menu (with the exception

of line errors), but the values they provide

concern only the specific workstation you have

selected.

These counters allow you to monitor the efficiency

and the error rate of the radio channel between the

base station and a specific workstation.

D.3) Event Configuration (Trap Customization)

----------------------------------------

To better present events that take place in the network, an extended traps

configuration file (trapd.conf.append) is provided for easy management of

the IBM Wireless Network.

As described in the "AIX NetView/6000 Administration Reference" (SC31-6196),

the standard NetView/6000 Event Configuration Tool was used to generate

these configurations.

E) Example of Utilization

----------------------

For example, if you want to monitor the radio link between a

base station and a wireless workstation, assuming that

you know the IP address of the base station and the name of

the wireless workstation:

- Select the base station as the device that you want to manage.

- On the Workstations submenu (located under the

Monitor...MIB Values...IBM Wireless LAN menu), select the

Configuration... option.  In the displayed table, you can see the

list of the workstations registered to the selected base station.

- Note the Adapter number and the Workstation number on the same

line as the Workstation name.

- Click on the Close pushbutton.

- You can now select the Workstations...Statistics... option (located

under the Monitor...MIB Values...IBM Wireless LAN menu). A graph is

displayed.

- Then, select the desired workstation in the 'Table ID' field (the

Stop command, available by clicking the right mouse button, gives you

access to this control), with the following syntax:

".Adapter number.Workstation number"

for example: 1.4 if adapter number= 1 and workstation number=4.

- After using the Restart command, the corresponding statistics will be

displayed.

F) Appendix: File List

-------------------

-- Files and Directories ------------------- Contents --------------

->WLSAMPLE directory        Wireless Network Management Sample Programs

README.DOC              General Installation Information

->WLMIB directory         Wireless MIB

IBM-WLAN.MIB         Wireless MIB Definition File

->NV6000V1                Samples for AIX NetView/6000 V1

INSTNV1.DOC          This explanation file you're reading

INSTNV1.AIX          Installation Script (for AIX shell)

->WLMENUS directory    Applications and Menus Files

WLAN.BS            Description of the Wireless Base Station

WLAN.BSC           Adapter Cards Configuration

WLAN.BSV           Adapter Cards Vital Product Data

WLAN.BSS           Statistics on Base Radio Protocol

WLAN.WKV           Wireless Workstations Vital Product Data

WLAN.WKC           Wireless Workstations Description

WLAN.WKS           Statistics on Wireless Workstations

->WLTRAPS directory    Wireless Alerts

IBMWLALR.CNF       Wireless Alerts Definition File (Appending)

->WLCODPT directory    Wireless Codepoints

WLCODPTS.DES       Wireless Codepoints Definition File