AIOTCL.NLM Version 1.04

Technology Concepts Ltd.

(19th April 1995)

TCL AIO Driver

*requires AIO.NLM v6.0 or later

(The abbreviation TCL is used throughout this document to refer to

Technology Concepts Ltd.)

The TCL AIO driver is a NetWare Loadable Module (NLM) which allows the

AIO.NLM to make use of TCL asynchronous multiport cards. The driver has been

certified by Novell Labs for use with the TCL Superport-186 stackable port-

layer system and TCL's 8-port PCC/8i cards. Speeds of up to 115,200 bps are

supported on all ports. The driver supports up to eight TCL cards which at

the present time gives a maximum of 320 ports in one PC with 8 X Superport-186

cards using 5 port layers (40 ports) each.

The TCL cards have been assigned the following hardware type numbers by

Novell :

Superport-186       Type 136

PCC/8i              Type 137

In case of problems, please contact TCL :

Technology Concepts Limited,

Grange Estate,

Cwmbran,

Gwent,

NP44 3XR,

United Kingdom.

Tel: 01633 872611

Fax: 01633 872329

BBS: 01633 877916

Load directives

===============

The driver consists of three NLM's. They must be loaded in the correct order

as follows :

1)      LOAD TCALMBX

LOAD TCALOAD

LOAD AIOTCL

OR

2)      LOAD TCALMBX

LOAD TCALOAD {KEEP | NOKEEP}

LOAD AIOTCL

There are no command line parameters for either TCALMBX or AIOTCL.

The KEEP or NOKEEP parameter for TCALOAD indicates whether or not TCALOAD

should display a screen showing how many TCL cards have been set up,

along with the number of ports and other information. If NOKEEP is used,

TCALOAD exits after all the TCL cards have been initialised leaving just

TCALMBX and AIOTCL loaded (NOKEEP is the default).

A description of other command line parameters parameters for TCALOAD can be

seen by entering 'LOAD TCALOAD ?'. These specify alternative filenames for

configuration files which are used by TCALOAD. In the majority of cases,

these parameters will not be needed.

Note that the driver need only be loaded once as TCALOAD searches for and

initialises all TCL cards in the system at once.

In addition to TCALMBX.NLM, TCALOAD.NLM and AIOTCL.NLM, three other files

are required for proper driver operation. These are used by TCALOAD when

initialising and configuring the cards :

PCCASYNC.SYS    This is the 'firmware' loaded onto the TCL cards by TCALOAD.

PCCHW.DEF       Text file describing capabilities of TCL cards.

PCCID.BIN       Software which identifies the TCL card type.

Unloading and Reloading the Driver

==================================

If it is necessary to reload the TCL AIO driver, the driver must first be

unloaded as follows :

1.      If the loader was loaded using the KEEP option, switch to the loader

screen (using ALT-ESC) and exit by pressing any key.

2.      Unload the driver by typing the following commands at the server

console (note that NetWare will display an error message if you try

to unload TCALMBX before AIOTCL) :

UNLOAD AIOTCL

UNLOAD TCALMBX

Now the driver can be loaded again in the normal way.

Performance Issues

==================

The TCL AIO driver allows speeds of up to 115200 bps on all ports. The

driver supports the full AIO specification and the driver operates in both

NetWare 3.x and 4.x environments.

Once the driver has been loaded, it is not possible to change the buffer

sizes allocated to a port. The default buffer size is 2047 bytes for receive

and 2047 bytes for transmit.

The maximum transmit or receive buffer size is 4095 bytes.

The following minor hardware limitations apply :

Superport-186 :

1) 5 or 6 bit data is not supported at 115200bps.

PCC/8i :

1) Mark and Space parity modes are not supported.

Warning and Error Messages

==========================

If the loader program TCALOAD.NLM encounters any problems when trying to

initialise the TCL cards and the driver, it will in most instances display a

message on the screen. Use the KEEP option on the TCALOAD command line if any

problems are occurring. The following warning messages may be displayed :

"*Error* Need to unload TCALMBX.NLM before loading TCALOAD.NLM again."

This message is displayed if you are trying to reload the loader program

without having previously unloaded and reloaded TCALMBX.NLM.

"Couldn't open file filename"

This warning message means that the loader couln't find the named file in the

SYS:SYSTEM directory. In most cases, the loader will continue with driver

initialisation substituting default values for information that might have been

contained in the missing file.

"Warning : Superport-186 at address xxxxx is not connected/not switched on"

This message is displayed if the Superport-186 external interface box is not

connected to the card or is not switched on. The loader will continue loading

and configuring other TCL cards in the system but will not attempt to

configure any ports on this card.

Once the external interface box has been connected and switched on, the driver

must first be unloaded and then reloaded in order to be able to use the

Superport-186 ports.

"No TCL cards found in the system"

This message will be displayed on the loader screen if there are no TCL cards

installed in the server. Note that if the loader is invoked using the NOKEEP

option (which is the default), then this message will not be displayed and

the only indication of an error is that subsequently loading AIOTCL.NLM will

cause a "Module initialisation failed. Module AIOTCL.NLM unloaded." message to

be displayed on the server console.