TCL AIO Driver

User Manual

(c)Copyright 1994,1995

Technology Concepts Limited

No part of this publication may be reproduced without the

written permission of Technology Concepts Limited.

Technology Concepts Limited make no representations or

warranties with respect to the contents hereof and

specifically disclaims any implied warranties of

merchantability or fitness for any particular purpose.

Technology Concepts Limited,

Grange Estate,

Cwmbran,

Gwent,

NP44 3XR.

Tel: 01633 872611

Fax: 01633 872329

BBS: 01633 877916

Contents

Page

1   Introduction. . . . . . . . . . . . . . . . . . .   3

2   Driver Installation . . . . . . . . . . . . . . .   4

3   TCL AIO Driver Files. . . . . . . . . . . . . . .   6

4   Driver Operation. . . . . . . . . . . . . . . . .   7

5   Loader Options. . . . . . . . . . . . . . . . . .   8

6   The Driver Configuration File . . . . . . . . . .  10

6.1   Configuration File Syntax . . . . . . . . . . .  11

6.2   The SETPCC Paragraph. . . . . . . . . . . . . .  11

6.3   The SETPORT Paragraph . . . . . . . . . . . . .  13

6.4   Using The Board Number Override Option. . . . .  16

6.5   Example Configuration File. . . . . . . . . . .  17

7   Loader Warning Messages . . . . . . . . . . . . .  19

8   Reloading the driver. . . . . . . . . . . . . . .  21

1   Introduction

The TCL AIO driver is a NetWare Loadable Module (NLM) which

allows the AIO.NLM to make use of TCL asynchronous multiport

cards. The driver is fully compatible with both the NetWare 3.12

and NetWare 4.1 operating systems. The driver has been certified

by Novell Labs for use with the PCC/8i and Superport-186 cards.

These cards have been assigned the following hardware type numbers

by Novell :

Superport-186       Type 136

PCC/8i              Type 137

The abbreviation TCL is used throughout this manual to refer to

Technology Concepts Ltd. and its products.

This manual is intended to be used by experienced NetWare

administrators who will be installing the TCL AIO driver for use

with products such as NetWare Connect and NetWare Global MHS.

For full details on installing TCL hardware, refer to the

appropriate TCL hardware manual.

2   Driver Installation

Before installing the TCL AIO driver on a NetWare Server, it is

necessary to install the appropriate system software such as

NetWare Connect making sure that all appropriate patches have

been applied (obtained from NetWire on Compuserv or any other

legitimate source). At the time of writing (19/04/95), the most

up to date patches are the following :

NetWare Connect version 1.0 : PTF440.EXE and PTF442.EXE

These patches must be applied together to upgrade

NetWare Connect version 1.0 to version 1.0.33.

NetWare Connect version 1.0.33 : NWC001.EXE and NWC002.EXE

These patches upgrade NetWare Connect version 1.0.33 to version

1.0.34.

The TCL hardware (Superport-186 or PCC/8i) should also have been

installed. Generally speaking, this consists simply of setting

the address switches on the card, plugging the card into the

server bus and connecting the external interface box to the TCL

card using the supplied cable. See the hardware reference manual

supplied with each TCL card for more detailed installation

instructions.

The TCL AIO driver is installed logging on to a workstation as

SUPERVISOR and copying the files from the driver distribution

disk to the SYS:\SYSTEM directory on the communications server.

The procedure to install the driver is as follows :

1.  Copy the following files to the SYS:\SYSTEM directory

on the communications server :

AIOTCL.NLM

TCALOAD.NLM

TCALMBX.NLM

PCCASYNC.SYS

PCCHW.DEF

PCCID.BIN

2.  Edit the AUTOEXEC.NCF file on the communications

server to add the commands necessary to load the

driver. These lines should be added after the line

which mounts the SYS volume and before any attempt to

load COMMEXEC. The following should be sufficient in

most cases :

LOAD TCALMBX

LOAD TCALOAD

LOAD AIOTCL

Note that the TCL AIO driver does not have to be

loaded from within AUTOEXEC.NCF, the above LOAD

commands can be entered at any time at the server

console.

See section 5 for details of the options which can be

used when loading TCALOAD.

4.  The system administrator may also want to create a

driver configuration file. As already mentioned, this

is optional but it is, for instance, the only way to

specify the use of different sized transmit/receive

buffers from the default size of 2047 bytes. Once the

buffer sizes have been set during driver

initialisation it is not possible to change them

afterwards without unloading and reloading the entire

driver.

5.  Down the server and reboot the machine so that the TCL

AIO driver will be loaded when the server comes up (if

loading the driver from AUTOEXEC.NCF).

3   TCL AIO Driver Files

As well as the driver itself, a number of other files are needed

for successful operation of the driver. These files are all

included on the driver distribution disk :

AIOTCL.NLM   The TCL AIO driver.

TCALOAD.NLM  This NLM is referred to as the loader program. It

loads and initialises all TCL cards in the system

and initialises the main driver data structures.

TCALMBX.NLM  This NLM allows the driver and the loader to

communicate during driver initialisation.

PCCASYNC.SYS This is the program that runs on the TCL card

during normal operation. It is loaded onto the

card by the loader progam.

PCCHW.DEF    This is an ASCII file describing all currently

available TCL hardware. It is used by the loader

program when identifying the TCL cards in the

system.

PCCID.BIN    This is a program which the loader loads onto all

found TCL cards during driver initialisation. It

is used to identify the exact card type and the

number of ports on the card.

AIOTCL.CFG   This file is optional and allows the system

administrator to specify the default port

configuration for each individual TCL port. If

present, it is used by the loader program when

initialising the ports and the driver data

structures. An example file is supplied on the

driver distribution disk but it is not copied to

the server during installation as the contents of

the configuration file depend entirely on a

user's particular requirements.

All the driver files must be located in the server's SYS:\SYSTEM

directory.

4   Driver Operation

The driver can only be loaded once the SYS volume has been

mounted on the server. This would normally be done during

execution of AUTOEXEC.NCF when the server is booted. The simplest

way to load the TCL driver is to add the following three lines

to the AUTOEXEC.NCF file, making sure they are placed after the

line mounting the SYS volume :

LOAD TCALMBX

LOAD TCALOAD

LOAD AIOTCL

If they are not already loaded, two system modules (AIO.NLM and

CLIB.NLM) will automatically be loaded by the NetWare operating

system when TCALMBX is loaded.

The three NLMs must be loaded in the order shown. TCALMBX.NLM is

loaded first and it just provides shared memory which the driver

and the loader use to pass messages to each other while the

driver is being initialised. Next, the loader program TCALOAD.NLM

is loaded. Once loaded, it waits doing nothing until it receives

the first message from the driver (AIOTCL.NLM). Finally,

AIOTCL.NLM is loaded and it sends a message to TCALOAD telling

it to initialise the TCL cards and the driver data structures.

The loader program then searches for all TCL cards in the system,

it identifies the exact type of each card and finds the number

of ports on each card. If a driver configuration file exists (the

default name for this file is AIOTCL.CFG), the information in

this file is used to set up the initial configuration for each

port, otherwise each found port will be configured to a default

setting (see page 9). Note that during this phase of the loader

program's operation, it allocates memory for the driver data

structures by sending messages to the driver asking the driver

to obtain the memory from the operating system itself. This is

done so that the allocated memory will still be valid and useable

by the driver once the loader program has finished its job and

been unloaded.

The loader program then loads PCCASYNC.SYS onto each TCL card and

initialises each port. Having done this, the loader program sends

a final message to the driver to indicate that it has finished

initialising the TCL cards and the driver data structures. The

loader program then terminates.

At this point, the driver registers itself with the AIO.NLM and

the configured TCL ports are available for use.

5   Loader Options

When the loader program (TCALOAD.NLM) is loaded using the LOAD

command, some command line options are available. There are no

command line options associated with TCALMBX.NLM or AIOTCL.NLM.

Help on the various command line options can be obtained by

typing the following at the server console :

LOAD TCALOAD ?

Otherwise the command line syntax for TCALOAD is as follows :

LOAD TCALOAD [HWDEF=filename1] [IDFILE=filename2]

[CONFIG=filename3] [KEEP|NOKEEP]

The options can be specified in any order. Where an option

requires a filename, there should be no spaces between the "="

and the option or the filename. If no command line options are

specified or if the specified files do not exist, the loader will

use the following default filenames :

HWDEF :  PCCHW.DEF

IDFILE : PCCID.BIN

CONFIG : AIOTCL.CFG

If any of the default files do not exist, the loader will then

use its own internal defaults for the information associated with

these files.

If the KEEP or NOKEEP option is not specified, the loader

defaults to NOKEEP.

All files are assumed to be in the SYS:SYSTEM directory and it

is not possible to specify an alternative pathname.

The HWDEF and IDFILE options are unlikely to be needed in normal

circumstances as most users will not need to use anything other

than the PCCHW.DEF and PCCID.BIN files supplied on the driver

distribution disk. The only situation envisaged where these

options might be useful is if a user is planning to use the

driver with new TCL hardware not yet certified by Novell Labs.

The CONFIG option gives the name of the file used to specify the

default configuration for each TCL port in the system. It is not

necessary for this file to exist as, if it doesn't, the driver

sets each found port to the following default setting :

Baud rate   : 19200

Data bits   : 8

Stop bits   : 1

Parity      : None

Flow control: None

TX buffer size: 2047

RX buffer size: 2047

After initialisation, each TCL port is set to its default

configuration each time an application calls the AIOAcquirePort()

or AIOReleasePort() NetWare API routine.

The KEEP|NOKEEP option is used to specify whether or not to

display a screen showing how each TCL card has been configured.

If this option is not given on the command line, the default is

NOKEEP which means that this screen is not displayed and that the

loader NLM terminates as soon as it has initialised each card.

If KEEP is specified, the loader displays this screen until the

user decides to close it by pressing any key. An example screen

for a configuration using two TCL cards is as follows :

===============================================================================

TCALOAD.NLM (C) Copyright 1994, Technology Concepts Ltd.

Loader program for TCL AIO driver.

Waiting for TCL AIO driver to be loaded ....

.......................

The card(s) have been loaded as follows :

Card 0, 0xd6000, PCC/8i, Configured Ports : 8, Found Ports : 8,

Novell Type : 137, Executive : sys:system\pccasync.sys

Card 1, 0xd8000, Superport-186, Configured Ports : 24, Found Ports 32

Novell Type : 136, Executive : sys:system\pccasync.sys

<Press Any Key To Exit>

===============================================================================

6   The Driver Configuration File

The driver configuration file is a user-supplied ASCII text file

which specifies the configuration and port setup information for

all TCL cards in the system. A configuration file can be created

using any text editor. The name of the driver configuration file

is specified by the user when the driver is loaded (see section

5 - "Loader Options").

It is not essential to use a configuration file as all ports will

be initialised to default settings if no configuration file is

specified when the driver is loaded.

The main uses of the driver configuration file are as follows :

- To specify non-default transmit and receive buffer

sizes. The default buffer sizes are 2047 bytes for the

transmit buffer and 2047 bytes receive buffer.

- To limit the number of ports used on a particular

card. The default is to use all ports found on the

card.

- To specify which TCL cards in the system will be used.

The default is to use all found cards up to a maximum

of 8.

- To specify the default port configuration that each

port will be set to when a port is acquired using the

AIOAcquirePort() NetWare API routine. Note that this

default value may be overridden by other system

software such as NetWare Connect.

- To specify the use of a non-standard executive to load

onto a TCL card. The default executive (and the only

one approved by Novell Labs) is PCCASYNC.SYS.

- To give each card a name that will be registered with

the AIO module when the driver is loaded.

- To override the default card numbers assigned by the

TCL AIO driver. This feature is provided to overcome

a known problem with early versions of the AIO module.

6.1   Configuration File Syntax

The configuration file consists of paragraphs which begin with

key words identifying the paragraph type. The following paragraph

types are permitted :

setpcc

setport

The setpcc paragraph is used to indicate the card number, the

card name, the number of ports to use on that card and the name

of the executive to load onto the card. Each setpcc paragraph may

be followed by a number of setport paragraphs.

The setport paragraph is used to specify port setup information

for the ports on the card mentioned in the immediately preceding

setpcc paragraph.

The paragraph type key word is followed by a number of optional

parameters as described in the following sections.

Note that if there is more than one TCL card in the system, the

administrator must know the address of each TCL card in the

system in order to be able to create a configuration file. This

is because the cards are referred to in ascending address order.

Also note that the driver refers to the first card as card 0

whereas cards first card is referred to as card 1 in the

configuration file.

6.2   The SETPCC Paragraph

The setpcc paragraph has the following format :

setpcc    [cX] nports=Y exec=Z

name=S node=N

[cX]

Here X is the card number and can have a value

from 1 to 8. Note that cards are sorted by their

physical address with the card at the lowest

address being Card 1. The [cX] parameter must be

present.

nports=Y

The value Y is used to indicate how many ports to

use on this card. The nports parameter is

optional. If not present, the number of ports

actually found on the card will be used. Note

that the ports used always start at the first

port on the card.

exec=Z

Here Z is the filename of the executive to be

loaded onto the card. The path to the file should

be specified, including the volume name.

name=S S is a string (not enclosed in quotes) giving a

name to the card. This name is registered with

the AIO module when the card is loaded.

node=N This option is only required when the driver is

used with early versions of the AIO module. N is

a decimal integer specifying the card number for

this card. By default, the driver assigns card

numbers according to the card's memory address,

with the card at the lowest address being card 0.

Some examples of valid setpcc paragraphs :

setpcc [c1] nports=32 exec=sys:system\myexec.sys

name=pcc8i_0

setpcc [c2] nports=8 sys:system\pccasync.sys node=0

name=sp186_0

If the driver finds a valid configuration file, then only those

TCL cards mentioned in the configuration file are loaded with an

executive and used by the driver. If a configuration file is not

found, all found TCL cards are loaded with PCCASYNC.SYS and all

ports are set up to default values as described in the next

section.

Note that a minimal configuration file just contains setpcc

paragraphs with card numbers saying which cards are to be used.

For example, if only card 2 and card 5 are needed, then the

following configuration file might be appropriate :

setpcc [c2]

setpcc [c5]

6.3   The SETPORT Paragraph

The setport paragraph has the following format :

setport   [PN] baud=B, wordlen=W, stopbits=S,

parity=P, prot=F, rxbuff=R, txbuff=T

[PN]

The value N indicates the port being referred to

and can take any value from 1 to the number of

ports specified in the preceding setpcc

paragraph. A group of ports is specified using

the format [PN-M] (where M >= N).

baud=B

The following are valid values for B :

50, 75, 110, 134.5, 150, 200, 300, 600,

1200, 1800, 2000, 2400, 3600, 4800, 7200,

9600, 19200, 38400, 57600, 76800, 115200.

wordlen=W

This parameter specifies the number of databits

and the value of W can be any of the following :

5, 6, 7, 8.

stopbits=S

The value of S specifies the number of stopbits.

Permissible values :

1, 1.5, 2.

parity=P

P indicates the parity mode with the following

permissible settings :

none, even, odd, mark, space.

prot=F

This parameter specifies the flow control method

to use for the port. If more than one flow

control method is needed, this can be specified

by specifiying multiple values seperated by a

plus sign (e.g. txxon+cts+dtr). Any sensible

combination of the following values is allowed :

txxon     Transmit XON/XOFF.

rxxon     Receive XON/XOFF.

dtr       Receive DTR.

rts       Receive RTS.

dsr       Transmit DSR.

cts       Transmit CTS.

modem     Modem flow control.

hdx       Multidrop mode (half duplex).

none      No flow control.

If modem is selected, the only other flow

modes allowed in conjunction with this are

txxon and rxxon.

If hdx mode is selected, no other flow modes

are allowed.

rxbuff=R

The size of the receive buffer on the card is

specified by the value R. The following values

are permitted :

255, 511, 1023, 2047, 4095, 8191, 16383,

32767, 65535.

(for PCCASYNC.SYS, 4095 bytes is the maximum

buffer size)

txbuff=T

The value T specifies the size of the transmit

buffer on the card. The permitted values are the

same as for rxbuff.

The baud, wordlen, stopbits, parity, prot, rxbuff and txbuff

parameters are all optional and may appear in any order.

Examples of valid setport paragraphs are as follows :

setport   [p15] baud=9600, wordlen=7, parity=odd,

txbuff=511

setport   [p16-17] baud=38400, wordlen=8,

stopbits=1, parity=none, prot=rts+cts,

rxbuff=255, txbuff=511

setport   [p18-19] baud=19200

The driver initially builds up default port setup blocks in the

card memory for all ports and then goes through the configuration

file to alter these setup blocks as required before telling the

executive to start the initial port setup.

The initial default values are as follows :

baud=19200, wordlen=8, stopbits=1, parity=none,

prot=none, rxbuff=2047, txbuff=2047

Note that the setport paragraphs are dealt with sequentially and

that if not all parameters are specified in a setport paragraph,

it is assumed that the rest of them have already been set to a

valid value and they are not actively set to the default values.

Therefore if a port is named implicitly or explicitly more than

once, any parameter not mentioned in the latest setport paragraph

will not be altered from what it was set to in the last one.

This is illustrated in the following example where the end result

is that port 1 on a card is set to 19200 baud, 8 bits, 1 stopbit,

even parity, modem flow, 255 byte receive buffer and 1023 byte

transmit buffer :

setport [P1-8] parity=even

setport [P1] prot=modem, txbuff=1023

6.4   Using The Board Number Override Option

The board number override option (node=) is only necessary when

there is more than one TCL card in the system and the TCL cards

are not all of the same type.

Even then, this option is only necessary when the driver is used

with older versions of the AIO module (e.g. AIO version 1.02

supplied on the original NACS 3.0 distribution disks) which

expect the cards of each different hardware type to be numbered

starting from 0.

AIO.NLM version 6.0 and subsequent versions do not have this

problem so there is no need to use this option with these later

versions.

If there were two PCC/8i cards and two Superport186 cards in the

server, the driver would number them Card 0 to Card 3 according

to their address. However, the (old version of the) AIO module

would be expecting a Card 0 and a Card 1 for the two different

hardware types as shown in the following example :

===================================================================

Card              Novell H/W      Address    Driver       Card No.

Type                       Default      Override

Card No.

===================================================================

Superport-186      136            CE000         0            0

PCC/8i             137            D4000         1            0

PCC/8i             137            D6000         2            1

Superport-186      136            D8000         3            1

===================================================================

A minimal configuration file for the above setup would have four

setpcc paragraphs containing a board number override :

setpcc [c1] node=0

setpcc [c2] node=0

setpcc [c3] node=1

setpcc [c4] node=1

Note that the card numbers used in the [cx] part of a setpcc

paragraph starts at 1 whereas the driver card numbers are zero-

based. Also note that the administrator must know the address and

Novell hardware type of each TCL card in the system in order to

use the board number override option correctly.

6.5   Example Configuration File

An example configuration file called EXAMPLE.CFG is supplied on

the driver distribution disk. This file is not copied across to

the server during installation since the contents of a driver

configuration file depend entirely on the user's individual

needs.

In the example configuration, a system with a 32-port NACS

license has 16 TCL ports as follows :

Superport-186, two layers (16 ports) (Address D8000)

PCC/8i (8 ports)                     (Address D6000)

There may also be other ports which the NACS can use, such as the

server's COM ports, or serial I/O cards from other vendors. These

would have no effect on the TCL driver configuration file.

The system administrator decides to use all the ports on the

Superport-186 but only ports 1 to 4 on the PCC/8i. The example

configuration file shows how the administrator can set this up

and at the same time set individual ports and groups of ports to

default to different configurations when they are acquired.

Note that the PCC/8i card is at the lower address so it is

referred to as card 1 in the configuration file. Also, it is

assumed that an up to date version of the AIO.NLM is being used

so that there is no need to use the node= option.

setpcc      [C1]        nports=4, name=PCC/8i

setport     [P1-8]      txbuff=4095, rxbuff=4095

setpcc      [C2]        name=SP186

setport     [P1-2]      txbuff=4095, rxbuff=4095, prot=rts+cts,

baud=300, wordlen=8, stopbits=1,

parity=mark

setport     [P3-4]      txbuff=4095, rxbuff=4095,

prot=txxon+rxxon+rts+cts,

baud=2400, wordlen=7, stopbits=1,

parity=mark

setport     [P5-6]      txbuff=4095, rxbuff=4095, prot=txxon+rxxon,

baud=4800, wordlen=7, stopbits=1,

parity=mark

setport     [P7-8]      txbuff=4095, rxbuff=4095, prot=none,

baud=9600, wordlen=7, stopbits=1.5,

parity=mark

setport     [P9-10]     prot=none,

baud=19200, wordlen=8, stopbits=1,

parity=mark

setport     [P11]       prot=dtr+dsr,

baud=38400, wordlen=7,

parity=mark

Some points to note about this example file :

- The nports=4 statement is necessary for card 1 since

not all the ports on the card are being used.

- The txbuff= statement is the only way to achieve

buffer sizes other than the default (2047 bytes). The

buffer size is set when the driver is loaded and

cannot be altered afterwards without reloading the

driver.

- It is possible to set ports to use non-standard flow

control methods (e.g DTR/DSR instead of the standard

RTS/CTS) by means of the configuration file. This may

not be advisable since it could cause problems with

other system software which uses the TCL driver.

- No exec= statements are used so both cards are loaded

with the default executive (PCCASYNC.SYS).

- Ports 12 to 16 of the Superport-186 are set to the

default configuration (see page 9) since they are not

mentioned in any setport paragraph.

- Ports 5 to 8 of the PCC/8i are not useable due to the

nports=4 statement. These ports will not be registered

with the AIO module and do not exist as far as the

system is concerned.

It should be stressed that although the driver configuration file

specifies the default configuration that each port will be set

to when the port is acquired, the port may well subsequently be

set to a different configuration by the software which has

acquired the port.

7   Loader Warning Messages

If the loader program encounters any problems when trying to

initialise the TCL cards and the driver, it will in most

instances display a message on the screen. Use the KEEP option

on the TCALOAD command line if any problems are occurring. The

following warning messages may be displayed :

"*Error* Need to unload TCALMBX.NLM before loading TCALOAD.NLM

again."

This message is displayed if you are trying to reload the

loader program without having previously unloaded and

reloaded TCALMBX.NLM.

"Couldn't open file filename"

This message means that the loader couln't find the named

file in the SYS:SYSTEM directory. In most cases, the loader

will continue with driver initialisation substituting

default values for information that might have been

contained in the missing file.

If the file PCCASYNC.SYS is missing, this will cause the

loader to terminate unless a different executive has been

specified using the driver configuration file using an

"exec=" statement.

"Possible error in config file, line %d, error token = %s"

This message occurs if there is a syntax error or an

invalid value in the driver configuration file. The line

number is given as %d and %s is replaced by the string that

caused the error. Note that in some cases, the actual error

may have been caused by whatever came just before the

string shown in this message.

If the error is due to an invalid keyword, the loader

displays this message but keeps on reading the

configuration file until it comes across a valid keyword.

If the error is due to an invalid value (e.g. txbuff=8095),

the loader will use the default value for the appropriate

parameter.

"Warning : Superport-186 at address xxxxx is not connected/not

switched on"

This message is displayed if the Superport-186 external interface

box is not connected to the card or is not switched on. The

loader will continue loading and configuring other TCL cards in

the system but will not attempt to configure any ports on this

card.

Once the external interface box has been connected and switched

on, the driver must first be unloaded and then reloaded in order

to be able to use the Superport-186 ports. The procedure for

reloading the driver is described in the following section.

"No TCL cards found in the system"

This message will be displayed on the loader screen if there are

no TCL cards installed in the server. Note that if the loader is

invoked using the NOKEEP option (which is the default), then this

message will not be displayed and the only indication of an error

is that subsequently loading AIOTCL.NLM will cause a "Module

initialisation failed. Module AIOTCL.NLM unloaded." message to

be displayed on the server console.

8   Reloading the driver

If it is necessary to reload the TCL AIO driver, the driver must

first be unloaded as follows :

1.If the loader was loaded using the KEEP option, switch

to the loader screen (using ALT-ESC) and exit by

pressing any key.

2.Unload the driver by typing the following commands at

the server console (note that NetWare will display an

error message if you try to unload TCALMBX before

AIOTCL) :

unload aiotcl

unload tcalmbx

Now the driver can be loaded again in the normal way.

*End*