AIOCOMX.NLM Version 2.13

Novell Inc.

*requires AIO.NLM v6.0, or later

This driver can allow AIO applications to use the built-in

serial ports on the server or those of an add-in card that

emulates standard serial ports.  Up to four ports can be

supported.  Each port requires it's own interrupt.  Speed

of up to 19,200 bps can be supported if the ports use 16550

UARTS (2400 bps with 8250/16450 devices).

In case of problems please call Novell technical support at

800 NETWARE (800-638-9273).

Load directives:

1)  Entering 'load aiocomx' will cause COM1 (port=3f8, int=4)

to be loaded if the hardware is available.  Entering

'load aiocomx', again, will cause COM2 (port=2f8, int=3)

to be loaded if hardware is available.

OR

2)  Entering 'load aiocomx {optional parameters ...}' will

allow the user to customize the driver according to the

options described in the help screen.

Most users will deem these options as unnecessary for the

majority of cases.

The optional load parameters and their descriptions are listed

below:

PORT=W,	        where W is the hexadecimal port address at

which the device resides.

INT=X,	        where X is the interrupt number assigned to

the particular device port.

NAME=Y,         where Y is the user-defined name which

correlates to the board loaded.

NODE=Z,         where Z is the decimal number which will

override the hardware-defined board number

which the driver determines.  Ordinarily,

COM1 is assigned board 0, and COM2 is

assigned board 1.

FORCE,          where FORCE overrides hardware device

validation, resulting in the driver using

the specified hardware resources regardless

of whether they exist.

NOFIFO,         where NOFIFO will override the automatic

selection of 16550 FIFO mode, regardless of

whether 16550 FIFO mode capability has been

detected.

RXT=A,          where A is the default override value to use

for receiver data interrupt threshold, when

16550 FIFO mode has been selected.

TXQ=B,          where B is the default override value to use

for transmitter data queue, when 16550 FIFO

mode has been selected.

MAXRATE=C,      where C is the maximum speed rating override.

An operator may select this option if a device

which emulates an 8250, 16450, or 16550 UART

is actually capable of higher speeds than

provided by those devices.

Performance Issues:

If your system has a 16550 UART device present communications

performance can be significantly enhanced.  The AIOCOMX driver

automatically configures itself to run in enhanced (fifo) mode

if it detects the device.  The receiver data threshold and

transmit queues for the fifos can be changed by the user if

this mode is allowed, but their default values are probably

optimal.  If a 16550 device is not present in your system the

user is prohibited from attempting to place the driver in

enhanced mode.

Characteristics of this version:

1)  Only ONE board (which, for AIOCOMX, implies an individual

port) is loaded for each instantiation of the driver.

2)  Driver operates in both NetWare 3.x and 4.x environments.

3)  Driver offers enhanced performance if 16550 hardware is

available.

4)  Driver rated at 2400bps for 8250/16450 hardware.

5)  Driver rated at 19200bps for 16550 hardware.

6)  Operator may override the driver speed rating (up to

115200bps) for 8250-style devies which can operate at

higher speeds without losing data.

7)  Multiple command line options available at driver load

time.

8)  Assigned board numbers are based on hardware configuration.

(This can be overridden by the NODE= option, as mentioned

below.)

9)  Board numbering can be overridden to accomodate a bug in

an early version of AIO.NLM which required that boards for

a particular driver be loaded in board number sequential

order, starting with 0.

10) Driver command line load information is available to the

user by entering 'load aiocomx ?' at server prompt.

(The driver will not be loaded.)

-------------------------------------------------------------------------------

AIOCOMX Console Messages

-------------------------------------------------------------------------------

Message      8250/16450 device detected

Driver rated at 2400bps

Explanation  AIOCOMX detected an 8250/16450 UART device in the port and

set the maximum port speed to 2400 bps.

Action       If 2400 bps is an acceptable maximum port speed, no action

required.  If you know the port hardware will support a

higher speed, you can use the MAXRATE= command line parameter

with the LOAD AIOCOMX command to set a higher maximum port

speed.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      8250/16450 device detected

Driver rated at 2400bps - override to 'X'bps

Explanation  AIOCOMX detected an 8250/16450 device in the port and set the

maximum port speed to `X'. The MAXRATE= parameter in the LOAD

AIOCOMX command line set the maximum port speed to 'X'.  If

the MAXRATE= parameter is not used, the default maximum port

speed is 2400 bps.

Action       If the port hardware supports no action is required. If the

port does not support the displayed speed, unload AIOCOMX and

reload it with the correct parameter settings.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      16550 device detected - FIFOs disabled

Driver rated at 2400bps

Explanation  AIOCOMX detected a 16550 UART in the port hardware and a

NOFIFO parameter in the LOAD AIOCOMX command. AIOCOMX set

the maximum port speed to 2400 bps.

Action       If 2400 bps is an acceptable port speed, no action is required.

If you know the port hardware will support a higher speed,

there are two ways to select a higher speed. You can use the

MAXRATE= command line parameter with the LOAD AIOCOMX command

to set a higher maximum port speed, or you can reenter the

LOAD AIOCOMX command without the NOFIFO parameter to select

the default for this configuration, which is 19200 bps.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      16550 device detected - FIFOs disabled

Driver rated at 2400bps - override to 'X'bps

Explanation  AIOCOMX detected a 16550 UART device in the port hardware,

disabled the 16550 FIFO mode, and set the maximum port speed

to 'X'.  The NOFIFO parameter on the LOAD AIOCOMX command

line disabled the FIFO mode, and the MAXRATE= parameter set

the maximum port speed to 'X'. If the MAXRATE= parameter is

not used, the default maximum port speed is 2400 bps.

Action       If the port settings are acceptable, no action is required.

If you want to change port operation, you must unload and

reload AIOCOMX.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      16550 device detected - FIFOs enabled

Driver rated at 19200bps (RXT='R'/TXQ='T')

Explanation  AIOCOMX detected a 16550 UART device in the port hardware,

enabled the 16550 FIFO mode, and set the maximum port speed

to 19200 bps. The receiver data threshold is 'R', and the

transmit FIFO queue is 'T'. Standard values of 'R' and 'T'

are 4 and 16, respectively, unless they have been altered by

means of the RXT= and TXQ= parameters on the LOAD AIOCOMX

command line.

Action       If the port settings are acceptable, no action is required.

If you want to change port operation, you must unload and

reload AIOCOMX.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      16550 device detected - FIFOs enabled

Driver rated at 19200bps (RXT='R'/TXQ='T') - override to 'X'bps

Explanation  AIOCOMX detected a 16550 UART device in the port hardware,

enabled the 16550 FIFO mode, and set the maximum port speed

to 'X' bps. The default maximum port speed (19200 bps) for

this configuration was overridden by the MAXRATE= parameter

on the LOAD AIOCOMX command line. The receiver data threshold

is 'R', and the transmit FIFO queue is 'T'.  Standard values

of 'R' and 'T' are 4 and 16, respectively, unless they have

been altered by means of the RXT= and TXQ= parameters on the

LOAD AIOCOMX command line.

Action       If the port settings are acceptable, no action is required.

If you want to change port operation, you must unload and

reload AIOCOMX.

You can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      *Error* All 4 Ports Are Already In Use

Explanation  A LOAD AIOCOMX command attempted to configure a fifth port

configuration; AIOCOMX supports up to four ports.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       To support more than the four ports managed by AIOCOMX, you

must use another driver and install hardware compatible with

the driver.

-------------------------------------------------------------------------------

Message      *Error* Board De-registration Failed (AIODeregisterBoard()=ccode)

Explanation  An internal program error occurred while AIOCOMX was unloading.

AIOCOMX might or might not be loaded.

Action       Write down the message and code number, unload AIOCOMX and

AIO, and contact your Novell(R) Authorized Reseller.

-------------------------------------------------------------------------------

Message      *Error* Board Registration Failed (AIORegisterBoard()=ccode)

Explanation  Either an internal program error occurred or the LOAD AIOCOMX

command specified a board number that is already in use.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

create a new port configuration.

Action       Write down the message and code number.

If the code number is -19, reenter the LOAD AIOCOMX command

using a different board number with the NODE= parameter.  You

can view the command line load parameters by entering the

LOAD AIOCOMX ? command.

If the code number is a number other than -19, contact your

Novell(R) Authorized Reseller.

-------------------------------------------------------------------------------

Message      *Error* Driver De-registration Failed (AIODeregisterDriver()=ccode)

Explanation  An internal program error occurred while AIOCOMX was unloading.

AIOCOMX might or might not be loaded.

Action       Write down the message and code number, unload AIOCOMX and

AIO, and contact your Novell(R) Authorized Reseller.

-------------------------------------------------------------------------------

Message      *Error* Driver Registration Failed (AIORegisterDriverA()=ccode)

Explanation  An internal program error occurred.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Write down the message and code number and contact your Novell(R)

Authorized Reseller.

-------------------------------------------------------------------------------

Message      *Error* Illegal Value for Maximum Rate Override (MAXRATE=)

Explanation  The LOAD AIOCOMX command line specified an invalid value for

the MAXRATE= parameter, which sets the maximum baud rate for

the port. You can set the MAXRATE= parameter to 9600, 19200,

57600, or 115200.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Reenter the LOAD AIOCOMX port with a valid MAXRATE= parameter.

Note that AIOCOMX queries the communications hardware during

initialization, and sets the maximum speed to 2400 bps for

8250/16450 UART devices and 19200 bps for 16550 UART devices.

However, internal modems that emulate these UARTs might

actually be capable of higher speeds because of additional

buffering. If you know your internal modem has this capability,

you can use MAXRATE= to increase the maximum allowed port

speed. If you experience data loss to the port, the maximum

port speed is set too high.

-------------------------------------------------------------------------------

Message      *Error* Illegal Value for Receiver FIFO Threshold (RXT=)

Explanation  The LOAD AIOCOMX command line specified an invalid value for

the RXT= parameter, which sets the receiver FIFO threshold

value.  For ports that use or emulate a 16550 UART, you can

set the parameter to 1, 4, 8, or 14. (The default RXT= value

for 16550 devices is 4). For all other types of ports, the

RXT= parameter must be set to the default value, 1.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Reenter the LOAD AIOCOMX port with an RXT= parameter that is

supported by your port hardware.

-------------------------------------------------------------------------------

Message      *Error* Illegal Value for Transmitter FIFO Queue (TXQ=)

Explanation  The LOAD AIOCOMX command line specified an invalid value for

the TXQ= parameter, which sets the transmitter FIFO queue

value.  For ports that use or emulate a 16550 UART, you can

set the TXQ= parameter to 1, 4, 8, 12, or 16. (The default

TXQ= value for 16550 devices is 16). For all other types of

ports, the TXQ= parameter must be set to the default value, 1.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Reenter the LOAD AIOCOMX port with a TXQ= parameter that is

supported by your port hardware.

-------------------------------------------------------------------------------

Message      *Error* I/O port address 0x'X' does not appear to be a valid

asynchronous hardware device.  If you know this address to

be correct, you may use the 'force' option to override the

COM port existence tests.

Explanation  AIOCOMX could not validate the existence of the port at

address 'X'.  The port might or might not be available at

that address.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Verify that the port address on the LOAD AIOCOMX commannd

matches matches the port address for the port hardware and

does not conflict with any other server hardware.

If there is an address conflict or error, correct the problem

and reload AIOCOMX. If there is no conflict, add the FORCE

parameter to the LOAD AIOCOMX command line to suppress the

port validation and force AIOCOMX to use the port.

You can view the command line load syntax, which includes

the FORCE parameter, by entering the LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      *Error* Load Line Parse Failed (ParseDriverParameters()=ccode)

Explanation  AIOCOMX detected an error on the LOAD AIOCOMX command line.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Reenter the LOAD AIOCOMX command using the proper syntax.

You can view the command line load syntax by entering the

LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      *Error* Port Memory Allocation Failed

Explanation  The available server memory cannot support another port

configuration.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Add more memory to the server or increase the available

memory using one of the methods described in Appendix A,

_Resolving Memory Problems._.

-------------------------------------------------------------------------------

Message      *Error* Request for NetWare Resource Tags Failed

Explanation  AIOCOMX could not get required server resources. The server

is probably running out of available memory.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Add more memory to the server or increase the available

memory using one of the methods described in Appendix A,

_Resolving Memory Problems._.

-------------------------------------------------------------------------------

Message      *Error* Requested Board Name is Already In Use (name)

Explanation  The LOAD AIOCOMX command specified a board name (which

appears in parentheses) that is already in use.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Reenter the LOAD AIOCOMX command using a different board

name with the NAME= parameter. You can view the command line

load parameters by entering the LOAD AIOCOMX ? command.

-------------------------------------------------------------------------------

Message      *Error* Requested Board Number is Already In Use (board)

Explanation  The LOAD AIOCOMX command specified a board number (which

appears in parentheses) that is already in use.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

create a new port configuration.

Action       Reenter the LOAD AIOCOMX command using a different board

number with the NODE= parameter. You can view the command

line load parameters by entering the LOAD AIOCOMX ?

-------------------------------------------------------------------------------

Message      *Error* Requested Hardware Is Already In Use

Explanation  There is a hardware resource conflict between the requested

port hardware and another device. For example, both devices

might be configured to use the same port address or interrupt.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Locate the conflict by examining the LOAD AIOCOMX command

line, the system hardware configuration (jumpers and DIP

switches), and the system setup programs (for example, EISA

configurator).  Resolve the conflict and reload AIOCOMX.

-------------------------------------------------------------------------------

Message      *Error* Transmit/Receive Buffer Allocation Failed

Explanation  The available server memory cannot support another port

configuration.

If AIOCOMX is already loaded, the previously loaded port

configurations remain active; the current load command does

not create a new port configuration.

Action       Add more memory to the server or increase the available

server memory using one of the methods described in Appendix

A, _Resolving Memory Problems._.

-------------------------------------------------------------------------------

Message      For AIOCOMX Driver Load Information, Enter 'LOAD AIOCOMX ?'

Explanation  This message appears each time you attempt to load AIOCOMX

and explains how to get more information on loading AIOCOMX:

enter LOAD AIOCOMX ?

Action       No action is required.

-------------------------------------------------------------------------------

Message      *LOAD PROCEDURE FOR NOVELL'S AIO SERIAL COMMUNICATIONS PORT DRIVER

Explanation  This message appears when you enter the following command:

LOAD AIOCOMX ?.  This message includes additional text that

summarizes the load options for AIOCOMX; it does not load

AIOCOMX.

Action       No action is required.

-------------------------------------------------------------------------------

Message      Installed I/O Address 0x'X' (Interrupt Number 'N') as Board 'B',

Port 0.

Explanation  This message appears after AIOCOMX successfully loads.  The

message indicates the port configuration of the loaded driver.

'X' is the I/O port address, 'N' is the interrupt number,

and 'B' is the board number.

Action       No action is required.

-------------------------------------------------------------------------------