Equinox

7775 West Oakland Park Blvd.

Fort Lauderdale, Fl 33351

(305) 791-5000

Equinox P/N 560083

04/05/95

Release Notes

for

Equinox SuperSerial

SST-2I, SST-4I, SST-8I, SST-64I,    SST-128I,

SST-4E, SST-8E, SST-64E,    SST-128E

SST-4M, SST-8M, SST-64M and SST-128M

Host Controllers

Novell AIO Driver

Version: 1.10L1

Equinox P/N 650131

Equinox, Equinox SuperSerial are trademarks of

Equinox.  NetWare, NetWare Access and Novell are registered

trademarks of Novell, Incorporated.  All other brand and product

names are the trademarks of their respective holders.

Copyright (c) Equinox Systems, Inc 1995

All Rights Reserved

Information in this document is subject to change without notice

and does not represent a commitment on the part of Equinox.

Equinox provides this document "as is", without warranty of any

kind, either expressed or implied, including, but not limited to,

the particular purpose.  Equinox may make improvements and/or

changes in this manual or in the product(s) and/or the program(s)

described in this manual at any time.

This product could include technical inaccuracies or typographical

errors.  Changes are periodically made to the information herein;

these changes may be incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used,

disclosed, and/or copied only as permitted under such license

agreement.  Any copy must contain the above copyright notice and

this restricted rights notice.  Use, copying, and/or disclosure

of the programs is strictly prohibited unless otherwise provided

in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to

restrictions as set forth in sub-paragraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause of DFARS

52.227-7013.

1.0  Introduction

1.1  Overview

Equinox's support for Novell compatible asynchronous

applications is provided through a universal driver

conforming to the "AIO" (Asynchronous Input/Output)

specification.  Use the AIO driver when Equinox SuperSerial

hardware is being installed in the file server

(or machine with run-time NetWare 3.X or 4.X).

AIO drivers are loaded on the file server from the system

console (or RCONSOLE) command line or the AUTOEXEC.NCF file.

Installation is simply a matter of copying of the driver

file AIOEQNX.NLM. The user should consult the Hardware Manual

for SST SuperSerial Technology Expandable I/O Subsystem

prior to installing the hardware.

All hardware should be installed prior to loading the driver.

1.2  Compatibility

The Equinox SuperSerial AIO driver supports the Equinox

SuperSerial SST-2I, SST-4I, SST-8I, SST-64I, SST-128I, SST-4E,

SST-8E, SST-64E and  SST-128E boards and therefore can be used on

ISA, or EISA servers.

The Equinox SuperSerial AIO driver also supports the Equinox

SuperSerial SST-4M, SST-8M, SST-64M and SST-128M which are

used in Micro Channel servers.

The driver will support up to 16 Equinox SuperSerial SST-2I, SST-4I,

SST-8I, SST-64I, SST-128I, SST-4E, SST-8E, SST-64E or SST-128E boards on ISA and

EISA bus servers, with up to 2048 channels (ports).

The driver will support up to 16 Equinox SuperSerial SST-4M,

SST-8M, SST-64M, and SST-128M boards on Micro Channel bus servers,

with up to 2048 channels (ports).

The Equinox boards may be configured to use the same memory

address or may be configured for unique addresses.

2.0  Installation

To install the Equinox SuperSerial AIO driver, simply copy the relevant

files to your file server (although the driver can be loaded

from the file server floppy, this will not be a typical way to

set-up a system).  The Equinox AIO driver file (AIOEQNX.NLM)

should be copied to the system directory

of the file server, typically called F:\SYSTEM.  Copy the files

from a workstation to the file server with the commands:

COPY A:\AIOEQNX.NLM F:\SYSTEM

2.1  SST-2I, SST-4I, SST-8I, SST-64I or SST-128I Installation

Equinox SuperSerial SST-2I, SST-4I, SST-8I, SST-64I or SS128-I

boards may be installed in either ISA or EISA platforms.  In either

case, the driver must be loaded for each board you wish to support.

The syntax for loading the driver is as follows:

load aioeqnx {port=xxx} {mem=xxxxx} [name=bname] [node=0] [sm=m]

port=xxx :

This parameter is the board's 3 digit hex I/O

port address.

For example: port=200 (default).

Valid port addresses are

200, 220, 240, 260, 280, 2a0, 2c0, 2e0,

300, 320, 340, 360, 380, 3a0, 3c0 and 3e0.

Each board must have a unique I/O port address, which

must not be used by any other device in the system.

mem=xxxxx :

This parameter is the 5 digit hex starting address of

where the Equinox SSP board's memory will be mapped.

Memory needs a 16k byte window.

For example: mem=d0000 (default).

Memory addresses supported by the driver are

A0000, A4000, A8000, AC000,

B0000, B4000, B8000, BC000,

C0000, C4000, C8000, CC000,

D0000, D4000, D8000, DC000,

E0000, E4000, E8000, and EC000

Typically C0000 and DC000 will be the only

addresses available in your server.

All SSP boards may share the same

memory addresses, since only one board's

memory is mapped at a time.

name=bname :

This parameter is optional and allows you to

specify up to a 10 character name to identify

the board.  bname may consist of any combination

of alphanumerical characters.  If you do not

specify a name, one will be assigned by the driver.

Following is a list of the board's I/O port

addresses and the associated default names and

board numbers.

Port         Default    Board

Address      Name       Number

-------      --------   ------

200h         Equinox00     0

220h         Equinox01     1

240h         Equinox02     2

260h         Equinox03     3

280h         Equinox04     4

2a0h         Equinox05     5

2c0h         Equinox06     6

2e0h         Equinox07     6

300h         Equinox08     8

320h         Equinox09     9

340h         Equinox10    10

360h         Equinox11    11

380h         Equinox12    12

3a0h         Equinox13    13

3c0h         Equinox14    14

3e0h         Equinox15    15

node=0 :

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These

versions of AIO need sequential board numbers

based on the load order of the driver.  (The

first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)

Using the node=0 parameter informs the driver

to assign board numbers sequentially.  If this parameter

is excluded, the driver will assign board numbers

based on the I/O port address as defined above.

sm=m  :

This parameter is optional and allows you to define

higher baud rates beyond 115,200 bps. It is only

supported on the SST-2I, SST-4I, SST-8I boards

and ignore otherwise.

The software selectable speed multipliers adjust the top

speed from 1 times the baud rate of 115,200 bps to 8 times

the baud rate of 115,200 bps to result in the top baud

rate of 921,600 bps. Novell's AIO interface's top baud rate

is 115,200 bps, so to obtain higher baud rates, the lower

rates 50, 75, 110 are replace with values beyond 115,200 bps.

When using speed multipliers above 2 times, the number of

channels available for use are halved.

The value of "m" is the following:

SST-8I     SST-4I     SST-2I

Value  Top  Baud    50       75      110     Maximum    Maximum    Maximum

m       Rate      Baud     Baud     Baud    Channels   Channels   Channels

-----   --------    ----    ----     ----   ---------   --------   --------

*  1      115,200       50       75      110      8          4          2

2      230,400  230,400       75      110      8          4          2

4      460,800  230,400  460,800      110      4          4          2

8      921,600  230,400  460,800  921,600      2          2          2

Note: * is the default setting.

2.1.1  SuperSerial SST-2I, SST-4I, SST-8I, SST-64I and SST-128I

DIP Switch Settings

I/O (Base) Port Address:

S1, Switch

4   3   2   1

Address     --- --- --- ---

200        OFF OFF OFF OFF

220        OFF OFF OFF ON

240        OFF OFF ON  OFF

260        OFF OFF ON  ON

280        OFF ON  OFF OFF

2A0        OFF ON  OFF ON

2C0        OFF ON  ON  OFF

2E0        OFF ON  ON  ON

300        ON  OFF OFF OFF

320        ON  OFF OFF ON

340        ON  OFF ON  OFF

360        ON  OFF ON  ON

380        ON  ON  OFF OFF

3A0        ON  ON  OFF ON

3C0        ON  ON  ON  OFF

3E0        ON  ON  ON  ON

2.2  SST-4E, SST-8E, SST-64E or SST-128E Installation

Equinox SuperSerial SST-4E, SST-8E, SST-64E or SST-128E boards must

be installed in an EISA platforms.

Prior to loading the driver, the EISA configuration utility

must be ran. This defines the Equinox board type and memory

address. The Equinox ".cfg" files should be copied to the utility

disk which contains the EISA configuration software.

If the EISA configuration is not done or done incorrectly the

driver will display an error message for the board which was not

configured, the board can not be used and will be skipped if

attempted to load it. Since all the configuration for the

EISA boards is in CMOS, the user need not supply any port or

memory parameters, in doing so this would imply an ISA type

of board. Equinox SuperSerial EISA boards are loaded in slot

order, the lowest slot first. The syntax for loading the driver

is as follows:

load aioeqnx  [name=bname] [node=0] [sm=m]

name=bname :

This parameter is optional and allows you to

specify up to a 10 character name to identify

the board.  bname may consist of any combination

of alphanumerical characters.  If you do not

specify a name, one will be assigned by the driver.

Following is a list of the board's I/O port

addresses and the associated default names and

board numbers.

slot         Default    Board

Address      Name       Number

-------      --------   ------

0            Equinox00     0

1            Equinox01     1

2            Equinox02     2

3            Equinox03     3

4            Equinox04     4

5            Equinox05     5

6            Equinox06     6

7            Equinox07     7

8            Equinox08     8

9            Equinox09     9

10           Equinox10    10

11           Equinox11    11

12           Equinox12    12

13           Equinox13    13

14           Equinox14    14

15           Equinox15    15

node=0 :

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These

versions of AIO need sequential board numbers

based on the load order of the driver.  (The

first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)

Using the node=0 parameter informs the driver

to assign board numbers sequentially.  If

this parameter is excluded, the driver will

assign board numbers based on the I/O port

address as defined above.

sm=m  :

This parameter is optional and allows you to define

higher baud rates beyond 115,200 bps. It is only

supported on SST-4E, SST-8E  boards and ignore

otherwise. The software selectable speed multipliers

adjust the top speed from 1 times the baud rate

of 115,200 bps to 8 times the baud rate of 115,200 bps,

to result in the top baud rate of 921,600 bps. Novell's

AIO interface's top baud rate is 115,200 bps, so to

obtain higher baud rates, the lower rates 50, 75, 110

are replace with values beyond 115,200 bps. When

using speed multipliers above 2 times, the number of

channels available for use are halved.

The value of "m" is the following:

SST-8E     SST-4E

Value  Top  Baud    50       75      110     Maximum    Maximum

m       Rate      Baud     Baud     Baud    Channels   Channels

-----   --------    ----    ----     ----   ---------   --------

*  1      115,200       50       75      110      8          4

2      230,400  230,400       75      110      8          4

4      460,800  230,400  460,800      110      4          4

8      921,600  230,400  460,800  921,600      2          2

Note: * is the default setting.

2.3  SST-4M, SST-8M, SST-64M or SST-128M Installation

Equinox SuperSerial SST-4M, SST-8M, SST-64M or SST-128M boards

must be installed in a Micro Channel platforms.

Prior to loading the driver, the Micro Channel Setup utility

must be ran. This defines the Equinox board type and memory

address. The Equinox ".adf" files should be copied to the utility

disk which contains the Micro Channel Setup software.

If the Micro Channel configuration is not done or done incorrectly

the driver will display an error message for the board which was not

configured, the board can not be used and will be skipped if

attempted to load it. Since all the configuration for the

Micro Channel boards is in CMOS, the user need not supply

any port or memory parameters.

Equinox SuperSerial Micro Channel boards are loaded in slot

order, the lowest slot first. The syntax for loading the driver

is as follows:

load aioeqnx  [name=bname] [node=0] [sm=m]

name=bname :

This parameter is optional and allows you to

specify up to a 10 character name to identify

the board.  bname may consist of any combination

of alphanumerical characters.  If you do not

specify a name, one will be assigned by the driver.

Following is a list of the board's I/O port

addresses and the associated default names and

board numbers.

slot         Default    Board

Address      Name       Number

-------      --------   ------

0            Equinox00     0

1            Equinox01     1

2            Equinox02     2

3            Equinox03     3

4            Equinox04     4

5            Equinox05     5

6            Equinox06     6

7            Equinox07     7

8            Equinox08     8

9            Equinox09     9

10           Equinox10    10

11           Equinox11    11

12           Equinox12    12

13           Equinox13    13

14           Equinox14    14

15           Equinox15    15

node=0 :

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These

versions of AIO need sequential board numbers

based on the load order of the driver.  (The

first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)

Using the node=0 parameter informs the driver

to assign board numbers sequentially.  If

this parameter is excluded, the driver will

assign board numbers based on the I/O port

address as defined above.

sm=m  :

This parameter is optional and allows you to define

higher baud rates beyond 115,200 bps. It is only

supported on SST-4M or SST-8M boards and ignore otherwise.

The software selectable speed multipliers

adjust the top speed from 1 times the baud rate

of 115,200 bps to 8 times the baud rate of 115,200 bps,

to result in the top baud rate of 921,600 bps. Novell's

AIO interface's top baud rate is 115,200 bps, so to

obtain higher baud rates, the lower rates 50, 75, 110

are replace with values beyond 115,200 bps. When

using speed multipliers above 2 times, the number of

channels available for use are halved.

The value of "m" is the following:

SST-8M    SST-4M

Value  Top  Baud    50       75      110     Maximum    Maximum

m       Rate      Baud     Baud     Baud    Channels   Channels

-----   --------    ----    ----     ----   ---------   --------

*  1      115,200       50       75      110      8          4

2      230,400  230,400       75      110      8          4

4      460,800  230,400  460,800      110      4          4

8      921,600  230,400  460,800  921,600      2          2

Note: * is the default setting.

2.4  SuperSerial XP Port Modules

The host adapter cable must be attached to

the SuperSerial  SST-64I or SST-128I or SST-64E or SST-128E or SST-64M or

SST-128M board, as well as to the required Port Modules prior to

loading the NLM. The NLM will unload if no Port Modules are found.

Note: A one-to-one replacement of Port Modules is supported after the

NLM has been loaded. if additional Port Modules are required

you must unload the NLM "unload aioeqnx" , then re-load the NLM

"load aioeqnx".

3.0  Error Messages

The following error messages may occur during the installation of the

driver.

*Error* 1, Invalid EISA Configuration for Equinox SuperSerial board in slot(%d)

*Error* 2, EISA Configuration for slot(%d) id4(%x) id6a(%x) id6b(%x) id6c(%x)

id6d(%x)

EISA configuration was not done after installing the Equinox

SuperSerial board, or done incorrectly.

If id4(%x) = id4(0)  then

EISA configuration was not run, or the board was

removed during the configuration, or non valid memory address

was found for the board.

if id6a(%x) = id6a(0) or if id6b(%x) =  id6b(0) or

if id6c(%x) not equal id6c(0) or if id6d(%x) not equal id6d(0)

then an invalid address was chosen to map the board,

which is not in the range 64k to 1 meg.

*Error* 3, Invalid Micro Channel Setup for Equinox SuperSerial board in slot(%d)

Micro Channel Setup was not done after installing the Equinox

SuperSerial board, or done incorrectly. Check the memory address

which should be in the range 64k to 1 meg.

*Error* 4, Equinox SuperSerial Micro Channel board not found

Attempt to load an Equinox SuperSerial board which does not

exists.

*Error* 5, Requested Board Name Is Already In Use (%s) num (%d)

The name given is already in use, choice a different name.

*Error* 6, Requested Hardware Is Already In Use

The requested board is already loaded, check the configuration.

*Error* 7, Illegal Value for Board Number (NODE=)

Board number is not in the range (0-7), check value.

*Error* 8, Illegal Value for SM (Speed Multiplier) 1,2,4,8

Speed Multiplier is invalid, check value.

*Error* 9, Maximum number of Equinox SuperSerial boards (%d) Installed !

Maximum number of Equinox SuperSerial boards (16) already in use.

*Error* 10, All 2048 Ports Are Already In Use

Maximum number of Equinox SuperSerial ports already in use.

*Error* 11, Requested Board Number %d Is Already In Use

Check, the board number given on the "load statement"

*Error* 12, Board at port=%x is Not an Equinox SuperSerial Board

*Error* 13, Board at port=%x is Not an Equinox Board, Unknown Board type (%x)

Board at Port=%x, is not an Equinox SuperSerial Board, or

no board found at that port address. Check the IO Port switch

settings on the board.

*Error* 14, No channels Found, Check Port Module/Adapter Cable

The host adapter cable is disconnected or Port Modules

are not attached to the adapter cable.

*Error* 15, Request For NetWare Resource Tags Failed

*Error* 16, Board Registration Failed (AIORegisterBoard()=%d)

*Error* 17, Driver Registration Failed (AIORegisterDriver()=%d)

*Error* 18, Driver De-registration Failed (AIODeregisterDriver()=%d)

*Error* 19, Board De-registration Failed (AIODeregisterBoard()=%d)

System does not have enough resource for allocation, check system

configuration.

*Error* 20, Port Memory Allocation Failed

*Error* 21, Board Memory Allocation Failed

*Error* 22, SSP Memory Allocation Failed

System, does not have enough free memory for Data Structure allocation.

Check the available free memory on the system.

*Error* 23, Equinox PRAM memory test failure. SSP(%d) offset(%x)

*Error* 24, PRAM memory range appears to be cached. SSP(%d)

*Error* 25, SSP(%d) Not Detected.

*Error* 26, Equinox SSP DRAM word memory test failure page(%x) offset(%x)

Check the memory address, where the Equinox SuperSerial board is

mapped, for conflicts with other system devices, or possible

hardware problem.

*Error* 27, Command Line Parse Failed (ParseDriverParameters()=%d)

Invalid Command line given

*Error* 28, AIOEQNX Driver, System Type(%d) Not Supported

Driver loaded in a system which is not ISA, EISA or Micro Channel based.