Installation and Configuration Guide

Cubix MP2008 Multiport Communications Board

October 5, 1993

Novell, NetWare and Connect are trademarks of Novell, Inc.

1  Introduction

The Cubix MP2008 Multiport Communications Board is a PC-AT form

factor, ISA bus, state-of-the-art combination of high speed

controller and UARTs that offers system administrators fast and

reliable shared serial resources for "port controlled" networks.

Using multiple MP2008 boards in a NetWare Connect or Cubix

Connect communications system allows the system administrator to

manage a "port pool" of simultaneous dial-in and/or dial-out

asynchronous connections.

Up to sixteen MP2008 adapters may be placed in a single system,

provided sufficient slots and power are available.  The MP2008

requires a single 16-bit ISA slot for each adapter.

Connect communicates with the MP2008 via a supplied software

driver AIOCUBIX, that provides an AIO (Asynchronous Input/Output)

interface to various applications.  This driver is loaded in a

Novell NetWare or RunTime NetWare server, and provides the

software interface for access to the MP2008 ports.

The heart of the MP2008 is the 20MHz 80C286 processor with

optimized code to move data between Connect and the 16550 class

UARTs quickly enough to keep the ports operating at their full

115Kbps rating. The 16C550 UARTs with 16 byte buffers ensure that

there will be no overruns or missed characters at full rated

speed.

The MP2008 is provided with an octopus cable with a 78 pin

connector on the MP2008 end and eight 6 foot long cables

terminating in DB-25 male "D" connectors, wired as Data Terminal

Equipment (DTE), for connection to external RS-232 compatible

devices such as modems, multiplexors and mainframe terminal

ports.

This manual is intended for the network systems administrator,

and provides the following information:

- Theory of Operation

- How to configure and install the MP2008 hardware for use

- How to load the MP2008 AIO software driver

- How to troubleshoot problems with the MP2008

1.1  Theory of Operation

NetWare Connect operates as a enhanced subset of Novell's file

server software. NetWare Connect consists of core elements of the

file server software, the NetWare Core Protocol (NCP), supplied

as "Runtime NetWare", to support non file server services in the

form of NetWare Loadable Modules (NLMs), and the Connect NLMs.

Connect NLMs provide security and form the virtual links between

a NetWare service, such as Remote Node Service(RNS) and IPX, or

NASI Connection Services(NCS) and the outside world via an

Asynchronous Input/Output(AIO), module.

The MP2008 and its associated NLM driver, AIOCUBIX, form the

actual Input/Output hardware/software module that moves data

to/from the network under control of the Connect software.

The Connect software moves data to/from the MP2008 board(s) via

the ISA bus as a two byte "programmed I/O" data transfer using a dual

ported RAM data buffer and "mailbox" on the MP2008 located at a

defined Connect I/O port address. This dual ported RAM has,

for each of the 8 ports:

8KByte buffers for transmit and receive data and

"mailbox" area for status and control information.

The MP2008 80C286-20MHz CPU moves data and control/status

information to/from the UARTs and the Connect host CPU through

the dual ported buffer RAM.

2  Hardware Configuration

This section describes the configuration of the MP2008 hardware for

operation and how to install the board.

The MP2008 has a single switch bank labelled SW1 on the top edge

of the adapter, near the end bracket. This switch bank consists

of eight switches, numbered one through 8.  These switches should

only be changed while the system in which the MP2008 is located

is powered off. A switch is in the ON position when it is toward

the board.

DEFAULT is ALL ON, or TOWARD THE BOARD

2.1 Base I/O Address Selection

The switches in bank SW1 set the I/O port address and the logical

board number to be used by the MP2008.  Switches 1 and 2 control

the I/O port address, while switches 4-8 control the logical

board number.  Switch 3 is always ON.

The MP2008 requires a series of 16 I/O locations to allow

communications between the software driver provided and the

hardware.  Switches 1 and 2 set the base I/O address to be used

for this I/O address range.  The MP2008 supports four different

I/O address ranges (numbers given in hexadecimal):

Base I/O Address    Range     Switch 1  Switch 2

default   200       200-20F   ON        ON

210       210-21F   ON        OFF

300       300-30F   OFF       ON

310       310-31F   OFF       OFF

Select any base I/O address that does not conflict with

other system hardware.  If a base I/O address other than the default

is selected, this address must be specified using a command line

option when the MP2008's AIO driver is loaded.  See section 3.

ALL MP2008 BOARDS IN YOUR SYSTEM MUST USE THE SAME BASE I/O

ADDRESS.  Furthermore, I/O addresses used by the MP2008 must not

conflict the I/O addresses in use by any other system hardware.

2.2 Logical Board Number Selection

Each MP2008 board installed in a server must be assigned a unique

board number among all MP2008 boards in that system.  The logical

board number is used (along with a port number) to identify

individual ports when the software driver is loaded.  If two

MP2008 adapters attempt to utilize the same logical board number,

driver failures will result.

Any board number from zero to fifteen may be selected for use by

a board.  Cubix recommends that boards be assigned numbers

sequentially, beginning with board number zero.  Switch bank SW1

switches 4-8 are used to set the board number for the hardware.

The MP2008 software dynamically detects the board numbers that are selected,

so there are no command line options for board number

specification for the MP2008 driver.  The switch settings for the

logical board number are:

Switch Settings

Logical Board    SW4  SW5  SW6  SW7  SW8

default         0        ON   ON   ON   ON   ON

1        ON   ON   ON   ON   OFF

2        ON   ON   ON   OFF  ON

3        ON   ON   ON   OFF  OFF

4        ON   ON   OFF  ON   ON

5        ON   ON   OFF  ON   OFF

6        ON   ON   OFF  OFF  ON

7        ON   ON   OFF  OFF  OFF

8        ON   OFF  ON   ON   ON

9        ON   OFF  ON   ON   OFF

10        ON   OFF  ON   OFF  ON

11        ON   OFF  ON   OFF  OFF

12        ON   OFF  OFF  ON   ON

13        ON   OFF  OFF  ON   OFF

14        ON   OFF  OFF  OFF  ON

15        ON   OFF  OFF  OFF  OFF

2.3  Installation

Before installation of the MP2008, be sure to turn the system off and

disconnect it from the power source.  Inserting the MP2008 into a live computer

may damage the MP2008 hardware and may severely injure you.

The MP2008 must be installed in a 16-bit ISA or 32-bit EISA

compatible slot.  Be sure to observe the overall system power

requirements for the hosts hardware configuration.

After installing the MP2008 in an available slot, be sure to use

a screw to secure the edge connector of the MP2008 to the system

chassis.  Firm installation of the adapter board helps to ensure

that loose connections do not cause system unreliability or

damage to the hardware.

Next, connect the provided MP2008 communications cable to the

MP2008 board.  IT IS VERY IMPORTANT THAT THE HOLD-DOWN SCREWS ON

THE CONNECTOR BE FIRMLY ATTACHED TO THE MP2008 ADAPTER BOARD.

Failure to observe this requirement often results in misalignment

of the connector, and intermittent communications signals which

can cause abnormal communications session loss (or inhibition of

communications altogether).

Be sure to double check the installation and configuration of the

MP2008 hardware before powering on the system or proceeding to

software installation and configuration.

3  Software Installation and Configuration

This section describes how to install and configure the MP2008

software for use.

The MP2008 Multiport Communications Adapter includes an AIO

software driver that executes as a NetWare Loadable Module (NLM)

in a Novell NetWare server.  This NLM, called "AIOCUBIX.NLM", is

loaded like other NLMs, with the server software.

The MP2008 driver checks for hardware base I/O address conflicts

with other types of hardware in its host system.  If a conflict is

detected, a prompt will ask for another selection. Remember to set

the hardware switch settings using switches 1 and 2 of switch

bank SW1 to match the selected software options.

3.1  Loading the Software using the Default Hardware

Configuration

Using the default hardware configuration (all switches

in the ON position), means that no command line options are necessary to

load the AIO driver.

To load the NLM in this configuration, issue the command:

LOAD AIOCUBIX

to your NetWare server.  The driver name, version, and date are

displayed.

The AIO driver attempts to locate all MP2008 boards in the

system using the default I/O address.  As each board is located,

a message is displayed that indicates the self test results and

whether or not the board was successfully loaded.  If all boards

do not load successfully, check the I/O base address settings and

refer to the Troubleshooting section of this manual.

If all boards pass the self test and are successfully loaded, the

MP2008 driver loads and the MP2008 is ready for use.

3.2  Using Non-default Base I/O Address Configurations

If non-default settings for the I/O base address are selected(as

described in section 2.1), they must be specified when

you load the AIOCUBIX.NLM driver.  Use the command:

LOAD AIOCUBIX PORT=<base I/O address>

where <base I/O address> is "200", "210", "300", or "310",

corresponding to the settings for switches 1 and 2 of switch bank

SW1.  Alternatively, the <base I/O address> may be omitted,

including only the parameter "PORT=", the operator will be prompted

with valid I/O addresses.  I/O addresses already in use by other

components in the system are not included in the valid options list.

ALL MP2008 BOARDS IN THE SYSTEM MUST USE THE SAME BASE I/O

ADDRESS.  Furthermore, I/O addresses used by the MP2008 must not

conflict the I/O addresses in use by any other hardware in the system.

3.3  Board Names

The MP2008 board automatically sets a default board name of "MP2008".

To override this name and specify a name, use the command line option

"NAME=<board name>".  For example, the command line:

LOAD AIOCUBIX  NAME=MY_NAME

establishes MP2008 boards using the name "MY_NAME".  Cubix

recommends using the default board name for the MP2008.

3.4  Unloading the MP2008 Driver from Memory

The MP2008 AIO driver may be unloaded from servermmemory using the

command:

UNLOAD AIOCUBIX

There are no options for the unload command.  Be sure to close

any active connections before unloading the MP2008 driver.

3.5  Command Line Help

An on-line description of command line options is available for

the MP2008 AIO driver.  Use the command:

LOAD AIOCUBIX HELP

to view this command usage information.  Note that the driver is

not loaded if command help is requested. The driver must be loaded

as described in this chapter.

3.6  Verifying that the Driver has Loaded Successfully

In addition to the messages printed on the system console, the

MP2008 provides a rear panel indicator lamp, visible through the

bracket, that may be used to confirm that the driver has loaded

successfully.  The lamp states are:

OFF SOLID       - indicates that the driver is not present,

that is, it has not yet been loaded, or that it has been unloaded.

ON/OFF FLASHING - indicates that the driver is present and

operating

normally.

ON SOLID        - indicates that the board failed self-test or

that the

software load was unsuccessful.

If the lamp is in the solid-ON state, consult the troubleshooting

section for assistance in identifying the problem.

3.7  Troubleshooting

This section provides tips on troubleshooting installation and

configuration problems with the MP2008 hardware and software.

(to be provided)

Appendix A - Specifications

Software:

Software Interface: AIO (as an NLM), name=AIOCUBIX.NLM

Unloadable:         yes

Bit rates:          50, 75, 110, 134.5, 150, 300, 600, 1200,

1800,

2000, 2400, 3600, 4800, 7200, 9600, 19200,

38400, 57600, 115200 (bits per second)

Word lengths:  5, 6, 7, and 8 bits

Stop bits:          1, 1.5, 2 (as appropriate for word length)

Parity types:       none, even, odd, mark, space

Flow control types: none, hardware (RTS/CTS), software

(Xon/Xoff),

hardware and software

Port deadman timerssupported

RS-232 break signal generated and detected

Firmware:      dynamically loaded from driver software

Hardware:

CPU:           80C286 (20 Mhz)

UARTs:4 each,  16C552 (dual port)

Receive buffer:     16 Byte FIFO

Transmit buffer:    16 Byte FIFO

Number of ports:    eight total, asynchronous, RS-232

RS-232 connectors:  25-pin male D-connectors, via octopus cable

RS-232 signals:     TxD, RxD, RTS, CTS, DSR, Gnd, DCD, DTR, RI

System bus:         16-bit ISA slot; full length, AT form factor

System bus data rate:    4MBytes per second (32 Mbits per second)

8MHz bus, zero wait states

Read buffers:  eight, (8191 bytes), SRAM, hardware assisted wrap

Write buffers: eight, (8191 bytes), SRAM, hardware assisted wrap

Mailbox buffer:     one (16k bytes) SRAM

(read buffers, write buffers, mailbox: dual

ported)

Local program memory     16k bytes, zero wait states

System interface:   dual-port memory, via pre-fetched and post-

written I/O mapped port access

Supported I/O ranges     200-20F, 210-21F, 300-30F, 310-31F (one

required)

IRQs required: none

Memory ranges required:  none

DMA lines required: none

Power requirements: 2.0a   +5 volts

0.2a  +12 volts

0.2a  -12 volts

FCC certifications: Class A

Temperature:   0 to 40 degrees Centigrade (operation)

0 to 70 degrees Centigrade (storage)

Humidity:      10 to 95% (non-condensing)

Appendix B - Cable Specifications

B.1 RS-232 Cable Pinouts

The Cubix MP2008 includes an "octopus" shielded cable that

converts the 78-pin high density connector on the adapter to

eight DB-25 male RS-232 D-connectors.  All ports are wired as the

DTE interface.  The pinouts for the RS-232 connections are

standard:

1 - chassis ground

2 - transmit data      (TxD) Transmit Data - output

3 - receive data       (RxD) Receive Data - input

4 - Request To Send    (RTS) Ready To Send - output

5 - Clear To Send      (CTS) Clear To Send - input

6 - Data Set Ready     (DSR) Data Set Ready - input

7 - signal ground      (GND) Ground

8 - Data Carrier Detect     (DCD) Data Carrier Detect - input

20 - Data Terminal Read (DTR) Data Terminal Ready - output

22 - Ring Indicator          (RI)  Ring Indicate - input

Leads denoted as "output" are outputs from the MP2008.

Conversely, leads denoted as "input" are inputs to the MP2008.

All other pins are "No Connects"

B.2 Connection to Modems

Since modems are DCE devices and the MP2008 provides a DTE

interface, a straight-through cable is sufficient to connect a

modem to the MP2008.

At a minimum, connections should be provided on the leads noted

in section B.1 above.  Connections on all 25 leads are

acceptable.

B.3 Connection to Hosts, Workstations, PCs, or Terminals

Direct connection of the MP2008's RS-232 ports to hosts,

workstations, PCs, or terminals configured as DTE equipment

(without a intermediate modem or DCE device) generally requires a

"null modem" cable. A "null modem" cable is so called, because it

eliminates the need for modems (DCE devices) by crossing RS-232

signal pairs between the two DTE devices, such that each "sees"

the other as a DCE device.

To make a "null-modem" cable, use the following connections:

MP2008 side            other DTE side

2  ---------------------  3

3  ---------------------  2

4  ---------------------  5

5  ---------------------  4

7  ---------------------  7

6  -----|---------------  20

8  -----|

20  ---------------|-----  6

|-----  8