Bug Fixes:

The 11/11/92 VLSI SC486 BIOS Release fixes the following bugs:

Memory Detection - On certain VLSI 480 boards the BIOS incorrectly

found memory when memory was not present in a given bank.

Auto-memory detection was rewritten to take care of all cases of bus

floating.  The problem was never reported with any SC486 board

although the problem could possibly occur.

Speed switching - The board would hang during CTRL-ALT-DEL if the

board was running at slow CPU clock speed but would work during

power-up.

Speed switching routines modified to fix problem in this release.

Enhancements:

Fast Gate A20 option removed from setup.  This was done to enhance

the performance of the benchmarks.  The BIOS always uses the Fast

Gate A20 port instead of the keyboard controller.