TECHNOTE.DOC                                            09-02-95

----REACHOUT DISTRIBUTION FILES----

INSTALL.EXE - The installation program for DOS or Windows.

INSTALLD.EXE - The Installation program for DOS.

RCHDOS.EXE - The DOS remote control executive, also provides

Viewer PC the ability to see the Host running DOS text based

applications.

DOSGRAPH.EXE - Provides the Viewer PC the ability to see the

Host running DOS graphics based applications.

RCHFILE.EXE - Provides File Manager capabilities on Host to

Viewer in Windows or DOS.

RCHFILED.EXE - Provides File Manager capabilities on Host to

Viewer in DOS.

AUDIT.EXE - Print/Display Audit Trail in Windows.

AUDITD.EXE - Print/Display Audit Trail in DOS.

RCHKBD.DRV - The ReachOut Windows driver that provides the

intercept for other ReachOut drivers.

RCHKYB.DRV - The ReachOut keyboard driver for Windows, is a copy

of KEYBOARD.DRV.

RCHMSE.DRV - The ReachOut mouse driver for Windows.

RCHOUT.DRV - The ReachOut display driver for Windows,

includes ReachOut executive for Windows.

VRCHOUT.386 - Windows virtual device driver to support

communications (both Network and Serial).

VRCHSRL.386  -  Windows virtual device driver to support serial

communications.

REACHOUT.EXE - The Host Windows Menu and User Interface.

REACHOUT.GRP - Group used to add ReachOut Windows Menu to

Windows Program Manager.

RCHSRL.EXE - The DOS TSR communications interface used to

support Serial lines and Modems.

RCHACS.EXE - The DOS TSR communications interface used to

support Serial lines, Modems, and ACS devices

RCHIPX.EXE - The DOS TSR communications interface used to

support Novell Networks ( IPX broadcasts ).

RCHSAP.EXE - The DOS TSR communications interface used to

support Novell Networks ( SAPS ).

RCHNETB.EXE - The DOS TSR communications interface used to

support NetBIOS Networks.

RCHBANV.EXE - The DOS TSR communications interface used to

support Banyan Vines Networks.

RCHFTP.EXE - The DOS TSR communications interface used to

support FTP's TCP/IP Networks.

HOSTMDM.EXE / HOSTNET.EXE - The Host DOS Menu and User Interface.

VIEWMDM.EXE / VIEWNET.EXE - The Viewer DOS Menu and User Interface.

VIEW_FM.OVL - The DOS Viewer file manager overlay.

SETUP.EXE - Setup Utility to configure ReachOut in DOS or

Windows.

SETUPD.EXE - Setup Utility to configure ReachOut in DOS only.

PHONEBK.EXE - Utility to edit the dialing phone book in DOS.

PASSWORD.EXE - Utility to edit the password security list in Windows.

PASSD.EXE - Utility to edit the password security list in DOS.

SECURITY.EXE - Utility to configure security for a ReachOut Host

in Windows.

SECURD.EXE - Utility to configure security for a ReachOut Host

in DOS only.

BWCC.DLL - Windows library to support ReachOut Windows utiltites.

MODEMCFG.EXE - Utility to modify the modem configuration list in

Windows.

MODEMD.EXE - Utility to modify the modem configuration list in

DOS.

MTABLE.RCH - Modem configuration list containing all modems and

their setup strings currently supported by ReachOut.

RCHTERM.EXE - Terminal emulation program for Windows

RCHTERMD.EXE - Terminal emulation program for DOS.

RCHVSCAN.OVL - Provides Virus scanning protection for file manager

in DOS

LPT$VPN.047 - Data file that contains information on the latest

viruses to be scanned for.

RCHWSOVL.DAT - Data file for Viewer.

REACHOUT.PRM - Initial parameter file for ReachOut installation.

SERIAL.INS - List of files and their sizes copied down for

installation.

RACLIENT.EXE - Remote Access client interface, allows a Viewer

or Host to access Serveup drives.

RASERVER.EXE - Remote Access server interface, allows a Viewer

or Host to Serveup its drives to a Client.

SERVEUP.EXE - Remote Access interface that determines which

drives and / or directories to be accessed by a Client.

SERVEUP.SRV - Remote Access data file that contains Serveup

information.

RAMAP.EXE - Remote Access mappiing utility, for changing Serveup's

drive designations with RACLIENT loaded.

DISC.EXE - A DOS disconnect executible used to disconnect a Host from

a Viewer

DICSRBT.EXE - A DOS disconnect and reboot executible to disconnect a

Viewer and reboot a Host.

VDISC.EXE - A DOS disconnect executible used to disconnect a Viewer

from a Host while using Remote Access.

*.HLP - Help files for their respective ReachOut modules.

READ.ME - Latest release notes concerning ReachOut. Notes are

displayed at install time.

Files created by Installation / Operation

RCH_DATA.DAT - Temporary data file.

RCHSCOVL.DAT - Temporary Viewer data file.

REACHOUT.AUD - ReachOut Audit Log.

GHM.BAT - Batch file to load the Host software.

GVM.BAT - Batch file to load the Viewer software. UNLOADMH.BAT -

Batch file to Unload the Host software. INSRO.BAT - Batch file

to install ReachOut in Windows .INI files.

REACHOUT.PRO - Text file that lists the DOS directories to be

protected by the Host file manager from being accessed by the

Viewer.

PASSWORD.MDM / PASSWORD.NET- Password security file for allowing

access to ReachOut Host.

PHONEBK.MDM / PHONEBK.NET - Phone book for making ReachOut

connections.

MODEM.RCH - Current modem configuration and serial port

parameters.

EMULATOR.RCH - Current configuration for Viewer terminal

emulator.

---------------------------------------------------------------

Batch files created by ReachOut are:

for Modem:

GHM.BAT to invoke the Host DOS and Windows support.

GVM.BAT to invoke the DOS Viewer.

WINLDM.BAT to invoke Host or Viewer Windows support.

WINUNLDM.BAT to unload the Windows support.

GT.BAT  to invoke DOS terminal emulator

UNLOADMH.BAT to unload the DOS and Windows support.

for Network:

GHN.BAT to invoke the Host DOS and Windows support.

GVN.BAT to invoke the DOS Viewer.

WINLDN.BAT to invoke Host or Viewer Windows support.

WINUNLDN.BAT to unload the Windows support.

UNLOADNH.BAT to unload the DOS and Windows support.

---------------------------------------------------------------

If your PC will be remotely controlled quite often as the Host

PC, you may want ReachOut Host loaded automatically each time

you reboot or turn on your computer.  Having the software load

every time the machine is booted insures that if the Host user

needs support, help is only a phone call away.  By loading

ReachOut in the AUTOEXEC.BAT, the Host does not have to leave an

application where there is a problem in order to load the Host

software, recreate the problem, and finally have the Viewer call

in to offer support.  It's more practical to have the Viewer be

able to call in at any time and provide assistance whenever the

Host requires it.

This is also useful if you will be remotely controlling an unattended

Host PC...

Perhaps the Viewer wants to use the option to reboot the Host PC

upon disconnect but still would like to call in again later.

Or the unattended Host PC may suffer a loss of power.

In either of these cases you may want the Host software to load

automatically after the PC reboots.

By making modifications to your Host AUTOEXEC.BAT file, you can

load ReachOut Host automatically.  Using a text-based editor,

add the following line to your AUTOEXEC.BAT file (This line

assumes ReachOut was installed on your C: hard disk drive):

CALL C:\REACHOUT\GHM.BAT

or

CALL C:\REACHOUT\WINLDM.BAT

If you are loading Windows via the AUTOEXEC.BAT file, you should

load ReachOut immediately after your networking software has

been loaded and, of course, before Windows loads.  This will

reduce the  chance of other TSR applications interfering with

ReachOut's communications to your networking software.

--------------------------------------------------------------------

The Communications Interface provides the link between the Host

and the Viewer computers.  For the modem version of ReachOut,

RCHSRL.EXE is the communications interface, it interacts with

your modem.  It uses parameters found in the MODEM.RCH file.

The MODEM.RCH is created at installation time and can be modified

by SETUP.  The RCHSRL.EXE is a TSR (terminate and stay resident)

program, and is invoked as a command in the GHM.BAT and GVM.BAT

files that were created by ReachOut INSTALL.

When RCHSRL is loaded, ReachOut looks at the MODEM.RCH file

(that was created by INSTALL or SETUP) and attempts to

initialize your communications port.  Commands are then sent to

your modem to put it into the correct state required by ReachOut.

For this reason, your modem should be turned on when ReachOut loads.

If it is not, you can initialize the modem by selecting the RESET

option under LINK in the ReachOut menu.

The following switches are available for RCHSRL, RCHACS, RCHIPX,

RCHBANV, RCHSAP, RCHNETB, RCHFTP:

NAME=

This name, up to 15 characters long, will be used for ReachOut

connections.  Your workstation will be known to others when

connected by this Name.

EXAMPLE:

RCHSRL NAME=SHARON

----------------------------------------------------------------

PERFORMANCE

If you are going to be doing a lot of remote control, especially

in the Windows environment, the benefits of using a 9600 or

14,400 baud modem with V.32bis capabilities cannot be emphasized

enough.  These modems are typically four to twelve times faster

than a standard 2400 baud modem. Prices of these modems have been

dropping.  If your connection involves paying long-distance

charges, you can easily cost-justify a 14,400 baud V.32bis modem.

Many V.32 style modems transmit data at a rate of 9600 bps

between modems, but realize higher throughput by using data

compression. Some V.32 modems are capable of transmitting data

at a rate of 14,400 (14.4K) bps between modems (50 percent

faster than their 9600 bps counterparts). These modems work

extremely well with ReachOut. Of course, both the Host and

Viewer PC would have to have modems capable of achieving this

data rate.  At connect time, the two modems will interrogate each

other for their capabilities and connect at the speed of the

slower of the two modems.

If your modems communicate at speeds of 9600 or 14.4K bps, they

most likely have built-in compression techniques to achieve even

faster throughput, depending on the data stream being transmitted.

In order to take advantage of these compression capabilities,

the speed at which your computer transmits to the modem must be

faster than the speed the modem transmits on the phone line.

Therefore, you would want your transmission speed (your ReachOut

data rate) to be 19.2K or 38.4K baud. In general, the

transmission speed of 19.2K should be used. A transmission speed

of 38.4K can be used to achieve even higher throughput, but you

must be cautious when using it.

Running at 38.4K has limitations depending on the speed of the

Host PC and the software applications that are being run on it.

If the HOST PC can't keep up with data being transmitted at

38.4K, then data will be lost and can actually cause throughput

to DECREASE. If your PC is not quick enough to keep up with

the baud rate, you will experience "overrun" errors in the

"Link" "Modem Diagnostics" or "Link" "Comm Stats" menu of

the Host or Viewer.

When the problem arises that the CPU can not keep up with data

being received on the serial port, (this is especially true

with Windows running and baud rates of 38.4 and greater), it

is suggested that you purchase a serial port card with a

buffered UART. These UARTs are known as a 16550 series UART.

Many systems come standard with them or they may already be

built into internal modems, to check the type of UART go to

the "Link" "Modem Diagnostic" menu on the Host or Viewer and

verify your UART type.

NOTE:  Problems may exist when running ReachOut at 38.4K while

remotely controlling Windows 3.0 on a Host PC with a 386DX 16MHz

or 386SX 16MHz processor. The symptoms of this problem include

jumpy, uncooperative mouse operations on the VIEWER or very slow

file transfer when copying from the VIEWER to the HOST. The

problem is that the computer is not able to keep up with the

data rate and information is being re-transmitted. The solution

is to back-off the communications speed to 19.2k or even

possibly 9600 baud.

---------------------------------------------------------------------

ADVANCED COMMUNICATIONS

There are three modes used by RCHSRL for establishing a

connection:  WATCHCARRIER, IGNORECARRIER, and OBEYCARRIER.

These can be set as an Advanced Communication Option in SETUP.

WATCHCARRIER:  This is the default mode for RCHSRL. In this mode

the Viewer will become CONNECTED if it sees the Carrier Detect

signal High.  The Viewer will also become CONNECTED if it sees

the string "CONNECT" received.  The string "CONNECT" is typically

returned from most modems when they become CONNECTED.  The Host

will look for the Carrier Detect signal to go from lo -> hi and

it will also look for the string "CONNECT" to determine when to

go online.  At this time, it will send out a ReachOut data packet

and proceed to remote control mode.  If the modems can't connect

in this default mode, proceed to the IGNORECARRIER mode.

OBEYCARRIER:  If the user knows that he will get a valid Carrier

Detect signal when he CONNECTs, and does not want ReachOut

looking for the string "CONNECT" to determine a connection, then

this mode should be used.  ReachOut has supported this mode

back to vers. 2.1.  When the Viewer sees Carrier Detect, he

will go into Terminal mode looking for a ReachOut data packet.

When the Host sees Carrier Detect he will go online and send out

a ReachOut data packet.  They will then become connected.

IGNORECARRIER:  If the user does not want his Carrier Detect

signal to be used to determine a CONNECT or his modem does not

return the string "CONNECT" when it goes online, then this mode

should be used.  The Viewer will become connected immediately and

go into Terminal mode.  It will remain in terminal mode, at which

time interaction can take place with TTY devices (the modem,

muxes, modem pools), until a ReachOut data packet is received

from the Host.  It will then proceed to ReachOut remote control

mode.

The Host will NOT use Carrier Detect to determine CONNECT.

The Host will look for the string "CONNECT" before it goes

online.  When "CONNECT" is received it will send out a ReachOut

data packet and go into ReachOut remote control mode.  The one

disadvantage of using this mode is that the Viewer will not

detect modem errors when dialing, those errors would be

returned in a raw state to the Terminal mode.

If the IGNORECARRIER and WATCHCARRIER modes are being used and

the Host is not seeing a "CONNECT" string from the modem (it may

not return one in some cases, ie. foreign modems), then the Viewer

can attempt a transition to ReachOut remote control mode by

hitting <F10>.  <F10> will send the "CONNECT" string to the

Host which should kick everything off, if the Host and Viewer

are actually physically connected.

---------------------------------------------------------------------

In a direct connect scenario (configured for direct connect,

null modem) the Viewer will become CONNECTED immediately and go

into Terminal mode.  It will remain in terminal mode, at which

time interaction can take place with TTY devices (muxes, modem

pools), until a ReachOut data packet is received from Host. It

will then proceed to ReachOut remote control mode.

The Host will NOT be able to call a Viewer in a direct connect

setup, unless the Carrier Detect signal is available.

If the Host and Viewer fail to connect, put both of them in

diagnostic mode and dial the other manually.  You should be able

to establish a connection between the modems.  With the

connection still active, the Viewer should exit diagnostic

mode and go into "Connect To Host".  Selecting any number should

connect the Viewer and Host.

----------------------------------------------------------------------

Host

When ReachOut is installed, the GHM batch file is created.  The

installation procedure includes in this batch file a number of

TSRs to implement your ReachOut session.  There is only one TSR

required to be loaded if you are only going to remote control

Windows, the communications interface RCHSRL.  This TSR will

consume approximately 11k of conventional memory.

The following TSRs are required for ReachOut to provide DOS

remote control capabilities:

RCHSRL

RCHDOS

They must be loaded in that order and will consume approximately

32k of conventional memory when loaded by DOS.  There are three

other TSRs which provide additional capabilities to ReachOut but

are not required:

HOSTMDM - provides the capability to "pop-up" on the Host and

modify session parameters as well as perform user interface

functions.

DOSGRAPH - provides the capability for the Viewer to

see DOS graphics applications.  It requires approximately 13k of

conventional memory when loaded by DOS.  If this TSR is not

executed in the GHN batch file, then the Viewer will get a text

message on his screen informing him that the Host has executed a

DOS graphics application.  The Viewer will return to normal when

the DOS graphics applications is exited on the Host.

RCHFILE - provides the capability for the Viewer to have

background access to the Host's files.  It consumes 6.4k when

using a 1k I/O buffer.  The I/O buffer can be set in increments

of 1, 2, 4, or 8k.  Increasing the I/O buffer can significantly

improve file transfer speeds, but will increase the amount of

conventional RAM it consumes when loaded.  RCHFILE consumes from

6.4k to 13.4k depending on the I/O buffer size.

- File Transfer can always be performed as a foreground

application, saving this memory. As a foreground application,

the user will be interrupted until the file transfers have been

completed.

Current approximate memory usage by the ReachOut Host software

is as follows:

RCHSRL   - 15k

RCHDOS   - 24k

DOSGRAPH - 10k

HOSTMDM  -  7k

RCHFILE  -  7K (1k buffer size)

All of these TSR programs for the ReachOut Host can be loaded in

HIGH memory when using QEMM, 386max, or DOS 5.0, thus freeing up

conventional memory.

All of these TSR programs can be unloaded from memory using the

"UNLOAD" parameter on their respective command lines.  INSTALL

creates an UNLOADMH.BAT file that will unload all the TSRs

automatically.

----------------------------------------------------------------------

TSR Conflicts

Check that the hot keys used by the programs do not interfere

with each other.

Avoid loading other TSR applications after loading your

networking software and before loading ReachOut, this may

cause problems with ReachOut file manager capabilities.

----------------------------------------------------------------------

DOS VIEWER NOTES

The Viewer executable is a stand-alone DOS application which

requires 250k of free memory to load.  Additionally, a 512k disk

cache file is created when the Viewer loads and two more

temporary data files totalling 512k will be created. Your

private ReachOut directory must have 1 megabyte of free disk

space in order to operate the Viewer correctly.

VIEWMDM.EXE runs on the Viewer workstation and allows the

support workstation to view the Host workstation.  The Viewer

workstation does not require that the Viewer machine have

Windows.

To start up the Viewer session, run the GVM batch file in the

\REACHOUT subdirectory.  This will run the communications

interface, and then VIEWMDM.EXE.  Most of the features in the

Viewer menu are self explanatory, hit <F1> for help on anything

you don't understand, it is context-sensitive.

If your Viewer PC is a Windows Workstation,  the Viewer can be

run from Windows by selecting the Viewer icon created at

installation time.  The properties of the .PIF file that

executes the DOS Viewer from within Windows should not be

changed.

-------------------------------------------------------------------

QEMM

If using QEMM with Windows, QEMM v. 5.11 or later must be used.

These are the only versions compatible with Windows.  All

ReachOut TSRs that are run on the Host can be loaded into upper

memory with the QEMM utility LOADHI.  We suggest that you do NOT

use the Stealth feature, although this feature has been tested

with ReachOut successfully.  You should allow OPTIMIZE to make

ALL changes to your CONFIG.SYS QEMM lines.  The procedure is as

follows:

1. Put the statement

CALL C:\REACHOUT\GHM.BAT

as the last line in your AUTOEXEC.BAT file.  If you normally

enter Windows last, put this line one line before the line

that says WIN, and remark the WIN line out with the letters

REM.

2. Reboot the computer.

3. Run QEMM Optimize.

This should allow QEMM to take ReachOut into account when

allocating upper memory blocks.

We have found the following command lines to work correctly in

the respective network environments with Windows:

device=qemm386.sys ram

-------------------------------------------------------------------

WINDOWS REMOTE CONTROL

If ReachOut was installed to support only Windows, the WINLDM

or WINLDN batch file should be invoked to load the ReachOut

communications interface. Windows can then be invoked and

the ReachOut Host or Viewer can be invoked from Windows.

WINLDM and WINLDN do NOT load the support for controlling

a Host PC outside of Windows or the full screen DOS box in

Windows. You must install for DOS support and run GHM or

GHN to provide DOS capability. Additional TSRs are loaded

in memory when running GHM or GHN.

When viewing a Windows Host, there is a 768k temporary cache file

that is created on the Viewer.  This cache file helps to improve

the speed of the Windows remote control and will cut down on

modem traffic.  You may want to disable the cache to save the

768k of disk space required.  If using a floppy disk, or if the

hard disk on your Viewer PC is extremely sluggish (slow access

time or very fragmented), then all those disk accesses to the

cache file may actually be slower that receiving the raw data on

the modem.

The disk caching can be controlled by the following method:

Use the /CACHE=0 switch on the DOS VIEWMDM command line to disable

disk caching entirely and prevent the creation of the 768k

temporary file.

In Windows caching is disabled from the Prefences menu of the

Viewer application.

If ReachOut was installed for Windows then the following

line was added to SYSTEM.INI under the [386enh] section:

timercriticalsection=10000

It is likely that a previous application already had a

timercritical section parameter, so ReachOut did not add one.

This parameter is necessary to successfully remote control

a Windows Host that will be switching to a full screen DOS

box, or a Windows Viewer that will be viewing a DOS Host.

Without this parameter in SYSTEM.INI, Windows can hang when

viewing or remote controlling DOS.

------------------------------------------------------------------

DOS Viewer

ReachOut currently provides video support for CGA, EGA 640x350,

Hercules monochrome, MCGA, VGA monochrome, VGA 640x480x16 color,

and SuperVGA 800x600x16 color with a DOS Viewer, and up to

1280x1280x16 with a 4.0 Windows Viewer.

Windows is not supported for CGA and Hercules cards.  In order

to properly remote control a Windows Host, the Viewer must have

at least the same or better video resolution as the Host.

The Viewer will automatically be put into the correct video mode

of the Windows Host that it is controlling (EGA, VGA, or

SuperVGA).  There may be some cases where the correct SuperVGA

mode is not selected for non-standard SuperVGA video cards. If

this is the case the Viewer display will be VERY muddled.

Super VGA Modes

ReachOut provides SuperVGA (800x600x16) graphics support for

Windows and Windows for Workgroups only, with a DOS Viewer.

ReachOut automatically attempts to determine the type of display

the Viewer is running on, and supports most of the popular displays.

If ReachOut does not correctly choose the correct video mode for

the 800x600 SuperVGA display mode on the Viewer PC, then the

correct video mode can be supplied as a parameter on the VIEWNET

or VIEWMDM command line of the GVN.BAT or GVM.BAT file.

This is specified in the following format:

viewmdm  /svgamode=<rax>,<rbx>

for the modem version, where rax is the AX value for the INT10

mode command and rbx is the BX value for the INT 10 mode command

to set the 800x600x16 video mode.

An example is:

viewmdm  /svgamode=52,0

where mode 52 is the Super VGA 800x600 mode.

As all SuperVGA cards have different modes for entering

800x600x16 color format, refer to your display card's user guide

to get the correct mode for your particular SuperVGA card.  The

following are SuperVGA parameters that we have used with certain

models of various manufacturer's display cards.  These may not

be correct for your particular card, but are provided as

examples of what to look for in your card's user guide:

Video 7, 1024i                  /svgamode=6f05,62

Western Digital / Paradise      /svgamode=58,0

STB, Powergraph ERGO-VGA        /svgamode=29,0

Compaq, CPQ                     /svgamode=29,0

Oak                             /svgamode=52,0

Realtek                         /svgamode=1f,0

Trident 8900C                   /svgamode=5b,0

Orchid Fahrenheit 1280          /svgamode=6a,0

Diamond Stealth                 /svgamode=6a,0

The only procedure required to support SuperVGA mode on the Host

is that an 800x600 16-color driver must be specified on the

"rodisplay.drv=" command line in Windows' SYSTEM.INI file.

--------------------------------------------------------------------

EXTENDED TEXT MODES

ReachOut supports all standard text modes, including the VGA

80x43 and 80x50. ReachOut also supports all non-standard DOS

text modes: such as, 132x43, 132x50, providing both the Host and

Viewer have display cards that can support the enhanced mode.

Extended display modes require more memory on the Host system.

To use an Extended Text Mode, the Host must add the following

"switch" to the RCHDOS.EXE command line in GHN.BAT or GHM.BAT.

The command line should read:

rchdos dspsize=<display size>

where <display size> is rows multiplied by columns.  For

example, to support a Host running 132x43 text applications, the

Host command  line would read:

rchdos dspsize=5676

Supporting these non-standard text modes will require more

memory on the Host when it goes TSR.  The default <display size>

is 2000.  The additional memory requirement is: <display size>

minus 2000  multiplied by 2.

----------------------------------------------------------------------

DOS GRAPHICS

ReachOut supports remote control of DOS graphics applications

with the DOSGRAPH.EXE TSR. It is not necessary that this TSR be

loaded if you do not require support for applications that run

in a DOS graphics mode. You will save approximately 10.5k of

conventional RAM by not loading this.

You will notice that DOS graphics applications will run

substantially slower than Windows and Windows applications. This

is to be expected.   Hopefully, all DOS graphic application

vendors will soon provide Windows versions of their products.

----------------------------------------------------------------------

MOUSE USE IN WINDOWS

In order for a Viewer to have mouse support when controlling a

Windows Host, MOUSE.COM must be loaded prior to starting the

DOS Viewer.

-Mouse Symptoms Indicating Need for Adjustment

When remotely controlling a Host using SuperVGA (800x600x16), a

Viewer may notice that the mouse has a tendency to move easier

in one direction than the opposite direction.  In some cases,

extremely slow movement of the Viewer mouse in one direction may

not move the mouse at all.  The problem can easily be fixed by

adding a parameter to the command that loads the mouse on the

Viewer.

Editing the Mouse Command

The Microsoft DOS mouse driver is usually loaded on the Viewer

in the CONFIG.SYS or AUTOEXEC.BAT file as the machine is booting

up.  The line usually says:

DEVICE=C:\WINDOWS\MOUSE.SYS /Y

in the CONFIG.SYS file, or

C:\WINDOWS\MOUSE /Y

in the AUTOEXEC.BAT file.

Use an editor such as Microsoft's DOS EDIT to add an additional

parameter following the /Y which usually appears at the end of

the command line.  Add:

/Sn

where n is a number between 50 and 100.   A typical line in

might read:

DEVICE=C:\WINDOWS\MOUSE.SYS /Y /S50

in the CONFIG. SYS file,

or

C:\WINDOWS\MOUSE /Y /S50

in the AUTOEXEC.BAT file.

Experiment with numbers between 50 and 100 to adjust for your

particular mouse.

DOS VERSIONS

ReachOut works with any version of DOS greater than 2.1

including DOS 6.2 and DRDOS 6.0.

DOS MOUSE USE

ReachOut supports full use of the mouse in both DOS and Window

applications. ReachOut's user interface is also menu driven with

mouse input. To use the mouse in DOS applications and within the

ReachOut menus, your MOUSE driver must be loaded before ReachOut.

Be careful of using MOUSE.COM as some versions of this program

have very thin interrupt stacks and do not function properly.

We found this to be a problem with MOUSE.COM v. 7.04 and Lotus

1-2-3 v. 2.01. This problem can be circumvented by using the

MOUSE.SYS driver.

---------------------------------------------------------------------

WORDPERFECT

If the Host is running WordPerfect versions 5.0 or 5.1, the

WordPerfect option, Cursor Speed, should be set to Normal. By

default Cursor Speed is set to 50 characters per second. In

WordPerfect 5.0, this option is under Setup (Shift-F1) and

Cursor. In WordPerfect 5.1, you will find it under Setup

(Shift-F1), Environment, and Cursor. If Cursor Speed is not set

to Normal and the Viewer is the only one typing, the keystrokes

in WordPerfect may awkwardly repeat themselves and the systems

may even hang.

--------------------------------------------------------------------

In addition to reading Technical Considerations, this section

can be used to solve problems you may have connecting or staying

connected while using ReachOut/Modem.  If you are unable to

resolve your problem after reviewing the following checklists,

please call ReachOut Technical Support at 407-563-2255, or FAX

407-770-2533.

Approximately half of the calls to Technical Support involve

installations where the user has made one of the following

mistakes:

For versions up to, and including 3.3.

1. Installed Reachout to a computer where Windows is using a

256 color display driver, or 1024x768 resolution.

2. Run Windows Setup over a ReachOut-modified Windows

SYSTEM.INI file, destroying ReachOut's modifications.

3. Attempted to run GHM.BAT from Windows or a Windows DOS

shell, or forgotten to run GHM.BAT completely.

4. Selected a COM port that is sharing its IRQ with another COM

port.

If you are having difficulty making or maintaining a connection

with ReachOut check the following:

1.  For a Host to successfully initiate a connection to a

Viewer, the Viewer must first choose "Link" and "Wait for Any

Host to Connect". The Host can then initiate a connection to the

Viewer. The Host does not need a password to connect to a Viewer

since the Host can not control the Viewer PC.

NOTE: When direct connected with a null modem cable a connection

can be established only by the Viewer calling the Host PC.  When

prompted for a phone number, enter "5" (or any single digit

number).

2. If having difficulty making a connection while the Host is in

Windows, you should attempt the connection with the Host running

just DOS or running Windows in Standard Mode. If the Host can be

remotely controlled successfully when running at the DOS prompt

or running DOS applications and then fails when attempting to

remotely control Windows:

Check that the SYSTEM.INI file on the Host PC contains your

system's equivalent of the following lines, for versions up to,

and including 3.3:

[boot]

display.drv=c:\reachout\rchout.drv

rodisplay.drv=vga.drv

keyboard.drv=c:\reachout\rchkyb.drv

rokeyboard.drv=keyboard.drv

mouse.drv=c:\reachout\rchmse.drv

romouse.drv=rchmouse.drv

[386enh]

device=c:\reachout\vrchout.386

device=c:\reachout\vrchsrl.386

The file RCHMOUSE.DRV referred to on the ROMOUSE.DRV= line is a

renamed copy of your original Windows mouse driver, usually

mouse.drv in your Windows\System directory.  Check that the file

was created during installation, and that it is the same size as

your original Windows mouse driver.

ReachOut version 4.0 does NOT affect Windows *.INI files, but does

require that no other softwares have altered them. A typical Host

SYSTEM.INI should appear:

[boot]

display.drv=vga.drv

keyboard.drv=keyboard.drv

mouse.drv=mouse.drv

When using version 4.0, display.drv can be equal to a display driver

with a resolution of 1280x1280x16 maximum.

Insure that your Windows default EMS page does not conflict with

the memory address space of your video card or the Host's

network card. You can  use the "emmexclude= " parameter in the

[386Enh] section of SYSTEM.INI to insure exclusion.

As discussed in Appendix A, memory conflicts could cause two PCs

to abnormally disconnect especially when running Windows on a

network and in Enhanced Mode. If your PC is running on a

network, please read the files SYSINI3.TXT and NETWORKS.TXT

provided with Windows.  Guidelines for loading networking

software are explained in these files.

3. Abnormal disconnects, in or out of Windows, may also be

caused by IRQ (Interrupt Request Level) conflicts on your

system. For example, if your serial port COM2 is set for IRQ 3

and your networking card is also on IRQ 3, you will most likely

run into problems. You should change the IRQ of one of the

devices to a unique IRQ level. A program such as QAPLUS can tell

you the IRQ levels of the devices on your PC.

Check the following on both PCs if you are unable to

successfully establish and maintain a connection using modems:

1. Make sure the modem is turned on.

If the modem was not on when ReachOut was loaded, you should

perform a modem reset using the "Reset Modem" command located

under "Link" on the ReachOut Menu. Certain commands essential

for proper ReachOut operation are sent to the modem when

ReachOut loads and during a reset.

2. Is the modem properly attached to your communications port?

If you have an external modem, it needs to be attached to the

appropriate COM port with a modem cable.  Attach one end of the

cable into your modem.  Attach the other end to a Serial (or

COM) Port. Make sure the connection is secure. The modem cable

should be a standard RS-232C modem cable, not a null-modem

cable. If you are using any adapters with the cable they should

use the pins a modem cable expects. Some adapters made for mice

do not work with modems.

If you have a cable that does not have the Carrier Detect signal

or are connecting through a device that does not provide Carrier

Detect (a mux or modem pool) you may need to use the

IGNORECARRIER switch on the RCHSRL command line. See Technical

Considerations for more information on the RCHSRL switches.

3. Is your modem connected or set to the correct COM port?

For example, make sure your modem is on COM1 if that is the COM

port your modem is installed on either through ReachOut

Installation or Setup.

If you have an external modem you can check that you are

connected to the correct COM port by looking at the lights on

the modem. They are usually on the front panel of the modem. You

have most likely chosen the appropriate COM Port if the lights

blink or change when you load ReachOut. They should also blink

if you perform a modem reset with the "Reset Modem" command

located under "Link" on the ReachOut Menu. If the lights do not

blink or change, use ReachOut Setup to change your COM Port

(choose COM2 if you had chosen COM1, or COM1 if you had chosen

COM2.)  Turn off both your computer and the modem, then turn

them both back on. Load ReachOut again, once again looking at

the modem lights.

If you have an internal modem verify that it is well-seated

inside your computer.

You may need to assure that the COM ports are correctly

installed in your PC. The COM port should use the IRQ and

address normally associated with that COM port. For instance,

COM1 should use IRQ 1 and address 3F8. Via Setup, you may define

a unique COM port IRQ and address if you need to .

If you continue to have problems here, make sure you have set up

ReachOut with the appropriate Data Rate. Though rare, ReachOut

may not be able to talk to the modem if it is set at the wrong

rate.

4. Are any other devices, like a printer or a mouse, set up to

use the same communications port that the modem is using? Is any

other software besides ReachOut "talking to" this COM port?

You should make sure that no other software will try to utilize

the COM Port you chose for ReachOut, at least while ReachOut is

loaded. For example, if you are using a mouse, make sure your

mouse software does not use ReachOut's COM Port. Serial Ports

can NOT be shared. Many of the problems users encounter with

ReachOut is that they load some other piece of software which

tries to access the port used by ReachOut. Quite often, it is

mouse-oriented software which presents a conflict.

If other software has sent data to the COM port,  you should

reset your modem with the ReachOut reset command. For extra

measure, you may need to turn off both your computer and the

modem, then turn them both back on. Load ReachOut again.

5. Does the calling modem get a dial tone just after the call is

initiated? Is the answering modem answering? You may need to

check that each modem is properly plugged into the phone jack.

The modem needs a wire with a phone jack on each end.  One end

should be plugged into a phone outlet (typically on a wall).

The other end should be plugged into a jack on your modem

usually labeled "Line", "To Line", or "Wall". (Check your manual

if the modem is not labeled.)

If the calling modem gets a dial tone after it goes "off hook,"

then it is properly plugged into the phone jack.  You may need

to adjust your modem's speaker volume to hear the dial tone. The

answering modem may not answer because it may not be properly

connected to the jack. You may want to have the answering modem

to  switch roles and see if it can initiate the call. If it can,

you know the underlying problem is due to something other than

not getting a dial tone.

6. Are you set for use with a modem and not for direct connect?

Are you installed for the correct modem type? If you defined

your own modem, have you included the proper command settings?

Using Setup, make sure you are set to use a modem (if that is

indeed your intention) and that you are set for the correct

modem type.

You may need to check the modem description by running MODEMCFG

as described in Appendix C, especially if you described your own

modem or made changes to a ReachOut-supplied modem type.  A

connection may not be possible if certain settings are incorrect

in the modem description.

You may need to "cross reference" the defaults of each modem to

make sure that the modems are able to negotiate a common form of

communication. For instance, one modem may be forced to connect

only using the V.32 protocol. If the other modem is incapable of

using V.32, a connection between the two will not be possible.

In this case, the V.32 modem would have to be told to settle for

a connection not employing V.32. The same situation could arise

if one modem were set to connect only if able to use protocol

MNP 5 and the other modem either did not have MNP 5 or was not

set up to use it.  Refer to Chapter 3 and Appendix C to make

sure your modem is set up properly.

7. In some cases, we have discovered that a user's phone line

has the polarity reversed.  While the modem may work just fine

calling Compuserve or a local BBS with other, less demanding

terminal software, it will not connect properly using

ultra-high-performance software such as ReachOut.  A simple test

using an inexpensive phone line tester available at Radio Shack

stores ("Line Tester," Catalog # 43-104, $5.49), can eliminate

phone-line polarity as a possibility.  We must emphasize that

with a phone line that has the wrong polarity, the modem may

work OK with other, less demanding terminal software, but may

not connect using ReachOut

8. Have you tried connecting at a lower baud rate?

Try reducing the communications speed to your modem. One of the

computers may not be able to receive data at the speed you have

specified for communication, which would subsequently cause

connections to be erratic. This may be especially true of data

rates at 38.4K.

NOTE: If running Windows in 386 Enhanced Mode on 16mhz 386

machines that do not have a serial chip with a FIFO, Windows

will not be able to receive data at 38.4k. This will cause

erratic mouse behavior and very sluggish file transfer.  Reduce

the baud rate to 19.2k.

Although you would like to connect the PCs at the highest

possible speed, it may be practical to assure that a connection

can be established at all.  Sometimes this is easier if the

modems try to connect at, say, 2400 baud instead of 9600 baud.

If they can connect at 2400 and not at 9600 you know that a

connection is possible but for some reason the modems can not

negotiate at 9600.

If you are sure each modem has the capability to connect at 9600

you should check the commands being sent to the modem in each

modem's ReachOut description.  You may even want to try

connecting the modems at 9600 baud with a program other than

ReachOut.  This way you can determine whether there is a

hardware or software problem.  You can often tell at what speed

the modem is communicating by the lights on the front panel of

an external modem. It can be difficult to get modems of

different brands or even models to connect.  You may need to

consult MODEMCFG and your modem manual for more assistance.

NOTE: Choose "Modem Diagnostics" under the "LINK" heading of the

Viewer or Host menu.  This will allow you to talk directly with

the modem to insure proper setup.  See Chapters 4 and 5 for

further information.

10. Is the Viewer using the correct password to gain entry into

the Host PC?

When a Viewer calls a Host PC, the Viewer PC must use a valid

password. The default ReachOut password is REACHOUT (not case

sensitive). But the Host PC may have set up one or more

different passwords with the Setup program.

11. If callback security is being used, are you using the

password associated with the correct callback number?

If the password typed in by the Viewer has a callback number

associated with it, the callback number should be that of the

Viewer otherwise the Host will call the wrong number.

12. Is the caller using the correct phone book entry?

If the call is being initiated by using the phone book, the

correct entry should have been chosen. The entry should include

the correct phone number and password.

13. Do the modems have enough time to negotiate a connection?

Modem negotiation time may need to be altered on one or both

of your modems depending on the situation. You may want to check

the defaults of your modem for the following settings:

The "duration of V.32 answer tones" may need to be lengthened if

the remote modem begins handshaking at 2400 bps and then shifts

to 9600 bps.

You can usually set the time that your modem "waits for a

carrier after dialing" before hanging up from the remote modem.

The default time is usually sufficient. But it may need to be

set for a much longer duration if, for example, the modem is

originating an international connection or if the two modems

require more time to negotiate.

These settings may be worded differently from modem to modem.

They are often controlled by using AT commands to change the

values of your modem's S-registers.  You can enter the

appropriate command in the modem reset string with the ReachOut

MODEMCFG program.  Please proceed with caution before altering

your modem settings.  Read your modem documentation thoroughly.

You may need to consult the technical support staff at Stac

ReachOut or your modem manufacturer.

14. The phone connection may be poor and the modem to which you

are connected is missing the signals your modem is transmitting.

You may try calling again.  If you still can not maintain a

steady connection, either wait awhile, or try calling another

modem.  If the connection lasts with second modem, the problem

may lie with the first modem you attempted to call.  (Be sure to

see item 7.)

15. Hardware flow control may not be set on your modem. This may

happen if the DTE rate (the rate data is transmitted between

the PC and the modem) is higher than the link rate (the rate

achieved between the two modems). Use the MODEMCFG program to

edit the modem's description and make sure that hardware flow

control is being used.

16. When a network Viewer cannot connect across a WAN, check your

network to be sure bridges and routers are not filtering out

ReachOut. On Novell networks, if filtering is not the problem, the

server name can be changed to help find a Host. Using any text

editor, edit the GVN.BAT. Change server=<server name> to the name of

a server closer to the Host.

SUPPORT ISSUES

1. Windows for Workgroups version 3.11 requires the following

changes to SYSTEM.INI, WINLDM.BAT, and GHM.BAT. Using any text

editor change:

[boot]

keyboard.drv=keyboard.drv

to:

[boot]

keyboard.drv=c:\reachout\rchkbd.drv

Using any text editor change the second line of GHM.BAT and

WINLDM.BAT to:

C:\REACHOUT\RCHSRL NAME=<computer name> NOHOST

Because Windows for Workgroups version 3.11 is somewhat different

than Windows 3.1, NOHOST will tell ReachOut to load its keyboard

driver from system.ini.

2. Using ReachOut Network with Windows for Workgroups version 3.11

requires loading network drivers from DOS. WfW 3.11 loads all network

support from Windows. ReachOut requires DOS support, therefor network

drivers must be loaded from DOS.

OPTIONAL COMMAND LINE PARAMETERS

VIEWMDM and VIEWNET (example C:\REACHOUT\VIEWMDM /ALTPOP=1).

/CACHE=0 Dissables 512K disk cache.

/SVGAMODE=<rax>,<rbx> If Viewer cannot view SUPERVGA.

/SVGAMODE=1 If modem Viewer hangs on VIEWNET in a gateway

/ALTPOP=1 Changes Viewer hot-keys to <left-ctrl><righ.t-shift>.

/NAME=<Host computer name> Automatic connect to that Host.

/PHONEBOOK="%1" Allows GVM <phonebook name>.

VIEWMENU and VIEWER

PHONEBOOK=<phonebook name> Automatic connection to that Host.

Use program manager to copy the icon. Edit the command

line to read C:\REACHOUT\VIEWMENU.EXE PHONE=<name>.

RCHDOS  (seperate by space ie...C:\REACHOUT\RCHDOS\ COLDBOOT).

COLDBOOT For machines that will not reboot on disconnect.

WARMBOOT For machines that will not reboot on disconnect.

HARDBOOT For machines that will not reboot on disconnect.

SOFTBOOT For machines that will not reboot on disconnect.

DSPSIZE=<display size> Result of rows multiplied by columns

for extended text modes.

I10ASYNC=1 For display problems with 25th line messages.

I09ENTRY=4 For "Internal Stack Overflow".

PHONYLPTS=<1, 2, or 3> To remote print in DOS when the Host

has no printer.

STDWIN For use with Windows in Standard mode (Windows 3.0).

VIDEO=0 Monochrome

1 Hercules

2 CGA

3 Composite CGA

4 EGA

5 VGA / SuperVGA

6 MCGA

RCHIPX INGTMO=<1 to 255> Time in second intervals that a network

Viewer will look for a Host.

OPNTMO=<1 to 255> Time wait for IPX connections to be made.

RCHIPX, RCHNETB, RCHSAP, RCHFTP, RCHACS APIINT=0 or 91 To change

ReachOut's API interupt.

3. When using ReachOut version 4.0 and higher with a modem on a

non-standard comm port and IRQ, requires using Windows Control

Panel, and Ports, to set the comm port in Windows to the correct

IRQ. The SYSTEM.INI is changed to;

COM3Irq=5

COM3Base=03E8

The example would be for a modem on comm 3 and IRQ 5.

---------------------------------------------------------------------

Remote Access

Automatic File Transfer

To set up Remote Access to automatically connect and transfer a

file to a Host the following is required.

Host

1. Run RASETUP from DOS, and be sure RASERVER is checked.

2. Run SERVEUP from DOS, enter the drive as you wish it to

appear to the Viewer, and the drive and / or

directory you wish to serveup.

3. Run GHM. the Host is ready.

Viewer

1. Run RASETUP from DOS, and be sure RACLIENT is checked.

2. Run GVM from DOS. Create a new phonebook entry for the

Host(s) you wish to transfer to.

3. Go to LINK, and ONCE CONNECTED, select EXIT TO DOS.

4. Edit your GVM.BAT as per the example;

SET RCHPATH=C:\REACHOUT

C:\REACHOUT\RCHSRL NAME=TEST

C:\REACHOUT\RACLIENT BUFFER_SIZE=8K

C:\REACHOUT\VWREXEC

C:\REACHOUT\VIEWMDM /PHONEBOOK="%1"

if errorlevel 3 goto exit

C:\REACHOUT\VWREXEC UNLOAD

C:\REACHOUT\RACLIENT UNLOAD

C:\REACHOUT\RCHSRL UNLOAD

:exit

COPY C:\REACHOUT\TECHNOTE.DOC D:\DOCS

VDISC

The command line switch /PHONEBOOK="%1" allows to viewer

to enter GVM <phone book name>. which will automatically

dial the phone number, and issue the password associated

with this phonebook name.

After connection the Viewer will exit to DOS and allow

the GVM.BAT to continue. The copy command will transfer

this file from the Viewers C drive to the Hosts C

drive, which appears as D to the Viewer. Then VDISC will

disconnect the Viewer. The copy can be done in either

direction, from C to D, or D to C.

---------------------------------------------------------------------

Tips for configuring ReachOut with Netware Connect

1. Determine your ReachOut version number by going into the Viewer

or Host help screen and select "About".

2. Edit the following batch files: GHM.BAT and WINLDM.BAT according

to the version number that you have shown below.

3.2 and below:

You will need to upgrade to version 3.3 or better. Then

follow the procedures for that particular version.

3.3:

Edit your GHM.BAT file and change the second line to

include the NMP switch. ie:

C:\REACHOUT\RCHACS NAME=pcname NMP

4.x and above:

Edit your GHM.BAT and WINLDM.BAT files, make sure that

the second line in your batch file (the RCHACS line) does

NOT contain the switches NMP or OLDNASI.

3. Load NASI from your NASI directory.  After loading NASI run

NASIDIAG and select "List of available ports." If you

have a list of ports go to step #5, otherwise continue on.

4. If you have no port listings then one of two things have

happened:

a: You have no groups created in Netware Connect.

b: You have entered an incorrect user name/password account

for your Netware Connect server.

Correct this problem and go to back to step #3

5. While viewing the list of available ports in NASIDIAG look for a

specific portname that has the name of your server with

the words DIALIN after it. ie: servernameDIALIN

(It may or may not have underscores between the servername

and the words DIALIN)

If you are intending to use ReachOut for dial in use record

the information on that entire line.  If you are intending

to use ReachOut for dial out use recorded the information

on the line containing the outbound group you wish to use.

6. Change directories to your ReachOut directory and run SETUP.EXE.

Select "Comm settings" and then NASI (Users of version 4.x

of ReachOut should select NETWARE CONNECT.)

Enter the information you recorded from step #5 into the

appropriate fields,  leaving the port name as an "*" for

any port.

7. From the ReachOut directory run your GxM.BAT file. Go to "Link"

and select "Modem Diagnostics." If you receive and error or

have no such menu selection check that:

a. you have a valid NASI connection

b. you have specified the proper server and group

name

c. Go to step 7B

7A. If you are able to select modem diagnostics then perform a modem

reset by hitting the <F8> key.  If you see activity on the

modem feedback data screen then ReachOut is ready for use.

Continue to step 8.

7B. From your Netware Connect Server Console select "Configure

Netware Connect", "Configure Services", "NCS", "Configure NCS."

Make sure that the "DIAL IN Group" has you inbound group name and

that "Modem Independent Group" is blank. *** THIS STEP IS CRITICAL ***

Repeat step 7

8. Reachout should now be ready for use in the envorinment you

selected. The following is a list of notes regarding the

use of ReachOut and NWC.

NOTES:

ReachOut version 4.x has a selection called "Netware Connect"

in the COM settings.  This will facilitate the use of the host or

the viewer without having to make setup changes on the fly.  When

ReachOut is used as a host it will use the parameters you have

supplied in the Inbound section,  when ReachOut is used as a viewer

it will use the parameters you have supplied in the Outbound section.

When using ReachOut with Netware Connect in a configuration where

the processors are being rebooted after disconnect, a user might

find that Netware Connect is "Dropping" the session from it's

Service Selector List. If this is the case a simple delay of

approximately 20-30 seconds before loading NASI should allow enough

time for Netware Connect to release the virtual connection between

NWC and the recently rebooted host PC.

For those users who are passing switches on the NASI command line

and are wanting to pass the number of virtual circuits (/V=x)

should pass at least 3. (/V=3)  Using less than 3 has caused

connection problems when Windows is involved. /V=3 will shave

off about 20K off the NASI TSR.

If you are experiencing difficulty getting ReachOut to open the

communications channel and you feel that you have Netware Connect

setup properly, according to the above proceedures, then you may

need to delete the file MODEM.RCH from your ReachOut directory.

Once having done this you will need to re-enter the information

in ReachOut setup. ie: Modem type, Netware connect ports, etc.

If everything seems to be working properly but ReachOut host

seems to "Hold on" to the port while waiting for a connection

one of two things have happend:

1. You have not selected the proper group as described

in step #5.

2. You have a group listed in the "Modem Independant

Group" as described in step #7B.

---------------------------------------------------------------------

Tips for using ATI Mach-32 video cards with ReachOut

Reachout version 4.02 has been tested extensively with the follwing

ATI MACH32 drivers:

Version 1.5 (build 59)

Version 2.2

Version 2.3

Here is a summary of what was found and the workarounds:

Version 1.5 build 59

This version of the MACH 32 driver exhibits severe problems

with Reachout when it is being used on either the host or

the viewer.

Specifically, one can observe:

- black or strangely colored icons

- pieces of the screen in the wrong place

(one customer described it as looking like a

puzzle)

- menus that don't completely go away when exited

- other various strange things

We spent extensive amounts of time debugging Reachout with

this driver version because of reports that this type

of failure was also happening in version 2.2

Although in the process we discovered some

things which we have elected to correct in Reachout, NONE

of them made any difference.  In fact it looks like version

1.5 is just plain not handling memory bitmaps correctly.

If our customers are trying to run with version 1.5,

insist that they upgrade to version 2.2 or higher.

Version 2.2

This version installs a section in win.ini called [Mach].

To work correctly as a host, one must add the following

line to that section:

StretchBlt=off

Reachout does not (yet) correctly intercept driver

accelerated StretchBlts, and without this flag, you will

notice that any bitmaps that require stretching such as the

toolbox in MS-Paintbrush or the card deck backs in the

Solitaire Deck...  dialog will be blank.  Setting this flag

solves that problem.

To work correctly as a viewer, another flag must be set:

DeviceBitmap=off

Reachout will fail as a viewer without this flag, usually

by causing some form of GP fault.  We recommend setting both

these flags regardless of whether it is a host or viewer.

In version 2.2, the user can disable device bitmaps using

the Mach FlexDesk control panel, but the StrecthBlt flag

will have to be edited by hand.

Once these two flags are set, then Reachout works quite

well with the ATI Mach drivers with the following

exceptions:

1.  Reachout at present has a problem (on any driver) when

one side is 16 colors and the other is 256.  It shows up

most repeatably in MS-Paintbrush when one selects Zoom In

from the View menu.  Trash will appear at the top of the

zoomed bitmap, sometimes causing a GP Fault.  This may

affect ATI users more than others because even when set for

16 colors, the ATI driver registers as an 8-bit/pixel

device.  This means that while the users may *think* they

have both sides set to the same color depth (like when one

one side is a standard VGA driver and the ATI has been set

for 16 colors), in fact the ATI is still running at 8bpp so

the problem just described will manifest itself.  The

solution is to try to have the customers set up both sides

to be 8bpp if possible.

2.  Hatched brushes (e.g.  go into Designer 3.1 and select

Pattern/Hatch...) may have the wrong background color when

the ATI driver is the host and the user is running a

non-ATI viewer.  Red is a color I see most often.  This

stems from a discrepancy between the way ATI handles their

hatched brush backgrounds and the way the Microsoft VGA

driver does.  In fact, some drivers use logical colors

(VGA, EGA) while some use physical colors (ATI, 8514) --

there appears to be no agreement on a standard.  This

problem may show up in other drivers as well (like 8514).

Fortunately, hatched brushes are almost defunct, being

replaced by patterned brushed in most mainstream products

like WORD, EXCEL, and POWERPOINT.  If a big deal rides on

having this work correctly, we know how to make it work for

ATI drivers but changing it for them will break the VGA

drivers.

3.  Occasionally, a slight misalignment of brush was noticed

origins when running with ATI drivers.  This may show up as

a rectangular "ghost-like" crease in dithered backgrounds

such as those used by WORD behind the page.  This is not

critical but we'll try to fix it in a future release.

Version 2.3

Everything said about Version 2.2 applies to this version

also, with the following exception:

The Mach FlexDesk control panel no longer allows users to

disable DeviceBitmaps.  However, you can still do it

manually in the win.ini file as mentioned above.  Again,

without this flag, Reachout will cause a GP fault as a

viewer.

We recommend having customers get version 2.3 from ATI if they don't

already have it.  If they report problems like those mentioned in

version 1.5, yet they insist they have version 2.2 or later, then

take very special note of their situtation.  We have NOT been able

to cause those types of problems in versions 2.2 or 2.3, so it will

be imperative we have as many details from them as possible.  We

may have lost some time chasing problems we thought customers were

having with in 2.2 but were really in 1.5.

Areas we did not spend any time testing include ATI Crystal Fonts,

Video for Windows Extensions, or the Virtual Desktop features.

With them all turned off, mainly because it has taken quite awhile

to debug these drivers when running in "vanilla" mode.  If

customers have problems with these on, make a note of it, but have

them disable them to see if the problem goes away.  Also, the

README file that shows up in the MACH program group documents a

plethora of flags one can set in the win.ini file to disable

various parts of the ATI driver.

ATI MACH 64

Four problems have been discovered while running ReachOut version

4.02 with the ATI MACH 64 card:

A.  Like the MACH 32 driver, the MACH 64 driver uses device

bitmaps.  This needs to be disabled as follows:

In the [Macx] section of the system.ini file (which can be

found in your main Windows subdirectory), add the line:

DeviceBitmap=off

Future releases of ReachOut will not require this modification.

B.  When any of the ReachOut TSRs are loaded (via GHM, GHN, WINLDM,

or WINLDN) and Windows is started, the MACH 64 Windows screen will

show incorrect colors for a number of screen elements, most notably

title bars and any color which is made of a pattern of dots.  This

system is otherwise OK, although in some cases instability has been

observed.

The problem has been corrected and will be available in the near

future from our Technical support group.  An interim solution that

works just fine follows:

In the [system] section of the reachout.ini file (which can

be found in your main Windows subdirectory), add the line:

maxbrush=128

C.  When trying to start the ReachOut Windows Viewer on a MACH 64

system, the Windows will issue a GP fault.  The problem has been

corrected and will be available in the near future from our

Technical Support group.  Unfortunately there is no easy interim

solution for this problem.

D.  The MACH 64 driver allows on-the-fly resolution switching

without rebooting Windows.  Because there is no Microsoft standard

for performing this action, it relies on tricks that will

inevitably uproot ReachOut if any of the TSR's are loaded and cause

the system to hang.

The solution is to avoid changing resolutions while ReachOut is

loaded.

---------------------------------------------------------------------

Netblazer ACS and ReachOut

Users who wish to use ReachOut with a Telebit Netblazer ACS

will need to select a modem type of "Telebit Netblazer ACS."

To avoid changing DTE speeds through the Netblazer, which can

cause connection problems through ReachOut and the Netblazer,

users must ensure that they utilize the "Modem Lock" feature in

the Netblazer ACS software.  A user can accomplish this by one of

two methods:

#1: Via Console Connection

From the "TOP" menu enter "ACS<ENTER>"

From the "Top>Configure>IPX>ACS>" enter "Lock <line-name> ON"

#2: Via Telebit's NBCFG program in Windows

Select "Protocols",  "IPX",  "ACS"

"Individual Line Control" section should contain a funciton

called "Line Lock", this needs to be set to "ON" for all

ports intended for dial out.

In both cases above the configuration should be saved for future use.

Users also should not use the "Specific Port" option in the

Telebit settings in ReachOut, this will cause connection problems

as well. Users should use "Pooled Port" or "Register Port."

---------------------------------------------------------------------

SAP  vs  IPX Broadcast

There are advantages and disadvantages to both forms of connection.

Broadcasts will provide fast connections, but may have problems with

WAN's, either because of speed issues, or routing problems.  SAPs

will easily handle WAN's, but may be very slow in establishing a

connection.

Below is an explanation of how each version operates.  It is not

exactly pseudo code, but it should give a pretty good idea of what's

going on.

SAP Host Operation.

SAP is Service Advertising Protocol.  ReachOut Hosts do a SAP

initialization, which means the host opens socket 452 and begin

monitoring it incoming service requests.   On initialization and

every 60 seconds thereafter ReachOut Hosts will broadcast an

identification packet.  All servers and bridges on the network

receive the packet.  They in turn keep a list of the available

services in their binderies.  IF the service fails to post an ID

broadcast for several minutes the servers assume the service is no

longer available and removes it from the bindery.

From this point on, the ReachOut Hosts continually monitor the

socket for service requests.

SAP Viewer Operation.

The ReachOut Host is, for all intents and purposes, a remote control

server. Servers are useless without clients, and so we come to the

ReachOut Viewer. The first job of the viewer is to find a Host to

connect to.  This can be done by using a network list, a phonebook

entry, or typing in the specific name of the HOST.  Whatever the

method the following procedure is pretty much how it will go.

The viewer will post a GENERAL SERVICE QUERY (a type 1 packet) to

socket 452, and it will specify a server type of B6A8.  All ReachOut

HOSTs on the local segment will see the request, verify that it is

a service request, and specifically that is for their type of

service, then respond with a ID packet to the Viewer.  If the HOST

is not on the local segment the SERVER will provide the information

to the client (ReachOut Viewer).  This ID packet contains the HOSTs

Name, Network, Node, and Socket Number.

Once this data is received the Viewer will issue an SPX Connect

Request to the Host.  When acknowledged the two systems will have

an SPX connection ID for communications.

IPX Broadcast Operation.

In the case of IPX the host will basically sit and monitor socket

B5A8 at it's local server.  The viewer will issue an IPX Open

Request broadcast packet.  The Host will respond with a an SPX

Connect Request.

If the HOST is on a different segment, the Viewer will need to

place the SERVER=<server_name> parameter on the RCHIPX line of the

GVN or WINLDN batch file.  If specified the viewer will request a

list of known networks from that server.  Then, using this list,

will broadcast it's open request to every workstation on each

network in the list.  The host will then respond and allow the

Viewer to from another list, this one of available hosts. Once a

host is selected, an OPEN Request is issued to the host, and the

rest follows the pattern described above.

The switch OPNTMO=<seconds> is the time to wait for an IPX

connection to be made. OPNTMO=13 is default. Try adding this to

your RCHIPX line if you can't connect, the new line might read;

C:\REACHOUT\RCHIPX SERVER=TEST NAME=TOMAS OPNTMO=60

---------------------------------------------------------------------

<end of document>