Chapter 1

This chapter provides detailed information about the Server-Based Setup program, which includes installing Windows 95 source files on a server, plus preparing for and installing Windows 95 to run as a shared copy on a client computer.

Server-Based Setup for Windows 95

Server-Based Setup: The Basics    2

¨ In This Chapter

Shared Installations: The Issues    5

Planning for Shared vs. Private Components    5

Planning the Requirements for Shared Installations    6

Planning Setup Scripts for Shared Installations    7

Task 1: Copying Windows 95 Files to a Server    8

Selecting the Server    9

Setting the Destination Path and Installation Policy    11

Creating a Default Setup Script    12

Installing Files on the Server    14

Task 2: Creating Home Directories for Shared Installations    15

Task 3: Installing Windows 95 for Shared Installations    17

Technical Notes for Shared Installations    19

Networking Support for Shared Installations    20

Setting Configuration Information for Shared Installations    21

Creating the Disk Image for a Remoteboot Workstation    23

Startup Process for Shared Installations    25

Startup Disks for Shared Installations    26

File Organization for Shared Installations    27

Other Notes on Shared Installations    27

# Server-Based Setup: The Basics

The purpose of the Server-Based Setup (NETSETUP.EXE) program is to prepare the server to run Windows 95 Setup (SETUP.EXE) on network client computers. Server-Based Setup replaces setup /a, which was used in Windows 3.x and Windows for Workgroups for administrative setup. Windows 95 provides new and better support for installing and running a shared copy of Windows 95 from a server, including support for the following kinds of shared installations:

·	Computers that start from the local hard disk and then run a shared copy of Windows 95 from the server.

·	Computers with floppy disk drives that start locally from a floppy disk and then connect to a server to run a shared copy of Windows 95.

·	Diskless workstations that remote boot (sometimes called RIPL or remote IPL) from Novell® NetWare® version 3.x and 4.0 servers and run a shared copy of Windows 95.

Currently, this option is available only for diskless workstations on Novell NetWare 3.x or 4.x networks. Support will also be available for remote boot of Windows 95 workstations in upcoming releases of Windows NT. For information about Windows NT support, contact your sales support representative.

After installing Windows 95 source files on a server, administrators can create and manage one or more home directories for shared installations. Home directories contain the specific configuration information for the client computers.

Administrators can also use Server-Based Setup to create setup batch scripts to make it easy to automate all kinds of installations, as described in Chapter 5, “Custom, Automated, and Push Installations.” As part of the process of creating setup scripts, Server-Based Setup uses the System Policy Editor user interface so that you can predefine installation settings. You can also use a text editor to edit scripts.

Server-Based Setup (NETSETUP.EXE and related files) is available in the ADMIN\NETTOOLS directory on the Windows 95 compact disc. You cannot run Server-Based Setup from the Windows 95 floppy distribution disks.

Important

¨	To install Windows 95 source files on the network and set up batch scripts

·	Run Server-Based Setup (NETSETUP.EXE) under Windows 95, and follow the instructions to do the following:

·	Copy the Windows 95 source files onto the server, as described in “Task 1: Copying Windows 95 Files to a Server” later in this chapter.

·	Optionally, for shared installations, create home directories for running a shared copy of Windows 95 from a server, as described in “Task 2: Creating Home Directories for Shared Installations” later in this chapter. This step is required for floppy disk-based and remoteboot installations.

·	Create setup batch scripts for automated installation on client computers, as described in “Task 3: Installing Windows 95 for Shared Installations” later in this chapter.

¨	To set up a computer from network source files to run Windows 95 from the local hard disk

·	Run Windows 95 Setup as described in Chapter 3, “Introduction to Windows 95 Setup,” specifying the server name as the distribution source and, optionally, the filename for a setup batch script.

¨	To set up a computer to run a shared copy of Windows 95 from the network

	Run Windows 95 Setup as described in “Task 3: Installing Windows 95 for Shared Installations” later in this chapter.

Server-Based Setup can only be run from a computer that is currently running Windows 95 or Windows NT. You cannot run Server-Based Setup from Windows 3.1, Windows for Workgroups, or MS-DOS.

Important

If you want to use Server-Based Setup to create setup batch scripts or to create home directories for running shared copies of Windows 95, you must run Server-Based Setup under Windows 95. If you run Server-Based Setup under the Windows NT operating system, you can only use this program to install the Windows 95 source files in a shared directory.

Installing Server-Based Setup on the Administrator’s Computer Server-Based Setup is not installed automatically with Windows 95. The files that make up Server-Based Setup are uncompressed files in the ADMIN\NETTOOLS directory on the Windows 95 compact disc. The required files for Server-Based Setup are the following: NETSETUP.ADM NETSETUP.LAY RPLIMAGE.EXE NETSETUP.DLL NETSUSRC.INF SETUPX.DLL NETSETUP.EXE POLEDIT.EXE SUWIN.INF NETSETUP.INF A network administrator can install Server-Based Setup on a local computer running Windows 95. To do this, insert the Windows 95 compact disc and switch to the ADMIN\NETTOOLS\NETSETUP directory on the CD-ROM drive. Then type copy netsetup.* c: at the MS-DOS Prompt.

The following illustration summarizes the main tasks that are performed with Server-Based Setup, as described in this chapter.

# Shared Installations: The Issues

This section provides some guidelines and summary information for planning the installation process for computers that will run a shared copy of Windows 95 (referred to as “shared installations” in this chapter). This information supplements that planning and implementation information presented in Part 1 of the Windows 95 Resource Kit.

Preparing to use Windows 95 on networks where users will run shared copies of Windows 95 includes several planning tasks:

·	Planning for shared vs. private components

·	Planning the requirements for shared installations, including the memory and hardware requirements for servers, the number of servers required, and the memory requirements for client computers in various configurations

·	Planning setup scripts for running Windows 95 Setup on client computers

This section includes information and guidelines for decision making and planning related to Server-Based Setup for running shared copies of Windows 95.

## Planning for Shared vs. Private Components

This section provides guidelines for placing files and configuring server and client computers for running a shared copy of Windows 95 on the network.

Running a shared copy of Windows 95 reduces the hardware requirements for the network computers and provides network administrators with a centralized location for managing users’ system configurations. The trade-offs that guide decision making for such system configuration relate to reliability, maintainability, and performance. The following summarizes the extreme ends of the possible range for where files are placed:

·	The most private configuration: All Windows 95 executable files and applications are stored on the local hard disk and run locally. Swap files and TEMP files are also located on the local hard disk. The network is used only to store commonly used data.

Typically, such computers are used for personal workstations, portable computers that occasionally connect to the network, and workgroups that only share data and applications such as word processors (not operating system software).

·	The most shared configuration: All Windows 95 executable files and applications run from the network. All data is saved on the network. Swap files and TEMP directories are placed on network drives.

Typically, such shared installations are used where there may be users who need to access various computers for tasks such as point-of-sale data entry. Shared installations are also typically used on enterprise networks with many thousands of similar computers.

The benefits of installing Windows 95 on the local hard disk of a computer include the following:

·	Performance is superior

·	The user can customize the system

·	The system can start and continue running whether or not the server is available

·	Less network traffic is generated

The benefits of a shared installation include the following:

·	Easier to customize and manage multiple desktops

·	More secure (each user has to log onto the network to use the computer)

·	Easier to upgrade, especially for multiple computers

·	Safer for novice users, because access to system files is controlled

·	Requires less local computer power and has other lower system requirements

Obviously, an entirely shared configuration makes it easy for the administrator to maintain the system and also requires less local disk space. However, a shared configuration also means more network traffic and requires larger servers.

## Planning the Requirements for Shared Installations

This section presents information that will help you plan for shared installations based on the server and workstation requirements for running Windows 95 from a shared source.

### Server Requirements for Shared Installations

Server-Based Setup operates on one server at a time. You specify a particular server where Windows 95 files will be installed, and you also specify where related home directories will be created for running a shared copy of Windows 95.

However, the home directories do not need to be created on the same server where the Windows 95 files are installed. In fact, to balance the network load, you may prefer to designate specific servers to maintain the Windows 95 source files, and other servers to contain the home directories and logon scripts.

Typically you will want to make Windows 95 source files available for installation from multiple servers, both to provide fault tolerance and to reduce network traffic. This section provides some basic guidelines for deciding the number of servers, the hardware requirements, and other server specifications to support client computers running shared copies of Windows 95.

To be provided: hard disk size; how many servers needed per # users

### Workstation Requirements for Shared Installations

The following summarizes the information known about the memory requirements for running Windows 95 at this writing.

Memory Requirements for Running Windows 95

|  |  | Third-party real-mode network clients |  |  |
| --- | --- | --- | --- | --- |
| Windows 95 location | Microsoft1 | NETX | VLM | Other |
| Local hard disk, local Windows 95 |  |  |  |  |
| Local hard disk boot, Windows 95 on a server |  |  |  |  |
| Floppy disk boot, Windows 95 on a server |  |  |  |  |
| Remote boot, Windows 95 on a server |  |  |  |  |
| 1 Microsoft 32-bit, protected-mode network client (either Client for Microsoft Networks or Client for NetWare Networks) |  |  |  |  |

## Planning Setup Scripts for Shared Installations

You can create default and custom setup batch scripts that predefine the settings for Windows 95 Setup. You will probably choose to use batch scripts at your site if you are installing Windows 95 on more than five computers. You will want to take advantage of the ease of use, manageability, and time savings that setup scripts provide.

Before users can run setup batch scripts to install Windows 95, the network administrator needs to do the following:

1.	Install the Windows 95 source files in a shared directory, as described in “Task 1: Copying Windows 95 Files to a Server” later in this chapter, and create home directories for users, as described in “Task 2: “Creating Home Directories for Shared Installations” later in this chapter.

2.	Create one or more automated setup batch scripts (MSBATCH.INF and variations), as described in “Task 1: Copying Windows 95 Files to a Server” later in this chapter.

3.	Optionally, create policy files, user profiles, and WRKGRP.INI files, as described in Chapter 5, “Custom, Automated, and Push Installations.”

4.	Optionally, create logon scripts for mandatory automated setup, as described in Chapter 5, “Custom, Automated, and Push Installations.”

The tasks for planning setup scripts, customizing Windows 95, and using push installations apply for both local hard-disk and shared installations. Most information about these planning activities can be found in Chapter 1, “Corporate Deployment Guide,” and Chapter 5, “Custom, Automated, and Push Installations.”

Note

After you have completed these preliminary tasks, then users can log onto the network and run Windows 95 Setup by specifying the batch file that contains the appropriate installation information or, optionally, by way of logon scripts.

# Task 1: Copying Windows 95 Files to a Server

To place the Windows 95 source files on a server, you must run the Server-Based Setup program. This was formerly setup /a in Windows 3.x and Windows for Workgroups.

The procedures for Task 1 are the only steps you need to complete if you want to install Windows 95 source files on a server for computers that will run Windows 95 from their local hard disks.

Note

The server where you place the source files for installing Windows 95 from the network can be a server on a NetWare or Windows NT network or a server on one of these other networks: Artisoft® LANtastic® 5.x, Banyan® VINES® 5.52, DEC™ Pathworks™ 5.x, IBM® OS/2® LAN Server 1.2 or greater, Microsoft LAN Manager 2.x, Novell NetWare 3.x or 4.x, or SunSelect PC-NFS® 5.0.

¨	To get ready to copy Windows 95 source files onto a NetWare server

1.	On the network administrator’s computer, log onto the NetWare file server.

This should be a network computer that can run Windows 95 from its local hard disk and that is used only by support personnel for network maintenance. Make sure you log on with security privileges that allow you to create directories and copy files.

2.	On the administrator’s computer, run castoff all (which is a NetWare-supplied utility) to make sure that server-to-workstation or workstation-to-workstation messages do not affect Server-Based Setup.

¨	To run Server-Based Setup

1.	Run Windows 95 or Windows NT.

If you plan to use Server-Based Setup to create setup scripts and user directories, you must run Windows 95.

Important

2.	Insert the Windows 95 compact disc in the CD-ROM drive, and make sure that it is the active drive.

For example, at the command prompt, type f: if the F drive is the CD-ROM drive, or switch to that drive in Windows Explorer or Network Neighborhood.

3.	Switch to the ADMIN\NETTOOLS\NETSETUP directory.

4.	At the command prompt, type netsetup and press Enter.

– Or –

In Windows Explorer, double-click NETSETUP.EXE.

The Server-Based Setup dialog box appears so that you can begin installing source files and other tasks.

The following procedure summarizes the steps for Server-Based Setup.

¨	To install Windows 95 source files on a server

1.	In the Server-Based Setup dialog box, click the Change Path button and specify the server path in the Source Path dialog box.

2.	Click the OK button. Server-Based Setup presents a dialog box (as described in the following sections) so that you can complete these actions:

·	Set the server path where the source files are to be installed

·	Specify whether you want to create a default setup script, and then define the settings in the script, if you choose to create one

·	Specify how users can install Windows 95 from the server — on a local hard disk, as a shared copy, or as the user chooses

·	Provide a product identification number

·	Install Windows 95 source files in the shared directory you specify

The following sections provide details about the steps required to set up the server.

## Selecting the Server

The first Server-Based Setup dialog box is used to specify a particular server, and to manage how users can set up Windows 95 from that server.

You cannot choose the Make Script, Add, or View buttons if you are running Server-Based Setup from Windows NT

In the Server-Based Setup dialog box, the Set Server Install Path box shows the name of the selected server. If you are running Server-Based Setup from a directory where Windows 95 is already installed, this shows that path. Otherwise, this box displays the mapped drive or the UNC path to another network location that you specify.

¨	To change the server path

1.	Use your preferred method to connect to the server where you will install the Windows 95 source files. Then run Server-Based Setup.

2.	In the Server-Based Setup dialog box, click the Change Path button if you need to specify a path to the server where you want to install Windows 95 source files.

3.	Type the drive letter for a mapped drive or the UNC path to the server you want. Then click OK.

For example, if the server where you want to install the files is named NWSVR1, type:

\\nwsvr1

If you are running Windows 95 with a NetWare client installed, you can use NetWare syntax to specify a NetWare server.

If the directory does not already exist, Server-Based Setup asks if you want to create the directory, and then completes this action if you confirm the message.

If you have insufficient privileges for connecting to or creating a directory on the specified server, a message warns you. You can specify another server or quit Server-Based Setup and log on under an account that has sufficient privileges on the specified server.

When Server-Based Setup is set to a new path, it looks for the Registry file (NETSETUP.POL) that it uses to keep track of computers that have been set up. When you use Server-Based Setup to create a home directory, an entry is added to the file. (No entry is added when a user runs Windows 95 Setup from that shared directory.)

## Setting the Destination Path and Installation Policy

After you set the server name, you can install the Windows 95 source files on that server. You must specify the source and destination paths and set the installation policy for users who will install Windows 95 from this server. Optionally, you can specify that a default setup script should be created.

¨	To set the destination path and installation policy for a specified server

1.	In the first Server-Based Setup dialog box, click the Install button to display the Source Path dialog box.

2.	In the Path To Install From box, type the path for the source of the Windows 95 distribution files.

The first time you install the source files, this is the path to the CD-ROM drive and directory that contains the Windows 95 compact disc.

If you are subsequently installing source files on other servers, you can specify the path to a shared directory that contains the Windows 95 source files.

3.	In the Path To Install To box, type a path to the directory where the files will be installed.

This must be the directory on the previously specified server where you want to copy the source files. (Server-Based Setup will create the directory if it does not already exist on the server.)

In either of these boxes, you can either type a drive and directory path or specify a UNC path name.

Note

4.	Select an installation policy option as described in the following table, and then click OK to continue to the next part of Server-Based Setup.

Server-Based Setup displays the Create Default dialog box, as described in the following section.

The installation policy options specify whether users running Windows 95 Setup from this server are installing Windows 95 to run from their local hard disks or are preparing to run a shared copy of Windows 95 from the server, as described in the following list.

| Installation policy option | Resulting installation capability |
| --- | --- |
| Server | Allows only shared installation of Windows 95 (shared Windows 95 files are stored on this server). Select this option if the source files on this server are to be used by client computers to run a shared copy of Windows 95. |
| Local Hard Disk | Allows installation only on a local hard disk. Select this option if all of the Windows 95 files are to be stored on each client’s local hard disk. |
| User’s Choice | Prompts the user to specify either shared or local installation. |

Server-Based Setup stores the settings made in this dialog box in the MSBATCH.INF file it creates as part of the Server-Based Setup process.

## Creating a Default Setup Script

Server-Based Setup can automatically create a default setup script. Server-Based Setup stores the setup options you specify in an MSBATCH.INF file, together with the Windows 95 source files on the server. This default setup script can be used to install Windows 95 on individual computers, or it can be used as a template to create other versions of the setup script.

This section summarizes the procedural steps related to creating a default script while installing Windows 95 source files on a server. For information about making a script based on choosing the Make Script button in the Server-Based Setup dialog box, see Chapter 5, “Custom, Automated, and Push Installations.”

Note

If you are running Server-Based Setup under Windows 95, when you click OK in the Install Server dialog box after specifying the destination path and installation policy option, Server-Based Setup automatically prompts you to specify whether a default setup batch script should be created. If you choose to create a default script, it will be saved as MSBATCH.INF on the server.

You cannot create a setup script if you are running Server-Based Setup under Windows NT.

Note

Your decision about whether to create a default setup script depends on the following:

·	Principally, whether you want to control installation settings for users who install Windows 95 from this server.

·	Whether you want to use the default settings to create custom scripts.

·	Whether you previously created the setup script to be used and do not require a new default script.

¨	To continue with Server-Based Setup without creating a default script

·	In the Source Path dialog box, click OK. Then, in the Create Default dialog box, click the Don’t Create Default button to begin installing Windows 95 source files on the server.

¨	To create a default script for automating Windows 95 Setup

1.	In the Source Path dialog box, click OK.

2.	In the Create Default dialog box, click the Create Default button.

Server-Based Setup displays the Policies page (the same one you see when you run System Policy Editor) for creating an MSBATCH.INF file that contains the specific configuration settings.

3.	Click options to check all the components you want to define in the setup script and, where required, type values in the Settings box. After you have defined all components for the script, click OK.

Although Server-Based Setup uses the System Policy Editor user interface, this procedure does not create system policies. Only a setup batch script is created.

Note

For information about using System Policy Editor while creating a setup script, see the context-sensitive Help. For information about the specific settings saved in the MSBATCH.INF file on the server, see Appendix D, “MSBATCH.INF Parameters.” See also Chapter 5, “Custom, Automated, and Push Installations.”

## Installing Files on the Server

After Server-Based Setup has the input it needs, it installs Windows 95 source files on the specified server, performing the following tasks:

·	Copies Windows 95 to the server using the same hierarchical directory structure used when files are copied to the hard disk, as summarized in the following.

·	Generates a default setup script (MSBATCH.INF) on the server, based on the default computer settings. This script is used whenever users run Windows 95 Setup from the shared directory on the server without specifying a particular setup script.

When Server-Based Setup is finished, the server should have the following directory structure (rather than the flat directory structure created by the Windows 3.x setup /a command):

destination directory\
	SUWIN\

COMMAND\
	CONFIG\
	FONTS\
	INF\
	MEDIA\
	SYSTEM\
		COLOR\
		IOSUBSYS\
		VIEWERS\
		VMM32\

# Task 2: Creating Home Directories for Shared Installations

When you run Windows 95 Setup to create a shared installation for a client computer, information is stored in the client computer’s home directory. The home directory contains the following kinds of configuration information:

·	Appropriate initialization and configuration files (including WIN.INI and SYSTEM.INI)

·	SYSTEM.DAT and USER.DAT files that make up the Registry for the shared installation

·	Files that define the Desktop, Start menu directories, and other programs

·	The spool directory for printing

Task 2 using Server-Based Setup involves creating home directories. You must create home directories for floppy disk-based and remoteboot shared installations. Using home directories is optional for computers that have hard disks but are running a shared version of Windows 95.

You can set up the home directory for a single computer or specify the filename of a text file that you created previously to define home directories for multiple computer names.

¨	To define multiple home directories for multiple computers

1.	Create a text file that contains a list of computer names and the related location of the home directory that will contain computer-specific files for Windows 95.

The entry for each home directory in this text file should appear on a separate line, in the following format:

computername,\\UNC_server_name\directory

For example:

acct1,\\nwsvr1\netsetup\group1
acct2,\\nwsvr1\netsetyo\group2

2.	Save the file in text-only (ASCII) format, using any filename and storing it in any shared directory.

Using a home directory is optional unless the user will be installing Windows 95 for a floppy disk-based computer or a remoteboot workstation.

Note

¨	To create one or more home directories

1.	In the Server-Based Setup dialog box, click the Add button.

2.	In the Setup Machine dialog box, click an option to select the mode for installing Windows 95 on computers from the currently specified server.

·	If you want to add a single computer to the current server, click Setup One Machine. Then specify the name of the computer and the path to its home directory.

·	If you want to add home directories in a batch, click Setup Multiple Machines. Then type the path and filename of the text file that contains the list of computer names and home directories. (This is the text file created in the previous procedure.)

In this case, you can type the name of a directory on the current server or type the UNC path name to a directory on another server.

3.	If you want Server-Based Setup to generate a setup batch script based on the values set in this dialog box, check the related Generate Setup Script check box.

A setup script for each computer will be generated based on the default script created in Task 1, as described in “Creating a Default Setup Script” earlier in this chapter.

4.	After you have specified all options for home directories, click OK.

After Server-Based Setup creates the specified home directories, it stores the setup batch script in each home directory, if you specify that a script should be created.

You can see a list of the home directories that have been created for the currently selected server.

¨	To view home directories for the current server

	In the Server-Based Setup dialog box, click the View button.

When you are done viewing home directories, click the Close button to return to the Server-Based Setup dialog box.

# Task 3: Installing Windows 95 for Shared Installations

Setup batch scripts can be used to install Windows 95 on a computer’s local hard disk, or to set up client computers to run a shared copy of Windows 95 from a server.

The setup script itself tells Windows 95 Setup what kind of installation is allowed, based on the installation policy specified by the administrator and stored in the script. The script can force a local hard-disk installation or a shared installation, or allow either. If the script is set to allow either, Setup asks the user to specify the type of installation the user wants.

This section provides specific information for using setup scripts to set up a shared installation of Windows 95 on client computers.

Server-Based Setup supports creating a shared installation of Windows 95 for the following kinds of systems:

·	Remoteboot workstations

·	Computers with a single floppy disk drive

·	Computers with local hard disk drives

To configure each computer and complete the installation process, each computer starting from a hard disk or a floppy disk must run Windows 95 Setup. For remoteboot workstations, run Windows 95 Setup once for each class of similarly configured computers. Similar computers can then use their own home directories with the same source files to run Windows 95.

As described in Task 2, for shared installations the administrator can use MSBATCH.INF (the default setup script stored on the server) plus a list of computer names and home directories to generate a setup script for each computer. This results in a setup script for each client computer. The administrator can also create custom setup scripts using the MSBATCH.INF format.

Whatever method is used to create setup scripts, the scripts can be used in logon scripts for push installations. Also, by archiving the batch script, the network administrator can quickly run Setup again for a particular computer, if a failure occurs (for example, if the Registry becomes corrupted).

After you create the MSBATCH.INF file with appropriate settings, the location of the appropriate MSBATCH.INF file is specified as a command-line parameter when running Windows 95 Setup.

You can use these basic approaches for installing Windows 95 with setup scripts:

·	Run Windows 95 Setup with a setup batch script from the command line after connecting to the server, as described in this section.

·	Use a logon script to run Setup with a setup batch script, automatically installing Windows 95 when each user logs on, as described in Chapter 5, “Custom, Automated, and Push Installations.”

·	Use Microsoft System Management Service (SMS) or other third-party network management software to run Windows 95 Setup with a setup batch script. For information, see Appendix K, “Microsoft Systems Management Server.”

For information about creating and running custom setup scripts, see Chapter 5, “Custom, Automated, and Push Installations.”

To customize Windows 95 Setup, you define settings in the setup batch script, rather than make changes to a series of INF files (as was done when customizing Windows 3.1). The format for the file used to customize Windows 95 Setup is described in Appendix D, “MSBATCH.INF Parameters.”

Note

How Windows 95 Setup Runs with a Setup Batch Script When Windows 95 Setup runs with a setup script, Setup performs the following for both local hard-disk and shared installations: · Looks for MSBATCH.INF in the current directory. If this file is found, Setup knows it is running from a server. · Runs detection and configures the hardware, storing the configuration in the home directory. · Prompts the user to specify or change any settings that are not defined in the setup script. · Copies MSBATCH.INF to the Windows directory to be used by Control Panel and the setup operations that run when Windows 95 is started for the first time. For shared installations, Setup performs the following additional tasks: · Prompts the user for the path to the home directory, if it isn’t specified in the script. · Formats the startup disk and then copies files to the startup disk. · If a protected-mode network client will be used to connect to the server, Setup configures the network for transition from the real-mode network, using settings from the setup script or by prompting the user.

# Technical Notes for Shared Installations

Computers that run a shared copy of Windows 95 from a server can connect to network resources using the Map Network Drive dialog box, Open and Save As dialog boxes, and other methods that were commonly used in Windows 3.x. Also, computers can share resources such as printers and directories when protected-mode network support is installed, such as Microsoft Client for NetWare Networks.

For earlier versions of Windows, network administrators created a server share with the complete Windows distribution (shared files) on it. Then, by running Windows Setup on each client, the administrator installed the real-mode network client software and configuration files specific to a computer, usually storing these configuration files in a directory on the network.

Note

To run a shared copy of the operating system, Windows 95 uses the shared files on the server, startup files, plus the configuration files in the home directory.

There are some basic differences between installing Windows 95 to run a shared copy across the network, and running Windows 95 from a local hard disk on a computer that is connected to the network:

·	No hard disk on the local computer is required for shared installations.

·	Network logon is required before running Windows 95 on shared installations, because network connectivity is required before Windows 95 can run across the network. This can be a general logon, rather than specific user logon.

·	Because most of the Windows 95 files are located on the server for shared installations, the Windows 95 components must be loaded across the network before they can run, which increases network traffic.

·	Home directories are required for remoteboot workstations and computers started from a floppy disk running a shared copy of Windows 95 from a server. For shared installations:

·	On a remoteboot workstation, the home directory is created on the server. The network adapter is considered the boot device, and the real-mode operating system files are stored with the disk image in the home directory.

·	On a computer started from a floppy disk, the home directory is on the startup disk.

·	On a computer with a hard disk running shared Windows 95, the home directory is optional and can be on the local hard disk.

## Networking Support for Shared Installations

For shared installations, the first access to the network must occur in real mode. After Windows 95 is loaded, the protected-mode drivers take over if the computer is configured to use a protected-mode client such as Client for NetWare Networks.

Also, a real-mode version of the IPX/SPX or NetBEUI protocol is required to connect to the network. Not all supported protocols allow Windows 95 to connect to the network in real mode. For example, the Microsoft TCP/IP protocol in Windows 95 runs only in protected mode, so the computer must have IPX/SPX or NetBEUI protocols loaded to make the real-mode connection to the network. After the system loads and switches to protected mode, then Microsoft TCP/IP can be used.

For Microsoft networks, your network adapter must have both NDIS 3.1 and NDIS 2.x drivers. You can also use your existing NDIS 2.x or ODI drivers for support of networks from other vendors.

The startup disk contains a real-mode network that connects to the server. The real-mode software is small enough to fit on a floppy disk. This allows Windows 95 to start from a hard disk, floppy disk, or a remoteboot disk image in the case of a diskless workstation.

Client computers can run a shared copy of Windows 95 from a server using real-mode network software other than the Microsoft real-mode network software when protected-mode network support is not available.

For additional information about running Windows 95 with a specific network, see Chapter 10, “Windows 95 on Other Networks.”

Network clients running a shared installation of Windows 95 can use protected-mode network software by making a transition from the Microsoft real-mode network to Client for Microsoft Networks or Client for NetWare Networks. Network clients that don’t use the Microsoft real-mode network clients cannot use protected-mode network clients (such as Client for Microsoft Networks or Client for NetWare Networks) to connect to the shared installation.

## Setting Configuration Information for Shared Installations

The configuration information in setup scripts for shared installations includes adding entries to CONFIG.SYS and AUTOEXEC.BAT. For shared installations on all remoteboot client computers, the following line is also added to AUTOEXEC.BAT so that it points to the correct source for COMMAND.COM:

@set compspec=bootdrv:\command.com

where bootdrv is the startup disk. On a computer started from a hard disk, bootdrv is usually drive C. On a computer started from a floppy disk, bootdrv is drive A. On a remoteboot workstation, bootdrv is the RAM drive on the server.

The following sections describe the specific configuration changes made, depending on the type of computer that is set up for a shared installation.

### Configuring a Computer that Starts from a Hard Disk

For a computer using a Microsoft network client, NETSTART.BAT is set up as follows for a shared installation on a computer with a local hard disk:

snapshot.exe

net start redir

net logon user password /y

where redir is either basic or nwredir.

For Client for NetWare Networks, NETSTART.BAT is as follows:

snapshot.exe

net start redir

net use \\server\directory

The following shows an example of AUTOEXEC.BAT for a computer that starts from its local hard disk:

To be provided.

Setup creates the home directory unless it already exists, and copies appropriate files into those directories. The following files are copied to the home directory:

·	All initialization files, including WIN.INI and SYSTEM.INI

·	SYSTEM.DAT and USER.DAT (must be in the home directory for user profiles to work)

·	WIN.COM

Because all Windows 95 source files are on the server, Setup does not need to copy Windows 95 locally as it does for installation on a local hard disk. The File Copy step in Setup should only copy about 2 MB of files.

### Configuring a Computer that Starts from a Floppy Disk

When running shared copies of Windows 95 on client computers started from a floppy disk, the client computer must be able to start Windows 95 and access the shared Windows 95 home directories, given limited file space on the floppy disk. Setup handles this problem by creating a “mini” Registry on the floppy disk.

The following shows an example of AUTOEXEC.BAT for a computer that starts from a floppy disk:

To be provided.

### Configuring a Remoteboot Workstation

Windows 95 can be installed on remoteboot workstations that start from Novell NetWare 3.x and 4.0 servers. Remoteboot workstations contain a Remote Boot PROM that queries the network for instructions, creates a RAM drive, copies a disk image from the server to the RAM drive, and continues booting from the RAM drive. Windows 95 Setup creates this disk image.

When you run Windows 95 Setup on a remoteboot workstation, a mini Registry is created and copied to the startup disk, which is a RAM drive. When Setup is finished, it creates a file containing a disk image of the RAM drive on the server.

Windows 95 Setup assumes that the destination directory is the root of the RAM drive, and copies files to it just as it does in the installations from a floppy disk only. In addition to the usual files, Setup copies IO.SYS, RPLBOOT.SYS, BOOTDRV.COM, and NWRPLTRM to the root directory.

For remoteboot workstations, Setup adds the following line to CONFIG.SYS to create the RAM drive:

device=a:\ramdrive.sys ramdrv: 1440

where ramdrv is the drive letter to assign to the RAM disk (the first available non-local drive).

For remoteboot workstations, the following lines must be added to AUTOEXEC.BAT:

@mkdir ramdrv:\

@copy a:\ ramdrv:\

@set compspec=bootdrv:\command.com

The following shows an example of AUTOEXEC.BAT for a remoteboot computer:

To be provided.

## Creating the Disk Image for a Remoteboot Workstation

Novell specifies that to allow a remoteboot workstation to start from a Novell NetWare server, the following actions must be completed.

1.	Create a boot disk, and then create a disk image file using the boot disk.

Windows 95 Setup installs a new disk image file, as described later in this section.

2.	Copy the remoteboot files to the server.

3.	Create a BOOTCONF.SYS file.

4.	Install the Enhanced Remote Boot PROM on the network adapter.

5.	Install the network adapter in the remoteboot workstation.

6.	Connect the remoteboot workstation to the network.

This section describes how Windows 95 Setup installs a new disk image file. It also provides information to help you create BOOTCONF.SYS file to support remoteboot workstations running a shared copy of Windows 95 from a NetWare server. For more information, see the Novell publication Installing Enhanced Remote Boot PROMs on Novell Ethernet Network Interface Cards. To obtain this document, contact your NetWare vendor.

When Windows 95 Setup is finished, it automatically creates a disk image on the server. This file contains a disk image of the RAM drive for the remoteboot workstation.

You can include the boot image filename plus the workstation’s network address and node address in the BOOTCONF.SYS file on the NetWare server that is the repository for boot image files. BOOTCONF.SYS is stored in the SYS:LOGIN directory of the server, and is a text file that contains one record for each remoteboot workstation or group of workstations. Multiple workstations can be specified by using wild cards or question marks within the network address.

The following shows the format for each record in BOOTCONF.SYS. The parameters are defined later in this section.

0x[network_address,]node_address = image_filename.sys [ack] [frame=ff] [gns] [noack] [nogns] [noprotect] [notro] [protect] [ps=server] [tro] [wait time=sss]

Each record ends in a carriage return or linefeed character.

The NetWare server that contains the boot image files should have Novell RIPL.NLM loaded and bound to the appropriate network adapter. The following procedures summarize this process.

¨	To load RPL on a NetWare 3.x or 4.x server

1.	At the command prompt, type load rpl

2.	Then type the following:

bind rpl to board [ack] [frame=ff] [gns], [nodefault], [protect], [ps=server], [tro], [wait time=sss]

The parameters for BOOTCONF.SYS records and for binding RPL are not case-sensitive. The parameters can be entered in any order, and can be separated by either blanks or commas. The following table briefly defines these parameters. For more information, see your Novell documentation on BOOTCONF.SYS.

Novell NetWare Parameters for BOOTCONF.SYS and BIND Commands

| Parameter | Description |
| --- | --- |
| ack | Requires a per-frame acknowledgment so slower workstations can pace RPL, when it sends frames in burst mode. |
| bind board | Binds RPL to a board configured for 802.2 frames. The board can be specified by the name of the network adapter board number. |
| frame=ff | Configures RBOOT to use the following frame types: 802.2 (default), EII (Ethernet_II), or snap. |
| gns | Causes the workstation to use a Get Nearest Server request after RBOOT is downloaded. Use this parameter when the workstation should find a server other than the one containing RPL. |
| nodefault | Causes RPL to ignore remoteboot requests when the node address is not in BOOTCONF.SYS. |
| nogns | Overrides gns. |
| noprotect | Overrides protect specified with BIND. |
| notro | Overrides tro with BIND. |
| protect | Adjusts memory size in BIOS data area to reflect the amount of memory used by RBOOT, reducing available memory by 12K. Do not use unless absolutely necessary. |
| ps=server | Specifies that RBOOT attach to a preferred server other than the server where RPL is located. |
| tro | Causes bootstrap program to perform This Ring Only Count Of 3 on all broadcast frames. Used in source routing environments. |
| wait time=sss | Specifies how many seconds (0000 to 665535) the workstation waits before selecting a Disk Image Name automatically, when multiple names are specified in BOOTCONF.SYS. |

MACHINES.DAT is a file on the server that, for each client, lists the location of the home directory and other drive letters to connect. Each computer has a section in the file with a section name that is the network adapter’s node address. This section contains information similar to the following:

[426127491902438912]

MachineDir=\\srv\shr

c=\\srv1\shr1

d=\\srv2\shr3

e=\\srv3\shr3

In this example, the Machine directory is set to \\SRV\SHR, and drives C, D, and E are set to their respective network locations.

## Startup Process for Shared Installations

The following briefly summarizes the software portion of the startup process for a computer that starts from a floppy or hard disk to run a shared copy of Windows 95 from a server.

·	IO.SYS starts and reads the Registry to determine whether the computer is running Windows 95 over the network.

·	COMMAND.COM executes AUTOEXEC.BAT and NETSTART.BAT, which starts the network.

·	If the network is configured to log on users in real mode, it asks for a user name and password, and then logs on the user. If the network is configured to log on users in protected mode, the network uses a generic user name.

·	The network connects to the shared Windows directory and connects to the home directory using the path stored in SYSTEM.DAT, then sets an environment variable to HomeDir\SYSTEM.DAT.

·	IO.SYS starts Windows 95.

·	If the computer is running a protected-mode Microsoft network client, the real-mode network transitions to the protected-mode network.

For a general description of the Windows 95 startup process, see Chapter 6, “Setup Technical Discussion.”

## Startup Disks for Shared Installations

The full copy of the Registry for a shared installation exists on the network in the home directory. For shared installations that are not remoteboot workstations, Windows 95 Setup creates a startup floppy disk by modifying the boot sector and copying IO.SYS. For shared installations on computers with hard disks, Setup modifies the startup disk for Windows 95 by changing the boot sector and copying new IO.SYS, COMMAND.COM, and MSDOS.SYS files. The following files are copied.

Files on Startup Disk for Shared Installations on Hard-Disk Computers

| AUTOEXEC.BAT COMMAND.COM CONFIG.SYS HIMEM.SYS IFSHLP.SYS | IO.SYS MSDOS.SYS NDISHLP.SYS NET.EXE NET.MSG | NETH.MSG PROTMAN.DOS PROTMAN.EXE SNAPSHOT.EXE NDIS 2 adapter driver |
| --- | --- | --- |

The following table lists the files on the startup disk for a computer with only a floppy disk drive running a shared copy of Windows 95 from a server.

Files on Startup Disk for Shared Installations on Floppy-Disk Computers

| AUTOEXEC.BAT CONFIG.SYS COMMAND.COM HIMEM.SYS IFSHLP.SYS IO.SYS | MSDOS.SYS NDISHLP.SYS NET.EXE NET.MSG NETH.MSG | PROTMAN.DOS PROTMAN.EXE SNAPSHOT.EXE NDIS 2 adapter driver |
| --- | --- | --- |

For remoteboot workstations, Windows 95 Setup copies the same files to the home directory as are copied in the case of floppy disk-based computers. In addition, Setup also copies the files listed in the following table.

Additional Startup Files for Remoteboot Installations

| BOOTDRV.COM NWRPLTRM.COM | RAMDRIVE.SYS RPLBOOT.SYS |
| --- | --- |

The following notes apply for startup disks for shared installations:

·	The contents of the startup disk fit on a 1.2 MB floppy disk.

·	A startup floppy disk can be removed with no problems after Windows 95 starts.

·	One startup disk can be used for similar computers (computers with the same kinds of network adapters and settings). The same configuration can be used to run a shared copy of Windows 95 on remote workstations with similar hardware configurations.

·	The startup disk can be marked read-only. The software on the startup disk does not require writing information to the startup disk.

·	Files shared by clients are marked read-only.

## File Organization for Shared Installations

Windows 95 files for a shared installation are stored in the locations described in the following table.

| File location | Description |
| --- | --- |
| Startup disk | Contains all the real-mode software necessary to start the computer and connect to the shared Windows directory. This can be a local hard disk, a floppy disk, or a remoteboot disk image stored on a server. For computers that start from a floppy disk or from a remoteboot server, there is a small copy of the computer’s SYSTEM.DAT file on the startup disk that contains the information needed by IO.SYS and the real-mode network. After connecting to the home directory, Windows 95 uses the full Registry stored there. |
| Home directory | Contains files specific to a particular computer. The home directory can exist on any network shared resource. |
| Shared installation directory | A central shared directory on a server that contains all the shared Windows 95 files. This directory is always marked read-only. |

## Other Notes on Shared Installations

##### MS-DOS Mode is disabled for shared installations.

When the user starts a program that runs in MS-DOS Mode, Windows 95 shuts down and exits back to real-mode MS-DOS to run the program. When the program quits, Windows 95 starts again. This mode is available as a last resort mode for compatibility with existing software. Computers that run a shared version of Windows 95 lose their network when Windows 95 exits. Because of the related problems, MS-DOS Mode is not available for computers running Windows 95 over the network. When a user tries to run a program in this mode, Windows 95 warns that the mode has been disabled. For more information, see Chapter 22, “Application Support.”

##### Hot docking for Plug and Play network adapters is not supported for shared installations.

When Windows 95 starts over the network, real-mode drivers control the network adapter. If the network adapter is a Plug and Play card, the driver is responsible for setting the computer to the active state. Computers that run over the network do not support hot Plug and Play disconnects, because the operating system is on the network.

##### Safe Mode system startup for network computers always runs configuration files.

To perform a Safe Mode system startup on a network computer, the network must be started. Thus, IO.SYS always runs AUTOEXEC.BAT, CONFIG.SYS, and NETSTART.BAT. This allows the network computer to detect the semaphores in the home directory that describe how and when to perform a Safe Mode startup.

##### Swap files for shared installations have special considerations.

For basic information about swap files, see Chapter 17, “Performance Tuning.”

·	The swap file is stored on the local hard disk for a client computer that starts from the hard disk.

·	The swap file is stored in the home directory for a client computer that starts from a floppy disk and runs protected-mode network software.

Client computers that start from a floppy disk and run real-mode network software do not have a swap file.