Chapter 1

This chapter describes the Windows 95 printing subsystem and explains how to set up printers with Windows 95. It also describes font support in Windows 95.

Printing and Fonts

Windows 95 Printing and Fonts: The Basics    3

¨ In This Chapter

Issues for Printing and Fonts    4

Windows 95 Printing Support Overview    5

Installing a Printer    6

Sharing a Printer    8

Connecting to a Remote Printer    9

Printing Documents    10

Printing to a Network Printer    11

Using a NetWare Print Server    11

Using Microsoft Print Services for NetWare    12

Using Digital Equipment Corporation’s PrintServer Software for Windows 95    15

Using Hewlett Packard’s HP JetAdmin Utility    15

Technical Notes of Windows 95 Printing    16

Enhanced Metafile Spooling    17

Bidirectional Communication Support for Printing    18

Printer Driver Support    18

Extended Capabilities Port Support    20

Improved Printing Support for MS-DOS–based Applications    21

Support for Deferred Printing    22

Image Color Matching Support    22

Windows 95 Fonts Overview    23

TrueType Fonts    23

Windows 95 Raster Fonts    25

Windows 95 Vector Fonts    27

Screen and Printer Fonts    27

Fonts Provided for Compatibility    28

How Fonts Are Matched in Windows 95    29

Loading Fonts in Windows 95    30

Installing Additional Fonts    31

Troubleshooting Printing Problems    32

Using the Online Print Troubleshooter    32

Correcting Problems with Printer Installation    33

Correcting Specific Printing Problems    33

Troubleshooting Font Errors    37

Windows 95 Printing and Fonts: The Basics

Windows 95 includes several significant improvements for printing over Windows version 3.x. These include the following changes.

The Windows 95 print subsystem is compatible with the Windows NT print subsystem, including monitors that facilitate bidirectional communications with printers.

###### A new 32-bit print subsystem.

Background spooling and the use of enhanced metafile (EMF) spooling can significantly decrease the time it takes to return control to the application, depending on job content.

###### Quicker “return to application” time.

Spooling from MS-DOS – based applications solves conflicts when multiple applications, or applications based on both MS-DOS and Windows, are trying to print at the same time.

###### Better conflict resolution.

Windows 95 provides hands-free printer installation. In addition, users can use Point and Print with Windows 95, Windows NT, and Novell® NetWare® printers.

###### Point and Print support.

Windows 95 includes a PSERVER, which can despool print jobs from NetWare queues to printers connected to computers running Windows 95. Using the PSERVER, a NetWare server printer queue can direct print jobs to a printer attached to a computer running Windows 95.

###### PSERVER Capability for NetWare print queues.

Windows 95 supports Image Color Matching (ICM), which allows applications to better match the color of images displayed on the screen with those generated by an output device.

###### Image Color Matching.

Deferred printing allows mobile and other users to generate print jobs when they are not connected to a printer. The print jobs are stored on their computer until a printer becomes available. Windows 95 detects the connection and automatically spools the print jobs as a background process.

###### Deferred Printing.

Windows 95 provides support for a parallel extended capabilities port (ECP). An ECP parallel port supports high-speed printing and ECP devices. Even if you do not have an ECP device, using an ECP parallel port will improve I/O performance.

###### Extended Capabilities Port.

Because the fonts are stored in the Registry, there is no limit on the number of TrueTypeÒ fonts that can be installed. Similarly, there is no limit on the number of fonts that can be used simultaneously, and no limit on the number of fonts that can be printed in the same document.

###### Font storage in the Registry.

As with earlier versions of Windows, Windows 95 offers key font support features such as: support for raster, vector, and TrueType fonts; the ability to handle downloadable soft fonts; and font substitution capabilities. Windows 95 also provides several enhancements for handling fonts, including the following:

·	Better integration of font handling with the Windows 95 user interface, optimized for the 32-bit environment.

·	Enhanced rasterizer for rendering and generating TrueType fonts. This 32-bit component delivers greater accuracy from the mathematical representation to the generated bitmap.

·	Smoother-looking on-screen TrueType fonts. By using a method called anti-aliasing, characters no longer appear slightly jagged when displayed in large sizes. Anti-aliasing support for smoother font images requires a 256-color display mode (or higher).

# Issues for Printing and Fonts

Before you configure your printers and install fonts in Windows 95, you should consider the following issues:

##### General Issues

·	When purchasing new printers, consider those with bidirectional communication support and extended capabilities port support.

·	Extended capabilities ports (ECP) are not automatically configurable. If your computer includes an ECP port, follow the procedure in this chapter to enable ECP support.

·	If your printer uses font cartridges, you will also need to install the fonts on your computer.

##### NetWare Printers

·	To take advantage of the Win32-based NetWare PSERVER capability, the computer must use Microsoft Client for NetWare Networks. However, the computer does not need to run File and Printer Sharing services.

·	If you are using printers connected to NetWare servers, be sure to configure the server to store Point and Print information for use by Windows 95 users. For information, see “Using a NetWare Print Server” later in this chapter.

##### Microsoft Networks

To share a printer, the computer must be running a 32-bit, protected-mode client, and File and Printer Sharing services must be enabled, as described in Chapter 11, “Logon, Browsing, and Resource Sharing.”

# Windows 95 Printing Support Overview

In Windows 3.1, print spooling functionality was handled by Print Manager and was supported by code in several different Windows-based components. In Windows 95, the print spooler is implemented as a series of 32-bit virtual device drivers and DLLs, and consolidates the spooler functionality into a single architecture.

The new spooler provides smooth background printing. Unlike Print Manager in Windows 3.1, which passed a fixed amount of information to the printer whether the printer was ready to receive it or not, the Windows 95 spooler passes data to the printer only when the printer is ready to receive more information, by using background thread processing. This helps to reduce what seemed to be jerky processing under Windows 3.1 Print Manager.

The new spooler provides quick return-to-application time is much more powerful and flexible. It allows the user to select printer attributes on a per-printer basis, instead of requiring global printing attributes as in Windows 3.1. For example, each printer can have a different separator page and each can specify whether jobs will be printed directly or using a queue.

The following diagram illustrates how Windows 95 prints documents. For flow diagrams showing specific printer configurations with Windows 95 as a client to other networks, see the “Printing Flowcharts” section later in this chapter.

For information about enhanced metafiles, printer drivers, extended capabilities ports, and other components of the printing subsystem, see “Technical Notes on Windows 95 Printing” later in this chapter. The following sections describe how to install and share printers and how to use print servers on Windows NT and NetWare networks.

# Installing a Printer

If you upgrade to Windows 95 from an earlier version of Windows, Setup will automatically migrate your previous printer selections to Windows 95. If no printer was previously installed, the Add Printer wizard runs automatically to let you install a printer. If you need to install a printer after Setup, everything you need to install a printer — whether you are installing a local or remote printer, manipulating a print job, or managing a printer — is all conveniently located in one place: the Printers folder.

You can access the Printers folder from several places:

·	From the setting menu in the Start button

·	From My Computer

·	From the Printers icon in Control Panel

Windows 95 uses a new INF format for printer installation. Although the previous OEMSETUP.INF file format is still supported for compatibility with previous versions of Windows, the new format offers added functionality, including supports for installation of print subsystem components such as the print driver and port monitor.

If the printer is Plug and Play-compliant, it reports its device ID to Windows 95 during installation. Windows 95 searches INF files to find the ID that matches the ID values reported by the printer. If multiple matches are found, Windows 95 checks for a driver which exactly matches the device, or for a compatible driver, if one is available. If an exact match is found, Windows 95 automatically installs the correct printer support. If an exact match is not found, but a compatible driver is found, Windows 95 displays a dialog indicating the device was found and a compatible driver is available. The user may then provide a diskette containing a Windows 95 driver which is an exact match, and either tell Windows 95 to install the compatible driver it has found, or tell Windows 95 not to install a driver at all. If multiple compatible drivers are found, Windows automatically determines the best one to use.

¨	To set up a new printer

·	From the Printers folder, double-click the Add Printer icon. The Add Printer wizard leads you though the process of setting up and configuring a printer.

Some printers take advantage of the built-in bidirectional communications ability (as described later in this chapter) and initially configure device driver settings such as available fonts and the amount of installed memory, without any user intervention.

The Add Printer Wizard provided in Windows 95 makes it easy to find the correct printer to install. The printer model choice provided in the Add Printer Wizard are grouped by manufacturer. When you select a printer manufacturer on the left side of the dialog box, the list of available printer models is displayed on the right.

¨	To install a local printer using the Add Printer Wizard

1.	When the Add Printer Wizard prompts you to select your printer, in the Manufacturer’s list click the manufacturer’s name. Then, in the Printer lists, double-click the model you want to install, or select the model and click Next.

2.	If the printer you are installing is not Plug and Play-compliant, the wizard prompts you for the port that you want to use with your printer. Next, you can choose to configure that port if necessary, as in the case of COM ports, where you may need to select the appropriate baud rate.

3.	When the Add Printer Wizard prompts you for a “friendly” printer name, you can type a name that people, not just computers, find useful. For example, if you plan to share this printer with others on your network, it’s a good idea to have the printer name reflect information users may need to know in order to use this printer properly, such as its physical location.

4.	Click Finish. Windows 95 begins copying the necessary files. If you already have all of the files needed to install the printer, you can either use the files you already have or you can choose to install new files. You can install new files from the Windows 95 source files or from a disk supplied by your manufacturer.

When installation is complete, you can print a test page to ensure that the printer is functioning properly.

¨	To print a test page

1.	Right-click on a printer icon in the Printers folder, and then click Properties.

2.	In the property sheet for the printer, click the Print Test Page button.

# Sharing a Printer

For information about using File and Printer Sharing services for either Microsoft or NetWare networks, see Chapter 11, “Logon, Browsing, and Resource Sharing.”

¨	To share a locally installed printer with other users

1.	From the Printers folder, right-click the icon of the printer you want to share, and then click Sharing in the context menu.

2.	From the Sharing property sheet, specify the options you want.

You can use WinPopup to send a message from a printer that a print job is done. The Microsoft Network must be running first. For information about how to use WinPopup, see Chapter 12, “Network Technical Discussion.”

Tip

# Connecting to a Remote Printer

The easiest way to set up a remote printer is simply to print to it. Windows 95 supports Point and Print to any Windows 95 printer and to printers managed by Windows NT, NetWare, or another network server.

The Windows 95 Point and Print feature allows you to do the following:

·	Install the driver for a printer over the network automatically.

To do this, drag the network printer icon from the server’s shared resource window to the Printer window or to the desktop. This works for printers on Windows 95, Windows NT, and NetWare networks.

·	Print to a networked printer.

To print an open document, select Print from the File menu and specify a network printer. To print an unopened document, drag and drop the document’s icon on the icon of the printer you want to use. In either case, Windows 95 determines the printer name and automatically loads the driver across the network to the local hard disk and configures the printer on the user’s system.

If you double-click a networked printer’s icon, Windows 95 automatically installs the printer driver (prompting you for information, if needed) and adds the printer to your Printers folder.

Tip

Point and Print functionality means Windows 95 attempts to identify the printer you want to install and to copy the driver and any printer-specific information from either the server or an alternate location on the network. If some of the information is not available, Setup prompts you for any missing information or, if the printer driver is not found locally, for a location of the printer driver.

What information is available and how Windows 95 determines the correct information depends on which print provider is used and how much information the provider can determine about the print device. The files needed to set up a Point and Print network printer are the printer drivers specified in the Windows 95 INF files.

If the print server you’re using doesn’t support Point and Print, you can use the Add Printer Wizard to select the printer driver you want to install. The only difference between setting up a remote printer and a local printer when using the Add Printer Wizard is that you will need to specify the name of the print server for a remote printer.

When you install a local printer under Windows 95, all of the information about the installed printer is stored in the Windows 95 Registry. If your computer is running File and Printer Sharing for Microsoft Networks or File and Printer Sharing for NetWare Networks, you don’t need to do anything other than share your printer and grant appropriate sharing permissions to configure Point and Print. When a user wants to set up and connect to your printer, the client computer asks for information about the installed printer from the peer server, and the peer server responds with the appropriate information. The printer driver is also copied from the server and installed on the local computer.

On a computer running Windows 95 and using user-level security, there are some additional steps you must take to enable Point and Print support.

¨	To enable Point and Print with user-level security

1.	From Windows Explorer, locate the Windows SYSTEM directory, and right-click its icon. Then click Properties.

2.	Click the Sharing tab, and then click Add.

3.	In the Add Users dialog box, add all users who should have access your printer, and assign them read-only access. Click OK.

It is not necessary to share the directory, just add the permissions.

Note

4.	When Windows 95 offers to share the directory after you add the permissions, click No.

## Printing Documents

Printing a document in Windows 95 has been made as easy as possible.

For easy access to a printer, use the right mouse button to drag the printer’s icon from the Printers folder to the desktop, and then click Shortcut on the popup menu that appears. Then you can quickly print a document by dragging the document’s icon onto the printer icon.

Tip

¨	To print a document

·	If the document is open, from the File menu, click Print.

– Or –

If the document is not open, drag and drop the document onto the printer icon for either a locally installed printer or a network printer.

¨	To view documents waiting to be printed

·	Double-click the icon for the printer.

This displays the print queue containing print jobs.

If you have administrative privileges for the printer, you can use the printer queue dialog box to manage the printer queue and print jobs remotely. For example, administrators can pause and purge printer queues.

¨	To change printer settings

·	Right-click the printer icon, and then click Properties in the context menu.

The settings you can change depend on the type of printer you have.

If you print a document to a file, you can change it to print to a printer in MS-DOS. At the MS-DOS prompt, type copy filename.ext/b prn.

Tip

## Printing to a Network Printer

With the Microsoft network clients and networks from other vendors supporting UNC paths, you no longer need a physical redirection to a printer to print. Instead, as soon as you double-click the installed network printer or drop a document onto a network printer in your Printers folder, the connection is automatically made for you.

You can quickly locate a printer on a particular server by choosing Run from the Start button, and then typing the server name (for example, \\myserver). You will be logged onto the server and prompted for a password, if needed. Then a Windows Explorer window appears so you can select the printer.

Tip

If you use a network client that does not support UNC connections, or if you need to have a redirected LPT port to support printing from a particular application, you can still make a connection to a printer by using the appropriate network commands (such as net use lpt1: \\server\printer or capture lpt1:).

# Using a NetWare Print Server

If you are using Client for NetWare Networks and are logged onto a server for which you have administrative privileges, you can configure a NetWare server to store Point and Print information in the NetWare Bindery.

¨	To configure the NetWare server to store Point and Print information

1.	Right-click Network Neighborhood, and then click Find Computer.

2.	In the Find Computer dialog box, type the name of the print server, and then click its icon in the name list box when it is found.

3.	In the window for that server, double-click the icon for the printer. The print queue for that printer appears.

4.	In the print queue for the NetWare server, click Point and Print Setup.

5.	In the context menu, click Set Printer Model.

6.	In the Select dialog box, click the printer manufacturer in the Manufacturers list, and the printer in the Models list. Click OK.

7.	Right-click the NetWare printer queue again and, from the context menu, click Set Driver Path.

8.	Type the UNC path (in the form \\server\volume\directory) for the driver files. For example:

9.	\\NOVSVR\SYS\DRIVERS\EPSON24

Copy the appropriate files (as specified in MSPRINT.INF) to the printer path.

¨	To use Point and Print to connect to a NetWare printer

1.	From Network Neighborhood, double-click the NetWare server icon.

2.	Drag and drop the print queue from the NetWare server window to your Printer Folders window, as shown here:

3.	Follow the online instructions. The Add Printer Wizard prompts you to type a name for this printer.

Windows 95 automatically copies the files for the printer driver (including .DRV, .DLL, .HLP, and other files, as needed) to the Windows SYSTEM directory.

# Using Microsoft Print Services for NetWare

Windows 95 includes a Win32-based PSERVER capability in Microsoft Print Services for NetWare (MSPSRV.EXE), which can despool print jobs from NetWare queues to printers connected to computers running Windows 95. This means that a NetWare server (running NetWare version 2.15 or later) printer queue can direct print jobs to a printer attached to a computer running Windows 95 with Microsoft Client for NetWare Networks.

Notice that the computer running Windows 95 does not need to run File and Printer Sharing for NetWare.

Although a normal PSERVER on a NetWare network must be a dedicated computer and can spool jobs to as many as 16 printers, the computer running Windows 95 with Microsoft Print Server for NetWare can despool jobs from the NetWare print queue while it performs other tasks.

When Microsoft Print Server for NetWare is set up, it tries to log onto the NetWare file server for which it is managing the queue. It can service one queue for printing to a single printer, locally attached to the computer running Windows 95.

Print Server for NetWare is not dedicated — it runs in the background, using no resource except for data packet polling at a set interval. This means it won’t interfere with other things the user wants to do. (This is a configurable setting.)

Microsoft Print Server for NetWare uses the queue management services (QMS) API for queue services. It logs onto the NetWare print server, attaches to the print queue, and gets a handle. Then it polls the NetWare print server and receives a header that includes information about how to complete the print job. For example, the header might include information about whether to use a banner, how many copies to print, and other information about printing the job. Next, Microsoft Print Server for NetWare reads data from the job in the queue and uses Win32 calls to print to the printer. When the job is printed, it returns a call to the NetWare print server.

The PSERVER capability is provided on the Windows 95 compact disc in the ADMIN\NETTOOLS\PRTAGENT directory.

¨	To install NetWare PSERVER

1.	In the Network option in Control Panel, click Add.

2.	In the Select Network Component Type dialog box, click Service, and then click Have Disk and type the path name to the ADMIN\NETTOOLS\PRTAGENT directory.

¨	To enable the Microsoft Print Server for NetWare

1.	Before you set up Microsoft Print Server for NetWare, check the NetWare print server and the MS-DOS computer configured as the PSERVER to make sure they are both working correctly.

If you see the following message, you will know that the NetWare print server isn’t configured correctly:

Cannot determine print queue name.

2.	In the Printers folder on the Windows 95 computer, right-click the printer to which the NetWare print queue will direct jobs. Then click Properties in the context menu.

3.	From the Printer Server property sheet, click Enable Microsoft Print Server For NetWare.

4.	From the list of servers, select the NetWare server on which the queue resides from the list.

You must have access to this server; only those NetWare servers to which you have access are listed. Access is determined by the user account under which you logged onto the network.

Note

When a NetWare server is selected, the available print server appears in the Print Server list.

5.	As appropriate, adjust the time interval for polling the print queue. This setting can be adjusted as high as 15 seconds for maximum print server performance, or as low as 3 minutes for increased local performance. The default is 30 seconds. Click OK.

# Using DEC PrintServer Software for Windows 95

Digital Equipment Corporation (DEC®) PrintServer Software for Windows 95 provides support for printing directly to Digital PrintServer printers from a computer running Windows 95. This software allows extensive bidirectional communication, which allows printers to communicate both job and printer status in real time.

Also, PrintServer Software for Windows 95 systems lets you choose the best way to print your file. You can select the input and output trays; choose to print either duplex (double-sided) or simplex (single-sided); and print to any PrintServer printer anywhere on the network. PrintServer Software for Windows 95 provides automatic recovery in case of a printer jam so that pages are never lost.

DEC PrintServer Software for Windows 95 is provided on the Windows 95 compact disc in the ADMIN\PRTTOOLS\DECPSMON directory.

¨	To install PrintServer Software for Windows 95

1.	In the Add Printer option in Control Panel, click the Add Printer icon.

2.	Use the Add Printer Wizard to connect to the Windows 95 compact disc. You will find PrintServer Software for Windows 95 in the ADMIN\PRTTOOLS
\DECPSMON directory.

For more information about the product, see the online Help file in the ADMIN\PRTTOOLS\DECPSMON directory.

# Using the Hewlett-Packard JetAdmin Utility

The HP® JetAdmin Utility is an administrative tool used to install and configure Hewlett-Packard® printers connected to a network using an HP JetDirect print server (network interface). The HP JetAdmin utility operates as a Microsoft Windows utility and can be used for networking when a Novell NetWare server is available or when the Novell NetWare file server (peer-to-peer) is not available.

From the HP JetAdmin main window you can:

·	Select New to set up a new interface and printer.

·	Select Modify to change an existing configuration.

·	Select Properties to modify printer settings.

·	Select Filter to determine which printers are shown in the list.

·	Select Sort to choose the order in which the printers are shown in the list.

From the New or Modify window you can:

·	Configure the HP JetDirect interface and printer.

·	Add or remove print queues.

·	Select Drivers to install and assign Windows printer drivers to a network printer.

·	Select notification to enable notification of printer error conditions to users or groups of users.

·	Select the printer operating mode.

·	Set or change the printer description.

The HP Jet Admin utility is provided on the Windows 95 compact disc in the ADMIN\NETTOOLS\HPJETADM directory. For more information about this product, see the Help file in that directory.

¨	To install the HP Jet Admin Utility

1.	In the Network option in Control Panel, click Add.

2.	In the Select Network Component Type dialog box, click Service, and then click Have Disk and type the path name to the ADMIN\NETOOLS\HPJETADM directory.

# Technical Notes on Windows 95 Printing

This section presents technical information about the following features of the Windows 95 printing subsystem:

·	Enchanced metafile spooling

·	Bidirectional communication support

·	Printer driver support

·	Extended capabilities port support

·	Improved printing support for MS-DOS – based applications

·	Support for deferred printing

·	Image Color Matching support

## Enhanced Metafile Spooling

Many changes to Windows 95 affect printing. The most noticeable of these is that all output to non-PostScript® printers will spool as enhanced metafiles (EMFs), instead of raw printer data as in Windows 3.1. When you direct an application to print, it prints more quickly (as much as twice as fast as Windows 3.1), so you can resume work much more quickly.

The following diagram shows how Windows 95 spools EMFs when printing from Windows-based applications. EMFs include instructions for how the document is to be printed. For example, if a document contains a solid black rectangle, the EMF would contain a command to draw a rectangle with the given dimensions, fill it in with a solid color, using the color black.

Instead of the raw printer data being generated by the printer driver, EMF information is generated by the Graphical Device Interface (GDI) before spooling. After the EMF is created, control is returned to the user, and the EMF is interpreted in the background on a 32-bit print subsystem spooler thread and sent to the printer driver. This results in control being returned to the user in significantly less time than having to wait for the print calls to be fully interpreted by the printer driver directly.

¨	To disable enhanced metafile spooling

1.	From the Start menu, choose Settings, and then click Printers.

2.	Right-click the icon for your printer, and then click Properties.

3.	Click the Detail tab, and then click Spool Settings.

4.	In the Spool Settings dialog box, select RAW from the Spool Data Format list. Click OK.

## Bidirectional Communication Support for Printing

Windows 95 detects Plug and Play-compliant printers that return device ID values (as described in the IEEE 1284 specification) through bidirectional parallel communication. This feature is of benefit to applications because they can use it to query the printer directly to find out about its physical attributes.

Bidirectional communication allows for the following:

·	Accurate identification of attached devices

·	Improved ease of use when installing new components

·	Optimized performance by enabling intelligent communication between host and peripheral

·	Dynamic event notification in real time

One of the immediate benefits of bidirectional communication is being able to initially configure device driver settings on the server without user intervention. The printer driver can automatically determine how much memory the printer has, what device fonts are available, and so on.

Bidirectional communication also allows printers to send unsolicited messages to Windows 95 and to applications. For example, the printer might send an “out of paper” or “printer offline” message. Bidirectional communication enables much more detailed status reporting on a wider variety of information such as toner low conditions, paper jams, maintenance needs, and so on.

## Printer Driver Support

The Windows 95 device driver model includes two parts, a universal driver and a minidriver. This device driver model makes it easier for printer manufacturers to create drivers for their printers. Windows 95 provides the universal printer driver which communicates with other parts of the operating system and includes information pertinent to all printers. The minidriver provided by the printer manufacturer communicates with this universal driver and with the printer itself. Windows 95 also provides a PostScript® minidriver.

Minidrivers written for Windows 95 also work with the Windows NT 3.5 universal printer driver.

Note

### Universal Printer Driver

The Windows 95 universal printer driver supports mainstream page description languages, including HP® PCL®, Epson® ESC P/2, Canon® CaPSL, Lexmark PPDS, monochrome HP GL/2, and most of the older dot matrix technologies. Almost all non-PostScript printer drivers provided with Windows 95 are based on universal driver technology.

The universal printer driver has full support for device-resident Intellifont and TrueType® scalable device fonts. It also supports downloading of TrueType outlines to PCL printers with TrueType rasterizers.

The universal printer driver includes the following:

·	Full 600 dots per inch (dpi) support. Changes allow future expandability to higher resolutions.

·	Support for downloading TrueType outlines to PCL printers with TrueType rasterizers. Enhancements offer support for downloading unbound TrueType outlines and of character sets with more than 256 characters.

·	Monochrome HP GL/2 support, providing full LaserJet® 4 functionality.

·	Generic Text support using TTY.DRV.

·	Support for ESC P/2 raster graphics directly through the universal driver, rather than through minidrivers.

·	Easy-to-use property sheets for configuring printer driver settings.

### PostScript Minidriver

Microsoft developed a PostScript minidriver by working closely with Adobe. This minidriver offers the following features:

·	PostScript Level 2 support. This feature is automatically enabled for printers reporting as Level 2 from their PPD.

·	Additional support for Image Color Matching (ICM). The PostScript driver allows the server to offload ICM to the PostScript processor to improve performance. This flexibility allows you to take advantage of color raster enhancements on either the printer or the server.

·	Control over output data format. The PostScript minidriver supports ctrl+d handling, Binary Communications Protocol (BCP) and Tagged Binary Communications Protocol (TBCP), and pure binary (8-bit) channels (AppleTalk®).

·	Support for version 4.2 PPD files. These are converted to driver-readable format. WPD files are still supported for compatibility.

·	Tracking of PostScript virtual memory availability in the printer. This allows you to print more complex documents than you could with Windows 3.1. However, if the printer sends a message saying it can’t print because it doesn’t have enough virtual memory, you can work around the problem by changing the PostScript options settings from Optimize for Speed to Optimize for Portability.

·	Easy-to-use property sheets for configuring printer driver settings.

·	Support for installable device options, as described in the .PPD file, through the user interface.

## Extended Capabilities Port Support

An extended capabilities port (ECP) provides high-speed printing and support for ECP parallel ports and ECP devices is included in Windows 95. If you have an ECP parallel port, you can connect either ECP or non-ECP devices to the port. In either case, using an ECP parallel port will improve I/O performance, although ECP devices will show the greatest I/O gains.

An ECP parallel port has five possible configurations (defined on the port’s Resources property sheet) as shown in the following list:

| Basic Configuration 0 | Standard I/O ranges for LPT ports only |
| --- | --- |
| Basic Configuration 1 | Standard I/O ranges for LPT ports and any IRQ |
| Basic Configuration 2 | Standard I/O ranges for LPT ports, IRQ, and any DMA setting |
| Basic Configuration 3 | Any I/O ranges for LPT ports only |
| Basic Configuration 4 | Any I/O ranges for LPT ports and any IRQ setting |

¨	To enable ECP support in Windows 95

1.	Consult your computer (or add-in card) manual to determine the IRQ and DMA settings selected for each of the ECP ports you want to use. You’ll need this information to enable ECP support.

2.	In the System option in Control Panel, click the Device Manager tab.

3.	Click Ports (COM & LPT), and then select the Extended Capabilities Port entry. (Notice that you will only see this entry if an Extended Capabilities port was detected on your computer. If you have multiple ECP ports, you must repeat Steps 6 through 10 to configure each port’s DMA and IRQ values.)

4.	Click the Properties button in the dialog box, and then click the Resources tab. This property sheet shows an I/O range that has automatically been detected.

5.	In the Settings Based On field, select Basic Configuration 2. (See the previous table for a description of possible settings for this field.)

6.	In the Resource Settings list, click Interrupt Request, and then click Change Settings.

7.	In the Edit Interrupt Request dialog box, type the IRQ value you noted in step 1. Click OK.

8.	In the Resources property sheet, click Direct Memory Access.

9.	In the Edit Direct Memory Access dialog box, type the DMA value you noted in step 1. Click OK.

10.	Shut down and restart Windows 95 for the changes to take effect. After restarting, you can take advantage of fast I/O capabilities offered by the ECP port.

## Improved Printing Support for MS-DOS–Based Applications

With Windows 3.1, users printing from MS-DOS – based applications could not take advantage of the Windows-based spooling functionality offered by Print Manager, and encountered device contention issues when trying to print from 
MS-DOS – based applications at the same time as printing from Windows-based applications.

Windows 95 addresses the print issues in Windows 3.1 by incorporating the functionality for an MS-DOS – based application to spool directly to the 32-bit Windows 95 print spooler. This support is integrated into a print spooler virtual device, which takes the output destined for a printer port and first places it in the Windows 95 print spooler before sending the data to the printer. This functionality works with all existing MS-DOS – based applications, and results in quicker return-to-application time through the use of the spooling mechanism. Although 
MS-DOS – based applications do not benefit from EMF spooling (which is supported only for printing from Windows-based applications), users won’t encounter device contention issues, and will instead experience smoother background printing and improved printing performance in Windows 95. The print spooling support for use with MS-DOS – based applications is automatically installed and configured. Handling is transparent to the user.

## Support for Deferred Printing

The Windows 95 print subsystem also features support for deferred printing. This capability allows users not connected to a printer to generate print jobs, which are stored on their local computers for later printing. This feature is handy for users working in a location away from the printer, and for users in the office who temporarily lose printer connections because of network or printer problems, for example. Users can create print jobs from Win16-based, Win32-based applications, or MS-DOS – based applications while on the road, and then print on a physical printer upon returning to the home or office. Items not immediately printed are held in the print queue until the user reconnects to a printer.

¨	To create a deferred print job

1.	In Control Panel, double-click the Printers folder, and then click a printer.

2.	From the File menu inn the Printers dialog box, click Work Offline. The printer will be dimmed in the Printers folder.

## Image Color Matching Support

Windows 95 includes Image Color Matching (ICM) support, enabling applications to offer better consistency between the color of images displayed on the screen and the color of images generated by an output device.

Using technology licensed from Eastman Kodak, Windows 95 includes Image Color Matching (ICM) support for display and printer drivers. ICM provides consistent, predictable color rendering from input, through monitor preview, to output. Applications that use ICM functionality enable portability of color information across applications, manipulating the graphic information; across users, to provide consistent use of colors; and across platforms, allowing color information to be easily moved to different systems where the ICM technology has been implemented.

Because Windows 95 includes ICM support as part of the operating system, application vendors can integrate ICM functionality into their applications, and thus take advantage of this new system service. To provide support for device-independent color matching, colors used in applications are tied to international (CIE-based) colorimetric standards, rather than in device-dependent form to specific hardware devices. The operating system then performs the appropriate color transformations to map the device-independent color representations to the colors supported by the physical device.

The key to ICM support is the use of a profile, which represents the color properties of a monitor, printer, or scanner device. The ICM profile, provided as part of the software for the particular monitor, printer, or scanner by its vendor, resides in the Windows SYSTEM\COLOR directory. The profile format used by the ICM support in Windows 95, is the result of an industry consortium called InterColor 3.0 and is made up of many industry hardware vendors (including Kodak, Microsoft, Apple Computer, Sun Microsystems, and Silicon Graphics, among others) and industry standard-setting bodies. The InterColor efforts provide for a consistent cross-platform color standardization process that will result in industry-wide standards for defining ICM properties of output and display devices.

# Windows 95 Fonts Overview

Fonts are used to print text, display text on screen, and send text to other output devices. The Windows 95 operating system provides a set of Win32-based functions that developers can use to install, select, and query different fonts.

Windows 95 provides three basic kinds of fonts, which are categorized according to how the fonts are rendered for screen or print output:

·	TrueType fonts are stored as mathematical models which define the outline of each character. They are much easier to work with than vector fonts because they appear in the same way on the screen and as on the printed page. TrueType fonts can be scaled and rotated.

·	Raster fonts are stored in files as bitmaps and are rendered as an array of dots for displaying on the screen and printing on paper. Raster fonts cannot be cleanly scaled or rotated.

·	Vector fonts are rendered from a mathematical model, where each character is defined as a set of lines drawn between points. Vector fonts can be scaled to any size or aspect ratio, but are much more limited than TrueType fonts. They are provided in Windows 95 to ensure backward compatibility with plotter devices.

Raster and vector font files have .FON filename extensions. TrueType font files have .TTF filename extensions.

In Windows 95, there is a single .TTF file, and the pointer information is in the Registry as described in “Loading Fonts in Windows 95” later in this chapter. For TrueType fonts in Windows 3.1, there were two files for each font: the .FOT file contains a relatively short header with pointer information and the .TTF file contains the actual font data.

Note

## TrueType Fonts

TrueType fonts are shapes that are described by their outlines. Instead of being composed of bitmaps (as raster fonts are) or lines (as vector fonts are), TrueType fonts consists of a series of contours.

The TrueType downloadable fonts included with Windows 95 support the Arial, Courier, Symbol, and Times New Roman font families. The following table shows TrueType files included with Windows 95.

| Font name | Normal | Bold | Bold/Italic | Italic |
| --- | --- | --- | --- | --- |
| Arial | ARIAL.TTF | ARIALBD.TTF | ARIALBI.TTF | ARIALI.TTF |
| Courier New | COUR.TTF | COURBD.TTF | COURBI.TTF | COURI.TTF |
| Times New Roman | TIMES.TTF | TIMESBD.TTF | TIMESBI.TTF | TIMESI.TTF |
| Symbol | SYMBOL.TTF | N/A | N/A | N/A |
| Wingding | WINGDING.TTF | N/A | N/A | N/A |

The Courier New TrueType Font also ships with the OEM character set to support use of Courier New TrueType fonts in MS-DOS-based applications.

Note

TrueType has many benefits over other kinds of Windows 95 fonts:

·	What you see is really what you get, because Windows 95 uses the same font for both screen and printer. You don’t have to think about whether you have a specific point size for a particular printer or for your display.

·	You can scale and rotate TrueType fonts, and they look good in all sizes and on all output devices that Windows 95 supports.

·	Your document will look the same when printed on different printers. And any printer that uses a Windows 95 universal driver can print TrueType fonts.

·	Your document will look the same if you move it across platforms. For example, the text you format in Microsoft Word for Windows 95 will look the same if you open the same document in Word for the Macintosh®.

·	Each TrueType typeface requires only a .TTF file to create fonts in all point sizes at all resolutions for all output devices. (Raster fonts need separate files for each point size, resolution, and display device).

·	TrueType is integrated with the operating environment, so all Windows 95 applications can use TrueType fonts like they do other Windows 95 raster fonts without changes or upgrades.

In many applications, TrueType fonts appear in the Fonts dialog box with a “TT” logo beside the typeface name. Typefaces that are device fonts have a printer icon beside their names in the list.

How TrueType Works TrueType fonts are stored as a collection of points and “hints” that define the character outlines. When a Windows 95 application requests a font, TrueType uses the outline and the hints to render a bitmap in the size requested. Hints are the algorithms that distort the scaled font outlines to improve how the bitmaps look at specific resolutions. Each time you run Windows 95, the first time you select a TrueType font size, TrueType renders a bitmap of the selected characters for display or printing. Because of this, the initial font generation may be slower than with Windows 95 raster fonts. However, Windows 95 stores the rendered bitmaps in a font cache, so each subsequent time the font is used during that Windows 95 session, display or printing will be just as fast as for a Windows 95 raster font.

Windows 95 users can choose any of the following methods for printing TrueType fonts on PCL printers:

Windows 95 downloads the TrueType font as a scalable outline font. The printer performs all rasterization. This option is available only on PCL printers which can rasterize TrueType fonts, for example, HP LaserJet 4 printers.

###### Download TrueType as outline soft fonts.

Windows 95 downloads the TrueType font as a bitmap soft font. Each set of characters for each font size used is rasterized by Windows 95, then sent to the printer as a separate bitmap font. This is the default method used for PCL printers that cannot rasterize TrueType fonts, for example, HP LaserJet II and III series printers.

###### Download TrueType as bitmap soft fonts.

Windows 95 rasterizes a whole page of TrueType fonts as a graphic before sending it to the printer. This option is used for PCL printers which cannot accept the format used for downloading TrueType as bitmap soft fonts, for example, the original HP LaserJet and LaserJet Plus printers. This option is also used sometimes for troubleshooting printing problems.

###### Print TrueType as graphics.

## Windows 95 Raster Fonts

Raster fonts are bitmaps supplied in different sizes for specific video display resolutions. The Windows 95 fonts MS Serif, MS Sans Serif, Courier, System, and Terminal are raster fonts.

A raster font file contains data that describes all the characters and style of a typeface for a specific display device. Windows 95 provides several raster font sizes for various display devices. For example, MS Serif comes in point sizes 8, 10, 12, and 14 for CGA, EGA, VGA, and 8514 display devices.

Windows 95 can scale raster fonts to even multiples of their supplied sizes. For example, MS Serif can be scaled to 16, 20, 24, and so on. Bold, italic, underline, and strikethrough styles can also be generated from a standard raster font, but if you try to scale them too far from their original size or style, they become jagged.

Six resolutions of raster screen fonts are shipped with Windows 95. If used for printing, raster fonts print text and graphics as bitmaps or raster lines. The resolutions are identified by a letter appended to the filename of the font as described in the following table.

Raster Font Files

| Letter | Output device | Resolution | x size* | y size* |  |
| --- | --- | --- | --- | --- | --- |
| B** | EGA display | 1.33:1 | 96 | 72 |  |
| C** | Printer | 1:1.2 | 60 | 72 |  |
| D** | Printer | 1.66:1 | 120 | 72 |  |
| E | VGA display | 1:1 | 96 | 96 |  |
| F | 8514 display | 1:1 | 120 | 120 |  |
| * x,y indicates the height/width aspect ratio, in pixels per inch. ** These fonts are not included on the Windows 95 installation disks. |  |  |  |  |  |

The letter that identifies the resolution is appended to the raster font filenames. For example, the files for the 8514 raster fonts are COURF.FON, SSERIFF.FON, SERIFF.FON, SMALLF.FON, and SYMBOLF.FON.

| Font | Filename | Character set | Font description |
| --- | --- | --- | --- |
| Courier | COURx.FON | ANSI | Fixed-width with serifs |
| MS Sans Serif | SSERIFx.FON | ANSI | Proportional-width sans serif |
| MS Serif | SERIFx.FON | ANSI | Proportional-width serif |
| Small | SMALLx.FON | ANSI | Proportional small size |
| Symbol | SYMBOLx.FON | Symbol | Math symbols |

Raster fonts can also be printed if their resolution and aspect ratio are close to what your printer requires. If you do not see raster fonts for your printer in a Fonts dialog box, check your printer’s horizontal and vertical resolution and compare it with the table above. If there is a close match, choose the Fonts icon in Control Panel and make sure the appropriate font set is installed. If there is no close match, you cannot print the Windows 95 raster fonts on your printer. You might be able to print raster fonts in a different resolution, if the other resolution has an aspect ratio that matches your printer. Some printer drivers cannot print raster fonts, regardless of the aspect ratio.

You can also purchase raster fonts as both screen and printer fonts that work with Windows 95. Font vendors include Bitstream® Fontware™, Hewlett-Packard Type Director, Adobe® Type Library, and SoftCraft WYSIfonts!®

## Windows 95 Vector Fonts

Vector fonts are a set of lines drawn between points, like a pen plotter drawing a set of characters. They can be scaled to virtually any size, but generally they do not look as good as raster fonts in the sizes that raster fonts are specifically designed for.

Vector fonts are stored in Windows 95 as collections of GDI calls and are time-consuming to generate. But these fonts are useful for plotters and other devices where bitmapped characters can’t be used. Before TrueType, vector fonts were also used in some applications to create large characters or characters that were rotated or distorted from the baseline.

Some Windows 95 applications automatically use vector fonts at larger sizes. Some applications allow you to specify at what point size you want to use vector fonts. For example, the “Vector Above” setting in Aldus® PageMaker® specifies the point size at which PageMaker will switch to vector fonts.

The Windows 95 fonts Roman, Modern, and Script are vector fonts. Although the vector fonts use the ANSI character set, they are marked internally as an OEM character set. The three vector font files are ROMAN.FON, SCRIPT.FON, and MODERN.FON.

## Screen and Printer Fonts

Besides the font-rendering mechanism (that is, raster, vector, or TrueType), Windows 95 fonts are described according to the output device:

·	Screen fonts are font descriptions that Windows 95 uses to represent characters on the display devices. (TrueType fonts, as listed in the following screen display, act as both screen and device fonts.)

·	Printer fonts are the font descriptions used by the printer to create a font.

Windows 95 applications can use three kinds of printer fonts:

·	Device fonts are fonts that actually reside in the hardware of your printer. They can be built into the printer itself or can be provided by a font cartridge or font card.

·	Printable screen fonts are Windows 95 screen fonts that can be translated for output to the printer.

·	Downloadable soft fonts are fonts that reside on your hard disk and are sent to the printer when needed.

Not all printers can use all three types of printer fonts. Plotters, for example, cannot use downloadable soft fonts.

## Fonts Provided for Compatibility

The user interface in Windows 95 relies on TrueType fonts. However, three fonts — System, Fixed, and OEM (or Terminal) — are installed to support display and output devices to support other applications that may require these fonts. Each of these fonts supports two display types — 8514/a (1024x768) resolution and VGA (640x480) resolution.

·	System is a proportional font used by default to draw menus, dialog box controls, and other text in Windows 95.

·	Fixed is a fixed-width font used in Windows 2.x and earlier versions as the system font (for menus and dialog boxes).

·	OEM font (also known as the Terminal font for U.S. and Europe) is a fixed-width font used, for example, to display the OEM text in the Windows 95 ClipBook Viewer. The OEM font also provides an OEM character set used by some Windows-based applications.

The system, fixed, and OEM fonts provided with Windows 95 are listed in the following table.

| Font name | 8514/a | VGA |
| --- | --- | --- |
| System | 8514SYS.FON | VGASYS.FON |
| Fixed | 8514FIX.FON | VGAFIX.FON |
| OEM | 8514OEM.FON | VGAOEM.FON |

Windows 95 also provides a set of fonts for displaying MS-DOS – based applications running in a window. By default, code page 437 (U.S.) fonts are installed. Other font files are included for international language support and are identified by the code page number appended to the filename.

The following font files are provided with the associated code page translation table files.

| Font file(s) | Code page | Configuration |
| --- | --- | --- |
| CGA40WOA.FON, CGA80WOA.FON, DOSAPP.FON, EGA40WOA.FON, EGA80WOA.FON | 437 | U.S. |
| APP850.FON | 850 | U.S. |
| CGA40850.FON, CGA80850.FON, EGA40850.FON, EGA80850.FON, VGA850.FON | 850 | Multilingual |
| VGA860.FON | 860 | Portuguese |
| VGA861.FON | 861 | Icelandic |
| VGA863.FON | 863 | French Canadian |
| VGA865.FON | 865 | Norwegian/Danish |

The TrueType fonts included on the Windows 95 compact disc contain approximately 650 characters that cover all the European languages. For more information about installing multilanguage support in Windows 95, see Chapter 34, “International Windows 95.”

Note

# How Fonts Are Matched in Windows 95

When an application requests characters to print or display, Windows 95 must find the appropriate font to use from among the fonts installed on your computer. Finding the font can be complex because, for example, your document may contain fonts that aren’t available on the current printer, or there may be more than one font with the same name installed on your computer.

The basic rules that Windows 95 uses for matching a font are:

·	If the font is a TrueType font, then TrueType renders the character and the result is sent to the display or to the printer.

·	If the font is not a TrueType font, then Windows 95 uses the font mapping table to determine the most appropriate device font to use.

When Windows 95 uses the font mapping table to match screen fonts to printer fonts, the characteristics used to find the closest match are, in descending order of importance:  the character set, variable versus fixed pitch, family, typeface name, height, width, weight, slant, underline, and strikethrough. The Windows 95 search algorithm for finding fonts is the same as the one in Windows 3.1. If the necessary size and bitmap are available, the algorithm proceeds as follows:

1.	Use the font found in the printer’s ROM.

2.	Use the font found in the printer’s cartridge slot.

3.	Use the downloadable soft font.

4.	Use the TrueType font.

You can also choose from among fonts by comparing similar ones in the Fonts folder.

¨	To manually match fonts

1.	Double-click the Fonts folder in Control Panel.

2.	In the Fonts folder, choose List Fonts By Similarity from the View menu.

If you choose almost any TrueType font, Windows 95 sorts the list of fonts in descending order with the least similar font listed last.

The following table shows the types of Windows 95 fonts that can be printed on different kinds of printers.

| Printer type | Device fonts | Raster fonts | Vector fonts | TrueType fonts |
| --- | --- | --- | --- | --- |
| Dot matrix | Yes | Yes | No | Yes |
| HP PCL | Yes | No | Yes | Yes |
| PostScript | Yes | No | Yes | Yes |
| Plotter | Yes | No | Yes | No |

# Loading Fonts in Windows 95

Windows 95 no longer checks the [fonts] section in WIN.INI to load fonts. The locations for all fonts are stored only in the Registry and are automatically moved when a legacy application installs a new font in the [fonts] section of WIN.INI.

When Windows 95 starts, it loads both the raster fonts and the TrueType fonts listed in the Registry.

·	The raster fonts are resolution-dependent and are listed in the Registry key named Hkey_Current_Config\Display\Fonts. (This supports multiple docking scenarios for portable computers where there can be a different resolution on the LCD screen vs. the docking station.) The master list of all possible resolutions for raster fonts is stored in the Registry under the following key:

Hkey_Local_Machine\Software\Microsoft\Windows\CurrentVersion\Fontsize

·	The TrueType fonts are loaded from the location specified in the key:

Hkey_Local_Machine\Software\Microsoft\Windows\CurrentVersion\Fonts

Printer drivers, when they are loaded later in the startup process, look in WIN.INI to load any possible soft fonts for enumeration to applications.

Because the fonts are stored in the Registry, there is no limit on the number of TrueType fonts that can be installed. In addition, almost 1000 fonts can be used simultaneously and the same number can be printed in the same document.

Notes

In Windows 95, the Enable TrueType Fonts option is no longer available, because the user interface depends heavily upon TrueType. The font installer is a special directory in the folder.

# Installing Additional Fonts

In Windows 95, fonts can be installed in your computer in several ways:

·	Windows 95 installs TrueType and its screen fonts automatically during installation. When you specify a printer and other options in the Printer Setup dialog box, Windows 95 includes information about font cartridges and built-in fonts for your printer.

·	Install more TrueType fonts from disks.

·	Install soft fonts from other vendors on your hard disk by using the utility supplied by the manufacturer. Then use the Fonts tool in Control Panel to install the fonts for Windows 95.

·	Install a new font cartridge in your printer.

For more information about using the Font Installer, see online Help.

¨	To install new fonts on Windows 95

1.	From the Control Panel, double-click the Fonts icon.

2.	From the File menu, choose Install New Font.

3.	Use the Directories and Drives fields to specify the location of the font file.

4.	Select the font you want to install and click OK.

If a TrueType font becomes corrupted, Windows 95 detects this and marks that font as unavailable during the remainder of that Windows session and prevents it from being rendered any longer. You can choose to uninstall the font at that point.

Note

¨	To install cartridge fonts available to Windows-based applications

·	Use the installation program that came with the cartridge.

– Or –

1.	If you are using an HP LaserJet or DeskJet® PCL printer, right-click the printer’s icon and click Properties in the context menu.

2.	From the printer’s Fonts property sheet, click the Install Printer Fonts button.

3.	In the HP Font Installer dialog box, select the cartridge fonts you want to install, and then click Exit. Click OK.

If you use the Install Printer Fonts button to install fonts, they will appear in the Cartridge list in the printer’s Fonts property sheet. If you use another installation program, they will not appear in this list, but will still be available for use with Windows-based applications.

¨	To make installed cartridge fonts available to Windows-based applications

1.	Make sure that the font cartridge is inserted properly in the printer.

2.	Right-click the printer’s icon and click Properties in the context menu.

3.	From the printer’s Fonts property sheet, select the cartridge fonts you want to use.

# Troubleshooting Printing Problems

This section describes how to identify and resolve printing problems.

## Using the Online Print Troubleshooter

Windows 95 provides an online Print Troubleshooter. Use this tool first to try to solve printing problems you may have. Check the following troubleshooting procedures in this section for additional help on resolving printing problems.

¨	To get online Help for printing

1.	Click the Desktop and press F1 to start Windows 95 Help.

2.	Click Index and type printing

¨	To use the Print Troubleshooter

1.	In Windows 95 Help, click Troubleshooting.

2.	Click If You Have Trouble Printing. Print Troubleshooter asks a series of questions.

3.	Click on the answers that apply and try the suggested steps to fix the problem.

## Correcting Problems with Printer Installation

This section describes conditions which may interfere with installing a printer, and how to fix them.

##### No printers are listed in the Print dialog box.

If you cannot select a specific model (because no list appears), verify that the printer INF file exists. The PRTUPD.INF file in the Windows INF directory stores the information displayed in the Manufacturer and Model lists.

##### Setup is unable to find printer driver files.

If the Add Printer Wizard cannot find or access the needed printer driver files, it checks the installation drive and directory. If it cannot locate needed files, a dialog box prompts you to specify the path to the required printer driver files. To continue, either type the location of the printer driver files (installation source directory), or click Browse to select the drive and directory location.

##### File Copy error occurs during printer installation.

If an error occurs during the installation process, the Add Printer Wizard displays the specific error information including the source and destination paths and filenames it was trying to copy when the error occurred. To continue, verify the location of the specified files, and retry the installation.

## Correcting Specific Printing Problems

This section describes problems or errors that may happen when printing and explains how to fix them.

##### Cannot print to a local printer.

·	Clear the print buffer. Turn the printer’s power off, wait about five seconds, then turn the printer on, and try printing again.

·	Try printing a test page as described earlier in this chapter.

·	Verify that there is paper in the printer, that the printer is not jammed, and that there are no problems with the printer cartridge or toner.

·	Trying printing to a file.

·	If you can print to a file, try copying the file to the printer port.

To copy a file to a printer port (assuming a printer output filename of TEST.PRN):

copy /b test.prn lpt1:

If copying the file to the printer port prints the document correctly, the problem is in the communication between Windows 95 and the printer. Check the following (and consult your printer’s documentation as needed for further information):

·	Check the printer, making sure it is plugged in, turned on, and online.

·	Check the printer’s self-test program.

·	Check the printer connection and printer cable.

·	Check the printer configuration.

·	Check the printer driver. (See the procedure following this list.) Try to print using the Generic/Text Only or Generic Laser Printer driver. If this works, check the driver version, and either reinstall or upgrade the driver.

·	Start the computer by pressing F8 and choosing the Command Prompt Only option. Check printing from real mode and check printer memory allocation. If you can print from real mode, run msd.

·	On the Utilities menu select Test Printer, and try printing.

If, at this point, you still can only print by starting the computer with SHIFT+F5:

·	Try temporarily renaming SYSTEM.DAT to SYSTEM.ORG.

·	Copy a backup SYSTEM.DAT to the Windows directory and restart the computer.

·	Reinstall the printer driver, and retry the printing operation.

·	If the printing operation fails, delete SYSTEM.DAT from the Windows directory.

·	Rename the SYSTEM.ORG back to SYSTEM.DAT.

·	Try to print again.

·	Check for conflicting applications. If you can print using a different application, check the failing application’s configuration, and reinstall the application if needed.

·	Try disabling all spooling and retry. (See the procedure named “To Clear Residual Spool Files” later in this chapter.)

·	Try disabling Enhanced Metafile spooling, as described earlier in this chapter, which is enabled by default for all non-PostScript printers and retry.

¨	To check the printer driver

1.	Right-click on the printer’s icon and click Properties.

2.	Click the Details tab and verify the driver name is correct.

3.	Click the Device Options tab, and then click the About button. Verify the driver version.

¨	To disable all spooling and print directly to the port

1.	Right-click on the printer’s icon and click Properties.

2.	Click the Details tab and click Spool Settings.

3.	Click Print Directly To The Printer So That No Disk Space Is Used.

##### Graphic images don’t print correctly or output is garbled.

·	Press F8 and choose option 3, Safe Mode, to start the computer. Then retry printing.

·	Try disabling Enhanced Metafile spooling (as described earlier in this chapter).

·	Try printing to a PostScript driver if supported by the printer (if this prints, then the problem is a UNIDRV.DLL error).

·	If PostScript fails, there’s either a problem with the graphic device interface (GDI) or with the application. If it’s an application problem, try to print another file or try to print from another application.

·	Try printing shorter jobs, or fewer jobs at a time. If you are printing a long document or several documents, the spooler may be printing one page over another.

##### The printer outputs partially printed pages.

·	If the printed page is missing part of a graphic image, this may mean that the printer has insufficient memory. To check for insufficient printer memory, try reducing print resolution.

·	Try printing the same graphic image from a different document and application.

·	Check the printable region by running a print test from the printer.

·	If the printed page is missing a section of text, check the font that is used, and verify that the font is valid, and correctly installed (check the Fonts folder).

·	Try printing from a different document with the same font.

·	Try printing from the same document with a different font.

·	Try enabling Print TrueType as Graphics.

##### Printing is slower than normal.

·	Press F5 to start the computer in Safe Mode and then retry printing.

·	Right-click on the printer’s icon and click Properties. Click the Details tab and click Spool Settings. In the Spool Settings dialog box, verify that Spool Data Format lists EMF. (If set to RAW, change it to EMF.)

·	Also try the following:

·	Use the Disk Defragmenter utility to check for excessive hard disk fragmentation, and defragment the hard disk drive.

·	Check for available disk space for temporary files.

·	Check for available system resources.

·	Check the printer driver and reinstall, if necessary.

·	Right-click the printer icon and click Properties, then click Fonts, and then click Download TrueType As Bitmap Soft Fonts to disable printing TrueType as graphics.

¨	To disable Print TrueType as Graphics

1.	Right-click on the printer’s icon and click Properties.

2.	Click the Fonts tab and click Download True Type as Bitmap Soft Fonts, if Print TrueType Fonts As Graphics are currently in use.

##### Computer stalls while printing.

¨	To troubleshoot a computer that stalls while printing to a local printer

·	Restart the computer, and press F5 to start the computer in Safe Mode.

·	Retry printing.

·	Check the printer driver version and reinstall the printer driver if needed as described earlier n this section.

·	Check the video driver and reinstall the video driver if needed.

·	Check for adequate free disk space on the TEMP drive.

·	Delete residual spool files and retry. (See “To clear residual spool files” later in this chapter.)

¨	To troubleshoot a computer that stalls while printing to a network printer

1.	Restart the computer, and press F6 to start the computer in Safe Mode with Network Support.

2.	Complete steps 2 through 6 of the preceding procedure.

##### You send a document to print, but nothing comes out at the printer.

When you send a document to print, but nothing comes out at the printer, you may need to disable Enhanced Metafile spooling (an option available for UNIDRV and DeskJet printers). You should also ensure that the spooler has cleared the unprinted spool file.

¨	To clear residual spool files

1.	Delete spool jobs if no output occurs from the print job (.SPL files in the Windows SYSTEM\SPOOL\PRINTERS directory).

2.	Check the TEMP directory and delete all .TMP files. Then restart the computer and retest printing.

3.	Check that the system has enough free hard disk space.

An error message may appear, although it may be hidden behind other windows, when you print from a Win32-based application. Press alt+tab to bring this message box to the foreground of your desktop.

Note

# Troubleshooting Font Errors

This section describes problems with fonts that may occur, and describes how to correct them.

##### Fonts do not print correctly.

Double-click the Fonts icon in Control Panel and ensure that the font you were trying to print is installed. Then double-click the font’s icon, and then click the Print button to print the font. If the Font icon has a small arrow in the lower-left corner, it is not resident in the FONTS directory.

Also try the following:

·	Print using a different font.

·	Print a different document using the same font.

·	Print with a different application using the same font.

·	Print to a different printer using the same font.

·	Verify the printer driver version, size, and date.

·	Print to a file and then copy the file to a port to see if the driver is creating the correct output, to help delimit whether the spooler is causing the problem or if the driver output is failing. For example:

copy test.prn /b lpt1:

·	If the printer supports PostScript and PCL, try printing in each format.

·	If it is a TrueType font, enable Print TrueType as Graphics.

·	Print with a different minidriver, such as the Generic/TTY.

·	Try using a different code path (UNIDRV, PostScript, and DeskJet color).

##### When printed, fonts appear distorted or unreadable.

·	In the application, change to Print Preview mode to see if the fonts appear correct on-screen.

·	Change the font size to see if the problem recurs with a larger or smaller size font.

·	Use a different font to see if the original font is corrupted or bad.

·	Cut and paste the formatted text into another application and print it. If the font errors still occur, the problem may be related to the specific font.

·	Load a PostScript driver and select Download as TrueType. If the job prints correctly, the problem was with the printer driver or UNIDRV.DLL. Otherwise, the problem is probably in the GDI.

·	For a laser printer, right-click the printer’s icon and click Properties. In the Fonts property sheet, click Send Fonts As. In the Send TrueType Fonts As list box, select Bitmaps. Click OK.

·	Check printer memory. If the image is large, you may need more memory.

##### When printed, fonts overlap.

·	Try different resolutions, using the same printer. If a higher or lower print resolution works, the printer driver is probably at fault. Try using another printer driver.

If the problem persists with more than one printer driver, the problem is likely to be at the GDI level.

Note

·	For a laser printer, right-click the printer’s icon and click Properties. In the Fonts property sheet, click Send Fonts As. In the Send TrueType Fonts As list box, select Bitmaps. Click OK.

·	Try printing the same information with a different font.

##### Fonts do not print properly when underlined or strikethrough text is selected.

·	If strikethrough or underlined text prints incorrectly, try a different application with the same font.

·	Print in a different orientation (that is, if portrait, change to landscape).

·	For a laser printer, right-click the printer’s icon and click Properties. In the Fonts property sheet, click Send Fonts As. In the Send TrueType Fonts As list box, select Bitmaps. Click OK.

##### Fonts are clipped when printed.

·	Recheck the printable region by running a print test from the printer. (Usually, there is a test button on the printer; press this to run a test.) Adjust the paper orientation if you can.

·	For a laser printer, right-click the printer’s icon and click Properties. In the Fonts property sheet, click Send Fonts As. In the Send TrueType Fonts As list box, select Bitmaps. Click OK.

·	Check the printer memory settings for the driver and printer. If you are printing large images, the printer memory may be insufficient; try printing small images.

##### Some parts of a TrueType font are rotated, but other parts are not.

·	If this occurs because the printer can only print 180-degree and 90-degree rotation (not odd-degree rotations), redefine the degrees of rotation for the image.

·	For a laser printer, right-click the printer’s icon and click Properties. In the Fonts property sheet, click Send Fonts As. In the Send TrueType Fonts As list box, select Bitmaps. Click OK.

·	If the problem is font-related, try using another character set. Or, download TrueType fonts to the printer and try again.

##### TrueType fonts do not display in an MS-DOS window.

Sometimes changing the displayed font to a TrueType font in an MS-DOS window does not change the font on the screen. TrueType fonts cannot be displayed in an MS-DOS window if the MS-DOS program is running in graphics mode. To work around this problem, run the MS-DOS application in text mode and use bitmap fonts in the MS-DOS window.

¨	To change the font used in an MS-DOS window

1.	In the MS-DOS window, click on the MS-DOS icon in the upper-left corner of the window (or press ALT + SpaceBar.)

2.	Click Properties, and then click the Font tab.

3.	Under Available types, click Both Font Types.

4.	Click the Font size you want to display, and then click OK.

You can distinguish TrueType fonts by the “TT” designation; fonts without this designation are bitmap fonts.