Chapter 1

This chapter describes how to install and use modems with Windows 95, how to configure HyperTerminal to connect two computers, and how to configure Phone Dialer to dial voice telephone calls with a computer.

Modems and Communications Tools

Modems and Communications Tools: The Basics    3

¨ In This Chapter

Issues for Modems and Communications Tools    4

Overview of Modems and Communications Tools    4

Setting Up a Modem    5

Configuring COM Ports For A Modem    6

Defining Your Location    7

Setting Modem Properties    7

Manually Dialing    11

Bringing Up the Terminal Window    12

Modem Registry Keys    12

Init Key    13

Responses Key    13

Settings Key    15

Using HyperTerminal    16

File Transfer Protocols    19

Terminal Emulation Types    20

Using Phone Dialer    20

Dialing With Phone Dialer    21

Storing Phone Numbers With Phone Dialer    22

Using Dialing Properties    23

Defining Calling Locations    24

Defining Calling Card Rules    25

Telephony Drivers From Other Vendors    27

Troubleshooting Communications Problems    28

Correcting Problems with Modem Installation    28

Correcting Connection Problems    29

Correcting Modem Access Problems of Applications    34

# Modems and Communications Tools: The Basics

Windows 95 improves the ease of using modems by making it possible to install and configure a modem once for all Windows 95 communications applications just as you would a printer. This provides the following benefits:

·	Centralized modem and COM port configuration for all Windows 95 communications applications through the Modems option in Control Panel.

·	Integration with Telephony API (TAPI) and Win32 Communications APIs.

·	Support for hundreds of brand-name modems

·	Modem connections and configuration using point-and-click instead of AT commands.

For earlier applications running under Windows 3.1 or MS-DOS, you still need to define the serial port, modem type, and other modem settings within each application.

Note

Windows 95 includes two other tools to expand your communications capabilities:

·	HyperTerminal offers the same base communications capabilities as Terminal in Windows 3.1, but has been better integrated into the Windows 95 user interface. HyperTerminal allows you to connect two computers through a modem and TAPI for purposes of transferring binary files. For example, HyperTerminal can be used to connect a computer to a computer bulletin board in order to download files. However, HyperTerminal, unlike Terminal, also includes autodetection of data bits, stop bits, and parity.

·	Phone Dialer allows you to use a computer to dial phone numbers for voice telephone calls. It includes a telephone dial pad, user-programmable speed dials, and a call log.

With Windows 95, you can:

·	Select and configure a modem just once for all Windows 95 applications in the Modems option in Control Panel, or in a Windows 95 communications application such as HyperTerminal when you run it for the first time. If you have not yet installed a modem, it prompts you to do so. In each case, Windows 95 launches the Install New Modem wizard, which automatically detects the type of modem and selects its default settings. If it does not detect a modem, it allows you to select one from a list and define its settings.

The modem settings that the Install New Modem wizard defines or that you manually define are then used by all Windows 95 communications applications. However, you can further adjust them, if necessary, from within a specific Windows 95 application.

Note

·	Send and receive faxes over a modem using Microsoft Fax. For more information, see Chapter 27, “Microsoft Fax.”

·	Configure HyperTerminal to predefine computers to which your computer can connect. Use HyperTerminal to transfer and receive binary files between computers and online services such as CompuServe®.

·	Configure Phone Dialer to make voice telephone calls.

·	Define the location of your computer in one place within Windows 95 — the Dialing Properties dialog box. After you define your location, all Windows 95 communications tools, such as Phone Dialer or HyperTerminal, reference that location when dialing out. The Dialing Properties dialog box can be found in Phone Dialer, HyperTerminal, and the Modems option in Control Panel.

·	Manually dial a phone call or bring up a terminal window before or after dialing. For more information, see “Using Dialing Properties” later in this chapter.

·	Connect to a remote computer using Dial-Up Networking. For more information, see Chapter 28, “Dial-Up Networking and Mobile Computing.”

# Issues for Modems and Communications Tools

Before you install and configure a modem for use with Windows 95, you should decide the following:

·	How many users need modems installed on their computers and what are the locations to which and from which they will be calling.

·	What kind of security restrictions to apply to modems installed on individual computer, as described in Chapter 14, “Security.”

·	How you want your modem to perform when it makes a connection, as described in “Setting Modem Properties” later in this chapter.

In Windows 95, you cannot connect to a modem installed on another computer on the network. You can, however, share a fax modem over the network. For details, see Chapter 27, “Microsoft Fax.”

Note

# Overview of Modems and Communications Tools

Through the Windows 95 Telephony API (TAPI) in the communications subsystem, a computer can simultaneously run several communications applications that ask for access to a modem or telephone device. For example, a computer can be waiting to receive a fax through Microsoft Fax, while HyperTerminal is dialing out. TAPI arbitrates multiple requests for the same device, allowing applications to share a communications device in a cooperative manner. For more information about the architecture of TAPI and how it fits within the Windows 95 communications subsystem, see Chapter 24, “Introduction to Windows 95 Communications.”

# Setting Up a Modem

You can install a new modem in one of three ways:

·	Using the Modems option in Control Panel.

·	Running a communications program before you have installed a modem. Windows 95 prompts you to install a modem at that time.

·	Adding a modem through the Add New Hardware option in Control Panel.

In all cases, the Install New Modem wizard appears and asks if you want Windows 95 to automatically detect the modem, or if you want to manually select a modem from the list of known manufacturers and modem models. If you choose the detection option, the wizard detects and then queries the modem to configure it. If it cannot detect the modem, it prompts you to manually select a modem.

When the proper modem has been selected, you can, if necessary, adjust its properties, such as the volume for the modem speaker, the time to wait for the remote computer to answer the call, and the maximum baud rate to use.

Depending on the type of modem you have, installing and configuring it up may vary slightly as follows:

·	If you install an internal legacy (non-Plug and Play) modem adapter, its built-in COM port must be configured by the Add New Hardware Wizard before it is installed in the Modems option in Control Panel. In most cases, the Install New Modem wizard will do this automatically for you. However, on some computers, you may also need to run the Add New Hardware wizard.

·	If you are using Windows 95 PCMCIA drivers, then Windows 95 will detect and configure PCMCIA modem cards automatically when they are first inserted. Otherwise, you may need to run the Add New Hardware wizard in Control Panel to configure the card's built-in COM port. Then, you should install the modem card in the Modems option in Control Panel. For more information, see Chapter 19, “Devices.”

¨	To install a new modem through the Modems option in Control Panel

1.	In Control Panel, double-click the Modems icon.

2.	If no modem is currently installed on your computer, the Install New Modem wizard starts automatically to lead you through the steps to install a modem. Follow the online instructions.

– Or –

If you are installing a second modem, click Add to start the Install New Modem wizard.

In most cases, it’s best to let the Install New Modem wizard detect the modem for you. If it cannot detect the exact manufacturer and model, the wizard picks a standard configuration that is usually compatible. Your modem will still function at its maximum speed and according to factory default settings. A few, more advanced features may be disabled, such as enabling and disabling compression, error control, and flow control.

For information on installing a modem if your modem is not detected or listed, or finding a better match than the standard modem, see the “Troubleshooting Communications Problems” section later in this chapter.

## Configuring COM Ports For A Modem

Windows 95 automatically assigns COM names to communications ports, internal modem adapters, and PCMCIA modem cards according to their base I/O port addresses as shown in the following list:

·	Basic Configuration 0: COM1 at 3F8 (Input/Output Range)

·	Basic Configuration 2: COM2 at 2F8

·	Basic Configuration 3: COM2 at 3E8

·	Basic Configuration 4: COM3 at 2E8

If a modem has a nonstandard base address or IRQ, or if all four standard ports have been assigned to devices, Windows 95 automatically assigns the modem to COM5 port or higher. Some Windows 3.1 applications might not be able to access ports higher than COM4. Consequently, you must adjust the base address in Device Manager in the Systems option in Control Panel, or delete other devices to make a lower COM port available.

For information about adjusting COM port configurations, see Chapter 19, “Devices.”

## Defining Your Location

The first time you set up a modem, the Install New Modem wizard prompts you for information about the location you will usually be calling from (the Default Location), including your area code and country code. This information is stored in Dialing Properties, a communications utility that is accessible from all Windows 95 communications applications and the Modems option in Control Panel.

¨	To type dialing location information

·	Run the Install New Modem wizard, and then type area code and country code information in the Location Information dialog box.

After modem installation, more specific location information, such as calling card numbers or the number you must dial to access an outside line, can be entered into the My Locations dialog box in Dialing Properties. For information, see “Using Dialing Properties” later in this chapter.

# Setting Modem Properties

In the Modems option in Control Panel, you can globally change default modem settings for all Windows 95 communications tools and applications. For example, if you do not want to listen to the modem speaker, you can turn off that feature in the Modems option for all tools and applications that use that modem. You can also adjust these settings within each application.

For Windows 3.1 or MS-DOS – based applications, you need to configure the modem settings within each application.

Note

¨	To view general properties for a modem

1.	In the Modems option in Control Panel, click a modem, and then click the Properties button.

2.	On the property sheet for the selected modem, click the General tab to view the properties. These are the default settings for the modem in all Windows 95 applications.

| Field | Comments |
| --- | --- |
| Port | A port is either a COM or LPT port to which an external modem is attached, or a COM port name which identifies an internal or PCMCIA modem for purposes of accessing it. Windows 95 automatically assigns a port name (COM1, COM2, COM3, or COM4) to any device it detects. Usually, the name is only adjusted if you move an external modem from one COM port to another. For PCMCIA modem cards, the port cannot be changed. |
| Speaker Volume | Sets the volume for the telephone speaker, which broadcasts the dial tone, modem connection, and voices, if applicable, on the other end. To change the volume, use the mouse to move the slider bar to the right or left. |
| Maximum Speed | This is the baud rate at which Windows 95 communicates with the modem. It is limited by the CPU speed of the computer and the speed supported by the communications port. Windows 95 picks a conservative default speed, so that slower computers don’t lose data during transfers. Set the baud rate lower if the faster rate causes data errors. Set it higher for faster performance if you are using a computer with a 80486 or a Pentium™ processor. For example, 57600 may work better than the Windows 95 default setting of 38400 for v.32bis (14400 bps) modems on fast computers. If applications report data errors, then step down the modem’s maximum speed (for example, from 38400 to 19200 for v.32bis modems). |

If you have a slower, older computer and an external modem, you can purchase and install a 16550A UART – based COM port adapter. It allows higher speeds for slower computers. Some internal modems have an integrated 16550A UART adapter.

Tip

¨	To change or view the connection properties

·	In the Properties for Modems dialog box, click the tab of the Connection folder. The settings for connection preferences are displayed.

| Option | Comments |
| --- | --- |
| Connection Preferences | Connection settings usually correspond to what the computer on the other end is using. Therefore, do not change connection settings using the Modems option in Control Panel. Rather, use a specific tool or application, such as HyperTerminal, to change these settings on a connection-by-connection basis. |
| Use Tone Dialing | Clear this field if you are using a local phone line or system that doesn’t support touch tone. |
| Wait for Dial... | Clear this if you are making calls from a country other than where your modem was purchased and your modem fails to properly detect the dial tone. |
| Cancel the Call... | Change the number of seconds listed in this field if, for some reason, connections are taking a long time to make; for example, this may occur when you are making an international call and there are long delays before the call is connected. |

You can also specify settings for data bits, parity, and stop bits in the Connection property sheet. For information about these values, see the Windows 95 Help file.

¨	To view or change advanced connection properties

·	Click the Advanced button in the Connections folder to display the Advanced Connection Settings dialog box. In this box you can set error control, flow control, and modulation, and audit the modem operations.

| Option | Comments |
| --- | --- |
| Use Error Control | Check this to boost file transfer speeds by eliminating errors caused by noise on the telephone line. This feature is available on most newer modems. When this feature is enabled, modems sometimes have trouble connecting. If this is true, clear the check box and try again. |
| Required To Connect | Choose this if you don’t ever want your modem to connect with another modem unless there is error control enabled on a call. This is useful in areas with very noisy phone lines. In these cases, connecting at a slower baud rate may actually improve data throughput. |
| Compress Data | Check this to boost transmission speeds by compressing data between the modems. This feature is available on most modems. When this feature is enabled, modems sometimes have trouble connecting. If this is true, clear the check box. Using modem compression can sometimes reduce performance if data being sent is already compressed by the application. |
| Use Cellular Protocol | Check this when you want your modem to use special protocols designed to reduce errors over cellular connections. You may want to clear this if you are making a call on a normal noncellular telephone line. |
| Use Flow Control | Check this for all modems to avoid loss of data. Whether to use hardware or software flow control depends on your modem cable. If your modem cable has RTS and CTS wires connected, you can use hardware flow control. |
| Low-Speed Modulation Settings | Check this if you are having problems when making an international call. Windows 95 provides three possible settings: Bell 103 and 212A, A (for calls in the United States), ITU-TV V.21 and V.22 (for international calls), and ITU-TV V.23 (for French Minitel calls) |
| Record a Log File | Check this to record commands and responses to and from the modem in the MODEMLOG.TXT file in the Windows directory. Reading this file along with a modem manual can help you solve problems. After you identify the problem, you can adjust the appropriate modem keys in the Registry. Only advanced users should use this feature. |
| Extra Settings | Check this field to type modem commands that Windows 95 will include in its initialization sequence sent to the modem before dialing. Do not include the “AT” prefix in this box. The Extra Settings field is intended only for debugging purposes, and should only be used by experienced modem users. |

## Manually Dialing

Windows 95 allows you to manually dial your modem if you are having difficulty making an international call or other connection. To manually dial your modem, you need a separate telephone headset and keypad. You can request manual dialing, using slightly different procedures from within any Windows 95-based communications applications, such as HyperTerminal, Dial-Up Networking, and Microsoft Fax. The following procedure uses Dial-Up Networking as its example.

¨	To manually control modem dialing

1.	In Dial-Up Networking, right-click a connection icon, and then click Properties.

2.	Click the Configure button, and then click the Options tab.

3.	Click the Operator Assisted or Manual Dial option, and then click OK.

4.	Double-click the connection icon in Dial-Up Networking, and then click Connect.

5.	When instructed, pick up the phone and dial the number.

6.	When you hear the other computer answer, click the Connect button and hang up.

## Bringing Up the Terminal Window

Windows 95 allows you to bring up a terminal window before or after dialing to type AT modem commands if a connection requires them. A terminal window can be used to logon for security purposes, to establish connections with servers that require a specific logon procedure, and for other reasons.

¨	To bring up a terminal window before or after dialing

1.	In Dial-Up Networking, right-click a connection icon, and then click Properties.

2.	Click the Configure button, and then click the Options tab.

3.	Click the Bring Up Terminal Window Before Dialing option, or the Bring Up Terminal Window After Dialing option, and then click OK.

# Modem Registry Keys

The following section describes information stored in specific modem Registry keys that might help advanced users debug and correct problems with the commands that Windows 95 uses to control a modem. To identify modem problems, you can enable Windows 95 to create a MODEMLOG.TXT file (as described in “Setting Modem Properties” earlier in this chapter), which records responses to and from a modem when a connection is being made. The MODEMLOG.TXT file might indicate when Windows 95 is sending an incorrect command string to a modem, or when a response code is not being correctly interpreted. After consulting the documentation for the modem, you might be able to adjust the modem’s Registry keys to restore proper operation.

Modem registry keys are stored under the following key:

Hkey_Local_Machine\System\CurrentControlSet\Services\Class\Modem

Windows 95 creates one Registry key (starting with \0000) for each installed modem, additional subkeys, which contain AT commands that Windows 95 uses to initialize, dial, and answer the modem, plus other entries that communications and modem drivers use.

Some of the more important entries that you can use to correct or optimize modem operation are described in the following sections. The full set of modem Registry keys and INF file format is documented in the Windows 95 Device Development Kit (DDK).

## Init Key

The multiple, modem-command string entries in the Init Key initialize the modem before Windows 95 uses it. The name of each entry is its sequence number, starting with “1”, and its data is the command which is sent to the modem. Usually, the Init key entry “1” is “AT<cr>”, which is sent to the modem to start it. Init entry “2” usually contains “&F” or a similar command to restore the modem to its default settings. Subsequent Init key entries contain miscellaneous commands to configure the modem to be compatible with Windows 95.

## Responses Key

The Responses key contains strings that the modem might report to Windows in response to a command, or during the connection process. The name of each subkey is the text of a single modem response, and its data is a 10-byte binary value, specifying the meaning of the response to Windows in a coded format. The first two characters (byte 0) specify the meaning of the response code, using one of the following values:

| Value | Type | Description |
| --- | --- | --- |
| 00 | OK | The modem accepted the previous command. |
| 01 | Negotiation Progress | Status information about a new connection is being reported. |
| 02 | Connect | A call is connected; the modem is data mode. |
| 03 | Error | The modem rejected the previous command. |
| 04 | No Carrier | The call was disconnected. |
| 05 | No Dial Tone | No dial tone is present. |
| 06 | Busy | The dialed modem is busy. |
| 07 | No Answer | The dialed modem did not answer. |
| 08 | Ring | There is an incoming call. |

The second two characters (byte 1) specify information about a connection that is being made. It is only used for response codes of type Negotiation Progress or Connect, and is one of the following values:

| Value | Error Control Negotiated | Compression Negotiated | Cellular Protocol Negotiated |
| --- | --- | --- | --- |
| 00 | No | No | No |
| 01 | No | Yes | No |
| 02 | Yes | No | No |
| 03 | Yes | Yes | No |
| 08 | No | No | Yes |
| 09 | No | Yes | Yes |
| 0A | Yes | No | Yes |
| 0B | Yes | Yes | Yes |

The next eight characters (bytes 2 – 5) specify the modem-to-modem line speed negotiated in bits per second (bps). The characters represent a 32-bit integer, double-word format (byte and word reversed). Common examples for this value include:

| Bits per second | String |
| --- | --- |
| 2400 | 60 09 00 00 |
| 9600 | 80 25 00 00 |
| 14400 | 40 38 00 00 |
| 19200 | 00 4b 00 00 |
| 28800 | 80 70 00 00 |

The last eight characters (bytes 6 – 9) indicate that the modem is changing to a different port (or “DTE”) speed. Usually, this field is not used, because modems make connections at a “locked” port speed, regardless of the modem-to-modem (or “DCE”) speed. However, for modems that only support “direct” modes, you can lower the DTE speed by specifying a negotiated DTE speed for a response code, using the same format as the DCE speed described in the preceding table.

## Settings Key

The Settings key commands configure various modem settings. After the Init subkey commands are sent, Windows 95 builds a dynamic configuration command string by concatenating various entries which are shown in the following table. The command string depends on the settings selected in the modem’s property sheets.

| Subkey | Description | Example |
| --- | --- | --- |
| Prefix | Configuration command prefix | AT |
| Terminator | Configuration command suffix | <cr> |
| DialPrefix | Dial command prefix | D |
| Dial_Pulse | Use pulse dialing | P |
| Dial_Tone | Use tone dialing | T |
| Blind_Off | Detect dial tone before dialing | X4 |
| Blind_On | Do not detect dial tone before dialing | X3 |
| CallSetupFailTimeout | Specify call setup time-out | S7=<#> |
| InactivityTimeout | Specify inactivity time-out | S30=<#> |
| SpeakerVolume_Low | Low speaker volume | L1 |
| SpeakerVolume_Med | Medium speaker volume | L2 |
| SpeakerVolume_High | High speaker volume | L3 |
| SpeakerMode_Off | Speaker always off | M0 |
| SpeakerMode_Dial | Speaker on during dial and negotiation | M1 |
| SpeakerMode_On | Speaker always on | M2 |
| SpeakerMode_Setup | Speaker on only during negotiation | M3 |
| FlowControl_Off | No flow control | &K0 |
| FlowControl_Hard | Hardware flow control | &K1 |
| FlowControl_Soft | Software flow control | &K2 |
| ErrorControl_Off | Error control disabled (normal mode, not direct) |  |
| ErrorControl_On | Error control enabled (auto reliable) |  |
| ErrorControl_Forced | Error control required to connect (reliable) |  |
| ErrorControl_Cellular | Cellular protocol enabled |  |
| Compression_Off | Compression enabled |  |
| Compression_On | Compression disabled |  |
| Modulation_CCITT | Use CCITT modulations for 300 and 1200 bps | B0 |
| Modulation_Bell | Use Bell modulations for 300 and 1200 bps | B1 |
| SpeedNegotiation_Off | Connect only at default modem speed, do not fall back | N0 |
| SpeedNegotiation_On | Use lower DCE speed to connect, if necessary | N1 |

# Using HyperTerminal

You can use HyperTerminal with a modem to connect two computers for purposes of sending and receiving files or connecting to computer bulletin boards and other information programs. For example, you can use HyperTerminal to connect to an online service, and then, when you’re ready, use HyperTerminal to download files from a bulletin board on the online service. You can also use HyperTerminal to connect a computer directly to another computer, for example, to a debugging terminal.

The easiest way to install HyperTerminal is to choose Custom Setup Type during Windows 95 installation and then perform the following procedure. You can also install it after Windows 95 installation in the Add/Remove Programs option in Control Panel.

¨	To install HyperTerminal during Window 95 installation

1.	Choose Custom Setup Type, and in the Select Components screen, click Communications, and then click the Details button.

2.	In the Details dialog box, click HyperTerminal.

HyperTerminal uses TAPI and the modem to communicate with another computer, whereas Dial-Up Networking uses TAPI and the modem to communicate with a network. For more information, see Chapter 28, “Dial-Up Networking and Mobile Computing.”

Note

¨	To create a new HyperTerminal connection

1.	From the Start button, select Programs, then Accessories, and then click HyperTerminal Connections.

2.	Double-click New HyperTerminal Connection.

3.	In the Connection Description dialog box, type the name of the connection and select an icon to associate with it. Click OK.

4.	In the Phone Number dialog box, specify the country code, area code, phone number, and modem for this connection. Click OK.

When you start HyperTerminal, if you haven’t previously installed a modem, you will be prompted to do so. For more information, see “Setting Up a Modem” earlier in this chapter.

5.	When the Dial dialog box appears, click Dial to begin a phone session, or click Dialing Properties to change your calling card or location information, and how you dial from this location. For more information about Dialing Properties, see “Using Dialing Properties” later in this chapter.

6.	If you chose to dial with a calling card in Dialing Properties, the Change Calling Card dialog box asks for the name and number of your calling card. If you click the Advanced button, the Dialing Rules dialog box appears. For more information about the Dialing Rules dialog box, see “Defining Calling Card Rules” later in this chapter.

7.	When you close the session, HyperTerminal prompts you to save your session definition. Click Yes and you’ll see an icon for the new connection in the HyperTerminal window.

¨	To dial up a remote computer using HyperTerminal

1.	Run HyperTerminal, and then double-click the icon of the connection you want to make.

2.	Click Dial. When the connection is made, the computer you are dialing will display its own access screen or prompt.

3.	When you are done using the connection, from the Call menu, click Disconnect.

¨	To edit HyperTerminal properties for the current session

1.	From HyperTerminal, click the File menu, and then click Properties.

2.	Use the Phone Number property sheet to correct the country code, area code, phone number, or modem information. Use the Configure button to update modem settings.

3.	Use the Settings property sheet to specify preferences for keyboard functions, emulation, and backscroll buffer. Click OK.

¨	To transfer files to another computer using HyperTerminal

1.	When the other computer indicates that it is ready to receive the file, click the Transfer menu, and then click Send File.

2.	Specify the filename to send, or click Browse to find the file.

3.	In the Use Protocol list box, select the protocol to use in transferring the file. Both computers must be using the same protocol. For details about supported protocols, see “File Transfer Protocols” and “Terminal Emulation Types” later in this chapter.

4.	Click Send to begin the file transfer.

¨	To receive binary files from another computer using HyperTerminal

1.	When the other computer indicates that it is ready to send the file, click the Transfer menu, and then click Receive File.

2.	Specify the destination directory for the file, or click Browse to find the appropriate directory.

3.	In the Use Receiving Protocol list box, select the protocol to use in transferring the file. Both computers must be using the same protocol.

4.	Click OK to begin the file transfer.

## File Transfer Protocols

HyperTerminal supports the following file transfer protocols:

| Protocol | Description |
| --- | --- |
| Xmodem | The most common error-correcting data communications protocol. Most communications packages support (and some only support) Xmodem. This protocol is also supported by most online services. Xmodem is slower than other protocols (for example, Zmodem). |
| 1K Xmodem | A variant of traditional Xmodem, which sends data in 1K (1024-byte) blocks instead of 128-byte blocks. On some bulletin boards, this protocol is called Ymodem. |
| Ymodem | A faster version of Xmodem, transferring data in 1K blocks. |
| Ymodem-G | A variant of Ymodem designed for use with modems that support hardware error control. If you cannot transfer files using Ymodem-G, your modem may not support error control. Use Ymodem instead. |
| Zmodem | The fastest data transfer protocol, and the primary choice of most bulletin board users. Zmodem dynamically changes its block size based on line conditions, and is extremely reliable. |
| Kermit | An extremely flexible protocol, found most often on DEC™ VAX™ computers, IBM® mainframes, and other minicomputers. However, Kermit is quite slow, and should not be used if faster options are available on the other computer. |

You can choose a file transfer protocol to use to send or receive a file in the Send or Receive File options in the Transfer menu as described in the preceding procedures.

Note

## Terminal Emulation Types

HyperTerminal supports the following terminal emulation types:

| ANSI | Viewdata (for the United Kingdom) |
| --- | --- |
| Minitel (for France) | DEC VT 100™ |
| Auto Detect | VT 52 |
| TTY |  |

¨	To choose a terminal emulation type

1.	Right-click a connection icon, and then click Properties.

2.	In the property sheets, click the Settings tab, and then select the emulation type.

# Using Phone Dialer

The Phone Dialer application that comes with Windows 95 allows you to use a computer to make voice telephone calls, using calling card and location information defined in Dialing Properties. It also stores frequently dialed numbers, dials stored phone numbers, and logs telephone calls.

The easiest way to install Phone Dialer is to choose Custom Setup Type during Windows 95 installation and then follow the following procedure. You can also install it after Windows 95 installation in the Add/Remove Programs option in Control Panel.

¨	To install Phone Dialer during Window 95 installation

1.	Click Custom Setup Type, and in the Select Components screen, click Communications, and then click the Details button.

2.	In the Details dialog box, click Phone Dialer.

## Dialing With Phone Dialer

With Phone Dialer, you can dial phone numbers in a number of different ways:

·	By clicking a speed-dial phone number (as described in “Storing Phone Numbers with Phone Dialer” later in this chapter)

·	By typing the number, and then clicking Dial

·	By clicking the digits on the Phone Dialer numeric keypad to indicate the phone number you want to dial, and then clicking Dial

·	By selecting from a list of most recently-used numbers

¨	To start Phone Dialer

1.	From the Start button, select Programs, and then Accessories.

2.	Click the Phone Dialer icon. The Phone Dialer dialog box appears with a telephone pad and speed-dial buttons.

3.	To make a call, type a phone number either from your keyboard or use the Phone Dialer numeric keypad.

## Storing Phone Numbers With Phone Dialer

When you dial a number with Phone Dialer, it is automatically stored in a list of your most recently used telephone numbers. All local, domestic long-distance, or international telephone numbers are stored when you type all the digits needed to dial in the Number To Dial text box. You can also store phone numbers as speed-dial numbers.

The Number To Dial text box can hold as many as 40 digits. Therefore, you can type a local, domestic long-distance, or international long-distance phone number in this text box. The same is true when you create speed-dial numbers.

Important

If you type the area code and local telephone number, Phone Dialer automatically precedes the dialed number with a “1” if you have typed this information in the My Locations folder in the Dialing Properties dialog box. Dialing Properties is a separate communications tool described in “Using Dialing Properties” later in this chapter.

If you type non-numeric characters (such as a hyphen or a parenthesis) in a phone number, Phone Dialer ignores the non-numeric character. An exception is when you type a plus sign (+) at the beginning of your number, which is international format. If you type a number in international format, you must put parentheses around the area code, for example, +1 (206) 882-8080.

¨	To create a speed-dial phone number

1.	Run Phone Dialer.

2.	In the Phone Dialer dialog box, click the Speed Dial button you want to configure.

3.	Type the name that will appear on the button and the phone number, then click Save. The name now appears on the selected button.

¨	To change or remove speed-dial settings:

1.	In the Phone Dialer dialog box, click Speed Dial from the Edit menu.

2.	Click the speed-dial button you wish to change.

3.	In the Program Speed Dial dialog box, change or delete the information in the Name and Number fields, and then click the Save button.

According to the North America Dialing Plan (a standard set of procedures that telephone companies in North America use), you must dial a “1,” followed by a three-digit area code, followed by a seven-digit local phone number to make a long distance call. There are a few cases where local phone companies have not followed this rule by allowing you to omit the initial “1” for certain numbers in adjacent area codes. Thus, the number you dial is 10 digits, not 11.

¨	To dial long-distance calls without a “1” prefix

1.	From the Tools menu in Phone Dialer, click Dialing Properties. (For more information about this tool, see “Using Dialing Properties.”)

2.	Verify that the area code listed here is correct for your location. (Change it if necessary.) Click OK.

3.	Using any method you prefer for storing phone numbers, type the phone number as a 10-digit number, omitting the “1” prefix.

The Windows 95 telephony number translation function sees the local area code in both the location and in the destination number, and assumes it should only dial the local number. For example, suppose you are dialing from area code 905 and can dial phone numbers with area code 416 without the “1” prefix. You would store those numbers as +1 (905) 416xxxxxxx where xxxxxxx is the local phone number.

# Using Dialing Properties

Dialing Properties is a utility that has been integrated with Phone Dialer, HyperTerminal, Dial-Up Networking, and other Windows 95 communications applications.

The options you set with Dialing Properties are stored in the TELEPHON.INI file (instead of the Registry) to ensure backward compatibility with older 16-bit Windows telephony-enabled applications.

With Dialing Properties, you can

·	Specify the location from which you are calling, or define a new location

·	Specify the area code and the country code

·	Specify in-house dialing rules

·	Define calling card rules

In Windows 95, you can access Dialing Properties from the Tools menu in Phone Dialer, or by clicking the Dialing Properties button when you make a new connection in HyperTerminal. For purposes of discussion, procedures in this section describe accessing Dialing Properties from Phone Dialer.

## Defining Calling Locations

A location is information that Dialing Properties uses to analyze telephone numbers in international format, and to determine the correct sequence of numbers to be dialed. It need not correspond to a particular geographic location, but it usually does. For example, a location could specify the procedures needed to dial calls from your office, or from a room in a hotel. You can name locations anything you choose to help you remember and select them later. Dialing Properties allows you to add new locations, edit existing locations, and remove locations you no longer need.

¨	To define a location in Dialing Properties

1.	From the Start button, select Programs, select Accessories, and then click the Phone Dialer button.

2.	In the Phone Dialer dialog box, click the Tools menu and select Dialing Properties.

3.	When the My Locations property sheet appears, type information about where you are calling from, including

·	Name of your location, (for example, home or office)

·	Area code

·	Name of country

·	Information about how you dial, including

·	Access number required to make a local or long-distance call

·	Whether you use a calling card to make calls. If you want to change calling card information, click the Change button. For more information about calling cards, see the “Defining Calling Card Rules” later in this chapter.

·	Whether your phone has call waiting and if it should be disabled

·	Whether your phone uses pulse or tone dialing

## Defining Calling Card Rules

A calling card is information that Dialing Properties uses to create the sequence of numbers to be dialed on a particular call. The calling card may include a calling card number which can be dialed at a specified time during call placement. However, the calling card does not have to specify a card number (in other words, calling cards can also be used to define alternative procedures for direct dialing without a calling card number). To help you remember them, you can name calling cards anything you choose.

Dialing Properties includes predefined settings for several popular calling cards used in the United States, including AT&T®, Sprint, MCI, British Telecom, France Telecom Mercury, Telecom New Zealand, and others. You can modify and use these directly or copy them to create your own calling cards. Dialing Properties allows you to add new calling cards, edit existing calling cards, and remove calling cards you no longer need.

The information stored in a calling card includes the name, card number, and the dialing rules, as explained in this section, for local, long-distance and international calls.

¨	To specify that you are using a calling card

1.	In the My Locations folder in the Dialing Properties dialog box, check the Dial Using Calling Card check box.

2.	In the Change Calling Card dialog box, select your calling card type from the list (for example, AT&T), and then type your calling card number. Click OK.

You can also customize Dialing Rules by clicking the Advanced button in the Change Calling Card dialog box. The following two examples illustrate how you might use dialing rules.

Suppose you are making a calling-card phone call from outside the United States and do not want Windows telephony applications to automatically add a “0” prefix to the number you are calling. You can do this by preceding the phone number with another numeric code (such as “144” in the following example).

¨	To redefine numbers that automatically precede the phone number

1.	In the Change Calling Card dialog box, click the New button, then select the name of the calling card and type the card number, and then click the Advanced button.

2.	In the Dialing Rules dialog box, complete the dialing rule fields. Insert a comma to include a fixed two-second pause; use multiple commas to increase the time. Use the following codes as appropriate:

| E | Country code | W | Wait for a second dial tone |
| --- | --- | --- | --- |
| F | Area code | @ | Wait for a ringing tone followed by five seconds of silence |
| G | Destination local number | $ | Wait for a calling card prompt tone (if your modem supports it) |
| H | Calling card number | ? | Display an on-screen prompt to the user to continue dialing |

For example, if you are using a British Telecom card and need to precede the number you dial with “144,” complete the fields as follows:

| Option | Type |
| --- | --- |
| Calls within the same area | 144,H,G |
| Long-distance calls | 144,H,0FG |
| International calls | 144,H,00EFG |

3.	Click OK, and then click OK again. In the My Locations property sheet, remove the leading zero from your area code (for example, “71” or “81” in London). Click OK.

4.	Using Phone Dialer or HyperTerminal (or another tool), store your destination number in international format with only the country code (44), area code (without the leading 0), and local number (+44(71)5551212).

When you choose the Dial option from Phone Dialer or HyperTerminal, the tool you are using automatically expands this number to include “144” and the card number, plus the pause codes.

In another scenario, suppose your organization in the United States uses a PBX system and you need to omit the “1” prefix before the area code. Create a calling card as in the following example:

¨	To omit the “1” prefix with a PBX system

1.	In the Change Calling Card dialog box, click New.

2.	In the Create New Calling Card dialog box, type a name for your calling card, such as “Direct Dial Through PBX.” Click OK.

3.	In the Change Calling Card dialog box, click Advanced.

4.	In the Dialing Rules dialog box, complete the text boxes and then click OK.

| Option | Type |
| --- | --- |
| Calls within the same area | G |
| Long-distance calls | FG |
| International calls | 011EFG |

## Telephony Drivers From Other Vendors

There are several ways to add telephony drivers from other vendors, if necessary. Some telephony drivers work over the network and consist of software only. If this is the case, the software could be installed through the Add/Remove Programs option in Control Panel.

Hardware drivers could be installed through the Add/New Hardware option in Control Panel, unless the hardware requires its own class installer. If this is the case, it would come with a standard Setup program to install it.

The Telephony Control Panel, available in the Windows 95 Software Development Kit, facilitates installation of telephony drivers from other vendors.

If you previously installed the Telephony Control Panel on your Windows 3.1 system, and you upgrade to Windows 95, you will receive the Windows 95 Telephony Control Panel.

Note

# Troubleshooting Communications Problems

This section describes how to solve problems with installing modems, making connections, and with using applications to access the modem. A general modem diagnostic tool is located in the Modems option in Control Panel.

¨	To use the Modems Diagnostics Tool

1.	Double-click the Modems icon in Control Panel, and click Diagnostics.

2.	In the Diagnostics property sheet, click the More Info button to find out additional configuration information about a modem, such as UART, Registry identifier, and what it supports.

In the More Info property sheet, the Port Information box displays the modem’s port assignment, IRQ, I/O address, UART, and highest baud rate. The Modem box displays its device identification and the modem’s response to AT commands.

## Correcting Problems with Modem Installation

This section describes conditions which may interfere with installation of a modem, and how to fix them.

##### The Install New Modem Wizard detected the modem as a Standard Modem.

This does not indicate a problem. Rather, it means that Windows 95 was unable to detect the exact make and model of a modem. Most communications applications work correctly with the Standard Modem option, that is, connections can be made successfully using the modem’s factory default settings. However, advanced control of some features, such as speaker volume, error control (for example, V.42 protocol), and compression (for example, V.42bis protocol) will be disabled in the modem’s property sheet.

If you don’t want to use the Standard Modem option, you can run the Install New Modem wizard to select a specific type of modem which is similar to the modem you are using. If you specify another, similarly named model from the same manufacturer — for example, if you configure the modem as a Practical Peripherals PM9600HC when a Practical Peripherals PM9600FX modem is actually 
installed — Windows 95 usually treats the models as being identical and the specified configuration will probably work well. To determine if other modem models are compatible with the one you’re using, check the modem manual. Many modems are compatible with Hayes®, Microcom®, Rockwell, or U.S. Robotics models.

##### The Install New Modem wizard did not detect any new modems.

·	Make sure the modem doesn’t already appear in the list of installed modems. Windows will not redetect modems that are already installed.

·	Make sure no other programs are running which might be using the modem or its COM port.

·	If the modem is external, check the connection between it and your computer, and reset it by turning it off and on again.

·	If the modem is internal, make sure that its built-in COM port has been configured properly in Device Manager in the System option in Control Panel. If it does not appear, run the Add New Hardware wizard to allow Windows 95 to detect and configure it.

·	Make sure the modem’s COM port is active and has a correct IRQ by checking its Resources configurations in Device Manager in Control Panel. Make sure the IRQ does not conflict with one in use by another device. For details, see Chapter 19, “Devices”.

If the Install New Modem wizard still does not detect a modem, then there is probably a problem with the port, the cable, or the modem itself. Try the modem with an MS-DOS based communications program, or with another computer, if possible.

##### The Install New Modem wizard does not detect a PCMCIA modem.

If the Windows 95 PCMCIA drivers are loaded, then Windows 95 should automatically detect and configure a PCMCIA modem when it is installed. Use the PCMCIA option in Control Panel to check the configuration of a PCMCIA socket driver.

If the Windows 95 PCMCIA drivers are not being used, then the modem card must be configured as a COM port before the Install New Modem wizard can detect and configure it as a modem.

¨	To detect and configure a PCMCIA modem when Windows 95 PCMCIA drivers are not used

1.	Configure the modem according to the instructions that came with your original PCMCIA driver software.

2.	Run the Add New Hardware wizard to detect and configure the card as a COM port.

3.	Run the Install New Modem wizard to detect and configure the card as a modem.

For more information about PCMCIA devices, see Chapter 19, “Devices.”

## Correcting Connection Problems

This section describes basic steps for troubleshooting communications and explains specific communications errors or problems and how to correct them.

##### Modem won’t dial or won’t answer.

If your modem is not set up correctly, communications features may not function properly. The following procedures list steps in verifying the correct operation of your modem and the Windows 95 communications subsystem.

Because some communications programs designed for Windows 3.1 install incompatible driver files, which may cause COM ports and modems to stop working, start by verifying that the correct Windows 95 files are being loaded.

¨	To verify that the required communications files are present

1.	Verify file sizes and dates of COMM.DRV and SERIAL.VXD in the SYSTEM directory against the original versions from the Windows 95 floppy disks or compact disc.

2.	Confirm that the following lines are present in SYSTEM.INI:

[boot]

comm.drv=comm.drv

[386enh]

device=*vcd

3.	To revert to the default communications drivers for Windows 95, delete communications port entries in Device Manager.

4.	Run the Add New Hardware wizard in Control Panel to detect and install the Windows 95 drivers.

Windows 95 does not load the SERIAL.VXD driver in SYSTEM.INI. Rather, Windows 95 loads it on demand using the Registry. Also, there is no corresponding file for the *vcd entry in SYSTEM.INI. This is an internal file built into VMM32.VXD.

Note

¨	To verify the modem configuration

1.	From Control Panel, double-click the Modems icon.

2.	Verify that the manufacturer and model for your modem is correct. If not, you might have changed the modem and failed to reconfigure it. In this case, run the Install New Modem wizard to detect the modem and confirm it with the current Registry configuration.

3.	If your modem does not appear in the list of installed modems, click Add, and then select the appropriate modem.

4.	If the manufacturer and model are not correct and are not available from the list, try the Hayes-compatible option or the Generic Modem driver option, set to the maximum baud rate supported by the modem, and click OK to accept the settings.

5.	Try removing any other modem entries in the list to eliminate any conflicts.

¨	To verify that the modem is enabled

1.	Double-click the System icon in Control Panel, and then click the Device Manager tab.

2.	Select your modem from the list and click Properties.

3.	Click The Device Is Present, Please Use It, if this is not already selected.

¨	To verify that the port is correct

1.	Double-click the Modems icon in Control-Panel.

2.	Select your modem, and then click the Properties button.

3.	On the General property sheet, verify that the listed port is correct. If not, select the correct port. Click OK.

¨	To determine if a serial port I/O address and IRQ settings are properly defined

1.	Double-click the System icon in Control Panel.

2.	Click the Device Manager tab, and then click Ports.

3.	Select a specific port (such as COM2), and click the Properties button.

4.	Click the Resources tab to display the current resource settings (IRQ, I/O) for that port.To find out the correct settings, consult the modem manual.

5.	In the Resources dialog box, check the Conflicting Devices List to see if the modem is using resources in conflict with other devices.

6.	If the modem is in conflict with other devices, click the Change Setting button, and then click a Basic Configuration that does not have resource conflicts.

Do not try to use a modem on COM3 if there is a serial mouse or other device on COM1. Usually, COM1 and COM3 ports use the same IRQ, meaning that they cannot be used simultaneously on most computers. The COM2 and COM4 ports have the same problem. If possible, change the COM3 or COM4 port to an IRQ setting that is not in conflict. Also, some display adapters have an address conflict with COM4 ports. The best way to work around this conflict is to use another COM port, or, replace your graphics adapter.

Note

¨	To check the port settings

1.	In the Modems option in Control Panel, click a modem, and then click Properties.

2.	In the modem’s property sheets, click the Connection tab to check the current port settings, such as bits per second (baud rate), data bits, stop bits, and parity.

Click the Advanced button to check error control and flow control. If you are using Win16-based applications, turn off these advanced features.

3.	Verify the UART type.

Data transmission problems may occur when a baud rate greater than 9600 is selected on a slower 80386-based computer not equipped with a 16550 UART, or when performing other tasks during a file download. If problems or errors occur during transmission, try lowering the baud rate. Attempting to use baud rates greater than 9600 on computers equipped with 8250 or 16450 UARTs will probably result in dropped characters.

¨	To check modem baud rate

1.	Double-click the Modems icon in Control Panel.

2.	Select the specific modem, and click the Properties button.

3.	Click the General tab.

4.	Set the baud rate to the correct speed. Lower speeds may work, especially when using an older, slower computer.

5.	Click Only Connect At This Speed if it is not already selected.

If the host system you are calling cannot communicate at the initial baud rate, it may or may not be able to negotiate a slower baud rate.

Note

To optimize communications performance, you can set the baud rate higher if your computer has an 80486 or a Pentium™ processor.

Tip

¨	To disable hardware flow control if your modem cable doesn’t support it

1.	Double-click the Modems icon in Control Panel.

2.	Click a modem, and then click the Properties button.

3.	Click the Connection tab, and then click the Advanced button.

4.	If a check appears in the Use Flow Control check box, click the box to clear it.

¨	To remove VxDs from other vendors which may be interfering with modem operation

·	Search for and comment out (type a semicolon as the first character of the line) any related entries in the SYSTEM.INI file. When commenting out a line in the SYSTEM.INI file, it is useful to add a comment line denoting this.

##### The initialization (dialing) string is improper for the modem.

If the modem won’t pick up the line and dial, it may be due to an improper initialization string. Typically, the manufacturer’s recommended dial command string is loaded from the corresponding modem INF file; however, if your modem driver was not available and you selected a compatible modem, the dial command string may not work correctly. Try using the modem types, and retest the modem dialing the selected number.

##### The modem repeatedly drops the connection.

1.	Check for a bad or loose serial cable to the modem. If all connections are tight, test for a faulty cable by replacing it with a working cable, and retest the modem communications.

2.	Check the connection between the RJ-11 phone outlet and the modem. Verify that the connection is firmly plugged in and well connected.

3.	Try using a different phone line. If you have ruled out other factors, consistent modem errors may be due to problems in the telephone line used for communication.

4.	Disable call waiting, if it is in use. The call waiting feature can interfere with remote connections and file transfers. If you use this feature regularly, only disable it temporarily, during the time the modem is in use.

5.	Check communications with host. The communications problems may be due to the host computer not connecting or repeatedly dropping the line.

6.	Try using a lower speed in the modem property sheets in the Modem option in Control Panel.

##### The COM Ports remain in Device Manager after the modem is removed.

After installing an internal modem and assigning it to a COM port that does not physically exist on your computer, the port appears in the Device Manager. After removing this adapter, you may also need to manually remove the port in Device Manager.

1.	Double-click the System icon in Control Panel.

2.	Click the Device Manager tab, and then click Ports.

3.	Click the Communications Port you want to remove, and then click the Remove button.

##### You are unable to dial international calls

Windows 95 allows you to set specific modem properties that assist you when making international calls over your modem. You set these properties in Dial-Up Networking, Dialing Properties, and in the modem property sheets.

¨	To check location and calling card settings

1.	In Phone Dialer, click Tools and select Dialing Properties.

2.	In the My Location folder, verify that your calling location and calling card settings are correct.

¨	To disable dial-tone detection if your modem fails to detect a dial tone

1.	Double-click the Modem option in Control Panel, and then click the Connection tab.

2.	In the Call Preferences field, click to clear the check box next to the Use Dial Tone option.

¨	To increase the time between dialing if connections are taking a long time

1.	Double-click the Modem option in Control Panel, and then click the Connection tab.

2.	In the Call Preferences field, increase the number of seconds in the Cancel the Call If Not Connected Within option.

¨	To manually control modem dialing

1.	In all Windows 95 communications applications, right-click a connection icon, and then click Properties.

2.	Click the Configure button, and then click the Options tab.

3.	Click the Operator Assisted or Manual Dial option, and then click OK.

4.	Double-click the connection icon, and then click Connect.

5.	When instructed, pick up the phone and dial the number.

6.	When you hear the other computer answer, click the Connect button and hang-up.

¨	To bring up a terminal window to type AT commands before or after dialing

1.	In all Windows 95 communications applications, right-click a connection icon, and then click Properties.

2.	Click the Configure button, and then click the Options tab.

3.	Click the Bring Up Terminal Window Before Dialing option, or the Bring Up Terminal Window After Dialing option, and then click OK.

When you make a connection for this number, a terminal window appears in which you can type AT commands.

## Correcting Modem Access Problems of Applications

##### You can’t send or receive binary files using HyperTerminal.

1.	Make sure that both computers are using the same file transfer protocol (that is, Xmodem, 1K Xmodem, Ymodem, Ymodem-G, Zmodem, or Kermit).

2.	If you are using the Ymodem-G file transfer protocol, ensure that your modem supports hardware error control. If it doesn’t, try using Ymodem instead.

3.	If you are trying to use an alternative protocol such as Kermit and encounter transmissions errors, try Xmodem instead. Most communications packages, bulletin-boards, and online services support Xmodem.

##### You cannot dial with Phone Dialer.

1.	From Phone Dialer, click the Tools menu, and select Dialing Properties.

2.	Make sure your area code and country code are correct.

3.	For each access number you want to use for calling out, specify, at a minimum, the country code, area code, and telephone number.

4.	Verify basic modem and port configuration.

##### Win16-based applications can’t access the modem, but MS-DOS – based or Windows 95-based applications can.

Make sure the communications driver for Windows 3.1 applications is COMM.DRV in the SYSTEM.INI file. Some applications replace this driver for various reasons.

##### MS-DOS -based applications can’t access the modem (especially PCMCIA modems), but all Windows-based applications can.

1.	If possible, adjust the IRQ setting in the MS-DOS – based application according to the application’s documentation.

2.	If the MS-DOS application’s IRQ settings cannot be adjusted, adjust the IRQ settings for the modem COM port as described earlier in this section.

##### MS-DOS – based and Windows 3.1-based applications can’t access the modem, but Windows 95-based applications can.

Make sure that Microsoft Exchange Remote Mail, Microsoft Fax, and Dial-Up Networking are not waiting for incoming calls. If they are, older applications cannot access the modem.

##### Errors occur during MS-DOS – based applications communications sessions, especially file transfers.

Increase the COMxBuffer setting in the SYSTEM.INI file in the 386 Enhanced section. The default value is 128 bytes.