+---------------------------------------------------------------------------+

|                                                                           |

|                    CADSIM keyboard macro scripts                          |

|                                                                           |

|  This file contains information about CADSIM macros (for all CADSIM       |

|  platforms).  As macros are new, they were not included in the CADSIM     |

|  User's Manual.  The information appears in chronological order, newest   |

|  last.  Where possible, forward references have been inserted next to     |

|  items whose information has been superceded by new information later in  |

|  this file.                                                               |

|                                                                           |

|  Refer also to other companion read-me files:                             |

|    CADSIM.DOC - CADSIM graphical simulation interface.                    |

|                                                                           |

+---------------------------------------------------------------------------+

----------------------- CADSIM Version 2.6 ----------------------------------

- KEYBOARD MACRO RECORD, PLAY AND PROGRAMMING:       -- (new) January, 1993

Keystrokes and mouse action can be recorded in keyboard macros by hitting

Ctrl-K on the keyboard.  Each macro can be named by a letter (K is the

default).  To end macro recording, hit Ctrl-K a second time.  The macro

can be played back by hitting Alt-x (where x is K for the default, or

other letters for permanently stored macros).  Logic and programming can

be added to macros using a text editor.  A text editor can also be used to

create a macro from scratch.

Each macro is stored in a file named ALT-x.MAC where x is the macro letter.

This macro letter can be any letter A to Z or any digit 0 to 9.

Macros can be stored in a central library using the  MACROS_LIBRARY path

command in the CADSIM.INI file (see the CADSIM.DOC readme file for further

information.  If the local CADSIM.INI file does not contain a

MACROS_LIBRARY command, macros will be looked for in the current working

directory.  However, the default Keyboard macro ALT-K.MAC is always local.

A macro can be launched automatically when CADSIM is started by including

the switch:

CADSIM [filename] -Sx

where "filename" could be any CADSIM drawing file and "x" could be any

letter which represents a previously recorded macro.  A good example of

how this can be set up is the DEMO macro which was installed in a

subdirectory (or sub-folder) of CADSIM called DEMO.  The DEMO macro

(Alt-d) can be started by typing DEMO at an OS/2 or DOS command prompt;

or by double clicking on the NEW.DRA icon in the DEMO folder and then

starting the demo macro by pressing the Alt and D keys on a Macintosh

keyboard.  This macro automatically draws and specifies a complete simple

balance from scratch.

Macro definition:

A macro is a set of keyboard strokes or mouse actions, mixed with

optional commands and programming, recorded in a script file, for replay

to automate CADSIM drawing tasks.  Macro files are text files that can

be edited with a text file editor.

The syntax of a macro script file includes:

- keystrokes (any letter, as if it had been typed at the keyboard)

- commands which are always enclosed in square brackets [ ].

- a set of commands and keystrokes which are to be processed together

as a condition of some command.  The set of commands and keystrokes

are always enclosed in braces { }.

The following sections list the possible contents of a macro file:

KEYBOARD:

Any alphanumeric character can appear in a macro.  For example: SLCD would

Set the Line Color to D.

[Left] - left cursor key hit

[Right] - right cursor key hit

[Up] - up cursor key hit

[Down] - down cursor key hit

[Enter] - enter key hit

[Altx] - any key 'x' hit while the Alt key is held down (ie. play macro x)

MOUSE BUTTONS:

[LMB] - left mouse button single click

[MMB] - middle mouse button single click

[RMB] - right mouse button single click

[PushLMB] - left mouse button push and hold

[PushMMB] - middle mouse button push and hold

[PushRMB] - right mouse button push and hold

[DoubleLMB] - left mouse button double click

[DoubleMMB] - middle mouse button double click

[DoubleRMB] - right mouse button double click

TIME:

[Wait nnnn] - wait nnnn milli-seconds (ms) for real time emulation.

[Pause] - waits for any key to be pushed on the keyboard before proceeding.

MOTION:

[RelMove dx dy nnnn] - move relative by dx and dy pixels

- use nnnn milli sec. to move to that location

[AbsMove x y nnnn] - move to the absolute x and y pixels

- use nnnn milli sec. to move to that location

[MenubarX nnnn] - move to the menubar item activated by the letter X

- use nnnn milli sec. to move to that location

- this only moves to the menubar; use [LMB] to activate

Movement examples:

[Comment: draw a line from point (100,200) to (200,200)]

[AbsMove 100 200 0]M[RelMove 100 0 0]L

PROGRAMMING COMMANDS:

[Comment Any string] - a comment using text such as "Any string"

[Display1 Any string] - displays the string "Any string" in display field 1

[Display2 Any string] - displays the string "Any string" in display field 2

[Display3 Any string] - displays the string "Any string" in display field 3

[End] - stop macro execution and return to user input

[SetNumber i nnn] - set the i'th integer variable to the number nnn.

- there are 20 integer variables that can be used

in commands.  Therefore i can be from 1 to 20.

- optionally nnn can be given as ?n which substitutes

the current value of the n'th integer variable for

the assignment.

- in fact ?n (where 1 <= n <= 20) can be used in any

command within the [ ] and it will be expanded into

the current value of the n'th integer variable before

the command is executed.

[AskNumber i Prompt string] - prompt for user input of a number for the

i'th integer variable

- "Prompt string" is any text to be used

in the prompt to clarify its purpose.

[AskPoint m n Prompt string] - prompt for the user to digitize a location.

- The x position of the point will be put in

the m'th integer variable

- The y position of the point will be put in

the n'th integer variable

- "Prompt string" is any text to be used

in the prompt to clarify its purpose.

[AddNumber i nnn] - add the number nnn to the i'th integer variable

- nnn can be a negative number to decrement

- optionally nnn can be given as ?n which substitutes

the current value of the n'th integer variable as

the added amount.

[MulNumber i nn.n] - multiply the i'th integer variable by the floating

point number nn.n, make an integer of the result,

and assign it to the i'th integer variable

- nn.n can be a fractional number to divide

- optionally nn.n can be given as ?n which substitutes

the current value of the n'th integer variable as

the multiplier.

Arithmetic examples:

[SetNumber 2 100][Comment: variable 2 now has a value of 100]

[AddNumber 2 100][Comment: variable 2 now has a value of 200]

[SetNumber 3  ?2][Comment: variable 3 now has a value of 200]

[AddNumber 2  ?3][Comment: variable 2 now has a value of 400]

[MulNumber 2 0.5][Comment: variable 2 now has a value of 200]

[if X < nnn]{ } - if the current x location of the graphics cursor is

less than nnn, perform the function given within the { }

- optionally nnn can be given as ?n which substitutes

the current value of the n'th integer variable as

the comparison.

[if X > nnn]{ } - if x is greater than nnn perform the functions in { }

[if X = nnn]{ } - if x is equal to nnn perform the functions in { }

[if Y < nnn]{ } - if the current y location of the graphics cursor is

less than nnn, perform the function given within the { }

[if Y > nnn]{ } - if y is greater than nnn perform the functions in { }

[if Y = nnn]{ } - if y is equal to nnn perform the functions in { }

[if ?i < nnn]{ } - if the i'th integer variable is less than nnn, perform

the function given within the { }

- optionally nnn can be given as ?n which substitutes

the current value of the n'th integer variable as

the comparison.

[if ?i > nnn]{ } - if the i'th variable is greater than nnn perform { }

[if ?i = nnn]{ } - if the i'th variable is equal to nnn perform { }

If example:

[if X > 100]{[RelMove -100 0 0][LMB]}

[Repeat nnn]{ } - repeat the functions in { } nnn times

- optionally nnn can be given as ?n which substitutes

the current value of the n'th integer variable as

the number of repeats.  The number of repeats are

initialized only once and are not modified if the n'th

variable is modified within the { }.

Repeat example:

[AskNumber 5 Number of parallel lines]

[Display2 You asked for ?5 parallel lines]

[AskPoint 6 7 Point to start drawing lines]

[Display2 Drawing lines from point (?6,?7)]

[Repeat ?5]{[Relmove 50 0 0]L[Relmove -50 -10 0]M}

[SetSwitch j X] - set the j'th Case switch to letter X

- there are 20 Case switch variables that can be used

in Case commands.  Therefore j can be from 1 to 20.

[AskSwitch j Prompt string] - prompt for user input of a letter for the

j'th Case switch

- "Prompt string" is any text to be used

in the prompt to clarify its purpose or

give example settings.

[Case j X]{ } - perform the functions declared within the { } if the j'th

Case switch is equal to letter X.

- Case switch commands are meant to be used in a series

where only one of the Case switches are performed.

- [Case j ?] is the default case and its functions are

performed only if one in a set of previous contiguous

[Case j X] commands were not performed.

Case example:

[SetSwitch 1 M]

[SetSwitch 2 Z]

[Comment: The following will do a middle mouse button single click]

[Case 1 L]{[LMB]}[Case 1 M]{[MMB]}[Case 1 R]{[RMB]}[Case 1 ?]{[Enter]}

[Comment: The following will do an Enter key hit]

[Case 2 L]{[LMB]}[Case 2 M]{[MMB]}[Case 2 R]{[RMB]}[Case 2 ?]{[Enter]}

More examples can be found in the MACROS sub-directory of the CADSIM

directory.  The example macros perform the following functions:

- ALT-1.MAC changes view to the top left hand corner of the drawing

- ALT-2.MAC changes view to the top right hand corner of the drawing

- ALT-3.MAC changes view to the bottom left hand corner of the drawing

- ALT-4.MAC changes view to the bottom right hand corner of the drawing

- ALT-V.MAC pans the drawing in steps from top left to bottom right

- ALT-Z.MAC zooms in steps between fine and medium zoom scales