3_Drives v2.6  Shareware  version Released August 20th, 1993

4_Drives v3.01 Registered version Released August 20th, 1993

������͵  3_Drives v2.6  �

�                                  ����������������Ŀ    ����������������Ŀ

�      Dustbowl Designs, Inc.      �     426 MB     �    �����0   MB�������

�      -=� NOT REGISTERED �=-      � ������������� �    ������������������

�    "Dedicated to enhancing the   ������������������    ������������������

�    world's personal computers."     E:  Installed         4_Drives only

�       (405) 741-2721 BBS

���������������������������������������������������������������������������Ŀ

�     Copyright 1992 by Dustbowl Designs, Inc. All rights reserved.         �

�         12600 S.E. 38th Street, Choctaw, Oklahoma USA 73020-6107          �

�   See 3_Drives.doc for v3.01 registration information. (405) 741-4705     �

�����������������������������������������������������������������������������

The universal device driver that lets you add an additional 16-bit

interface to your system.  Add up to two **= MORE =** MFM, RLL, ESDI

or AT/IDE hard disk drives regardless of the first interface type.

��������������������������������������������������������������������Ŀ

���� What's New: v2.6 ������������������������������������������������

����������������������������������������������������������������������

Happy Birthday, 3_Drives!!  In celebration of a full year in

shareware, we've decided the best way to thank everyone was to move

one of the Registered version's features into the Shareware release.

Now, both Registered and Shareware versions support the new READ FAST

routine that can be used on all drive interface types (MFM, RLL, ESDI

and IDE). This fast I/O routine yields data transfer rates that equal

the drive's normal performance on the Primary Port Address! (See

Command Line Switches for instructions on setting this option.)

Included in this ZIP file is 4DRIVES.PCX, in a standard graphics file

format used by most paint programs and picture viewers, that shows an

example of one drive's performance results. (BTW, if you'd like to

graphically measure your own drives, look for 4_Speed soon on your

local BBS or call ours. 4_Speed is Freeware and works on all

interfaces and both port addresses!)

Beginning with release v2.6, the device driver's filename will change

to a format that reflects the current version number.  Older releases

were simply named 3_DRIVES.HDD... now it is named 3DRVS260.HDD.  If

you're still evaluating an older version of 3_Drives, then be sure to

update your CONFIG.SYS file to the new name. Do not rename the new

device driver to the old name and please consider registration of the

shareware.

Beginning with release v2.50, a new utility is provided called

4DRVUTIL.EXE. This is an AT/IDE Identify Drive utility that supports

both the Primary and Secondary Port Addresses.  The emphasis with

4DRVUTIL is to help show what advanced features on your AT/IDE drives

are available and then give the appropriate command line switches for

both 3/4_Drives and our new product, DiskQwik. See 4DRVUTIL.DOC for

details.

Instant Upgrade(/Conscience?) Satisfaction.  Registration AND Download

of the registered versions can be accomplished in only ONE call to our

BBS!  Using our custom Credit Card Charge/Download Door, you can move

into our product line's most advanced features in just a matter of

minutes.

��������������������������������������������������������������������Ŀ

���� Introduction ����������������������������������������������������

����������������������������������������������������������������������

3_Drives v2.6 is a shareware CONFIG.SYS block device driver that

allows the utilization of a SINGLE hard disk drive on the standard

Secondary Port Address using the DOS operating system giving you the

potential total of FOUR fully functional hard drives in your system!

This unique device drive also permits two incompatible controllers

and/or host adapters to COEXIST in the same system.

*  With 3_Drives you can combine MFM, RLL, ESDI, AT/IDE and

SCSI hard disk drives......

Primary Port       |       Secondary Port

Address         |           Address

|

Drive 1   Drive 2   |

�����Ŀ   �����Ŀ   |   ������� 4_Drives �����ͻ

� RLL �   � RLL �   |   � � 3_Drives �         �

�������   �������   |   � � �����Ŀ  � �����Ŀ �

�����Ŀ   �����Ŀ   |   � � � IDE �  � � IDE � �

� MFM �   � MFM �   |   � � �������  � ������� �

�������   �������   |   � � �����Ŀ  � �����Ŀ �

�����Ŀ   �����Ŀ   |   � � �ESDI �  � �ESDI � �

�ESDI �   �ESDI �   |   � � �������  � ������� �

�������   �������   |   � � �����Ŀ  � �����Ŀ �

�����Ŀ   �����Ŀ   |   � � � MFM �  � � MFM � �

� IDE �   � IDE �   |   � � �������  � ������� �

�������   �������   |   � � �����Ŀ  � �����Ŀ �

�����Ŀ   �����Ŀ   |   � � � RLL �  � � RLL � �

�SCSI �   �SCSI �   |   � � �������  � ������� �

�������   �������   |   � ����������;         �

|   ����������������������ͼ

|

Our systems handle   |   Any one of these interfaces

this side already.   |   can be added using 3_Drives

|   or 4_Drives.

Any of these works...>>>>> with any of these!

*  This means you DO NOT have to retire your older MFM  or RLL

drives and controllers if you purchase a new AT/IDE hard

disk drive!!!

*  Allows you to still boot to a SCSI C: drive in a system

with IDE, MFM, RLL or ESDI.

*  If you have two IDE drives that will not work together due

to a manufacturer's Master/Slave incompatibility, 3_Drives

lets them coexist without conflict as both being Masters.

*  With 3_Drives you can easily use an AT/IDE, MFM, RLL or

ESDI drive to backup data and/or transport large quantities

of data between systems at hard drive speeds.

��������������������������������������������������������������������Ŀ

���� Upgrade Offer ���������������������������������������������������

����������������������������������������������������������������������

As noted in the screen representation and at the beginning of this

document, 3_Drives v2.6 is shareware and supports a SINGLE primary

partition on a SINGLE hard disk drive at the Secondary Port Address.

In the event that you want to activate the High Speed Data Transfer

Rate options, add a SECOND drive to the Secondary Port Address, desire

access to logical drives in extended partitions, we offer the

programs: Registered v3.01 3_Drives and 4_Drives.

Registered version benefits and features:

* MULTIPLE sector Block Mode commands supported on newer AT/IDE

drives for data transfer rates that exceed the primary port

address!

* FAST WRITE companion to the FAST READ rountine for all interfaces.

* Support for up to 10 logical drives per physical drive.

* Ultra-fast program loading without delays or required user

intervention. (If you're getting tired of the random character

starter and prolonged registration reminder, then it is probably

time to register <grin>.)

* Includes 4_Park, a four drive head-parking/screen-saver utility.

* Postscript version of the documentation.

* Advanced technical support and continuous upgradability to all

future registered versions via our BBS.

��������������������������������������������������������������������Ŀ

���� System Requirements ���������������������������������������������

����������������������������������������������������������������������

A few simple conditions must be met in order to utilize the 3_Drives

device driver.

They are:

1. Computer system MUST be either a 286, 386sx, 386dx, 486sx or

486dx.

2. DOS level must be v3.0 or higher, including Digital Research

DR DOS 6.0, with a single Primary partition (3_Drives v2.6

shareware version, only).

** Registered 3_Drives or 4_Drives v3.01 also support Extended

partitions with up to 10 logical drives per physical unit.

3. A second Controller or Host Adapter which must have the option

to set a Secondary Port Address to 170h and to disable IRQ14

monitoring.  The Secondary Port Address feature is available on

practically every hard drive controller made and the disable

IRQ14 feature is found on about half of the adapters, but is

"do-able" on 100% of them using a simple tape-over method (See

App Note #10 for "Tape-Over Method"). Jumpers and settings for

the Seagate ST-07A, ST-08A, Acculogic SIDE-3 AT/IDE Host

Adapters and Western Digital WD1006-xxx, Adaptec ACB-231x,

Adaptec ACB-237x, Data Technology DTC728x MFM/RLL ST506/ST412

Interface controllers are included in this doc file to show you

how this is done. (See Controller and Host Adapter Setup

Instructions).

��������������������������������������������������������������������Ŀ

���� Preparing "Secondary Port" Disk Drives ��������������������������

����������������������������������������������������������������������

The disk drive that will be attached to the Secondary Port Address

needs to be partitioned and formatted on the Primary Port Address

prior to using the 3_Drives/4_Drives device driver.  Use the standard

DOS utilities, FDISK and FORMAT, or automatic installation software to

accomplish the task.  This will probably require temporarily removing

the existing hard disk drive controller or preparing the "Secondary"

drive in a different system.

BEFORE disengaging the "Primary" drives, BE SURE to write down their

CMOS drive type(s). If 'User Defined' or 'Custom' drive types are

being used, write down the associated cylinder, head, write precomp,

lzone and sector per track values.

(If the drive you intend to install on the Secondary Port

Address is prepared and ready to go, jump to step #9 below)

The installation steps to take for an unprepared drive without data

are roughly as follows:

.5  Finish reading the rest of this document.

1. Satisfy yourself that BACKUPs of your data are current and in

good condition.

2. Prepare a bootable floppy diskette and copy (or simply have

available on a separate diskette) FDISK and FORMAT to this

floppy.

3. Install (on the default Primary Address) the controller or host

adapter for the drives you eventually intend to use on the

device driver and set the appropriate drive type number in the

system CMOS setup. Remember, each controller will support one or

two drives.  The drive jumpers that establish drive selects or

Master/Slave relationships follow the same rules as they would

in a single controller system. Do not set a MFM or RLL to Drive

Select three!

4. Boot to the floppy diskette.

5. Use FDISK to define the Partition(s). The system will

automatically reboot to the A: drive after concluding FDISK.

6. FORMAT C: /S (Registered versions: including any other logical

drives in extended partitions without the /s).

7. Confirm that the hard drive boots without the floppy in A: and

can perform reads and writes, and is good working condition.

8. Restore the CMOS drive types to the ones used on the original

1st (and 2nd) drives. Power down the system and reinstall the

controller for the original drives.

9. Set the second Host Adapter for the "Secondary" drive to

Secondary Port Address 170h and disable IRQ14 monitoring. (see

below for examples of popular Host Adapter/Paddle Board and

Controller jumper settings). You do not have to make any changes

to the drive select jumpers or Master/Slave jumpers.

10. Reboot. The system should start in the same way it always has.

11. Copy the 3DRVS260.HDD program into the root directory of the C:

drive and Edit the CONFIG.SYS file to include the line:

DEVICE=C:\3DRVS260.HDD

This line should precede any other block device drivers (such a

RAMDRIVE.SYS). If a Disk Manager (tm Ontrack Computer Systems)

partition is installed on the original "Primary" drives, the

DEVICE=C:\3DRVS260.HDD line should follow after DMDRVR.BIN.

12. Reboot. An 3_Drives installation screen (shown above) will

indicate the loading status of the device driver. The drive

letters assigned to the "Secondary" drive(s) will depend on the

number of partitions on your C: and D: drives.

��������������������������������������������������������������������Ŀ

���� Command Line Switches �������������������������������������������

����������������������������������������������������������������������

These switches can be set up in the CONFIG.SYS file after the complete

driver name and should be separated by spaces for clarity.

The following command line switch have been put into place for BOTH the

Shareware and Registered versions:

No Command Line Switches - Functionally equivalent to our previous

releases.  Without any of the Fast I/O

options.

ATA  - This switch enables new reset and recovery features that are

available on the newer IDE 'ATA' spec drives. You can try this

switch for improved error recovery on MFM/RLL/ESDI controllers

as well but if the driver locks up remove this switch.

@(?) - The ? is defined as 0 for drive 0 and 1 for drive 1. This

switch implements a solution to a reported problem where the

IDE drive being setup on the secondary port address would not

initialize properly unless it knew it's drive parameters first;

this condition does not follow the ATA specification correctly

and is like "putting the cart in front of the horse."

Nonetheless, this switch allows 3_Drives/4_Drives to initialize

drives that require this reversed wakeup call.

RF(?) - Where RF = READ FAST, and the ? is defined as 0 for drive 0

and 1 for drive 1. (drive 1 only available on registered

v3.01) This option can be used on all interfaces.

Examples:

device=driver.hdd rf0   - enables the read fast I/O

algorithm for drive 0 only.

device=driver.hdd rf0 RF1 - enables the read fast algorithm

on both of the driven drives.

The following command line switches have been put into place for the

Registered versions:

M(?)(+) -  Where M selects the MULTIPLE Sector Block data transfer mode and

the ? is either a 0 to select this mode on the first drive

attached to the secondary controller or a 1 to select the

multiple sector data transfer mode for the second drive

attached to the secondary controller. This capability is

usually found on newer AT/IDE drives.

The + is used to set the block mode to the maximum that the

drive can handle. Without the + switch, 1/2 of the maximum

available block size is taken.

(CAUTION! Use the "+" switch with care as some drives will

reserve the entire onboard read/write cache ram for a

multi-block data area. This will slow the drive's overall

data xfer rate to an all time low. Use 4DRVUTIL.EXE to see

exactly what resources the drives have available.)

Examples:

device=driver.hdd M0     - selects multiple or block mode on

drive 0 only.

device=driver.hdd M0+ m1 - selects the multiple mode to be

enabled on both of the driven

drives.  The first drive will

utilize the full capacity given

for block mode xfers.

WF(?) - Where WF = WRITE FAST and the ? selects drive 0 or drive 1.

This is the companion to READ FAST and can be used on all

interfaces.

Examples:

device=driver.hdd wf0     - enables the write fast algorithm

for drive 0 only.

device=driver.hdd wf0 wf1 - enables the write fast algorithm

for both drives.

Q    - This switch enables a Quick driver init without delays usually

generated for display recognition.

P    - This switch enables a "press c to continue.." pause after the

driver loads for debug and command switch confirmation.

3*   - This switch stops 4_Drives from looking for a non-existent

fourth drive.  Normally, the routines can determine the

presence or lack of the fourth drive unit. During our beta

testing, we ran in a drive/controller combination that

stubbornly reported a "phantom" fourth drive, this switch puts

a stop to that nonsense. (to clarify the switch, it is 3

asterisk)

MORE EXAMPLES:

device=driver.hdd ata @0 m0 rf1 wf1 p

these switches will enable the following:

ATA reset and recovery routines with a reversed "wakeup call"

on drive 0. Multiple mode on drive 0, both Read Fast and Write

Fast on drive 1, the driver will Pause after loading until the

letter c is pressed.

device=driver.hdd wf0 p rf1 wf1 q

these switches will enable the following:

Write Fast on drive 0, a Pause until 'c' is pressed after the

driver loads, both Read Fast and Write Fast on drive 1, no

screen delays will be utilized in the init code.

CONFIRMATION of Device options:

Visual confirmation of the I/O option combinations will be indicated

by the COLOR of the "led" indicator on the 4_Drive banner screen drive

icon....

����������������Ŀ

�     426 MB     �

� �*������������ �

���^��������������

|

|

RF(?)  - will be denoted by a BLUE * (led) indicator for the drive

selected.

WF(?)  - will be denoted by a YELLOW * (led) indicator for the drive

selected.

RF(?) & WF(?) - will be denoted by a CYAN * (led) indicator for the

drive selected.

M(?)(+)- will be denoted by a RED * (led) indicator for the drive

selected and the size of the multiple block will be printed

below the drive capacity information.  This KiloByte size

will usually double if the + switch is included.

P      - The "Press c to continue", message will be displayed in the

bottom left corner of the yellow text box and the 'c' key

must be pressed before the driver's init code is terminated.

Q      - The screen display will not slow to allow recognition of

drive capacity, I/O option configuration confirmation or

Multi-mode buffer size option confirmation.  This switch will

have a more noticeable affect on drives that have multiple

logical partitions installed on them.

ATA    - Sorry, but we decided not to give a visual indication of the

implementation of this switch.

��������������������������������������������������������������������Ŀ

���� Background ������������������������������������������������������

����������������������������������������������������������������������

In the past, three major factors have stood in the way of achieving

the elusive goal of supporting more than two hard disk drives or even

two disk drives of dissimilar interfaces.  First, DOS versions prior

to the v5.0 would support a maximum of two physical hard disk drives.

The v5.0 DOS FDISK supports up to eight physical hard disk drives if

the Host Adapter is designed to follow the FDISK conventions of DOS

v5.0. (Some SCSI interface Host Adapters are now out in the

marketplace which have this support.)

Second, standard interface hard disk drive controllers (except for

SCSI) are limited to two physical drives.  In the case of MFM, RLL and

ESDI, the drives usually have a Drive Select 3 and 4 jumper (DS3 &

DS4), but the controllers for these interfaces use the industry

standard "Daisychain" configuration which allows only two drives to be

connected. (The rare "Radial" configuration uses the DS3 and DS4.) In

the case of the new AT interface for hard disk drives, also known as

IDE, the controller is built onto the drive itself and is limited by

design to two physical drives per Host Adapter - Master and Slave.

Third, controllers for these interfaces usually require a drive type

number to be set in CMOS and only two slots are ever given by the

system BIOS.

Until 3_Drives/4_Drives, two AT controllers have generally been unable

to coexist because of a condition known as a "port address conflict."

In effect, both controllers canceled each other out because they

needed to use the same Hard Drive Primary Port Address, 1F0h.  In its

default configuration, DOS only recognizes hard disk drives that are

attached to this Primary Port Address and communicates with them

through the system bios and INT13.  A block device driver is required

to service drives that are attached to the common Secondary Port

Address, 170h; the 3_Drives/4_Drives device driver serves this

function.

��������������������������������������������������������������������Ŀ

���� Outline ���������������������������������������������������������

����������������������������������������������������������������������

An brief outline of how 3_Drives works is as follows:

After partitioning and formatting in the DOS environment, every hard

disk drive, regardless of interface, controller, or CPU has a very

special sector of information called the Master Boot Record (MBR). The

MBR is always the very first physical sector on the disk (cyl 0, hd 0,

sector 1), the contents of which 1) preserve the defined partition

structure, 2) contain the first executable instruction to begin

loading the active operating system, and 3) a final word signature in

the sector, AA55h, indicating a valid Master Boot Record.

Once a valid MBR is identified, the values stored in the partition

record of the MBR indicate the location of the DOS Boot Record (DBR)

which is generally located at logical cyl 0, hd 1, sector 1.  This DOS

boot sector contains two vital components - the BIOS Parameter Block

(BPB) structure and a loader routine for the operating system .  The

BPB provides a logical description of the hard disk drive's geometry

(logical blocks, heads, sectors per track, bytes per sector, etc.),

while the operating system loader establishes the names of the system

files IO.SYS and MSDOS.SYS, their root directory locations and then

loads them into memory .

Next, the IO.SYS file is read into memory, followed by the MSDOS.SYS.

These are the names of the two hidden files which accompany

COMMAND.COM after formatting a bootable drive. These programs begin an

initialization routine which establishes the resident character and

block devices found during the Power On Self Test (POST) and

motherboard ROM BIOS initialization.  Once the basic hardware has been

identified and the DOS kernel has been positioned into low memory, the

next stage is to run a system module within IO.SYS called SYSINIT.  It

is at this time that the first of the Primary Port Address drive

letter designations are assigned (A:, B:, C:, etc.). These block

devices are given special handling at this point.  For each, a pointer

to a BPB array is established in memory.

Next, begins the process we are most interested in, namely the

processing of the CONFIG.SYS file.  The entire file is read into

memory and analyzed for block and character type device drivers.  When

block device drivers are identified, like 3_Drives, their entries

are added to the drive translation table and given the next available

drive letter designation.

Device drivers, like regular programs (but not quite like regular

programs), have initialization code and resident code.  The

initialization code in 3_Drives is the display banner and secondary

drive identification, while the resident portion is a collection of

routines handling very specific AT register set commands. The AT

register commands "talk" directly to the Secondary Port Address, 170h,

and therefore to the drive itself, and cover drive specific operations

like Read, Write, Recal, and Reset. It should be noted that the Format

command operation is not supported by 3_Drives, which is the reason

why the drive must be partitioned and prepared in a normal C:/D:

environment.

After all components of the CONFIG.SYS file are processed, SYSINIT's

final function is to load and execute the MS-DOS shell program,

COMMAND.COM.  How DOS manipulates the resident (critical error

handling, for example) and transient portions (batch file processes,

COPY, and RENAME, etc.) of the shell are beyond the scope of this

overview.

Thereafter, during the normal course of operation of the computer

system, DOS commands which are directed towards the drive letter(s)

assigned to the 3_Drives block device(s) are executed through the

memory resident portion of 3_Drives.

3_Drives v2.5 will occupy a minimum of 4.1Kbytes of memory under DOS

5.0. In the registered versions of 3_Drives/4_Drives (v3.0), the

amount of memory required for the device driver will climb slightly by

�48 bytes/drive, depending on the number of logical drives in extended

partitions, if any.

��������������������������������������������������������������������Ŀ

���� App Notes �������������������������������������������������������

����������������������������������������������������������������������

Some Application Notes:

1. The effect of the program on the performance of the system is

negligible.

2. The use of 3_Drives on XT's is DISALLOWED, since it relies

exclusively on AT register set commands which are only available

on 286's or higher.

3. If power supply leads are in short supply, a simple "Y" adapter

that splits two leads off of a single, are available at most

computer supply stores.

4. If two controllers are set to the SAME port address or share the

same IRQ, either floppy or hard, the system will hang during the

POST (Power On Self Test).

If both controllers have support for floppy drives, disable the

floppy support on the second. If a specific "disable floppy"

jumper is unavailable, then set the floppy to a secondary port

address (usually 370h) which effectively hides the conflict.

5. 3_Drives can be moved into upper memory under DOS 5 with

DEVICEHIGH=C:\your...path\driver.HDD

6. 3_Drives shows all signs of complete stability with Microsoft

Windows (tm).  In fact, our entire Windows subdirectory

structure is located on the "Secondary" drive! We have thrown in

everything we can think of to test Windows compatibility, and

all tests have proven successful. If you intend to use the drive

supported by the 3_Drives for the Swap File, then in must be of

the Temporary Swap File Type. (A Permanent Swap File requires

Int13 support.)

7. We've run three of the most popular file defragmenters without

problems.  In any case, ALWAYS follow the software

manufacturer's instructions and BACKUP you data first.

8. DOS 2.x and above will run with the same partition and capacity

limits allowed by that version of DOS, however only DOS 3.0 and

above will show the proper drive letters on the 3_Drives

initialization screen.

9. 10 partitions per physical drive are allowed under 3_Drives

support. Registered v3.0 only.

10. "Tape-Over Method":  It is possible to install a single MFM, RLL

or ESDI drive on the secondary port address and utilize

3_Drives.  This process, however, is not as straight forward as

putting the AT/IDE on the secondary because typical ST412

interface controllers (MFM and RLL) usually do not have the

built-in ability to disable the IRQ14.  If no jumper on the MFM

controller exists to disable IRQ14, then the line can usually be

disabled manually where it plugs into the motherboard.

On a 16-bit controller, the IRQ14 line is found on the back side

card edge, on the "short" 16-bit extension.  It is the 7th

contact from the left or 12th from the right (D7). Usually,

placing a piece of simple transparent tape over the contact will

cause it to be disabled when the card is plugged into the

motherboard.  It is advisable that you refrain from attempting

this configuration unless you have prior experience with

modifications of this sort. See below:

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts.

11. As noted earlier, some programs will not operate through

3_Drives (i.e. FDISK, FORMAT).  This is because these programs

bypass DOS and do not update the partition's FAT (File

Allocation Table).  Other examples of this are usually drive

diagnostics utilities.  If programs of this sort are deemed

necessary to run, the drive will need to be reinstated on the

primary port address in order to be "seen" by the program.

12. The DOS ASSIGN utility can be used to set a drive on the

Secondary Port Address to C:, if desired.  Simply execute the

command    ASSIGN C:=E: E:=C:   This will effectively swap the

assigned drive letters.  Remember that CHKDSK will not run on

assigned or substituted drives.  Running ASSIGN again, without

any command line parameters, will clear the assignment.

13. Popular drive compression programs work well with 3_Drives and

4_Drives.  (In fact, this document is being written on drive

number 4, and is a compressed drive.)  The important points to

remember are that 3_Drives needs to be loaded before the

compression drivers IF the Secondary Port Address drives are

being compressed.  If the system is already compressed prior to

installing 3_Drives, then confusion could arise when it comes

time for the compression driver to assign the drive letter

designations, especially with the drive letter swapping and

reserved drive settings that are typical of compression

programs.  To be absolutely safe, BE SURE to backup all data

prior to creating compressed drives.

14. RLL and ESDI controllers usually operate on the Primary Port

Address with an enabled on-board bios.  This bios needs to be

disabled when used with 3_Drives on the Secondary Port Address.

The drive does not have to be lo-level formatted with the bios

disabled.

15. The driver installs in it's raw state eg., device=driver.hdd

will set the driver up in the "old", slow but sure I/O mode of

operation. If the driver loads ok with the Old switch but hangs

with data errors use the ATA switch. Use of the ATA switch

implements some of the new drive data error recovery techniques

employed by this version.  The omission of the ATA switch is

provided as a bridge between these technologies.

16. The M(?) switch will only be enabled, indicated by a RED *

(led), if the drive selected by the (?) is capable of performing

multiple block mode data transfers.  If the * (led) indicator

remains GREEN, then the drive rejected the multi-mode. You will

have to then edit the config.sys file to implement Fast Reads or

Writes as desired or utilize no I/O switches to default to the

old slow but sure mode.

17. Utilization of the M(?)+ plus switch addition to select the

maximum block xfer size will prove to be detrimental to the data

xfer rate on some drives.  As the 'AT' world is ever changing,

some early drive models gave totally "unworkable" maximum block

sizes for this selection.  If one were to select the size that

the drive offered, some drives would immediately lock up as part

of this overall buffer is utilized as drive firmware scratch

pad!  We utilize an algorithm to (hopefully) select a workable

compromise of the total buffer area.

18. Multiple Block mode on some early drives though enabled, will

not work AT ALL with the driver as the early implementations of

this multi mode were faulty in their microcode.

19. Data xfer rates with the use of the Multi-mode will be more

noticed in systems that ask for large blocks of data in a single

request.

20. Fast Reads and Fast Writes are not mutually exclusive. Therefore

any or all combinations of Fast I/O will be allowed.

21. We will check the first 60 bytes of text after the = sign in the

device=driver.hdd line in the config.sys file for command line

switches.  This should be adequate for all possible

configurations as described above.

22. If your AT/IDE drive handles an inquiry ok from 4DRVUTIL but

will not respond to 3_Drives, then try using the @(?) command

line option, where ? is replaced by 0 or 1 depending on your

case.

��������������������������������������������������������������������Ŀ

���� Error Messages ��������������������������������������������������

����������������������������������������������������������������������

The device driver may display a small assortment of error messages

during the loading process of the CONFIG.SYS file. These usually

relate to unformatted status, jumper, cabling and power issues.

The most common error messages are descriptive and will explain the

specific problem encountered. (i.e. "No controller found at the

Secondary Port Address" or "No valid DOS boot record found").

The less common errors are listed by number:

Error code #1  : Address Mark not found. cyl=xx hd=xx sect=xx

Drive may need lo-level format. Scan for defects.

Error code #2  : Drive Fails Recal

Check cables and drive select jumpers, etc.

Error code #4  : Aborted Command.

Invalid command request or drive fault problem like a

seek error.

Error code #5  : Write Fault.

Usually caused by power supply voltage fluctuations or

problems with termination resistors.

Error code #6  : Time-out.

General time out.

Error code #7  : Time-out while waiting for drive DRQ.

Error code #8  : Read Fault.

Check cables, connections and drive jumpers.

Error code #16 : Sector ID Not Found. cyl=xx hd=xx sect=xx

May need a lo-level format.

Error code #64 : CRC Error.

Scan for defects.

Error code #128: Block Marked Bad.

��������������������������������������������������������������������Ŀ

���� Legal Disclaimer ������������������������������������������������

����������������������������������������������������������������������

LICENSE AGREEMENT

3_Drives 4_Drives 4_Park 4_Drive Utilities All versions.

Dustbowl Designs, Inc. provides  the accompanying object code software

("Software")  and nonexclusively  licenses  its  use on  the following

terms and conditions. The Software is copyrighted by Dustbowl Designs,

Inc. with all rights reserved.  YOU ASSUME FULL RESPONSIBILITY FOR THE

SELECTION OF THE  SOFTWARE TO ACHIEVE YOUR INTENDED  PURPOSES, FOR THE

PROPER INSTALLATION  AND USE. Dustbowl Designs,  Inc. DOES NOT WARRANT

THAT THE  SOFTWARE WILL MEET  YOUR REQUIREMENTS, THAT  THE SOFTWARE IS

FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE SOFTWARE WILL BE

ERROR FREE. Dustbowl Designs, Inc. EXPRESSLY DISCLAIMS ALL WARRANTIES,

WHETHER  ORAL  OR  WRITTEN,  EXPRESSED  OR  IMPLIED, INCLUDING WITHOUT

LIMITATION WARRANTIES  OF MERCHANTABILITY OR FITNESS  FOR A PARTICULAR

PURPOSE. IN  NO EVENT WILL  Dustbowl Designs, Inc.  BE LIABLE TO  YOU,

YOUR  CUSTOMERS   OR  OTHER  USERS   FOR  ANY  INDIRECT,   INCIDENTAL,

CONSEQUENTIAL,  SPECIAL  OR  EXEMPLARY  DAMAGES  ARISING  OUT OF OR IN

CONNECTION WITH THE USE OR INABILITY TO USE THE SOFTWARE.

3_Drives is a trademark of Dustbowl  Designs, Inc. The 3_Drives

software and documentation are Copyright 1992 by Dustbowl Designs with

all rights reserved. The Registered 3_Drives, 4_Drives and 4_Park

software programs ARE NOT shareware or freeware and unauthorized

redistribution is STRICTLY PROHIBITED.

End of License agreement.

*****************************************************************

��������������������������������������������������������������������Ŀ

���� Revision History ������������������������������������������������

����������������������������������������������������������������������

v1.1    08/92 - public release of 3_Drives SHAREWARE v1.1.

08/92 - public release of 3_Drives and 4_Drives REGISTERED

v1.1.

v1.5    02/93 - public release of 3_Drives SHAREWARE v1.5. Smaller

memory resident size (37% smaller!). Optimized for

faster execution.  Improved error handling and added

new error messages.  DOS support beginning with v3.0.

Stopped hang problem when driver installed and

controller isn't. Documentation upgraded. Introduces

new company name and BBS. Support terminated for

NON-registered v1.1 users.

v2.1    02/93 - public release of 3_Drives and 4_Drives REGISTERED

v2.1. Same as shareware v1.5 including extended

partition support with up to 10 logical drives per

physical unit. Now includes 4_Park utility.

v2.12   04/93 - private release of 3_Drives and 4_Drives REGISTERED

v2.12. This version incorporated new reset and error

handling routines and was released to the Beta Testing

Team.

v2.50   07/93 - public release of 3_Drives SHAREWARE v2.50. Now

implements new ATA-spec drive/data recovery

techniques. Now includes 4DRVUTIL.EXE AT/IDE dual port

address drive inquiry utility. New mailing address.

Announce new BBS online Registration/Download

capability. Support terminated for NON-registered v1.5

users.

v3.0    07/93 - public release of 3_Drives and 4_Drives REGISTERED

v3.0. Same as shareware v2.5 including High Speed I/O

Options which are: SET READ/WRITE MULTIPLE SECTORS PER

INTERRUPT for AT/IDE drives and SET FAST READ/WRITE

for older AT/IDE and all other interfaces.  These new

fast I/O routines are 35-50% faster than any previous

release.

v2.6    08/93 - public release of 3_Drives SHAREWARE v2.6.  Now

implements new RF0 READ FAST switch that was

previously only available in the Registered versions.

New @0 command line switch.  Support terminated for

NON-registered v2.5 users.

v3.01   08/93 - public release of 3_Drives and 4_Drives REGISTERED

v3.01.  Same as v3.0 above but with new modified I/O

algorithms and new @(?) command line switch (many

thanks to Ron in Kansas City for helping with this).

Contents of the 3_Drives Distribution Set. 3DRVS260.ZIP (3DRVS.ZIP on

CompuServe (tm) go IBMHW forum Lib 1)

3DRVS260.HDD    Device driver file. Compiled 08-20-93 06:20:00.

3_DRIVES.DOC    Documentation file with installation instructions.

4DRVUTIL.EXE    An AT/IDE Identify Drive inquiry utility that shows

which 3_Drive/4_Drive and DiskQwik command line

switches work best. Designed to work on both Primary

and Secondary Port Addresses. Freeware.

4DRVUTIL.DOC    Auxiliary documentation.

DBOWLDEM.EXE    Simple demo that gives a look at registered

4_Drives v3.01 and 4_Park v1.1, and our new DiskQwik

programs.

4DRIVES.PCX     Graphics picture file showing Data Transfer Rates with

various command line options. Test and graphics file

created by our new 4_Speed Data Transfer Rate Test

utility. Freeware.

ORDER3.FRM      Mail-in registration form

README3.TXT     User shareware policy

VENDOR3.DOC     Shareware vendor distribution policy

*****************************************************************

��������������������������������������������������������������������Ŀ

���� Controller and Host Adapter Setup Instructions ������������������

����������������������������������������������������������������������

===== AT IDE Host Adapters ===================

=====Seagate ST-07A and ST-08A================

The ST-08A is a combination floppy drive and hard drive host adapter,

while the ST-07A is for hard drives only.  The floppy support on the

ST-08A can be disabled making it functionally equivalent to the

ST-07A.

J2

Diskette

Connector     J3 LED           J1 AT Interface

(ST08A only)  �                Hard Disc Connector

�  ������������������������������������������������������Ŀ �����

�  �   1      1                   �                    ������

�  �   ::     ����             ::::::::::::::::::::    ������

�  �   ::                      1                          � �

������ ::                           �������������������������� JP2

�   ::                           1 JP2                 � �  Disc/Diskette

�   ::                          ��                     � �  Address

�   ::         (ST08A only)                            � �  Select

�   ::  �1���� JP4 Diskette Controller Enable          � �

�       :                                   ��������������� JP6 I/O

�       ����}�� J4 Aux DC Power to          1       ������  Channel

�            1�    Hard Disc                ��      ������  Ready

�          ������Ŀ                                    � �

� � � � � � � �����Ŀ � � � � � � � � � � � � � ������� �

���������������� �  ������������������������������      �

�� JP5 Disable Interrupt or            �

Interrupt While Busy IRQ14

If set up as the 2nd Host Adapter for use with 3_Drives:

1.  Add a jumper to JP2 (Disc/Diskette Address Select) which sets the

port to the secondary address at 170h. (New Seagate documentation

does not describe this jumper.)

2.  Remove jumper from JP5 (Interrupt While Busy, IRQ14). The jumper

can be conveniently stored on a single pin.

3.  If ST-08A, then remove jumper from JP4 (Diskette Controller

Enable) to disable the Floppy Controller).

=====Acculogic SIDE-3================

�� 40-pin IDE Hard drive

�    connector (pin-20   �� 34-pin Floppy drive     �����

�   removed for keying)  �    connector (pin-5      �

J1      �����Ŀ                  �  removed for keying)     �

LED Ŀ        �                  �                          �

�ĳ�������ĳ�����������������ĳ�����������������������Ŀ �

�1����     �         ::::::   � :::  (see below)       � �

�          �      JP 123456   � 789                 ������

� :::::::::�::::::::::      ::�::::::::::::::       ������

� 1    J2                   1    J3                    � �

�                                                      � �

�                                                      � �

�               :                                   ������

�               JP10                                ������

� � � � � � � �����Ŀ � � � � � � � � � � � � � ������� �

����������������    ������������������������������      �

�

Detail on JP    1 2 3 4 5 6     7 8 9

: : : : : :     : : :

� � ����� �     � � �

*  Normal   IRQ 14     1 0   �   �     � � 0 Precompensation = 125ns *

Delayed  IRQ 14     0 1   �   �     � � 1 Precompensation = 187ns

!  Disabled IRQ 14     0 0   �   �     � �

�   �     � 0   Single speed floppy *

Reserved ������������������   �     � 1   Dual speed floppy

�     �

* Primary Floppy Address (3F7h)  0     0 Primary IDE Address (1F7h) *

! Secondary Floppy Address (370h)  1     1 Secondary IDE Address (170h) !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

===== ST412 MFM RLL Controllers ==============

Adaptec ACB-2370B  ST412 Interface (RLL)

�����������������������������������������������������������������������Ŀ �����

� ����                    Data 1      Data 0      Hard               ������

� ����                    %::::::::%  %::::::::%  %:::::::::::::::%  ������

� LED                     1           1           1                     � �

�                                                                       � �

�    1��                                                                � �

�    2���� JP6                                                          � �

�    3��                                               JP14             � �

�    4��                                              1���              � �

�    5��                                                                � �

�                                                                    ������

�                      JP13                                          ������

�                     1����                                             � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP6  Hard   Drive Port Address:   Primary 1F0h jumper 1   open *

Secondary 170h   ""     closed !

JP13 Hard Drive Interrupt Req :    IRQ14       pins 1-2 closed *

IRQ15       pins 2-3 closed

Disabled    pins all   open !

JP14 Bios Memory Address      :    C800-CBFFh  pins 1-2 closed *

CC00-CFFFh  pins 2-3 closed

Disabled    pins 1-2-3 open !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

Adaptec ACB-2372B  ST412 Interface (RLL)

������ Hard Drive Controller

����������������������������������������ĳ�����������������������������Ŀ �����

� ����     1234567              1�  1�  1�  1�� Floppy Drive         ������

� ����     :::::::              ::  ::  ::  ::                       ������

� LED            �              ::  ::  ::  ::                          � �

�            JP2 �              ��  ��  ::  ::                          � �

�                                �   �  ::  ::                          � �

�                   HDD 1 Data ���   �  ::  ::                          � �

�                   HDD 0 Data �������  ��  ��                          � �

�                                                                       � �

�                                    �4 JP7                             � �

�                                  1���3                             ������

�                      JP13   JP22       JP21   JP20                 ������

�                     1���    ���        ���    ���                     � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP2  Hard   Drive Port Address:   Primary 1F0h jumper 1   open *

Secondary 170h   ""     closed !

JP2  Floppy Drive Port Address:   Primary 3F0h jumper 2   open *

Secondary 370h   ""     closed !

JP7  Bios Memory Address      :    C800-CBFFh  pins 1-2 closed *

CC00-CFFFh  pins 2-3 closed

Disabled    pins 2-4 closed !

JP13 Hard Drive Interrupt Req :    IRQ14       pins 1-2 closed *

IRQ15       pins 2-3 closed

Disabled    pins 1-2-3 open !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

Adaptec ACB-2372C  ST412 Interface (RLL)                      ���� JP8

ACB-2370C                                             � �� JP9

JP7 Ŀ    � �

��������������������������������������������������������ĳ���ĳĳ������Ŀ �����

� ����           1          1�  1�  1�  1�              1�    � �    ������

� ����    ::::::::          ::  ::  ::  ::               �    � �    ������

� LED       �               ::  ::  ::  ::               �              � �

�           �JP2            ��  ��  ::  ::                              � �

�                            �   �  ::  ::                              � �

�                            �   �  ::  ::                              � �

�                            �   �  ��  ��                              � �

�                            �   �   �   �� Floppy Drive                � �

�                            �   �   ������ Hard Drive Controller       � �

�                            �   ���������� HDD 0 Data               ������

�                            �������������� HDD 1 Data               ������

�                                                                       � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP2  Hard   Drive Port Address:   Primary 1F0h pins 1-2 open   *

Secondary 170h pins 1-2 closed !

JP7  Bios Memory Address      :    C800-CBFFh  pins 1-2 closed *

CC00-CFFFh  pins 2-3 closed

Disabled    pins 1-2-3 open !

JP8  Floppy Drive Port Address:            Primary 3F0h open   *

Secondary 370h closed !

JP9  Floppy Drive Enable      :               Enabled   open   *

Disabled   closed !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts. (READ App Note # 10)

Adaptec ACB-231x  ST412 Interface models.

�����������������������������������������������������������������������Ŀ �����

� ����           1          1�  1�  1�  1�                           ������

� ����      ::::::          ::  ::  ::  ::                           ������

� LED       �               ::  ::  ::  ::                              � �

�           �JP6            ��  ��  ::  ::                              � �

�                   JP10     �   �  ::  ::                              � �

�                   �  1     �   �  ::  ::                              � �

�                   ��� ooo  �   �  ��  ��                              � �

�                   ��� ooo  �   �   �   �� Floppy Drive                � �

�                   �  1     �   �   ������ Hard Drive Controller       � �

�                   �        �   ���������� HDD 0 Data               ������

�                   JP11     �������������� HDD 1 Data               ������

�                                                                       � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP10 Floppy Drive Port Address:   Primary 3F0h pins 1-2 closed *

Secondary 370h pins 1-2 open   !

JP11 Hard   Drive Port Address:   Primary 1F0h pins 1-2 closed *

Secondary 170h pins 1-2 open   !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts. (READ App Note # 10)

Data Technology Corp. (DTC) DTC728x ST412 Interface models

�����������������������������������������������������������������������Ŀ �����

� ����                      1�  1�  1�  1�             W4 �  oo      ������

� ����       JP6 �          ::  ::  ::  ::             W7 �  oo      ������

� LED            oo         ::  ::  ::  ::                              � �

�                           ��  ��  ::  ::                              � �

�                           �    �  ::  ::                              � �

�               HDD 1 Data ��    �  ::  ::         ��� W2               � �

�               HDD 0 Data �������  ��  ��         �                    � �

�    Hard Drive Controller �����������   �       : : :                  � �

�             Floppy Drive ���������������       �   � W3               � �

�                                                �                   ������

�                                                ����� W5            ������

�                                                                       � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

Primary Port Addresses Hard 1F0h and Floppy 3F0h : W2 and W3 open *

Secondary Port Addresses Hard 170h and Floppy 370h : W2 and W3 closed !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts. (READ App Note # 10)

Western Digital WD1003V, WD1006V ST412 -MM (MFM) Interface models

�����������������������������������������������������������������������Ŀ �����

� ���� Data 1      Data 0      Hard               Floppy             ������

� ���� %::::::::%  %::::::::%  %:::::::::::::::%  %:::::::::::::::%  ������

� LED  1           1           1                  1                     � �

�                                                                       � �

�                    JP1                                                � �

�                    ::::                                               � �

�                   1                              oo � JP3 Hard        � �

�                                                  oo � JP4 Floppy      � �

�                                                  oo - JP5             � �

�                                                                    ������

�                                                              :     ������

�                                                              JP6      � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP3 Hard   Drive Port Address:   Primary 1F0h  = open   *

Secondary 170h  = closed !

JP4 Floppy Drive Port Address:   Primary 3F0h  = open   *

Secondary 370h  = closed !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts. (READ App Note # 10)

Western Digital WD1003V, WD1006V ST412 -SR (RLL) Interface models

�����������������������������������������������������������������������Ŀ �����

� ���� Data 1      Data 0      Hard               Floppy             ������

� ���� %::::::::%  %::::::::%  %:::::::::::::::%  %:::::::::::::::%  ������

� LED  1           1           1                  1                     � �

�                                                     � JP2 Bios        � �

�                    JP1                              � enabled open    � �

�                    :::                              � disabled off    � �

�                   1                              oo �                 � �

�                                                  oo � JP3 Hard        � �

�                                                  oo - JP4 Floppy      � �

�                                                                    ������

�                                                              :     ������

�                                                  oo - JP5    JP6      � �

������������������Ŀ � � � � � � ����Ŀ � � � � � � � � � � � � � ������� �

����������������   ������������������������������      �

�

JP2 On-board Bios status:       enabled = open   *

disabled = closed !

JP3 Hard   Drive Port Address:   Primary 1F0h  = open   *

Secondary 170h  = closed !

JP4 Floppy Drive Port Address:   Primary 3F0h  = open   *

Secondary 370h  = closed !

* = Default Setting, ! = 3_Drives Setting, 1 = ON, 0 = OFF.

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                               D7 (IRQ14) Tape over

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts. (READ App Note # 10)

*********************************************************************

Dustbowl Designs, Inc.          |

12300 S.E. 38th Street          |

Choctaw, Oklahoma  73020-6107   |

U.S.A.                          | CIS: 71062,2542

phone:

(405) 741-4705 Ordering, tech support, general information

(405) 741-2721 BBS (9600 HST/v.everything) Tech Support, Hard disk

drive shareware, drive specs, Online Shareware

Registration/Download for VISA or MasterCard orders!

NAME : ___________________________________Password:_________________

ADDRESS : _____________________________________________________________

___________________________________________ ZIP :____________

TELEPHONE: ___________________________________________ (Day & Night)

COMMENTS : _____________________________________________________________

_____________________________________________________________

___________________________________________________________

Qty    Description                            Each       Total

____   Special Upgrade for Registered users    $0.00    THANKS!!

to v3.01 4_Drives

____   3_Drives v3.01 Registration            $30.00     ________

____   4_Drives v3.01 Registration            $40.00     ________

____   Upgrade from Registered 3_Drives       $10.00     ________

to v3.01 4_Drives

Subtotal ________

(Oklahoma residents add 6.5% sales tax)                 Tax ________

U.S. diskette orders add $3 shipping & handling, Foreign diskette

orders add $5 U.S. shipping. This also applies to Special Upgrade for

Registered users (If you want to download from our BBS put $0.00

dollars for shipping and write the password you'll be using next to

your name above.  Allow 7 days for Postal transit.)

Shipping ________

TOTAL $________

PAYMENT:

[ ] Visa/MasterCard  [ ] Check (number ____________)

(see below)       (make checks payable to Dustbowl Designs, Inc.)

[ ] Cash US$         [ ] US$ Money Order (No foreign  currency)

Total amount enclosed or authorized: $________________

_______________________________________________________________________

If Payment by Visa or MasterCard:

We can only accept Visa or MasterCards (no American Express, please.)

We will need the following information:

Credit card number: ______________________________________________________

Expiration date: _________________________________________________________

Bank the card's drawn on: ________________________________________________

The name as it appears on the card: ______________________________________

The signature of its owner authorizing us

to debit it for the amount indicated above: ______________________________

_______________________________________________________________________

THE FOLLOWING INFORMATION IS REQUIRED FOR REGISTRATION:

HARD DISK  #1 MAKE ____________________MODEL_________________SIZE________

HARD DISK  #2 MAKE ____________________MODEL_________________SIZE________

HARD DISK  #3 MAKE ____________________MODEL_________________SIZE________

HARD DISK  #4 MAKE ____________________MODEL_________________SIZE________

CONTROLLER #1 MAKE_____________________MODEL_________________

CONTROLLER #1 MAKE_____________________MODEL_________________

COMPUTER   286 ( )   386 ( )  486 ( )      OTHER ( ) ______________________

SX ( )    DX ( )  DX2 ( )  OverDrive ( )

OPERATING SYSTEM / DOS VERSION ___________________

VIDEO CARD  Standard VGA ( )  Super VGA ( )  Coproccessed/Accelerator  ( )

MOUSE      YES ( )  NO ( )

JOYSTICK   YES ( )  NO ( )

MEMORY     640 ( )  1 meg ( )   2 meg ( )   4 meg ( )  more ( ) ______

MODEM      YES ( )  NO ( )  .... BAUD RATE ___________________________

HOW DID YOU ACQUIRE THIS PROGRAM: bbs name &#__________________________

Fold the pages at the break below with print facing out.  Fold letter style

with mailing address exposed.  Tape/staple edges, enclosing payment.

___________________________                                          .-----.

|Place|

___________________________                                          |Stamp|

|Here |

___________________________                                          `-----'

Dustbowl Designs, Inc.

12600 S.E. 38th Street

Choctaw, OK    73020-6107

U.S.A.

3_Drives v2.6

-=EOF: 3_DRIVES.DOC=-