Windows™  95 Internet FAQ

The following procedure details how to setup your Windows™  95 Dial-Up Networking (DUN) for accessing an Internet Service Provider, ISP.  Windows™  95 DUN has built-in support for TCP/IP, dialup PPP/SLIP, and includes a 32 bit Winsock API.  The setup was verified using the Microsoft Preview

Program version of Windows™  95, the release version may have changes that affect this procedure.

I have included screen snapshots for connecting to the CompuServe PPP service.  I recommend that you start by selecting an Internet Service Provider and then request a PPP type account.  Windows™  95 will support SLIP type accounts but they are more difficult to configure.  The following instructions are

believed to be correct but I assume no liability or responsibility for your usage of these procedures, use them at your own risk.  If you have suggestions for improving these instructions please send me email

using one of the addresses listed at the end of the document.

Request the following information in writing or fax format from your provider:

1.) User Name

2.) Password

3.) Telephone Access Number

4.) If you have fixed IP type accounts request your IP Address and Subnet Mask

For Dynamic (bootp) type accounts PPP will request your IP Address and Subnet mask

each time you login.

5.) DNS Primary

6.) DNS Secondary (Optional)

7.) Gateway IP Address (Optional)

8.) Subnet Mask

9.) SLIP/PPP type account

10.) Authentication method

Terminal login procedure

If you have a PPP type account ask if

the ISP is using PAP or CHAP

authentication

Install Dial-Up Networking

Double click on “My Computer”

Double click on “Control Panel”

Double click on “ Add/Remove Programs”

Click on Windows™  Setup TAB

Select Communications

Click on Details button

Check Dial-Up Networking

Click on OK button

Click on OK button

Setting up the TCP/IP Protocol

Double click on “My Computer”

Double click on “My Control Panel”

Double click on “Network”

Verify that you have “TCP/IP -> Dial-UP Adapter” listed as a protocol

if not follow the steps below to install it (If you do have ““TCP/IP -> Dial-UP Adapter”

listed select Properties for the Dial-Up Adapter and click on the Bindings tab,

confirm that the TCP/IP box is checked.):

Click on Add button

Select on Adapter

Click on Add button

Select Microsoft®  from the manufacturer list

Select Microsoft®  TCP/IP from the network protocol list

Click on OK button

Click on OK button

Click on the Properties button

Click on the IP Address TAB

If you have a fixed IP address

Select Specify an IP address

Enter your IP address and subnet mask

If your provider is using dynamic IP addresses

Select Obtain an IP address automatically

Click on the Wins Configuration TAB

Select Disable WINS Resolution

Deselect Use DHCP for WINS Resolution

Click on the Gateway TAB

If your provider specified a Gateway IP enter it in the New Gateway box then click on the

Add button If your provider did not provide a Gateway IP address you can leave this entry

blank and let the PPP protocol negotiate it for you when you dial-in

Click on DNS Configuration TAB

Enter your Host Name (If you are using dynamic IP addressing you can use your

SMTP email name for a host address)

Enter your Domain Name

Enter the primary DNS in the DNS Search Order box then click on the Add button

Enter the secondary DNS in the DNS Search Order box then click on the Add button

Click on the Advanced TAB

Select “Set this protocol to be the default”

Click on Bindings TAB

Verify that the Client Microsoft®  Networks is checked

Click on OK button

Click on OK button again to confirm that you want

to allow the computer to shutdown and restarted

Creating a New DialUp Networking Connection Profile

Double click on “My Computer”

Double click on “Dial-Up Networking”

Double click on “Make New Connection”

Type the name for your ISP, my example uses the name CompuServe

Select the Modem you wish to Install if you are using a Widows 95 supported PCMCIA

modem plug it in with the PC powered up and it will auto-install itself using Plug-N-Play.

Confirm the proper modem type is displayed in the “Connect using” dialog box

then Click on the Config button

Select the General TAB

Select the Com Port for your modem Com1, Com2, etc.

Select the desired Speaker Volume

Select the Maximum speed

Note: I recommend you use 57600 or 38400 if you have an internal modem.

If you have an external modem use 19200 or 9600 baud if you do not

have a 16550 UART.  If you have a 16550 UART use can use a higher

baud rate of 38400 or 57600 baud.

Select the Connections TAB

Data bits - 8

Parity - None

Stop bits - 1

Call preferences

Check “Wait for dial tone before dialing”

Check “Cancel the call if not connected within 60 secs

Deselect “Disconnect a call if idle for more than XX mins”

Now Click on the Advanced button and confirm the following items are checked:

Use error control

Compress data

User flow control

Hardware (RTS/CTS)

Select the Options TAB

Under Connection control

Check “Bring up terminal Windows™  after dialing”

Under Dial control

DeSelect Operator assisted or manual dial

Status control

Select Display modem status

Click the Next button

Click on the OK button to close the dialog

Enter the local access telephone number for your ISP in the telephone number box

Click the Next button

Click the Finish button

Select the icon for your ISP (Enet in the example)

Click on the right mouse button

Select Properties

Click on Server Type button

If you are using a PPP connection:

Select PPP: Windows™  95, Windows™  NT, Internet

If you are using a SLIP connection:

Select SLIP: UNIX Connection

Under Advanced

Select - Log on to the Network

Select -Enable Software compression

DeSelect - Require encrypted password

Under Allowed Network protocols

Deselect  - NetBeui and IPX/SPX compatible

Select  - TCP/IP

Click on the OK button to close the Server Types dialog

Click on the OK button to close the DialUp Networking General dialog

Using the DialUp Networking Profile

Double click on “My Computer”

Double click on “ Dial-Up Networking”

Double click on “CompuServe”

Enter your PPP/SLIP account Username and Password

(For CompuServe you can leave both Username and Password blank)

If your provider uses PPP with PAP authentication you must enter the proper

username and password.

Check - Save password if you want to retain the password without typing it in again

the next time you dial your provider

Click on the connect button

Click on the Dial-UP Networking

Note: You can create a Dial-Up Networking Short Cut by

dragging the CompuServe icon to the desktop.

When you get tired of manually logging  in every time you access your ISP, and you can automate your PPP/SLIP login using a fantastic freeware dialup Network Script utility developed by Mark Gamler, Email - CompuServe ID 76450,2754.  You can download this program from the CompuServe WINNEWS Forum.  The current version of this utility is titled “RoboDUN Version 0.61, Windows™  95 Dial-Up Network Script Utility”.

What is RoboDUN? (From Mark’s RoboDUN help file) “Many dial-up resources such as internet providers require a "clear text" login. This requires you to request a post-dial terminal screen to enter your user name, password and possibly other information in order to log into the system. SLIP further complicates the matter by also requiring you to wait for an IP address from the server and provide the address when prompted by Windows. RoboDUN (Robotic Dial-Up Networking, if you will) eliminates all that by running a script for a given connection, the script guiding the connection automatically.”

Installing RoboDUN

Download the latest version of the RoboDUN ZIP archive file, unzip the file and copy all the files in the

package to a common directory (C:\robodun) and then start them as you would any other Windows™  program. Note that for script functionality, ROBODUN.EXE must be started before any dial-up connection is attempted.  You should read the readme.txt file included with RoboDUN for the

latest version of the install instructions.  You can install RoboDUN by following the steps below:

Double click on “My Computer”

Double click on “Control Panel”

Double click on Add/Remove Programs

Select Windows™  Setup TAB

Click on Have Disk

When prompted, enter the full filename and path for robodun.inf and then Click on OK

Click on OK

You should see “RoboDUN X.XX Utility”

(Where X.XX is the current version number)

Make sure the check box next to RobDUN is checked then Click on Install

The RoboDUN utility will be installed and RoboDUN will be added to the Startup Group

Run the RDUN32.exe program

Select the Dial-Up Networking entry for your ISP

Select Edit Script

Type in your script (Helpful hint you can use the clipboard and paste in the RoboDUN script examples and then simply edit them to meet your specific login requirements)

You must complete the creation of the script by selecting Save

Here is the actual RoboDUN script I use for connecting to CompuServe using DUN PPP, be sure and substitute your CompuServe ID in place of the 00000,0000 below, and your CIS password in place

of pass.word below.

----------------------------------------------------------------------

# CompuServe login script

# Specify 7 bit operation (required for CIS)

7bit

# Kick off the game with a Ctrl-C

Send "<ctrlc>"

# Wait 90 seconds for "User ID:" prompt

WaitFor "User ID:", 90

# send ID and service type

Send "00000,0000/go:pppconnect<cr>"

# Wait 20 seconds for password prompt

WaitFor "Password:", 20

# Send CompuServe password and quit

Send "pass.word<cr>"

Done

----------------------------------------------------------------------

Here is an example RoboDUN script for connecting to a Xylogics Annex Three dialup server

using PPP.

-----------------------------------------------------------------------

WaitFor "sername:", 10

Send    "MyUsername<cr>"

WaitFor "assword:", 10

Send    "MyPassword<cr>"

WaitFor "ine>", 10

Send    "PPP<cr>"

Done

-----------------------------------------------------------------------

Here is an example RoboDUN script for connecting to a Xylogics Annex Three dialup server

using SLIP.

# Example SLIP script for RoboDUN

# Send CR/LF to get the ball rolling

Send ""

# Wait 10 seconds for "login" prompt

WaitFor "ogin:", 10

# Send user name in response to "login"

Send "username<cr>"

# Wait 10 second for "password" prompt

WaitFor "assword:", 10

# Send password in response to prompt

Send "userpwd<cr>"

# Wait for "service ready" prompt

WaitFor "Ready", 10

# Specify service...SLIP in this case

Send "SLIP<cr>"

# Pause for the terminal server to return your IP address

Pause 2

# Get the second SLIP IP address from server

GetSlip 2

# End the script here

end

-----------------------------------------------------------------------

Here is an example RoboDUN script for connecting to a 3Com Access Builder dialup server

using PPP.

-----------------------------------------------------------------------

#3Com Access Builder

Pause 2

Send "<cr>"

Pause 1

WaitFor "ogin:", 10

Send    "username<cr>"

WaitFor "ord:", 10

Send    "password<cr>"

WaitFor ">", 10

# Specify service...PPP in this case

Send "PPP<cr>"

Done

-----------------------------------------------------------------------

Installing Windows™  95 SLIP DialUp Networking

First add TCP/IP to the dial up adapter card in control panel/networks

1.)  In control panel open add/remove programs

2.)  Select the middle tab - Windows™  setup

3.)  Select - have disk

4.)  Use browse to locate admin95\apptools\slip\rnaplus.inf

5.)  Select ok then ok again until dialog with "UNIX Connection for SLIP" etc is

visible

6.) Make sure you select the UNIX SLIP connection by checking the check box at left

of name, just highlighting will not install it.

7.)  When the check mark is visible, then select install

8.)  You should now see the progress bar as it installs

The text of this document is under copyright by Steve Scoggins, May 10, 1995, This document may

not be published, reproduced , distributed, recorded on CD-ROM, or posted on other on-line services with out my written permission.  This document is posted to the CompuServe WINNEWS Forum with

my permission.  (Steve Scoggins, CompuServe ID 74431,1332, sscoggin@enet.net)

Microsoft® is a registered trademark of Microsoft Corporation, and Windows™  and Windows™  95

are trademarks of Microsoft®  Corporation. 3Com® is a registered trademark of 3Com Corporation. CompuServe is a registered trademark of CompuServe, Inc.