CGEN 4.2

This version of CGEN supports all generations of TI's TMS320

family of DSP processors through the latest floating point TMS320C30.

Topic Covered:

1. Installation

2. Enhancements

---------------------------------------------------------------------------

1. INSTALLATION

To install CGEN you should run the install program located on

the floppy.  This will setup the proper directories within the

DFDP3/plus environment.

Examples of how to use CGEN to generate a complete

executable filter on one of ASPI's processor cards are given in the

README.DOC files located in the MAIN_IO and BANSHEE directories on

the release diskette. These instructions are for using CGEN without

DFDP3/plus.  If you use CGEN with DFDP3/plus from the CGEN Menu, you

must first copy the ADP.PAR associated with your hardware

installation, get into DFDP3/plus and select an I/O loop.  This will

set up the proper I/O loop and the proper TMS320DL (for downloading),

and TMS320CL (for compiling with the C interface), and TMS320AL (for

compiling with the ASM interface) files.

If you are using a BANSHEE and have problems running the GO._MC file

that is generated, then you should check that the MACRO environment

variable (as defined in the ASPI_ENV.PAR file in the DFDP3/bin

directory) includes the Banshee macro directory.  You may change

this line as:

MACRO=C:\DFDP3\MAC

may be changed to:

MACRO=C:\DFDP3\MAC;C:\BANSHEE\MACRO

In order for CGEN to operate properly, you must set a DOS

environment variable 'CGEN'.  CGEN uses this variable to locate

parameter files that it needs to generate code.  If the 'CGEN'

environment variable is not set, CGEN uses the 'PATH' environment

variable.  The following example sets the 'CGEN' environment variable

to point to the directory C:\ASPI\UTILITY.

SET CGEN=C:\ASPI\UTILITY

'CGEN'  can be set so that more than one directory is searched just

like 'PATH'.  See your DOS manual for further details.

Example:

SET CGEN=C:\BIN;C:\ASPI\UTILITY

If the CGEN environment variable does not exist, PATH is used.

-------------------------- WARNING ----------------------------------

To use CGEN with the BANSHEE or CHIMERA processor cards you should

have a revision 2.23 or newer TI_LOAD.  If you do not, please contact

ASPI about obtaining a new release of the BANSHEE or CHIMERA

software.

You should also run 320_SAVE.COM (provided your BANSHEE or CHIMERA

software) at boot time.  This program is used in conjunction with

TI_LOAD to save the sample clock rates.  It is a memory resident

program that occupies about 300 bytes of PC memory.

----------------------- End of WARNING ------------------------------

---------------------------------------------------------------------------

2. ENHANCEMENTS

Another way to execute CGEN from the command line is given below:

CGEN FILTER_FILE ASSEMBLY_FILE UNIQUE_NAME {Y|N} PROCESSOR_IMPL

where

FILTER_FILE        is the name of the filter file (.flt default ext).

ASSEMBLY_FILE      is the name of the output source file.

UNIQUE_NAME        is the name of the 5 character unique identifier.

{Y|N}              indicates entering either a Y or an N to select a

full implementation (including the main I/O) or not.

PROCESSOR_IMPL     is the processor implementation number

In other words the answers to the questions that CGEN asks when run

interactively

Ex: CGEN IIR IIR IIR Y 1

---------------------------------------------------------------------------