[contour]

1 01250FLT       // Tool reference

2 1              // Tool number

3 1              // Tool diameter offset number

4 1              // Tool length offset number

5 0.125          // Tool diameter

6 0.0            // Tool corner radius

7 0              // Compensate to tip (0 = no, 1 = yes)

8 L              // Cutter compensation in computer

9                // Cutter compensation in control

10 0.0           // Amount of stock to leave

20 1.0           // Rapid depth

30 40.0          // Feed rate

31 20.0          // Plunge rate

32 5000          // Spindle speed

33 NONE          // Material

40 100           // Starting sequence number

41 2             // Sequence number increment

42 0             // Program number

50 0             // Coolant

60 0             // Linear array: X axis: Steps

61 0.0           // Linear array: X axis: Distance

62 0             // Linear array: Y axis: Steps

63 0.0           // Linear array: Y axis: Distance

64 0             // Depth cuts: Rough: Steps

65 0.0           // Depth cuts: Rough: Distance

66 0             // Depth cuts: Finish: Steps

67 0.0           // Depth cuts: Finish: Distance

70 0.01          // Roll around sharp corners

71               // Rotary axis

72 0.0           // Rotary axis diameter

101 1            // Infinite look-ahead (0 = no, 1 = yes)

102 0            // Entry line is perpendicular (0 = no, 1 = yes)

103 0.0          // Entry line length

104 0.0          // Entry arc radius

105 1.570796     // Entry arc angle in radians

106 0            // Number of roughing cuts

107 0.0          // Rough cut spacing

108 1            // Number of finish cuts

109 0.0          // Finish cut spacing

110 0.001        // Linearization tolerance

111 0            // Contour depth is incremental (0 = no, 1 = yes)

112 0.0          // Incremental contour depth

113 0.005        // Maximum depth variance

114 1            // Finish all (0 = no, 1 = yes)

[drill]

100 0            // Drill cycle

102 0            // Initial height is incremental (0 = no, 1 = yes)

103 0.2          // Incremental initial height

104 0            // Reference height is incremental (0 = no, 1 = yes)

105 0.1          // Incremental reference height

106 0            // Drill depth is incremental (0 = no, 1 = yes)

107 0.0          // Incremental depth

108 0.11         // First peck increment

109 0.1          // Subsequent peck increment

110 0.009        // Peck clearance

111 0.001        // Retraction distance

112 0.0          // Dwell

113 1.0          // Absolute initial height

114 0.1          // Absolute reference height

115 0            // Adjust depth per drill tip (0 = no, 1 = yes)

115 2.05945      // Drill tip angle in radians

[pocket]

300 0            // Cutting method (0 = zig zag, 1 = spiral inside out, 2 = spiral outside in)

301 1.570796     // Roughing angle

302 0.1          // Roughing step size

304 0            // Number of finish passes

305 0.0          // Finish pass step size

307 1            // Entry line is perpendicular (0 = no, 1 = yes)

310 0.0          // Entry line length

311 0.0          // Entry arc radius

312 1.570796     // Entry arc angle in radians

313 1            // Machine by region (0 = no, 1 = yes)

314 0            // Finish allowance

315 0            // Use additional finish parameters (0 = no, 1 = yes)

316 0            // Break arcs (0 = full circles, 1 = arcs, 2 = lines)

317 0.001        // Chordal deviation for breaking arcs into lines

320 1.570796     // Maximum angle for breaking arcs into arcs

321 0            // Optimize finish passes

330 0            // Tapered wall pocketing (1=yes, 0=no)

331 0.05235      // Tapered wall pocketing: Base taper angle in radians

332 0.05235      // Tapered wall pocketing: Island taper angle in radians

333 0.0          // Tapered wall pocketing: Initial Z

334 -1.0         // Tapered wall pocketing: Final Z

335 -0.25        // Tapered wall pocketing: Z step

350 0            // Finish: Cutter compensation in control (0 = no, 1 = yes)

351 0.0          // Finish: Roll around sharp corners

353 NONE         // Finish: Material

354 01250FLT     // Finish: Tool reference

355 1            // Finish: Tool number

356 1            // Finish: Tool diameter offset number

357 1            // Finish: Tool length offset number

360 0.125        // Finish: Tool diameter

361 0.0          // Finish: Amount of stock to leave

362 40.0         // Finish: Feed rate

363 20.0         // Finish: Plunge rate

364 5000         // Finish: Spindle speed

365 0            // Finish: Coolant

366 1.0          // Finish: Rapid depth

367 0.0          // Finish: Home position x

370 0.0          // Finish: Home position y

371 0.0          // Finish: Home position z

372 1            // Finish: Fillet for cutter compensation in control

380 1            // Helix: 0 = off, 1 = on

381 0.5          // Helix: maximum radius

382 0.087266     // Helix: ramp angle in radians

383 0.02         // Helix: maximum error

384 0            // Helix: 0 = CW, 1 = CCW

385 0.02         // Helix: xy clearance

386 0.5          // Helix: depth

387 0            // Helix: 0 = plunge to start, 1 = rapid to start

388 0.0          // Ramp direction

[flowline]

100 0            // Use chordal deviation (0 = no, 1 = yes)

101 0            // Cutting method

102 0            // Cutting direction

103 0.05         // Along distance

104 0.05         // Across distance

105 0.001        // Chordal deviation

106 0.001        // Scallop height

107 0            // Zig-Zag vectors: 1=yes, 0=no

108 0.0          // Zig-Zag vectors: Angle in radians

109 0.0          // Zig-Zag vectors: Length

[mfinish]

200 0.785398     // Cutting angle in radians

202 1            // Use chordal deviation (0 = no, 1 = yes)

203 0.051        // Along distance

204 0.00125      // Chordal deviation

205 0.052        // Across distance

206 0.053        // Checking distance

207 0.0002       // Vertical wall tolerance

208 0            // Cutting method (0 = zig zag, 1 = one way)

210 1            // Display intersections (0 = no, 1 = yes)

213 1            // Find self-intersections (0 = no, 1 = yes)

220 0.1          // Maximum short gap length

221 0            // Short gap method (0 = direct, 1 = broken)

222 0            // Long gap method (0 = broken, 1 = feed to rapid, 2 = rapid to rapid)

223 0            // Prompt for starting point (0 = no, 1 = yes)

224 1            // Find tool edge intersections (0 = no, 1 = between surfaces, 2 = all)

225 0.001        // Tool-edge intersection error

226 0            // Use check surfaces (0 = no, 1 = yes)

227 0.0          // Stock to leave on check surfaces

228 0            // Prompt for 2d tool center containment boundaries (0 = no, 1 = yes)

229 2            // Connecting motion along one cut (0 = direct, 1 = broken, 2 = retract)

230 2            // Connecting motion between cuts (0 = direct, 1 = broken, 2 = retract)

231 3            // Retract feedrate to rapid plane (2 = feed, 3 = rapid)

232 0            // Use (finish) multiple pass roughing (0 = no, 1 = yes)

233 1            // Allow negative z motion along surface (0 = no, 1 = yes)

234 1            // Allow positive z motion along surface (0 = no, 1 = yes)

235 1            // Plunge control (0 = multi-plunge, 1=one side, 2=both sides)

252 0            // Write UV document

280 0.0          // Slice value

281 0            // Intersection method

[mrough]

401 0.0          // Entry arc radius

402 1.570796     // Entry arc angle

403 1            // Incremental depths (0 = no, 1 = yes)

404 0.10         // Top clearance

405 0.05         // Bottom clearance

407 -0.25        // Maximum depth

408 -0.25        // Minimum depth

410 0.1          // Distance between constant-z levels

412 0            // Roughing option: 0 = contour, 1 = pocket

413 0            // Type of surface: 0 = cavity, 1 = boss

414 1            // Ordering of cuts: 0 = bottom-to-top, 1 = top-to-bottom

415 1            // Cutting direction: 0 = conventional, 1 = climb

416 1            // Display intersections (0 = no, 1 = yes)

418 0.50         // Maximum stitching gap

420 0            // Prompt for starting point

421 0            // Graphics cleanup: 0 = none, 1 = redraw, 2 = clear screen

423 0.001        // Minimum contour length

424 0            // Connect stitched gaps: 0 = direct, 1 = sharp corners

426 0            // Find peaks and valleys (0 = no, 1 = yes)

427 0.25         // Checking distance for peaks and valleys

428 0.5          // Maximum short gap length

429 1            // Short gap path: 0 = rapid, 1 = broken

430 0            // Long gap path: 0 = rapid, 1 = broken

431 0            // Cutting direction for open contours: 0 = zig zag, 1 = one way

435 1            // Use chordal deviation:

436 0.01         // Stepping method: 0 = distance increment, 1 = chordal deviation

437 0.05         // Along distance

438 0.05         // Checking distance

439 0            // Generate toolpath from: 0 = surfaces, 1 = NCI file

440 0            // Simulate spherical cutter compensation

[coons]

100 0.1          // Along cut distance

101 0.1          // Across cut distance

102 0            // Blending: 0 = linear, 1 = parabolic, 2 = cubic, 3 = cubic with slope matching

103 0            // Cutting method: 0 = zz, 1 = o.w., 2 = circ., 3 = 5ax zz, 4 = 5ax o.w., 5 = 5ax circ.

104 0            // Cutting direction: 0 = along, 1 = across

105 1            // Graphics: 0 = striped, 1 = solid

[loft]

100 0.1          // Along cut distance

101 0.1          // Across cut distance

102 3            // Cutting method: 0 = zig zag, 1 = one way, 2 = circular, 3 = 5 ax zz, 4 = 5ax o.w., 6 = 5ax circ.

103 0            // Cutting direction: 0 = along, 1 = across

104 1            // Graphics: 0 = striped, 1 = solid

[revolution]

100 0.1          // Step size

101 -1.0         // Axis depth

102 1            // Shape: 0 = convex, 1 = concave

103 0            // Retract to: 0 = nothing, 1 = reference plane

104 0.1          // Reference plane height

105 N            // Trim surface to: N = nothing, H = height, W = width

106 0.1          // Trimming height

107 0.0          // Trimming width 1

108 0.0          // Trimming width 2

109 1.0          // Trimming width direction 1

110 -1.0         // Trimming width direction 2

[ruled]

100 0            // Constant z cutting (0 = no, 1 = yes)

101 0.0          // Initial z

102 -1.0         // Final z

103 -0.25        // Z step

104 0            // Cutting method: 0 = zig zag, 1 = one way, 2 = circular

105 0.05         // Across distance

106 1            // Graphics: 0 = striped, 1 = solid

107 0            // Gouge checking: 0 = off, 1 = parallel, 2 = perpendicular

110 N            // Trimming plane 1: X, Y, Z or N (none)

111 0.0          // Trimming plane 1 depth

112 0.0          // Trimming plane 1 point: x

113 0.0          //                         y

114 0.0          //                         z

115 0.0          // Trimming plane 1 normal: x

116 0.0          //                          y

117 0.0          //                          z

120 N            // Trimming plane 2: X, Y, Z or N

121 0.0          // Trimming plane 2 depth

122 0.0          // Trimming plane 2 point: x

123 0.0          //                         y

124 0.0          //                         z

125 0.0          // Trimming plane 2 normal: x

126 0.0          //                          y

127 0.0          //                          z

[2d swept]

100 0.15         // Across distance

101 0.785398     // RZ: Roll around sharp corners

102 0.785398     // XY: Roll around sharp corners

[3d swept]

100 0.1          // Along distance

101 0.1          // Across distance

102 0            // Rotate (0) / translate (1) the across contour

103 0            // Cutting method: 0 = zz, 1 = o.w., 2 = circ, 3 = 5ax zz, 4 = 5ax o.w., 5 = 5ax circ.

104 0            // Cutting direction: 0 = along, 1 = across

105 1            // Graphics: 0 = striped, 1 = solid

[project]

100 t           // Name of NCI file to project

101 0.0002      // Minimum error from theoretical surface

102 0.001       // Minimum step size

103 100         // Maximum step size

104 0.0         // Cutter radius fort 3d cutter compensation

[projsurf]

100 t           // Name of NCI file to project

101 0           // Add Z depths (0 = no, 1 = yes)

102 1           // Rapid depth

103 0.02        // XY step increment

104 0.0         // Offset distance

105 1           // Graphics (0 = striped, 1 = solid)

[projxsec]

100 t           // Name of NCI file to project

101 0.001       // Minimum step size

102 100         // Maximum step size

103 0.0002      // Minimum error from theoretical surface

[edxform]

100 1            // Number of copies

101 0            // Ignore blocks

102 0            // Xform tool plane

103 0            // Delete the original items

104 1            // Reverse cutter compensation direction

110 90.0         // Rotation angle in degrees

111 0.0          // Rotation center: X

112 0.0          // Rotation center: Y

113 0.0          // Rotation center: Z

120 0.0          // Polar angle in degrees

121 1.0          // Polar distance

130 1.0          // Scale factor: X

131 1.0          // Scale factor: Y

132 1.0          // Scale factor: Z

133 0.0          // Scale center: X

134 0.0          // Scale center: Y

135 0.0          // Scale center: Z

140 0.0          // Translation distance: X

141 0.0          // Translation distance: Y

142 0.0          // Translation distance: Z

143 0.0          // Translate between points: From point: X

144 0.0          // Translate between points: From point: Y

145 0.0          // Translate between points: From point: Z

146 0.0          // Translate between points: To point: X

147 0.0          // Translate between points: To point: Y

148 0.0          // Translate between points: To point: Z

[ofsctr]

1 1             // Offset direction: 1=Left, 2=Right

2 0.5           // Offset radius

3 1             // Roll around corners (1=none, 2=sharp, 3=all)

4 1             // Use infinite look ahead (1=yes, 0=no)

5 1             // Number of copies

6 2             // Delete old (1=yes, 2=no)

[subfig]

1 t             // Geometry file name

2 1             // Scale

3 0.0           // Rotation angle (in radians)

4 0             // Mirror X axis (1=yes, 0=no)

5 0             // Mirror Y axis (1=yes, 0=no)

6 0             // Mirror Z axis (1=yes, 0=no)

7 0             // Use main color and level (1=yes, 0=no)

[backplot]

1 1             // Tool geometry file name

2 0             // Draw tool reference (1=yes, 0=no)

3 0             // Draw auto tool (1=yes, 0=no)

4 1             // Draw toolpath (1=yes, 0=no)

5 0             // Interpolate along toolpath (otherwise endpoint)

6 0.05          // Interpolation step size

7 1             // Animated tool display (otherwise static)

8 0.0           // Run delay

9 0             // Spin the tool (1=yes, 0=no)

10 0            // Save geometry (1=yes, 0=no)

11 0            // Fit toolpath to screen

12 0            // Stop at NULL tool changes?

13 0            // NULL tool change cleanup (0=none, 1=repaint, 2=clr scrn)

14 0            // Simulate cutter compensation (1=yes, 0=no)

[surfshade]

701 892         // Image horizontal size

702 631         // Image vertical size

703 1           // Output to screen (1=yes, 0=no)

704 10          // Always 10 (do not change)

705 20          // Number of pixels per facet

706 50          // Diffuse reflection (0-100)

707 30          // Shiny reflection (0-100)

708 20          // Shinyness of the material (0-200)

709 10000       // Perspective distance

710 0.0         // Background color #1 (R)

711 0.0         // Background color #2 (G)

712 0.0         // Background color #2 (B)

713 4           // Number of palettes

714 100         // Palette #1 - Color (R)

715 100         // Palette #1 - Color (G)

716 100         // Palette #1 - Color (B)

717 0           // Palette #1 - Associate color

718 100         // Palette #2 - Color (R)

719 0.0         // Palette #2 - Color (G)

720 0.0         // Palette #2 - Color (B)

721 12          // Palette #2 - Associate color

722 0.0         // Palette #3 - Color (R)

723 100         // Palette #3 - Color (G)

724 0.0         // Palette #3 - Color (B)

725 10          // Palette #3 - Associate color

726 0.0         // Palette #4 - Color (R)

727 0.0         // Palette #4 - Color (G)

728 100         // Palette #4 - Color (B)

729 1           // Palette #4 - Associate color

730 3           // Number of Lights

731 0.0         // Light #1 - X position

732 0.0         // Light #1 - Y position

733 1000        // Light #1 - Z position

734 100         // Light #1 - Intensity (0-100)

735 1           // Light #1 - State (1=on, 0=off)

736 1000        // Light #2 - X position

737 1000        // Light #2 - Y position

738 1000        // Light #2 - Z position

739 100         // Light #2 - Intensity (0-100)

740 0           // Light #2 - State (1=on, 0=off)

741 -1000       // Light #3 - X position

742 1000        // Light #3 - Y position

743 1000        // Light #3 - Z position

744 100         // Light #3 - Intensity (0-100)

745 0           // Light #3 - State (1=on, 0=off)

[curvature]

801 892         // Image horizontal size

802 631         // Image vertical size

803 1           // Output to screen (1=yes, 0=no)

804 0           // Always zero (not used)

805 20          // Number of pixels per facet

806 4           // Curvature type (1=Gauss, 2=Avg, 3=Abs, 4=Min)

807 0.01        // Minimum radius of curvature

808 10          // Maximum radius of curvature

809 0           // Always zero (not used)

810 0.0         // Background color #1 (R)

811 0.0         // Background color #2 (G)

812 0.0         // Background color #2 (B)