MP.EXE Fixes (includes MPL.EXE and MPWIRE.EXE)                 FIXES.MAN

----------------------------------------------

1.04   Correct -.nci line number count (bug4).  Needed reset to zero

after reading tooltable.

Correct circular with scaling (by scalex, scaley, scalez)

Correct some problems with postline subroutine structure

(MP got lost on some 'subroutine' calls - calls to other p'lines)

Copy xh, xr to x after output, and x to xh, xr after x output, etc.

(maintains modality of X, Y, Z)

Output unformatted var's absolute, even if FS 1 is incr.

Allow literal var's (i.e. .5) to start with dec. pt.

( .5 is now OK instead of 0.5 )

Allow pause and wait for response (FQ function)

Major dia. threading in MPL (see THREAD.MAN)

4 APR 1991

1.05   Allow upper case in all lines of MP.

25 APR 1991

1.06   Reduce no. of user-named variables to 100 to get post size of 113k.

MPL - fixed circular to output arc center in i,k variables.

1.07   Correct some problems with out-of-range subscripts which can cause

untraceable difficulties.

8 MAY 1991

1.08   Smaller size. MP, MPL, MPWIRE fit in 110k.

Added Auxiliary file (-.aux).

Sub and Aux files can be merged with main with MERGESUB and MERGEAUX.

MPL corrected for scalex, scalez operation.

Paths can be added to filenames.

All file names and extensions can be overridden (see FILES.MAN).

Questions can input strings. Define string before FQ (see QUESTION.MAN)

13 MAY 1991

1.09   Correction for Lookup Tables (they failed in 1.08)

MPWIRE - added TRIMPLANE1, TRIMPLANE2 to 1015 lines.

added flags LISTTPR, LISTRAD to list taper and radius tables.

added postlines PTPRLIST, PRADLIST to write taper and

radius tables.

BLDNXTOOL must be 'yes' or '1' if you want to list taper or

radius tables.

added postline PCONCHG to 1010 lines (control settings change)

MP, MPL - NEXT_TOOL works (again)

15 MAY 1991

1.10    Added EX to exit a primary postline.  EX can be called in a

primary (pre-named) or secondary (user-named) postline to exit

the primary postline without making any more tests.

Allow prefixes to start with a number if within quotes.

MPWIRE - corrected CANTEXT flag for 4-axis wire.

28 MAY 1991

1.11    Drill cycles (gcode=81 or 100) skip motion test in linear(PLIN,PRAPID)

Added error message 'Postline cannot be forced with "*" or "!"'

11 JUNE 1991

1.12    No. of user-assigned VAR names is now 200 for MP, MPL, MPWIRE

No. of user-assigned POSTLINE names is now 200 for MP, MPL, MPWIRE

Allocate 115k for MP, MPL in NCCONFIG.

18 JUNE 1991

1.13    SWEEP is now in degrees (circular - central angle of arc)

19 June 1991

1.14    TRFLAT is now a found from 1013 -.NCI lines (TRFLAT = TLRAD - TCR)

XZ arcs (G18) work better (signs of I, J are reversed)

5 JULY 1991

1.15    IF and WHILE are now case insensitive.  The boolean operator

(<, >, etc.) must still be separated with spaces.

8 JULY 1991

1.16    Long drill cycle corrected to ignore blocks with no motion.

25 JULY 1991

1.17    Built-in character conversion using file MPTRANS.TBL and new

variables TRANSCHR and OMITCRLF.  See TRANS.MAN.

1.171   MPL corrected for threading the "backside" of a thread by using

negative THDA1 and THDA2.

1.18    SPACES can now be changed in postlines.

Formats with a decimal point and no digits after the decimal point

now work correctly (FS 4 1.0) yields 123. for decimal 123.

26 AUG 91

1.19    200 strings are now available  ( changes LIMITS.MAN ).

27 AUG 91

1.20    FIRST_TOOL is now picked up correctly when using build-next-tool-

table option (BLDNXTBL = yes).

30 AUG 91

1.21    Delay output of initial comment lines (1005, 1006) until after the

header (PHEADER).

06 SEP 91

1.22    Output manual entry comments (again).

Outputting prv_(var) no longer updates prv_(var) with current variable.

09 SEP 91

1.23    ATAN2 is now corrected so that the order of arguments is y, x.

This may cause trouble in posts currently using atan2, but

better to fix it now rather than later.

Addition to circular to detect small arcs.

New var. DMAX contains largest x, y, or z delta.

Post can compare DMAX against some tolerance and call PLIN

if arc is too short.

Omit dec. pt. if fraction is zero - ^ replaces dec. pt in FS

01 NOV 91

MP, MPL - DRILLCUR = 1 if gcode is 81 or 100, otherwise is 0.

MILL ONLY -

Update Z or X,Y to rapid or home position in linear. This

affects rapid incremental output on dogleg only.

MILL ONLY -

Output updated Z in circular (formerly wasn't updated in cir. ).

TLRAD is now calculated from TLDIA in postline PTOOLTBL.

TOOLTABLE can now be used in a postline, by itself, to output

the tool table other than in the header.  TOOLTABLE still

retains its former property as a flag variable, so it

can be used either way, but generally will not be used

both ways in one -.pst file.  See SPECIAL.MAN.

18 DEC 91  V1.27   MP, MP386

NEXTOP holds the gcode of the next motion when GETNEXTOP : yes.

23 DEC 91  V1.28   MP, MP386

Five axis routine, while still pretty much of a dummy, outputs

x,y,z and i,j,k, the normalized tool axis vector.

31 DEC 91  V1.29   MP, MPL, MP386

Scratch file for getnextop/nextop NCI2.SYS is now deleted

automatically.

13 JAN 92  V 1.30  MP, MPL, MP386

PROGNAME is now a flag as well as a command. If PROGNAME is

initialized to 1  (progname : 1), the progname will be output

upper case.

14 FEB 92  V 1.31  MPL, MP386

G18 arcs (PLANE=2) had reversed sign of Y (Y is the plane of the arc)

03 MAR 92  V 1.32  MPL, MP386

G18 and G19 arcs (PLANE = 2 or 1) do not have a valid depth in the

-.NCI file, so the current depth (Y or X) must be used.  This should

correct all G18 and G19 problems.

04 MAR 92  V 1.33  MPL, MP386

1011, 1012, 1013, and 1014 data were not being restored properly

after writing tool table (TOOLTABLE = yes).

Currently, a temporary work-around is needed for ZR in PSOF and PTLCHG

in posts using tool origin and toolplanes.  ZR is correct in

CAM386/A10 and later.

12 MAR 92 V 1.35  MP, MPL, MP386

Pre-start-of-file comments are now limited to 40 characters, but

now 40 comments are allowed.  Single comments can still have 80

characters.

VTOL is now active in circular.  It should be set to the resolution

of the output format (if FS=1.5, vtol : .00001).

ROTAXTYP flag is now active for toolplanes (1014 lines in -.NCI),

which call postline PROT.  See TOOLPLN.MAN.  If Toolplane in on,

postline PROT will be called.  PROT must be added to old posts

which are going to process toolpaths with Toolplanes (1014's).

17 MAR 92 V 1.36 MP, MP386

ROTAXTYP flag didn't quite match the document (toolpln.man) in 1.35.

1.36 contins the correction and the seeds of a logical extension

for 5-axis posts:

ROTAXTYP = 0 - First toolplane view (1014) is zero for rotary axis.

ROTAXTYP = 1 - Top view is zero for rotary axis.

ROTAXTYP = 2 - First toolplane view (1014) is zero for 5-axis.

ROTAXTYP = 3 - Top view is zero for 5-axis.

06 APR 92 V 1.37 MP, MP386

Postline PROT0 added - In some cases it is necessary to input data

to -.PST when the first 1014 is read (Pre-SOF 1014).

1014 lines after SOF (1001) still use PROT.

08 APR 92 V 1.38  MP, MP386, MPL

SWEEP now works correctly with BREAKARCS=YES.

MPL only - THDFIRST is now active.  Order of calculating THDNCUTS

is now THDNCUTS, THDFIRST, THDLAST  (first non-zero var. wins).

THREAD.MAN is updated.

06 APR 92 V 1.39  MP, MP386, MPL

Format for ldg. zeroes omitted, no dec. pt., will now leave '0'.

Duplicate tools have been removed from TOOLTABLE.

NTOOLS contains no. of tools, including duplicates.

TTBLEND = 1 when last tool has been called.

26 MAY 92 V 1.40  MP

300 User Variables, 300 User Postlines, 500 Strings[15]

27 MAY 92 V 1.41  MP, MP386

PROTOFS postline added for 1014 toolplanes and ROTAXTYP = 4.

This gives the -.pst a chance to set up ROTOFSX,Y,Z when

toolplane changes.

02 JUN 92 V 2.00  MP (now inclues toolplane optimization of MP386)

300 user variables, 300 user postlines, 500 strings.

All strings and comments are now 80 characters long.

50 pre-start-of-file comments will be allowed.

Error reporting is improved.  If there are any -.PST errors,

the post will not run, and error messages will stay on screen.

PCOMMENT and PCOMWITH postlines added, so that comments can

now be formatted on a postline.  The name 'scomm' is now

reserved for the comment string.  A separate postline

PCOMWITH was needed in case comments are to be output with

a line (1007).  In this case, the sequence no. 'n' should

not appear in the postline, since it will be in the motion

part of the -.nc line.

'scomm' is now a reserved string name.

Note:  Toolplane optimization is not finished yet, but the new calculator,

expanded arrays and increased string length are available now, so we

will have some 1.9.. versions until the toolplane optimization can be

included in MP.

17 JUN 92 V 1.91 MP

Like V 2.00, but without toolplane optimization (former MP386).

Variables now have 15 char prefix (letters) in FMT function.

( V 2.00 with Toolplane Optimization isn't ready yet. )

06 AUG 92 V 1.92 MP

Power is added to calculator with '^'.

Example: llen = sqrt(dx^2 + dy^2 + dz^2)

19 AUG 92 V 1.93 MP

Tombstone mapping is again correct.

26 AUG 92 V 1.94 MP, MPL

Correction in comment handling.  All comments are now 'formatted'

by postline PCOMMENT.  PCOMWITH no longer exists.  A typical

comment postline is as follows:

pcomment     # Format comments

"(", scomm, ")"

08 SEP 92 V 1.95 MP, MPL

Check for illegal characters in variable names.  This will also

trap such illegalities as !prv_(var).  Only legal characters

in a variable name now are letters, numbers, and underscore.

Variable names must start with a letter.

14 SEP 92 V 1.96 MP

M1 thru M9 rotation matrix is now available.

Toolplane matrix is now initialized to Top View for -.nci files

which have no 1014 before Start-of-file.

24 SEP 92 V 2.00 MP

MP now contains Toolplane Optimization and Tool Optimization

(formerly found in MP386).  A second -.NCI file is generated

by the optimization, but is named -.NCO so that it will not

overwrite the original -.NCI file.  This file can be backplotted,

but the extension -.NCO will have to be specified.

Sometimes a -.TMP file is generated and deleted.

If a -.TMP file exists by the same name, it will be wiped out.

Toolplane Optimization is enabled by QTOOLPLN = yes.

Tool Optimization is enabled by QTOOLOPT = yes.

MP.OVR must be copied along with MP.EXE, and reside in the

same directory.

26 SEP 92 MP, MPL V 2.01

Special variable names DAY, MONTH, MON, YEAR have been added.

Now the date can be expressed in any order. DAY, MONTH, and YEAR

are numeric, while MON is a 3-letter abbreviation for month,

removing any ambiguity of which is day and which is month.

Since these are 'special' variables, SPACES should be set to zero

and then back to the current value (usually 1).  These variables

cannot be used in any equation(=) or assignment(:).

spaces = 0

day," ", mon, " ", year

spaces = 1

LINARC : yes  will now linearize arcs to chord height tolerance

specified by VTOL (a variable tolerance definable in -.PST).

Duplicate equations and postlines are now flagged.  All variables are

initialized to zero, so re-initializing to zero will not be flagged.

Special variables cannot be used in an equation.

01 OCT 92 MPL V 2.011

Correction for taper threading.  Depth of thread is now accurate for

threads with extreme tapers.  Z location of start of all threads

may change a little.  Z of last cut is now at Z location described

by THDZCLR.  This value is set by "Stock clearance" on the parameter

page.  For clarity, you may wish to change line 3268 in LATHE.TXT to

"Stock clearance in Z".

06 OCT 92 MP, MPL V 2.02

DATE and TIME both have trailing blank removed so that they can

be concatenated any way desired.  DATE, DAY, MONTH, MON, YEAR and TIME

are not really variables, they are special strings, so they cannot be

used in the same way that variables can.

MP - It's now OK to have ROTDIA = 0 in the -.NCI file.  It will no

longer cause a divide-by-zero error.

09 OCT 92 MP, MPL V 2.03

DAY, MONTH, and YEAR have been changed to be normal variables.

This means that they can be formatted, concatenated, used as

selector variables, etc.  On the downside, they MUST be formatted,

or their format will be FS 1, and the output probably won't look right.

DATE, DAY, and MON are still special strings.  SPACES=0 must still

be used to concatenate MON.

EXAMPLE:

FS 6 2 0ln

FMT    6 day

FMT    6 month

FMT    6 year2       # don't use 'yr' here - that's y-rapid

year2 = year - 1900  # Change 1992 to 92

pheader     # File header

spaces = 0

day, month, year2

091092       -.NC file output for Oct. 9, 1992

spaces = 1

day, mon, year

09 OCT 1992   Another option

27 OCT 92 MP V 1.98 (without toolplane opt.)

MP V 2.04 (with toolplane opt.)

Added axis substition XS and YS to linearized arcs (LINARC:yes)

See AXISSUB.MAN

06 NOV 92 MP, MPL V 2.05 (with toolplane opt.)

Correction in write tool table routine (wrtt).  A null tool change

(g=1000) before a drill cycle (g=81) could cause MP to abort.

11 NOV 92 MP, MPL, MPFAB V 2.06

A small revision on duplicate declarations (initializations).

It is now OK to have 'YES : 1' in the initializations.

It will be accepted because it is the same value as the internal init.

DAY, MONTH, YEAR now output correctly.  You still must supply a space

if you want one, because these values have no prefix letter,

therefore 'spaces : 1' is not applied.

NQC is a new variable which tells the number of characters in a

question string (q1, q2, etc.).  You must capture NQC in another

variable before the next question overwrites it.

29 DEC 92 MP, MPL V 2.07

Another correction in write tool table routine (wrtt).  There was a

problem with comment (1007) following rapid (following toolchange

or SOF).

14 JAN 93 MP, MPL V 2.08

I,J,K are cleared in circular initially and after each arc segment.

25 JAN 93 MPL only v 2.081

Several more variables are made available in PTHDEXT and PTHDINT:

THDDEPTH, THDFLAT, THDA1, THDFIRST, THDLAST.

Changing THDA1 allows the infeed to be different from thread angle

'a1' without affecting the equal-area-per-cut calculations.

See NCI201.MAN and THREAD.MAN.

08 FEB 93 MP only V 2.09

New parameter is read from G=11 -.NCI lines.  This is split into

two new variables CUTPOS and CUTTYP.  CUTPOS is the first digit.

CUTTYP is the second digit.

CUTPOS = 1 - Start of cut

CUTPOS = 2 - Middle of cut

CUTPOS = 3 - End of cut

CUTTYP = 1 - Zig-zag

CUTTYP = 2 - One way

CUTTYP = 3 - Circular

CUTTYP = 4 - Swarf

There are now 600 user-named variables.

01 MAR 93 MP, MPL V 2.10

New special variable COMMENT outputs up to 50 comments (1006 or 1007)

in PSOF, PTLCHG, or PEOF.  Comments are still formatted by PCOMMENT,

and comment string is still SCOMM.

03 MAR 93 MPL V 2.10

Correction in threading.  Initial THDA1 from -.NCI file is in

radians, but if THDA1 is entered from PTHDEXT or PTHDINT, it is

the infeed angle in degrees.  This makes it possible to make the

infeed angle different from the thread angle.  The thread angle in

the parameter page should be the correct thread angle.  Both thread

angles from the parameter page are used to calculate equal area cuts.

04 MAR 93 MP, MPL V 2.11

Correction in formatter - in FS (format statement), the combination of

leave trailing zeroes and omit dec. pt. failed to leave trailing zeroes

when the value of the number was zero.

11 MAR 93 MP, MPL V 2.12

Tool numbers can now be up to 999 for tooltable output.  An error

message was added to prevent crashing if this number is exceeded.

Tool number can be any integer (+-32000) if tooltable is not output.

01 APR 93 MP      V 2.13

ARCRAD is internally set to zero for each -.NCI line.  Only a circular

line will change ARCRAD to be non-zero.  ARCRAD can then be used to

distinguish linear from circular in PLIN when LINARC is YES.

06 APR 93 MP, MPL  V 2.14

Calculator should be bulletproof now.

A new math function FINC is added.  This function increments the

output column of a lookup table.  This makes it possible to keep count

of the number of times a tool is used.  The incremented count is

returned.  The count may also be accessed by FLOOK.

ntimesused = finc(1, t) # Increment the count for number of

# times t is used.

flktbl   1     99            # Lookup table for tool usage (99 entries)

0      1   # tool no.

0      2

0      3

...

04 MAY 93 MP, MPL, MPWIRE  V 2.15

Correction for comments.  When the command COMMENT was added to allow

pre-SOF comments to be positioned anywhere, later comments were dropped.

12 MAY 93 MP, MPL, MPWIRE  V 2.16

PQ postline added for questions which supply variables needed before

SOF.  For instance, if PROT0 (called by 1014 toolplane) needed

TLNGA for rotation offsets, the 'question' to input TLNGA would

have to be called in postline PQ.

COMMLNG variable added - gives length of comment line - can be tested

in -.PST to see if a comment exists.

Tooltable can now accommodate tool numbers up to 9999.

11 JUN 93 MP, MPL, MPWIRE V 2.17

New compilation needed to make PQ postline accessible.

PQ postline is accessed only if PQ appears in the post.  PQ postline

can be used like PHEADER.  PQ is called only once, before any -.NCI

lines are read or processed.

26 JUL 93  MP V 2.18

Read ZNCI depth in circular for all planes (PLANE).

New flag ROLLDIE : YES must be set in order to enable the test in

linearizing arcs (with LINARC : YES).  Now the error message

'Arc radius ARCRAD is less than tool radius TLRAD' will only

occur if both flags are set and the error condition exists.

The reason for the change is that the error message was created

mainly for rolldie work.

COMMENT command in a postline now works correctly.  COMMENT should

appear in PSOF, PTLCHG, and PEOF if it appears anywhere

in the -.PST file.

G18 arcs (ZX plane) are corrected (again).

30 JUL 93  MP MPL V2.19

Allow boolean operations on strings.

Only named strings allowed.  STRTOOL is now a pre-named string,

and is the same as TOOLSTR.  TOOLSTR can still be used in a postline,

but STRTOOL must be used in a string boolean.

Only '=' and '<>' operations are allowed on strings.

Rotaxtyp = 4 will map linear, circular, drill, and 5-axis data

to TOP view.  This is for both MP and MPL.

13 AUG 93 MP, MPL V 2.19

Error correction to booleans using literal var's.

17 AUG 93 MP V 2.20,  MPL V 2.201

More corrections on booleans.

15 SEP 93 MP V 2.21

Allow OMITROUND:YES to not affect motion test, which skips calls

to linear postlines when there is no motion.  Formerly, OMITROUND:YES

caused motion calculation to be skipped, which required SKIPMOTEST

to be set to 'yes'.

09 NOV 93 MP, MPL V 2.23

Add third scratch file -.EXT.  This file has properties identical with

-.SUB and -.AUX.  Write to -.EXT with SUBOUT = 3.

MERGEEXT merges -.EXT to main (-.NC).  CLEAREXT clears -.EXT.

15 NOV 93 MP, MPL V 2.24 MP

Add code for lathe toolplanes with ROTAXTYP = 3.  Spindle axis is C

(like tombstone), and angle perp. to spindle is A.

Add duplicate postline label detection.

11 DEC 93 MP, MPL, MPWIRE   V 2.26

Add NEXTCCOMP, which is the next CCOMP (0..4) cutter comp. code.

This is only available when GETNEXTOP:YES.

Circular doesn't depend on prv_x anymore.  Start of arc from previous

linear or circular move is saved internally.  Difference between

startpoint and current x is only checked if ARCCHECK : YES.

There will probably still be a discrepancy unless OMITROUND:NO.

In other words, there is often a rounding difference between the

-.NCI file start of arc (the point before the arc) and the start of

arc which is output to the -.NC file.

01 JAN 94 MP V3.00

Added vector math routines. See -.VEC files for more information.

Added a new feature to allow a way of protecting posts from being

copied and distributed to people other than the original

customer.  See MPBIN.VEC for more information.

Added a third subfile -.EXT similar to -.SUB and -.AUX.

Fixed bugs in the output format.

User defined questions can now contain the characters "=" and ":".

Program execution will now stop after two errors have been found

in the -.PST file.

27 JAN 94  MP V3.00

WRTT now reads 1011 and 1012 lines.

1000,1001,1002 lines call PWRTT.

09 FEB 94  MP V3.01

Drill cycles ('simple' drilling) can now be used to drill from

the back.

USECANDRILL... flags can now be changed in postlines.

Changed product code from 128 to 129 for MP.

14 FEB 94  MP V3.01

Change T to neg. tool no. rather than -1 in PWRTT.

28 FEB 94  MP V3.02

In long drill cycles, skip last retract move to REFHT if

INITHT = REFHT.  This is done so that NEXTOP will be correct on

the last move of a long drill cycle.

01 MAR 94  MP V3.03

There is no longer an error message for missing postline names,

unless you set TESTPLN : 1.

Added memory allocation error message that will tell how to allocate

more memory.

Fixed bug in the third subfile -.EXT.

The words 'startbin' and 'endbin' may now be upper case or lower case.

See MPBIN.VEC for more information.

17 MAR 94  MP V3.04

Correct circular with breakarcs : yes in the YZ plane.

22 MAR 94  RP V3.01

First release of reverse post processor.

21 APR 94  MP V3.05, MPL V3.01

Changed round() function to round numbers up to 2 billion instead

of the current 32,000.

Fixed comments in MPL to work the way they do in MP.

Fixed 'nextop' in MPL.

Fixed bug in peck and fixed bug in drill in MP.

Fixed 'linarc' to work in planes 1 and 2.

The words 'startbin' and 'endbin', which are used with MPBIN, will be

ignored if they appear in a post file with extension .psm.

See MPBIN.VEC for more information.

Added a new feature called buffer files.  See documentation in

BUFFERS.MAN for more information.

Added a new feature to round a variable according to the format

statement without updating the previous value.  The symbol '@'

before the variable name will cause the rounding.

01 MAY 94  MP V3.06, MPL V3.01

Fixed math routines for intersections: 'xll' (line-line), 'xla'

(line-arc), 'xaa' (arc-arc).  Added documentation for intersection

routines see INTERSEC.VEC .

Added a new feature called 'brklines' to break a line into many small

line segments. (MP only)

Fixed error message for insufficient memory when running MPL from

MasterCam.  The message will now give the proper solution to the

problem.

Fixed tool plane mapping. (MP)

09 MAY 94  RP V1.02

Fixed bug finding 'nextop' and 'nextccomp'.

Fixed bug outputting a blank line with g80 and g1003.

Changed the conditions that cause a tool change.  A change in the tool

number is no longer sufficient to cause a tool change.  The only way

to get a tool change is to set the variable 'tlchng' = 1 in the

string table.

18 MAY 94 RP V1.03

Added postlines 'ppretlchg', 'pposttlchg', 'ppreparsetool',

'ppostparsetool' and 'ppostproc'. See RPPOSTLN.MAN for details.

Fixed output NCI format, when feedrate is unchanged -1 is output for a

linear move and -2 is output for a rapid move.

03 JUNE 94 RP V1.03

Added feature to strip the control-in and control-out characters

from comments.

Fixed bug where the look ahead variable 'size_sof' is larger than the

size of the NC file.

10 JUNE 94 MPL V3.02

Fixed bug in threading and changed the version number to 3.02.

10 JUNE 94 RP V1.03

Fixed bug: Do not output tlchng before drill cycle if start-of-file

block has not yet been output and skiptotool1 is set to yes.

15 JUNE 94  MP V3.06, MPL V3.02, RP V1.03

Added additional debug information.  If bug2 is set equal to a negative

number, for example,

bug2 : -30     # Turn on debug info

then a long list of postlines that are called will be written on the

NC line starting at column 30.  If bug2 is set equal to a positive

number it work the same as before.

Added a special character '~' which will output a variable with no

update of the previous. For example,

~x, ~y, ~z

will cause x, y and z to be output with no update of the previous.

This is particularly useful in debugging.

Changed the way buffer files work.  The buffer files are now saved

as text files. See BUFFERS.MAN for details.

Fixed a bug in rounding.

27 JUNE 94  MP V3.07

Fixed five axis tool mapping by delaying the mapping until after the

tool length offset is found.

Changed MP version number to V3.07.

28 JUNE 94  MP V3.07, MPL V3.03, RP V1.04

Fixed bug in buffer files so they can have more than 32,000 records.

Fixed bug in lookup tables so they can have more than 128 entries.

Changed RP Version number to V1.04.

Changed MPL version number to V3.03.

06 JULY 94  MP V3.07, MPL V3.03, RP V1.04

Fixed bug that can occur when two strings are tested for equality.

For example: if sstr1 = sstr2, "strings are equal"

08 JULY 94  MP V3.07

Fixed bug that occurs when an arc is in a plane but the depth

changes from the start point to the end point.

14 JULY 94  MPL V3.03

Fixed bug where 'scalex' scaled 'z' and 'scalez' scaled 'x'.

01 AUGUST 94 MP V3.07

The flag 'brklines' is now a variable and a flag.  As a flag 'brklines'

works just as before.

brklines : no    # don't break lines into segments

brklines : yes   # break line into segments using angle and radius

If 'brklines' is given a value other than 0 (no) or 1 (yes) then

that value is the length of the segments that the line should be

broken up into.  For example,

brklines : 0.05  # break line into segments 0.05 long

08 AUGUST 94 MP V3.07

Added a new variable 'nextdc', the next drill cycle.  This variable

works like 'nextccomp', the next cutter comp.  Both variables

only look as far ahead as the next nci line.

08 AUGUST 94 MP V3.08, MPL V3.04, RP V1.05

Increased the number of buffers from 5 to 10.

Added initialize flag to buffer definitions.  With this flag you can

specify whether to initilize the buffer from a text file or not.

Added string buffer files.  These work just like the buffer files of

real numbers except with strings there is only one string per record.

See BUFFERS.MAN for detailed information.

Added string-equals-string function.  For example, str1 = str2

where str1 and str2 have been initialized.

Changed MP version number to V3.08.

Changed MPL version number to V3.04.

Changed RP Version number to V1.05.

11 AUGUST 94 MP V3.08

Fixed bug in scaling arcs in plane 1 and 2 when scalez is different

from scalex and scaley.

'linarc' will now linearize arcs where the height changes from the

start point to the end point.  These arcs are also called a helixes.

15 AUGUST 94 MPWIRE V3.00

As of version 3.00 MPWIRE is a C language version with most of the

functions of MP mill including vector math functions and buffer files.

17 AUGUST 94 All Post Processors

Fixed bug in the set extension variables: sextnc, sextsub, sextaux,

and sextext.  The optional period, '.', will now work.  For example,

sextnc  .ncc  # Set NC file extension to 'ncc'

sextnc  ncc   # Set NC file extension to 'ncc'

are both valid.

24 AUGUST 94 MP V3.09

Changed five axis drilling.  On the first hole the routine pdrill,

ppeck, pchpbrk, ptap, pbore1, pbore2, pmisc1 or pmisc2 is called and

on all sebsequent holes pdrill_2, ppeck_2, pchpbrk_2, ptap_2,

pbore1_2, pbore2_2, pmisc1_2 or pmisc2_2 is called.

01 September 94  All Post Processors

Added "Enter to continue" after error message to give time to read

the message.

Fixed bug where the output line would exceeded 255 characters if

'bug2' was set to output a long list of postline calls.

08 September 94  All Post Processors

Subprogram names can now be entered with a user defined question.

See QUESTION.MAN for details.

Predefined variable 'time' now has hours, minutes and seconds

in the form hh.mmss, when 'time' is given a format assignment.

(This is not available for reverse post processors.)

19 September 94 MP V3.09a

Fixed bug where some toolplanes matrices caused round off errors.

04 October 94  MP V3.09b

Added new variable 'rtnrefht'.  This variable is used to determine

the return height at the end of a canned drill cycle and must be set

before the postblock 'pcanceldc', if it is used.

If rtnrefht = 0, then prv_z = initht, initial height (default).

If rtnrefht = 1, then prv_z = refht, reference height.

If the z position is going to be any other value then it must

be set in the postblock 'pcanceldc', set prv_z to the desired height.

05 October 94  MP V3.09c

Fixed cantext.  See CANTEXT.MAN for information on how the use cantext.

11 October 94  MP V3.10, RP V1.05, MPL V3.04, RPL V1.01, MPW V3.01

Fixed bug that kept frange and the binary file from working in DLL.

Fixed bug in MPW where the first point was lost.

New version numbers for all post processors.

21 October 94 MP V3.10a

Tool numbers can now be up to 8 digits.

Fixed math function 'rotp'.  The axis of rotation is now automatically

unitized before the rotation calculated.

27 October 94 MP V3.10b, MPL V3.04b, MPW V3.01b

Fixed bug when tooltable is used.

27 October 94 RP V1.06b, RPL V1.01b

The toolname found on the 1013 line can now be output with

either the string 'strtool' or the variable 'tlname', if the string

is empty.  The sting 'strtool' can be set in the post if it is used.

If the variable 'tlname' is used it will now be output without a

trailing period '.', for example:

old: 10.

new: 10

01 November 94

The new 'Post Processor Users Guide' has been printed.  This guide is

written specifically for mill but includes general information

about any post written in the MP language.

03 November 94 MPL V3.04c

Added postblock 'pthread0'.

Added 'linarc', linearize arc, to lathe.

11 November 94  MP V3.10c

Added variable 'cctotip'.  This variable is read from the NCI file

on the 1013 line as the fourth number.  This variable is output

from Mastercam.

cctotip = 1, compensate to tool tip

cctotip = 0, compensate to tool center

11 November 94  All Post Processors

MP V3.10d, MPL V3.04d, MPW V3.01d, RP V1.06d, RPL V1.01d

Fixed bug in format statement.  When incremental was used with no

decimal value and no decimal point extra spaces where added between

the prefix and number.

Added math routine bitwise 'and' (and), bitwise 'exclusive or' (xor)

and bitwise 'inclusive or' (ior).  Bitwise math is done by

converting two numbers to binary then combining the individual bits

of the two numbers one at a time in the following way,

X  Y    (X and Y)  (X xor Y)  (X ior Y)

0  0        0          0          0

1  0        0          1          1

0  1        0          1          1

1  1        1          0          1

For example, 1023 in binary is 0011 1111 1111

255 in binary is 0000 1111 1111

1023 xor 255 is 0011 0000 0000 which is 768

Thus, 1023 xor 255 = 768.

19 December 94  All Post Processors

When using string buffers a blank line will now be treated as a

string of spaces, ' ', instead of being ignored.

Fixed bug in windows versions of MP.  This bug will cause the following

to fail: all posts that use string buffers and all posts that use

hexidecimal output.  Posts that use real number buffers are not

effected by the bug.

Added feature to reverse post processors RP and RPL.  Previously, all

comment read from the NC file where output to the NCI file.  You can

now read a comment from the NC file and not output it to the NCI file

by setting the variable 'comm' to '3'.  This allows a way to skip

lines in the NC file.  For more information on reverse post

processing comments see RPSTRTBL.MAN, located in RP.MPK on the BBS.

New version numbers: MP V3.11, MPL V3.05, MPWIRE V3.02,

RP V1.07, RPL V1.03.

15 February 95

New version numbers: MP V3.11a, MPL V3.05a, MPWIRE V3.02a,

RP V1.07a, RPL V1.03a.

You can now set the variable 'vtol' in the initialization section

without getting an error message.

2d vector math functions are no longer protected.  You do not need an

access code or a *.psb file to use 2d vector math functions.

Fixed bug when a five axis toolpath is being post processed with

leadin and leadout set to zero. (mill only)

Added variables 'brklinestype' and 'brklineslen'.  These variables are

be used to automatically break lines into small line segments.  For

more information see BRKLINE.MAN. (mill only)

Added more types for 'rotaxtyp'.

rotaxtyp = -1 or 1, top plane is C zero

rotaxtyp = -2, front plane is C zero

rotaxtyp = -3, side plane is C zero

See the Post Processor User Guide for more information on 'rotaxtyp'.

(mill only)

MPWIRE: The following variables are now available: 'nextwc', 'nextwt'

and 'skimpass'.  See the Post Processor User Guide.

There are now MP.dll's available for Mill Version 5.5.  Note: Version

5.0 c-hooks are not compatable with Mill version 5.5 and version

5.5 c-hooks are not compatable with Mill version 5.0.

07 March 95

New Versions: MP V3.12, MPL V3.06, MPWIRE V3.03,

RP V3.08, RPL V3.04

You can now hit ESC or Ctrl-Break to stop a post while it is running.

There is now a beta version of RPWIRE.EXE, the reverse wire post

processor.  This is located in RPWIRE.MPK.

Fixed bug in reverse posting when a line of NC code is read and part

of the line has a comment and part of the line has code.  It will no

longer ignore the code on the NC line.

Fixed bug in reverse posting when 'skiptotool1' is set to 0 (no).  It

will no longer read the NC file twice if a start-of-file tool change

is never found.

23 March 95

New Versions: MPWIRE V3.04, RPWIRE V3.01

MPWIRE and RPWIRE have been updated for Mastercam Wire 5.0 NCI.

The following predefined variables have been added:

reg1 : auxillary register 1

reg2 : auxillary register 2

reg3 : auxillary register 3

reg4 : auxillary register 4

reg5 : auxillary register 5

cstop  : stop flag

cgstop : optional (glue) stop flag

cend   : end of contour flag

cstart : start of contour flag

The auxillary registers appear as entries 5 to 9 on the g1010 line and

entries 11 to 15 on the g1015 lines.

The control flags ('cstop', 'cgstop', 'cend' and 'cstart') are combined

into one number as follows:

1's : cstop

10's : cgstop

100's : cend

1000's : cstart

The control flags appear on the following NCI lines as the last number:

g0, g1, g2, g3, g11, g20, g21, g22, g23, g30, g31, g23 and g33

02 May 1995

New Versions: MP V3.13, MPL V3.07, MPWIRE V3.05

RP V1.09, RPL V3.05, RPWIRE V3.02

Fixed bug in 'frange' function where numbers at the high end of the

range, within 0.5 of the max, were rounded up instead of down.

(all post processors).

Fixed parsing bug in all reverse post processors.  This bug caused

the wrong value to be scanned from the NCI line when the string being

scanned appeared twice in the NCI line.

Fixed start-of-file bug in all reverse post processors.  If you set

'skiptotool1' to 'no' then a start-of-file tool change block will now

be written out at the start of the file before any other NCI lines

are output, except comments.  Comments at the start of the NC file

will still be output to the NCI file before the start-of-file block.

Fixed bug in MPWIRE to output correct Q angle with Agie advanced conics.

Fixed bug in MPWIRE to output immediate wire change angle with 4 axis

taper and 'qrtyp' set to 2.

17 May 1995

New Versions: MP V3.13, MPL V3.07, MPWIRE V3.05

Fixed bug where the null character was not output on the NC line.

For example: n, x, y, z, 0

The number '0' will now properly output the null character.  There

is a limit of 10 null characters per post line.

(all forward post processors)

Fixed bug in MP.EXE and MP.DLL where the variable 'fg1014' was not

always set properly.  The variable 'fg1014' is set to '1' if a

1014 line is read from the NCI file at the last tool change block

and set to '0' if there is no 1014 line at the last tool change

block.