Converter v5.3                                Extra Technical Documentation

---------------------------------------------------------------------------

-- -x now traces every line processed. -t show's tables and elements.

-- -X now allows you to set the way the xyz data from the input file is

translated to the output file. -X takes the following form :

-X [+ / -][x / y / z] [+ / -][x / y / z] [+ / -][x / y / z]

examples:

-X +x+y+z : this is the normal case

In    Out

+x >> x

+y >> y

+z >> z

-X +y+x-z : this will mirror the z co-ordinate and swap the x and y

In    Out

+y >> x

+x >> y

-z >> z

by making a simple model fi. four colored balls on the origin and the xyz axis,

noting the colors and rendering this, one can determine which axis have to be

swappend and/or mirrored. Options for DesignCad3D DXF files are  -x+z+y

-- 'Stray' objects in the block section as generated by MicroStation.

except for holes, dxf's from MDL dxfout 4.1.5 work ok.

-- The -o option will generate an file for standard output but not echo

anything to the screen anymore.

-- Support for Personal Designer 4.1 (GCD) v. Computervision.

-- Support 3D-Studio Release-2 DXF files (backplane cullable).

-- Support for AME-2 DXF polyfaces (backplane cullable).

-- AutoCAD 10,11,12 DXF Support (and DXF compatible programs)

-- Handles polyfaces with more then 400 vertices and/or polygons. these are

expanded to 'large'-polyfaces just like with large meshes.

-- Closed pline's with WIDTH are handled correctly where end and begin meet.

-- Pline's off which the begin and end vertex are the same but are not closed are

not treated as closed anymore.

-- Arcs are supported in FACE, NORMAL and WIDTH POLYLINE's.

-- Pline widths joints have been cleaned up.

Reminder:

The polyline is split in the converter into a number of different types :

Type           Used when:              with -n                     Limits     ->

-----------------------------------------------------------------------------------------

FACE           width = 0 & thick = 0   treated as complex face     vertices < 400 otherwise skipped

NORMAL         width = 0 & thick != 0  capped if closed            vertices < 200

WIDTH          width != 0                                          vertices < 100

MESH           71 != 0 & 72 != 0                                   vertices < 400

LARGE-MESH     71 != 0 & 72 != 0                                   vertices => 400

POLYFACE       70 = 64                                             vertices < 400 polygons < 400

LARGE-POLYFACE 70 = 64                                             vertices => 400 polygons => 400

-- With (-n), cylinders with thickness of zero are triangulated.

-- Max unresolved inserts raised to 1000.

-- Frozen layers are turned off.

-- Polyfaces with 3 and/or 4 vertices per polygon are supported.

-- -3 (split faces) now also works with 4 vertex pface face's.

-- Inserts in layer '0' while this layer is turned off are now inserted.

-- Total blocks raised to 1000.

-- Polylines which have in their header BOTH mesh counts and width

are treated as width polylines and not as meshes anymore.

the mesh count is ignored.

-- -u option to update an existing cmf file, use in combination

with -a. it can handle adding layers but not re-ordering so don't

purge or delete. check the cmf as output by the converter to see

how it works.

-- AutoCad Dview's with perspective now come across to rs2 correctly.

-- Cylinders with zero thickness become disc's always.

-- Material store now set at 1024.

-- Layers table store now set at 2048.

-- Now with 5.1 pharlap.