+---------------------------------------------------------------------------+

|                                                                           |

|                               VIDEO Support                               |

|                                                                           |

|  This file contains information about video display devices (for all      |

|  CADSIM platforms) which has been changed since the last revision of the  |

|  CADSIM User's Manual.  The information appears in chronological order,   |

|  newest last.  Where possible, forward references have been inserted next |

|  to items whose information has been superceded by new information later  |

|  in this file.                                                            |

|                                                                           |

|  The CADSIM drawing window will generaly be displayed at the highest      |

|  screen resolution possible for your particualr hardware.  Specifically,  |

|  on the following platforms:                                              |

|    Macintosh -- CADSIM will use the full screens on displays attached.    |

|    OS/2 -- CADSIM suports any resolution supported by the Presentation    |

|            Manager                                                        |

|    DOS -- CADSIM tries to open the highest screen resolution supported by |

|           your hardware.  For more information see the notes under CADSIM |

|           Version 2.4 below.                                              |

|                                                                           |

+---------------------------------------------------------------------------+

----------------------- CADSIM Version 2.1 ----------------------------------

----------------------- CADSIM Version 2.2 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.1

and Version 2.2:

- MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS:

See enhancements in Version 2.4 notes for more details about accessing

the various video modes.

- CURSORS MADE BOLDER WITH NEW DOS VERSION:

Cursors have been made bolder or accented so that they are more visible

on the mono or grey-scale screens of laptop computers.  Therefore,

cursors have a slightly different appearance than that shown in the

manual.  Their functions remain the same.

- OS/2 VERSION OF CADSIM IS RELEASED:

An 8514/A compatible graphics card may require a full 1 or 2 Mb of RAM

in order for the Presentation Manager to recognize it in 1024 x 768

resolution.

----------------------- CADSIM Version 2.3 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.2

and Version 2.3:

- OS/2 CADSIM AND THE 8514 HIGH RESOLUTION VIDEO (1024 by 768) DISPLAY:

After installing OS/2, copy \CADSIM\OS28514.INI to \CADSIM\CADSIM.INI if

you will be using an 8514 display card.  This will give better color

definition.  Note that for an 8514 to be used with OS/2 it must have the

256 color option.  Clones of the 8514 card that fit in the AT style of

chassis are becoming available from companies such as ATI and Paradise --

they work with DOS and OS/2 on IBM compatible machines and are very fast.

17" monitors are a good match with the 8514/A adapter.  The Sony 17"

Trinitron monitors are highly recommended.

----------------------- CADSIM Version 2.4 ----------------------------------

The following notes outline the changes between CADSIM Release Version 2.3

and Version 2.4:

- ENHANCED MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS:

A new version of CADSIM supports multiple graphic standards from a single

version.  This should make changing between hardware easy. The standards

supported are:  CGA, EGA, VGA, Super VGA, 8514A, VESA, TIGA, Hercules and

Toshiba.  Resolutions up to 1280 x 1024 are supported.  CADSIM uses

auto-sensing.  For most hardware it will automatically select the highest

resolution and number of colors (max 16) supported by your graphics card.

If however starting CADSIM causes the screen to go blank, auto-sensing may

have failed to identify your particular hardware.  If this happens, or

if you wish to try different resolutions, a particular mode can be set by

either one of two methods.  Examples follow for both methods (assuming a

VGA 16 color display operation is required for this example):

Method 1:  Place the following line in the AUTOEXEC.BAT file:

SET FG_DISPLAY=VGA12

Method 2:  Add the mode number as the second number on the first line of

the CADSIM.INI file.  For example, To force video mode # 18, the first

line of CADSIM.INI might be changed from:

0.26                Screen pitch (mm/pixel)

to:

0.26 18             Screen pitch (mm/pixel), Video mode (forced) to VGA

Other Display names for method 1 and mode #s for method 2 follow:

Display        mode  mode  Resolution  Colors   Description of Standard

Name           #     hex   hor x ver

HERCHALF         2   002   720 x 348    2(mono) Hercules 1 display page

HERCFULL         3   003   720 x 348    2       Hercules 2 display pages

CGAHIRES         6   006   640 x 200    2       IBM CGA in highres mono

EGACOLOR        14   00E   640 x 200   16       EGA with IBM color monitor

EGAMONO         15   00F   640 x 350    4       EGA with monochrome monitor

EGAECD          16   010   640 x 350   16       EGA, enhanced color display

VGA11           17   011   640 x 480    2       IBM VGA monochrome

VGA12           18   012   640 x 480   16       IBM VGA

ATI61         2657   A61   640 x 400   16       ATI VGA Wonder+ 640 x 400

ATI62         2658   A62   640 x 480   16       ATI VGA Wonder+ VGA

ATI63         2659   A63   800 x 600   16       ATI VGA Wonder+ Super VGA

TSENGHIRES      41   029   800 x 600   16       Tseng Labs/Orchid Pro VGA

PARADISEHIRES   88   058   800 x 600   16       PARADISE Super VGA

TRIDENTHIRES    91   05B   800 x 600   16       TRIDENT Super VGA

VEGAVGAHIRES    98   062   800 x 600   16       VIDEO 7 VEGA Super VGA

DFIHIRES       100   064   800 x 600   16       Diamond Flower Super VGA

VESA6A         106   06A   800 x 600   16     * multi vendor Super VGA

EVGAHIRES      112   070   800 x 600   16       Everex EVGA

TOSHIBA        116   074   640 x 400    2     # Toshiba T/J-3100

VESA0          256   100   640 x 400   16     * multi vendor 640 x 400

VESA1          257   101   640 x 480   16     * multi vendor VGA

VESA2          258   102   800 x 600   16     * multi vendor Super VGA

VESA3          259   103   800 x 600   16     * multi vendor Super VGA

VESA5          261   105  1024 x 768   16     * multi vendor 1024 x 768

VESA7          263   107  1280 x 1024  16     * multi vendor 1280 x 1024

TIGA_8_640   13313  3401   640 x 480   16       TIGA 2.0

TIGA_8_1024  13314  3402  1024 x 768   16       TIGA 2.0

TIGA_8_640   13315  3403  1280 x 1024  16       TIGA 2.0

8514A_8_640  34049  8501   640 x 480   16       IBM 8514A & clones

8514A_8_1024 34050  8502  1024 x 768   16       IBM 8514A & clones

* The VESA modes will work on a variety of graphics adaptors which are

made to be compatible with the VESA standards for graphic adapters.

The VESA terminate and stay resident program for your board is required

for all of the VESA modes except 0x6A and 0x102.  Refer to the manual

for your graphics adapter for more information.

# No auto-detection.  You must use Method 1 or Method 2 above.

To determine what graphics modes your computer hardware supports type

SET FG_DISPLAY=??? at the command line prompt, (substituting different

display names from the table above for ???), then run CADSIM.  Refer also

to the manufacturers specifications for your graphics card and monitor to

determine which modes are supported by your hardware configuration.

----------------------- CADSIM Version 2.5 ----------------------------------

- ENHANCED MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS:

The above table of DOS graphics modes supported has been updated:

The ATI, Diamond, TIGA and some of the VESA modes were added in Oct. 1991.

The 8514A, TSENGHIRES and TIGA modes were changed in August 1993.

----------------- CADSIM Version 2.6 / COACH Version 0.9 --------------------

The following notes outline the changes between CADSIM Release Version 2.5

and Version 2.6:

- ENHANCED MULTI-VIDEO MODE VERSION OF CADSIM FOR DOS AND 8514:

DOS CADSIM no longer requires that the HDI drivers are loaded to support

the 8514 graphics mode.  8514 is normaly a very fast high resolution mode

for DOS and is recommended.  However, this mode does run slower than other

modes only when run in a DOS session under OS/2.  For this reason, you may

wish to experiment with other modes if running DOS CADSIM under OS/2.  The

native OS/2 version of CADSIM supports 8514 well.

- COACH BETA TEST CODE FOR OS/2 RELEASED:            -- (new) October, 1993

The COACH debugging mode employs many dialog boxes and charts.  A low

resolution screen such as VGA will soon get too crowded.  A screen

resolution of 1024 by 768 is a recommended minimum, however, 800 by 600

may be useable.