F i l e    I n f o r m a t i o n

* DESCRIPTION

Documentation for TPSPOOL.

* ASSOCIATED FILES

TPSPOOL.PAS

TPSPOOL.DOC

TPSPOOL.EXE

* KEYWORDS

PASCAL V4.0 DOCUMENTATION SPOOL PRINTER BUFFER

==========================================================================

}

TPSPOOL - Simple Print Spooler written in Turbo Pascal 4.0

Released to the Public Domain AS IS!

Written on or about 12/5/87

VERSION 0.8!!!  You get what you pay for.

Uses TurboPower's Turbo Professional 4.0.

This program captures printer output and saves it in a file.  The printer

output is captured by trapping the BIOS interrupt 17h and hijacking the

characters.

If you do not specify a number on the command line, TPSPOOL will use a large

buffer.  The buffer should be large enough to store the most characters that

will be written at once.  Future versions may not impose these requirements.

In practice, I believe 16K, 32K, or the default of about 64K should work in

most cases.  The next version will use EMS for the big buffers, freeing

valuable main RAM.

In keeping with the Turbo Pascal tradition, Hex numbers may be passed on the

command line by preceeding them with a $.

The program will display some text, then terminate and stay resident.  To

activate, hit Alt-Tab (Alt key and Tab key at the same time).  This "hot key"

toggles the spooler on and off.  When the spooler is off (the default), hitting

Alt-Tab will beep and turn it on.  When the spooler is already on, typing

Alt-Tab will disable the spooler and dump the buffers to the spool file.

All printer output is spooled (stored) in the disk file SPOOL01.TMP.  If a

file by this name exists, it is appended, if not, it is created.  Future

versions will allow other file names.

Some examples of command line options:

TPSPOOL<cr>

This would use the default (biggest possible) buffer size.

TPSPOOL $4000

This would use a 16K buffer

TPSPOOL $8000

This would use a 32K buffer

TPSPOOL 1024

This would use a 1K buffer.  This is probably too small unless you can

guarentee that only Int 17h will be used to send output to the printer.

See technical notes.

TECHNICAL NOTES

The most common problem experienced with this program is running

it but forgetting to enable the spooler by pressing Alt-Tab.  This will

cause output to go to a printer if one is attached just like normal.  If a

printer isn't attached, YOU MUST ENABLE THE SPOOLER BEFORE TRYING TO PRINT.

If a single write to the PRN device via a DOS Write Handle or Device call is

larger than the specified buffer size, this program will hang.  This is why

the default is almost a full 64K.  In reality, most programs send small chunks

of 1K or less to the PRN device.  If the program uses Int 17h directly (like

most Word Processors), any buffer size of 1K or more will work safely.  I

recommend 4-16K if memory is tight, or 16-64K if it's not.

#