XIRCOM CREDITCARD ETHERNET ADAPTER IIps

Version 1.02

June 21, 1994

========================================================================

Thank you for purchasing the Xircom CreditCard Ethernet Adapter IIps.

Xircom has done everything to ensure that our CreditCard Ethernet

Adapter IIps is compatible with notebooks that support the Personal

Computer Memory Card International Association's (PCMCIA) 2.0 to 2.1 I/O

slot configuration.  Xircom is committed to making all of its networking

products compatible with the widest range of notebook and laptop PCs

possible.  As a company, we are equally committed to customer

satisfaction and timely response to the needs and concerns of our

customers.

As the PCMCIA technology evolves you can be assured Xircom will continue

to provide the highest level of hardware and software compatibility in

the industry.

If you have compatibility issues related to your specific environment,

or you have general questions, please contact Xircom Technical Support.

Information on Xircom Technical Support is located at the end of this

file.

Remember to register your Xircom CreditCard Ethernet Adapter IIps

to receive information about future software upgrades and

new products.

CONTENTS

========

1)  CreditCard Ethernet Adapter IIps Characteristics

2)  Features

3)  CE2CFG.EXE notes

4)  Network Driver Notes

5)  Special Software Notes

6)  Machine Specific Notes

7)  Xircom Technical Support

1) CREDITCARD ETHERNET ADAPTER IIps CHARACTERISTICS

================================================

This release of Xircom's CreditCard Ethernet Adapter IIps has the

following characteristics:

- Card and Socket Services set versions 2.0 to 2.1 compatibility.

Card and Socket Services is a software specification defined by the

PCMCIA organization.  Its purpose is to serve as an intermediary

between the network device driver and the system resources required by

the PCMCIA Adapter.

- Intended for use with Intel 82365 PCIC, Databook TCIC,

Cirrus Logic CL-PD6710 or CL-PD6720 or Toshiba ICCNT PCMCIA

interface chips.

- Variable interrupts, 3 to 15, default interrupt 5.

- Variable I/O window, 16 bytes needed, default base at 300h.

- Requires a 4Kbyte memory window on a 4Kbyte boundary, within segment

C000h to E800h, default is D100h. WE DO NOT SUPPORT THE D000-D0FF

SEGMENT SEE SECTION 5 UNDER MEMORY RANGE NOTES.

- Keyword support to disable link integrity.

- ODI, NDIS for OS/2 2.1 and DOS support, and PACKET drivers.

2) FEATURES OF RELEASE

===================

Features of release 1.02 of the Xircom CreditCard Ethernet

Adapter IIps Network drivers:

-  Windows and DOS-based installation program automatically configures

PC and NOS setup

-  Continued support of Advanced Power Management for suspend/resume

and HotSwapping through the use of Card and Socket Services

-  Power On/Off - turn power to the CreditCard on or off at the slot

using a Power On/Off feature through CE2CFG

-  The CreditCard Ethernet Adapter IIps requires only a 4K memory

window leaving more upper memory blocks free

-  Full Duplex capabilities increases bandwith to 20Mbps.

-  Advanced Look-Ahead Pipelining drivers for superior adapter

performance

3) CE2CFG.EXE CONFIGURATION UTILITY

================================

Supplied on the Network Drivers disk is a utility called CE2CFG.EXE.

The utility can be run from the DOS command line or included in the

AUTOEXEC.BAT file or the CONFIG.SYS file.  It is recommended that

CE2CFG.EXE be run prior to loading any DOS Xircom network driver.

This utility is intended for DOS usage only.

The CE2CFG.EXE utility must be informed about which type of network

driver your network uses: NDIS, ODI, or Packet driver.  The CE2CFG.EXE

file must be located in the same directory as the network driver.

The CE2CFG.EXE allows the Xircom CreditCard Ethernet Adapter IIps to

become a client of Card Services.  This is mandatory to support the

functions of Card and Socket Services.  CE2CFG.EXE must be loaded prior

to loading a network driver when using Card and Socket Services.

4) NETWORK DRIVER NOTES

====================

- The Microsoft Windows NT driver was not available at the time of this

release and will be available in a future release of the drivers.

- The Novell NetWare Server driver was not available at the time of this

release and will be available in a future release of the drivers.

- The diagnostic utility CE2TEST was not available at the time of this

release and will be available in a future release of the drivers.

5) SPECIAL SOFTWARE NOTES

======================

***                     MEMORY MANAGERS                           ***

*** When using a memory manager be certain to exclude the memory  ***

*** range being used by the CreditCard adapter.  A 4k range will  ***

*** need to be excluded within C000 to E800.                      ***

example:   device=c:\dos\himem.sys

device=c:\dos\emm386.exe x=D100-D1FF

*** This example excludes 4k of the D100 segment for use by the   ***

*** CreditCard in a default configuration.  If the CreditCard is  ***

*** configured for a memory address different from the default,   ***

*** be certain to exclude that memory address range in use.       ***

- The OS/2 drivers available on the disk are intended to support OS/2

versions 2.0 and above.  These drivers do not support Microsoft OS/2

version 1.3.

- During installation of NetWare Requester version 2.01 for OS/2, when

prompted for the source drive, which has default A:, insert the

Network Drivers disk and press return following prompted instructions.

The CE2ODI.OS2 driver is located in the root of this diskette as well

in the ODI subdirectory for an easier installation.

***                    MEMORY RANGE NOTES:                        ***

***                                                               ***

*** The CreditCard Ethernet Adapter IIps does not support the     ***

*** memory range:                                                 ***

***                    D000-D0FF                                  ***

***                                                               ***

*** Please use a different one such as:                           ***

***                    D100-D1FF                                  ***

***                    FULL DUPLEX NOTES                          ***

***                                                               ***

*** To utilize full duplex capabilities, your network must have   ***

*** full duplex hardware in place and enabled.  If your network   ***

*** connection does not have full duplex, your network will       ***

*** become unusable do to excessive collisions.                   ***

***                                                               ***

6) MACHINE SPECIFIC NOTES

======================

Dell 320SLi

-----------

The Dell laptops may have an early version of Socket Services, built

into the BIOS, that is not compatible with newer PCMCIA software

specifications.  In this case, either the NET.CFG file must be edited to

include the keyword string 'socketservices NO', or the PROTOCOL.INI

file must include the keyword string 'socketservices = NO'.

The Dell must also be run in IO mode.  To do this add the following line

to the NET.CFG or PROTOCOL.INI or run the Packet Driver as shown below.

NET.CFG example:

Link Driver CE2ODI

SOCKETSERVICES NO

MODE IO

PROTOCOL.INI example:

[XIRCMAC]

SOCKETSERVICE = NO

MODE = IO

PACKET DRIVER example

ce2cfg driver=packet socketservices=no mode=IO

ce2pd  socketservices=no mode=IO

Gateway Colorbook 40MHz and 75Mhz

---------------------------------

The above laptops are not supported on this release of the drivers.

They will be supported in future releases of the drivers.

Grid Convertible

----------------

The above laptops must be run in 8-bit mode.  This requires that the

'iowordsize' keyword be set to '8' in either the NET.CFG or PROTOCOL.INI

file, or on the command line for Packet driver.

NET.CFG Example:

IOWORDSIZE 8

PROTOCOL.INI Example:

IOWORDSIZE = 8

PACKET DRIVER Example:

ce2cfg driver=packet iowordsize=8

ce2pd

HP Omni Books 300 and 4XX series

--------------------------------

The network drivers require a special configuration to support these

machines.

A sample NET.CFG and PROTOCOL.INI is given and all possible

configurable values are shown.

Sample NET.CFG:

Link Driver CE2ODI

Frame Ethernet_802.3

MODE IO

INT 11

MEM D4000 (Other options: D8000, DC000)

IOWORDSIZE 8 (Note: CE2CFG must be run to interpret this line)

Sample PROTOCOL.INI

[XIRCMAC]

MODE = IO

INT  = 11

MEM  = 0xD400 (Other Options: 0xD800,0xDC00)

IOWORDSIZE = 8 (Note: CE2CFG must be run to interpret this line)

IBM 720

-------

The IBM 720 needs to be run in 25MHz mode in order to support this

card.  It will be fully supported in a future release.

NCR 3150

--------

We are unable to support Card and Socket Services with these drivers at

this time on the above machine.  It will be supported in future releases

of this software.  To use the card, remove the Card and Socket Services

in the CONFIG.SYS and AUTOEXEC.BAT.  There is no need to run the CE2CFG

program; simply run the appropriate network driver, such as CE2ODI,

CE2NDIS, or CE2PD.

Sharp 6781

----------

We are unable to support this chipset "PHIC" at this time; it will be

made available in future releases of this software.

Toshiba T3300SL

---------------

We are unable to support Card and Socket Services with this chipset

"ICC/NT" at this time; it will be made available in future releases

of this software.  To use the card remove the Card and Socket Services

in the CONFIG.SYS and AUTOEXEC.BAT.  There is no need to run the CE2CFG

program; simply run the appropriate network driver, such as CE2ODI,

CE2NDIS, or CE2PD.

Toshiba 1950, 4500, 4600, 4700, 4800 series

-------------------------------------------

The Toshiba laptop may have a version of Card and Socket Services that is

not compatible with the newer specifications.  When using Socket Services

1.0, and/or Card Services release 2.0 version 1.0 from Toshiba do the

following.  Use the Keyword 'SOCKETSERVICES' and 'CARDSERVICES' set to

'NO' to allow the Xircom CreditCard Ethernet Adapter IIps to bypass

Card and Socket Services.  See the Xircom User's Guide 'Appendix C' for

configurations of these keywords for the various drivers.  Or remove

Card and Socket Services from the CONFIG.SYS of the laptop.

Toshiba 1900, 1950CS

--------------------

This model will not initialize properly at IO port 300.  Choose another

IO port such as 310.

Zeos

----

The Zeos laptops need to have the keyword "Mode IO" for the NET.CFG or

"Mode = IO" for the PROTOCOL.INI.

7) XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call or send

a facsimile to the appropriate number listed below.  Access the Xircom

INFOSYS BBS at up to 14400 bps, No parity, 8 data bits, 1 stop bit

(N-8-1).

Corporate Offices        Technical Support and Information

------------------------ ---------------------------------------------

Xircom, Inc.             United States............. TEL: 800-874-4428

Corporate Headquarters   International............. TEL: 818-878-6280

26025 Mureau Road        24-Hr Fax-on-demand.(USA). TEL: 800-775-0400

Calabasas, CA  91302     24-Hr Fax-on-demand (Intl) TEL: 818-878-6400

TEL: (818) 878-7600   24-Hr Fax................. FAX: 818-878-7175

FAX: (818) 878-7630   24-Hr BBS Data Lines...... BBS: 818-878-7618

24-Hr Internet E-mail..... INT: cs@xircom.com

24-Hr CompuServe.......... CIS: GO XIRCOM

HOURS:  5:00 a.m. to 6:00 p.m., Monday through Friday (Pacific)

Xircom Europe NV         Europe, Mid-East, Africa.. TEL: 32(0)3 360 38 60

Bisschoppenhoflaan 82-84 24-Hr Fax................. FAX: 32(0)3 326 31 50

2100 Antwerp, Belgium    24-Hr BBS Data Lines...... BBS: 32(0)3 326 23 68

TEL: 32(0)3 360 38 11

FAX: 32(0)3 326 31 50

HOURS:  8:30 a.m. to 5:30 p.m., Monday through Thursday

8:30 a.m. to 4:30 p.m., Friday

Xircom Asia Ltd.         Asia Pacific.............. TEL: 852-525-2078

1604 Century Square      24-Hr Fax................. FAX: 852-525-8955

1-13 D'Aguilar Street    24-Hr Data Lines.......... BBS: 852-537-6048

Hong Kong

TEL: 852-525-2078

FAX: 852-525-8955

HOURS: 9:00 a.m. to 5:00 p.m., Monday through Friday

---------------------------------------------------------------

Thank you for making Xircom a part of your network.

------------------------------------------------------------------------

Xircom acknowledges all tradenames and trademarks used in this document

as the property of their respective owners.

===============================[  END  ]================================