Artisoft AE-2 MC Ethernet Adapter Installation Notes

====================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, NE2000.COM is the only driver you need.

You will have to set the card to NE2000 emulation before installing

PhoneNET PC.  To do this simply boot your machine with your reference

diskette, and go to the section to change the configuration of the AE-2

card.  You will be prompted to insert the diskette that came with the

AE-2.  In the configuration section go to the 'AE-2 MODE' option and

set it to 'NE-2000 COMPATIBLE MODE'.  Save your changes and quit.  The

machine will reboot reconfigured with your changes.

2. GENERAL INFORMATION

-------------------

During this installation, the installer will prompt different I/O base

Addresses and Interrupt Lines for your selection.  In all cases, the

actual settings on the card (the previously configured software

settings) and the parameters specified in the INSTALL program must

match or the MLID will not work.

The Artisoft AE-2 MC Ethernet Adapter works with thinwire Ethernet

cabling (connected via a BNC coax connector) or with an external

transceiver (MAU) connected via a DB-15 AUI connector.  NOTE: you will

have to set the one jumper block on the card to indicate which port

on the card you wish to use.

For more information about installing and configuring your adapter,

please refer to the manual that comes with your adapter.