Cabletron 21xx Series Ethernet Adapter Installation Notes

=========================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, E21ODI.COM is the only driver you need.

To use the lower port on a E2110 or E2112 adapter, you must edit the

NET.CFG file and add the line

MEDIA SECONDARY

in the Link Driver portion.  On the E2110 adapter, this will allow use

of the AUI port.  On the E2112 adapter, this will allow use of the BNC

port.  Otherwise, the board will default to the top twisted pair port.

2. GENERAL INFORMATION

-------------------

The E21xx series boards contain jumper blocks to configure the Base I/O

Address.  The Interrupt and Shared Memory are set by the MLID and

NET.CFG files when they load.

During this installation, the installer will prompt different Base I/O

Addresses and Interrupt Lines for your selection.  In all cases, the

actual jumper settings on the card and the parameters specified in the

INSTALL program must match or the MLID will not work.

Not all shared available memory addresses are presented in the install

program.  If you need to use a shared memory address other than those

provided, you will need to edit the NET.CFG file manually for the

desired address.

For more information about installing and configuring your adapter,

please refer to the manual that comes with the adapter.