Farallon PhoneNET PC Card II (MCA) Installation Notes

=====================================================

1. IMPORTANT NOTES

---------------

To use this MLID, PHNETMC2.COM is the only software driver you need to

run PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The Farallon PhoneNET PC Card II (MCA) uses I/O ports and interrupts.

The card does not use any DMA channels or shared memory.

The Farallon PhoneNET PC Card II (MCA) uses one of 6 I/O ports.  The

card's I/O address can be set by using the reference diskette that

comes with your computer system and the .ADF file that comes with the

adapter.

The PhoneNET install program allows the following possible values for

I/O ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The Farallon PhoneNET PC Card II (MCA) uses one of 3 IRQs.  The card's

IRQ number can be set by using the same files mentioned above.  The

interrupt numbers allowed are :

3, 4, 5

For more details about configuring Farallon's PhoneNET PC Card II (MCA)

and installing the driver, please refer to the "Farallon PhoneNET PC

Card II User's Guide" that comes with your adapter.