PhoneNET PC version 3.1 Read Me file for DOS Users

February 14, 1995

This file contains important information to supplement the PhoneNET PC

User's Guide and PhoneNET PC Release Notes. Please see the user's guide

for basic product information, and the release notes for special

installation instructions and other late-breaking product news.

This file includes the following topics:

List of supported network adapter cards

Using PhoneNET PC with NetWare versions 3.12 and 4.0

Adding your User Name to PostScript print jobs

Copying files from an AFP server to your PC

List of extension mappings

The information in this file applies to the use of PhoneNET PC in DOS.

You may also wish to see the PhoneNET PC Read Me file for Windows users.

LIST OF SUPPORTED NETWORK ADAPTER CARDS

Ethernet Cards

Many of these Ethernet adapters have both coaxial and 10Base-T versions.

Timbuktu for Windows and PhoneNET PC v3.1 support both versions.

Allied Telesis AT-1500T

Artisoft AE-2 & AE-2 MC

Artisoft AE-3

Asante 2000 Series

Asante 2100 Series (MCA)

Cabletron E2000 Series (E2010, E2012, E2019)

Cabletron E2100 Series (E2110, E2112, E2119)

DEC EtherWorks Turbo DE200 Series (DE200/AC, DE201/AC, DE202/AB)

Eagle/Novell NE1000

Eagle/Novell NE2000

Farallon EtherWave ISA Card (PN880)

HP PC LAN Adapter NC/16 (HPJ2405P)

HP PC LAN Adapter/16 TP Plus (HP27247B)

Intel EtherExpress 16 (PCLA8110)

3COM EtherLink (3C501)

3COM EtherLink II (3C503)

3COM EtherLink II/16 (3C503-16)

3COM EtherLink 16 (3C507)

3COM EtherLink III (3C509)

3COM EtherLink III/MC (3C529)

3COM EtherLink III/EISA (3C579)

3COM EtherLink /MC (3C523)

Ungermann Bass PC-NIU/EX (2261A)

Ungermann Bass NIUpc/EOTP (PC4035)

Ungermann Bass NIUps/EOTP (PC3035)

SMC (WD) EtherCard Plus Elite (8003 Series)

SMC (WD) ETherCard Plus Elite 16 (8013 Series)

Xircom Pocket Ethernet Adapter (PE10BT)

Xircom Pocket Ethernet Adapter II (PE2-10BT)

Xircom Pocket Ethernet Adapter III

Xircom CreditCard Adaptor (PCMCIA)

Zenith Z-Note Portable Computer (Built-in Ethernet)

Token Ring Cards

IBM Token Ring 16/4 Adapter

IBM Token Ring 16/4 Adapter /A

IBM Token Ring II 16/4 Adapter

LocalTalk Cards

Apple LocalTalk Card (AppleShare PC Card)

Dayna DL2000 LocalTalk Card (ISA)

Dayna DL2 LocalTalk Card (MCA)

DayStar (COPS) LT200 PC Card

DayStar (COPS) LT200 MC Card

Farallon PhoneNET Card PC (PN380)

Farallon PhoneNET PC Card II (ISA PN381& MCA PN382)

Farallon PhoneNET PC Card III (1/2 Card ISA PN381& MCA PN382)

USING PHONENET PC WITH NETWARE 3.12 or 4.0

This section discusses four known issues with using PhoneNET PC with

NetWare 3.12 or 4.0.

First, you may have difficulty copying files between your NetWare client

drive and an AFP server. To overcome this difficulty, copy the files from

the source server to your local hard disk, and then re-copy them to the

destination server.

Second, you may be unable to launch NetWare Management utilities located

in the SYS or PUBLIC subdirectories. To use any such utility, copy it to

your local hard disk or to your home directory on the server, and run it

from the new location.

Third, the NetWare custom installation tips in the PhoneNET PC User's

Guide assume that your PC uses the NetWare 3.11 client software file

called NETX. If you wish to use the VLM client software available with

NetWare 3.12 or 4.0, find each instance of NETX in the NetWare custom

installation tips in your PhoneNET PC User's Guide, and substitute VLM.

Fourth, the NetWare custom installation tips in the PhoneNET PC User's Guide

assume that both your PC and your NetWare server use the 802.3 Ethernet

frame type. If you are using NetWare 3.12 or 4.0, the Ethernet frame type

used on your PC or server may be 802.2.

Your PC must use the same Ethernet frame type as its NetWare servers. A PC

can only use one frame type, though NetWare servers can often be configured

to use more than one.

Different versions of NetWare have different default Ethernet frame types.

By default, NetWare 3.11 servers use 802.3, NetWare 3.12 servers use 802.2,

and NetWare 4.0 servers use both 802.2 and 802.3. Please ask your NetWare

administrator which frame types each of your NetWare servers use.

If your NetWare administrator is willing to add 802.3 support to any NetWare

server that does not already have it, or you do not need to communicate

with any 802.2-only servers, you can use the NetWare custom installation

tips in the PhoneNET PC User's Guide, without further modification.

However, if you need to communicate with 802.2-only NetWare servers, you

must add the following instructions to the custom installation tips in the

PhoneNET PC User's Guide. These instructions configure your PC to use the

802.2 frame type, rather than 802.3, so you will not be able to communicate

with any 802.3-only NetWare servers.

1. Install PhoneNET PC according to the basic installation instructions and

custom installation tips in the PhoneNET PC User's Guide, but do not perform

the final reboot at the end of the custom installation tips.

2. Use a text editor to open your NET.CFG file, and add the following line

to the Link Support section of the file:

MAX STACKS 6

3. With the NET.CFG file still open, find the following lines in the Link

Driver section of the file:

FRAME ETHERNET_802.3

PROTOCOL IPX 0 ETHERNET_802.3

Change these lines to read as follows:

FRAME ETHERNET_802.2

PROTOCOL IPX E0 ETHERNET_802.2

Please note the addition of an "E" in the second line, above, as well as the

change in frame type.

4. Add the following lines at the end of your NET.CFG file:

NetWare DOS Requester

FIRST NETWORK DRIVE = F

USE DEFAULTS = OFF

VLM = CONN.VLM

VLM = IPXNCP.VLM

VLM = TRAN.VLM

VLM = SECURITY.VLM

;   VLM = NDS.VLM

VLM = BIND.VLM

VLM = WNP.VLM

VLM = FIO.VLM

VLM = GENERAL.VLM

VLM = REDIR.VLM

VLM = PRINT.VLM

VLM = NETX.VLM

Save and close the file.

5. Make sure you have the following lines in your CONFIG.SYS file. If you

already have FILES set to a larger value than 20, please continue to use the

larger value.

LASTDRIVE = Z

FILES = 20

6. Inspect your AUTOEXEC.BAT file. If there is a line to run ABOTH.BAT or

some other PhoneNET PC batch file, comment that line out.

7. Reboot your PC.

8. If your PC boots correctly, change to the PhoneNET PC directory and load

PhoneNET PC by typing ABOTH at a DOS prompt.

9. If you followed the custom installation tips in the PhoneNET PC User's

Guide, the NetWare files you need to load should already be present in the

PhoneNET PC directory. The following example shows the commands you should

type at the DOS prompt if you are using the VLM client software. You can use

NETX in place of VLM, if you wish.

IPXODI

VLM [/PS=Servername]

10. Copy the LOGIN.EXE file from drive F: (your first network drive) to your

PhoneNET PC directory.

11. Run the login program from your local hard disk, as shown below:

LOGIN [Server/] [Username]

Type your password when prompted to do so. Use this local LOGIN command

whenever you wish to work with a NetWare server.

12. If you wish to load PhoneNET PC whenever you start up your PC, use a

text editor to open your AUTOEXEC.BAT file, and uncomment the line for

ABOTH.BAT or another PhoneNET PC batch file.

13. Reboot your computer. The Novell client should automatically load along

with PhoneNET PC.

ADDING YOUR USER NAME TO POSTSCRIPT PRINT JOBS

If you wish, you can add your User Name to PostScript print jobs that are

produced by applications running under DOS. To accomplish this, edit your

ABOTH.BAT file or APRNT.BAT file, and find the following line:

LH C:\PHONENET\APRINT

If you installed PhoneNET PC into a directory other than C:\PHONENET, the

name of your directory will be substituted in this line. Without modifying

the path name, add a "/N" parameter at the end of the line, as shown here:

LH C:\PHONENET\APRINT /N

To put the change into effect, save the file, exit from your editing

program, and reboot your PC.

COPYING FILES FROM AN AFP SERVER TO YOUR PC

If you experience poor performance when copying files from an AFP server to

a PC, particularly on LocalTalk networks or very busy Ethernet or Token Ring

networks, you may be able to improve performance by adding a parameter to

your ABOTH.BAT file or AFILES.BAT file. To do this, open the file with your

favorite text editor, and find the following line:

LH C:\PHONENET\ATALK

If you installed PhoneNET PC into a directory other than C:\PHONENET, the

name of your directory will be substituted in this line. Without modifying

the path name, add a "/FASTATP" parameter at the end of the line, as shown

here:

LH C:\PHONENET\ATALK /FASTATP

To put the change into effect, save the file, exit from your editing

program, and reboot your PC.

LIST OF EXTENSION MAPPINGS

When you use PhoneNET PC to copy a file from your PC to an AFP file server,

PhoneNET PC attributes Macintosh file types and creators to files with

certain DOS file extensions. The following list shows the DOS file

extensions that receive this service, and the Macintosh creators and file

types they receive.

Timbuktu uses the same list and maps both ways; from PC extension to

Macintosh file type/creator and from Macintosh file type/creator to PC

extension.

Extension       Creator Type            Application

---------------------------------------------------------

RPL             REP!    FRL!            Farallon Replica

DOC             MSWD    WDBN            MS Word

RTF             MSWD    TEXT            MS Word RTF

XLS             XCEL    XLS3            Excel 3 Spreadsheet

XLC             XCEL    XLC3            Excel 3 Chart

XLM             XCEL    XLM3            Excel 3 Macro

XLA             XCEL    XLW3            Excel 3 Workbook

XLT             XCEL    sLS3            Excel 3 Template

WKS             L123    LWKS            Lotus 123

WPD             WPC2    WPD2            Word Perfect

AP1             PLP2    PRS2            Aldus Persuasion

APT             PLP2    PRT2            Persuasion Template

API             PLP2    PRIF            Persuasion Interchange

PM4             ALD4    ALB4            PageMaker 4

PT4             ALD4    ALT4            PageMaker 4 Template

PM5             ALD5    ALB5            PageMaker 5

PT5             ALD5    ALT5            PageMaker 5 Template

AI              ARTZ    EPSF            Adobe Illustrator

PSD             8BIM    8BIM            Adobe PhotoShop

FM              FMPR    FMPR            FileMaker Pro

CWP             BOBO    CWWP            Claris Works WP

CWS             BOBO    CWSS            Works SpreadSheet

CWG             BOBO    CWGR            Works Graphic

CWD             BOBO    CWDB            Works Database

QXD             XPRS    XDOC            QuarkXPress

APP             O7$$    O7$A            Omnis 7 App

DF1             O7$$    O7$D            Omnis 7 Data

DBF             FOX+    F+DB            FoxPro/FoxBase

PPT             PPT2    SLD2            MS PowerPoint

MPP             MSPJ    MPP             MS Project

MPV             MSPJ    MPV             MS Project View

DAT             GPEB    GDAT            Dynamics data

IDX             GPEB    GIDX            Dynamics index

DIC             GPEB    DICT            Dynamics dictionary

WKZ             WNGZ    WZSS            Informix Wings

MOV             TVOD    MooV            Apple QuickTime

TIF             MDOS    TIFF            TIFF graphics

GIF             GIFF    Bozo            GIF graphics

SIT             SIT!    SIT!            Alladin Stuffit

C               ttxt    TEXT            Text files

(No extension)  ttxt    TEXT

CPP             ttxt    TEXT

H               ttxt    TEXT

INI             ttxt    TEXT

RC              ttxt    TEXT

DEF             ttxt    TEXT

RSC             ttxt    TEXT

ASM             ttxt    TEXT

INC             ttxt    TEXT

RSA             ttxt    TEXT

RSC             ttxt    TEXT

TXT             ttxt    TEXT

BAT             ttxt    TEXT

EXE             MDOS    BINA            Binary files

COM             MDOS    BINA

DLL             MDOS    BINA

BIN             MDOS    BINA

OVL             MDOS    BINA

SYS             MDOS    BINA

ZIP             MDOS    BINA