Ungermann-Bass NIUps/EOTP Ethernet Adapter (Microchannel) Installation Notes

============================================================================

1. IMPORTANT NOTES

---------------

Before you load the driver UBENODI.COM, you MUST run LOADNIU.EXE and

TCPBIOS.COM that come with the adapter card, otherwise the adapter will

not work correctly.

The file LOADNIU.EXE MAY NOT WORK CORRECTLY if you have some other

software loaded in DOS High memory.

Please also check the file CONFIG.SYS that comes with the driver diskette,

and the file CONFIG.SYS that your system is using.

When you load the drivers you may get a beep and see a warning message

regarding a line in the NET.CFG file. The software will continue loading

correctly, though. You may safely ignore the warning message or you may

insert a pound sign (#) at the beginning of the line in the NET.CFG file

that reads                   PS/2 Slot ?

so that it looks like     #  PS/2 Slot ?

2. GENERAL INFORMATION

-------------------

The NIUps/EOTP is a Microchannel host interface Ethernet Adapter. It

supports Twisted Pair Ethernet (connected via an RJ-45 connector) and

Thick Ethernet cabling (connected via a DIX connector).

For more details about configuring the adapter card, please refer to

"NIUps/EOTP User's Guide" that comes with the adapter card.