Farallon PhoneNET PC Card III (ISA) Installation Notes

======================================================

1. IMPORTANT NOTES

---------------

To use this MLID, PHNETPC3.COM is the only software driver you need to

run PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The Farallon PhoneNET PC Card III (ISA) uses I/O ports and interrupts.

The card does not use any DMA channels or shared memory.

The Farallon PhoneNET PC Card III (ISA) uses one of 6 I/O ports.  The

card's I/O address can be set by changing switches 5-8 of the Switch

Bank.  The install program allows the following possible values for I/O

ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The PhoneNET PC Card III (ISA) uses one of 3 IRQs.  The card's IRQ

number can be set by changing switches 1-3 of the the switch bank.

The following IRQ numbers are allowed:

3, 4, 5

If you choose to install this card the install program will show a

picture of the switch bank.  Use the arrow keys to make the screen

selection match the switch bank on the card.

For more details about configuring Farallon's PhoneNET PC Card III

(ISA) and installing the driver, please refer to the "PhoneNET PC Card

III User's Guide" that comes with your adapter.