3Com EtherLink 16/TP Adapter ( 8 bit slot ) Installation Notes

==============================================================

1. IMPORTANT NOTES

---------------

If you choose to use this MLID, you MUST first run 3C507.EXE to

configure the adapter before you load driver 3C507.COM.  You should NOT

install or load any other programs supplied by 3Com for your EtherLink

16 TP adapter.  (You may have to delete the line in your CONFIG.SYS

that loads 3Com's ETH507.SYS ("DEVICE=ETH507.SYS") or its equivalent.)

The connector used by the EtherLink 16/TP adapter is selected with the

3C507.EXE configuration program.

2. GENERAL INFORMATION

-------------------

3C507.EXE is a diagnostic and configuration program which is supplied

by 3Com to configure the adapter card.  After you install PhoneNET PC

in your system you MUST run 3C507.EXE to configure the adapter.

Note that the actual settings you select when you run 3C507.EXE and the

parameters specified in the INSTALL program must match, otherwise the

MLID will not work.

The EtherLink 16/TP adapter will work on either Twisted Pair Ethernet

cabling (connected via an RJ-45 connector) or thick Ethernet cabling

(connected via a DIX connector).

For more information on installing and configuring your adapter, please

refer to the "3Com EtherLink 16/TP Installation and Configuration

Guide" that comes with your adapter.