6/3/92

Instructions for LOCKDRIVE Filter

---------------------------------

Purpose

-------

LOCKDRV.FLT converts Removable DASD and R/W Optical

drives to fixed disks which are managed by the

OS/2 DASD Manager (OS2DASD.DMD).

Disks supported by the OS/2 DASD Manager may be

paritioned and formatted with either FAT or HPFS

file systems.

The LOCKDRIVE filter also issues commands to

prevent media removal while OS/2 is running.

LOCKDRV Filter ADD Installation

-------------------------------

1.) Copy the LOCKDRIVE filter ADD - LOCKDRV.FLT to

\OS2 directory of the boot drive.

2.) Add the following statement to CONFIG.SYS on

the boot drive.

BASEDEV=LOCKDRV.FLT /UNIT:0,1.....n

Where:

/UNIT: - Specifies devices to be locked.

Devices are numbered starting with the

first Removable DASD or R/W Optical Device

(SCSI Types: 0 or 7).

Notes: 1.) CDROM and Diskette (A:, B:) devices

are not included in this count.

2.) The BASEDEV= stmt for LOCKDRV.FLT may

be put in any where in CONFIG.SYS.

However, if other Filter ADDs are installed,

then the LOCKDRV stmt must appear in the

proper order with regard to other Filter ADD

statements.

3.) Ready the devices to be locked. A cartridge

must be present in the device during the

OS/2 boot process.

4.) Reboot the system.

5.) Use OS/2 FDISKPM, FORMAT utilities to partition

and format the media.

Note: FDISKPM may show incorrect partition sizes for

media which was previously non-paritioned.

This condition may be corrected by selecting

and deleting the incorrect parition entries.

When all the entries have been deleted, FDISK

should show the correct size for the media

in its FREESPACE entry.

6.) Media in the drive will be unlocked after a SHUTDOWN

or Ctrl-Alt-Del reboot of OS/2.

If the LOCKDRV filter cannot lock the devices specified

on its command line in CONFIG.SYS the following message

will be issued:

LockDrive Filter: Some devices requested could not be locked.

If the syntax of the command line in CONFIG.SYS is incorrect,

the LOCKDRIVE filter will uninstall with a SYS1201 error.

Installing OS/2 on Removable Media

----------------------------------

OS/2 may be booted from a locked cartridge device provided,

the DASD adapter provides INT 13 support for removable

devices.

The LOCKDRIVE Filter must be manually installed at the

completion of OS/2 installation and prior to rebooting the

system following the procedures above.

The installation diskettes can be booted to access HPFS

partitions or DOS may be booted to access FAT partitions.

It is STRONGLY RECOMMENDED that OS/2 be installed on a

Primary Partition rather than an Extended Parition

(Logical Drive).

If you install in  an Extended Partition, and your

ADD recognizes addtional drives (beyond what the adapter's

BIOS recognizes) or additional drives are later added to the

configuration, then the drive letter of the bootable partition

will change and you will probably not be able to reboot

the system!

Restrictions and Known Problems

-------------------------------

1.) Locked drives may unexpectedly become unlocked if an ADD

issues a SCSI Device Reset or SCSI Bus Reset to clear

an abnormal SCSI bus condition.

This will be addessed by having the LOCKDRV filter

reestablish the lock condition when it detects a

UNIT ATTENTION (Device Reset) from the device.

2.) Certain devices (especially large optical devices) may

timeout during a FAT format, which issues large

a Read Verify operations. This will be addressed by

the DASD Manager by setting a non-default IORB timeout

for these operations.

3.) Devices with other that 512 byte default sector

sizes are not supported.

The LOCKDRV Filter does not currently set block

sizes to 512 (via a SCSI Mode Select).

4.) There is currently no low-level formatting support in

OS/2 2.0.

Support of low-level formatting is being investigated.

5.) The GA version of (IBM2SCSI.ADD) does not currently support

the Lock IORB Command Code and will not work with the

LOCKDRV filter.

This is being addressed.