Bizcomp PC:IntelliModem

The Bizcomp PC:IntelliModem modem card can be used with Softerm by

selecting PC:IntelliModem as the modem dialer type.

Although the PC:IntelliModem allows connections to be made at 110,

300, and 1200 baud, the board always communicates at 1200 baud

with the software.  The Softerm Speed communications parameter in

terminal setup should be set to 1200 baud for proper operation

when using the modem manually.  Phone numbers dialed using the

Softerm phone book will force the serial interface speed to 1200

but will issue the proper command for dialing at 110, 300, or 1200

baud.

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.