Multi-Tech MT212AH Auto-Dial Modem

The Multi-Tech MT212AH auto-dial modem can be used with Softerm by

selecting Multi-Tech MT212AH as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch-3  Setting	Function

1	   User Option	Enable/Disable auto-answer

2	   Open		Normal DTR Operation

3	   Open		Normal DCD Operation

Switch-4  Setting	Function

1	   User Option	Word/Digit result codes

2	   User Option	Enable/Suppress command responses

3	   User Option	Enable/Disable command echo

4	   User Option	Enable/Disable command mode